# Load Profile Metering

## Introduction

This section defines the requirements for metering with regard to Load Profiling and IDR activities.  Metering data in this context encompasses monthly kWh consumption, kW demand and interval meter data.  The Transmission and/or Distribution Service Providers (TDSPs) are the only Entities authorized to provide settlement meter data to ERCOT in accordance with ERCOT Protocols Section 10, “Metering.”

Each ESI ID in ERCOT must be assigned to a Load Profile ID. Metering data is necessary to perform this assignment because the information used for assignment of the Load Profile ID are kWh and kW data. Only metered data provided to ERCOT (thus, available to market participants with legitimate interest in the ESI ID) shall be used to assign the Load Profile ID.  The primary uses of metering data are:

- To assign each ESI ID;

- To allow validation for Load Profile ID assignments;

- To ensure profile models are appropriately specified and;

- To allow for profile model development.

If an advance meter is installed on a customer’s premise and has the capability to function as an IDR or lower level metering, data shall be supplied to ERCOT in accordance with its intended purpose to meet the needs of ERCOT billing and settlement activities.

This section will address the following topics:

- IDR Requirement

- Demand Meter Changes

- Load Research Samples

- Supplemental Profiling

Details for metering activities can be found in ERCOT Protocols Section 10, “Metering.”

When metering becomes competitive, the scope and nature of metering as input to Load Profiling may change and will require review to ensure consistency and completeness.

## IDR Requirement

IDRs will be installed in accordance with ERCOT Protocol Section 18.6, “Mandatory Installation and Use of Interval Data Recorders.”

For all ESI IDs that do not currently have an IDR meter installed for settlement purposes, ERCOT will monitor each ESI-ID’s demand data to determine if 1000 kW threshold requirement for IDR installation is met. ERCOT shall publish a MIS report, IDR Requirement Report, monthly, to each CR.  The list shall consist of ESI IDs that do not currently have an IDR meter installed for settlement purposes, which meet the threshold requirement. The CR is responsible for placing an IDR installation request with the appropriate TDSP within thirty (30) days. The TDSP is responsible for installing the IDR within sixty (60) days. Any ESI ID listed in the IDR Requirement Report, which has not had an IDR meter installed within 120 days will be reported as “overdue”.  It is the responsibility of the Market Oversight Division of the PUCT to monitor market participant compliance with IDR installation requirements.

If the CR or TDSP disputes the IDR requirement for any ESI-ID, the CR or TDSP must provide disputed evidence to ERCOT in accordance with the dispute resolution process described in Protocols Section 9.5, Settlement and Billing Dispute Process.

Costs associated with mandatory installation of IDRs by TDSPs will be the responsibility of the TDSP and be in accordance with approved TDSP tariffs.

## Demand Meter Changes

Section 8.2.4. “Load Profile ID Changes Resulting from Meter Changes” presents the procedure for changing profile ID assignment. The following provides brief discussion regard the circumstances, which may involve a meter change.

There will be circumstances where customer’s demand requirements diminish to a minimum and they would be better represented on a Business No Demand Profile.  When this situation occurs, the CR or the TDSP shall request the ESI ID be reassigned from a Business Demand Profile to a Business No Demand Profile through the appropriate TDSP in accordance with the LPOG Section 8.  If the CR has not made this request, the CR can require the demand meter to stay in place for billing purposes.  TDSPs will enforce appropriate thresholds and tariffs requiring the removal of a demand meter.  It is at the discretion of the TDSP whether to physically remove the demand register/meter or do a virtual meter change in their system.  A virtual meter change means that no demands will be reported to ERCOT.

Conversely, the customer’s load growth may warrant a demand register.  ERCOT shall make this determination and request the TDSP to install a demand register/meter. TDSPs will enforce appropriate thresholds and tariffs requiring the installation of a demand meter.  If the demand is such that it exceeds the 1000 kW threshold, then refer to Section 2.

Once it has been determined that a demand meter change is warranted, the TDSP will make appropriate changes in accordance with ERCOT Protocols Section 18.4, “Assignment of Load Profile Types and Weather Zones.”  The TDSP will notify the CR of the completed changes as well.

CRs may request the installation of a demand meter for their customers, regardless of TDSP thresholds, when required for application of the CR billing.  The CR is responsible for any costs associated with the demand meter installation and monthly meter reading in accordance with the approved TDSP tariffs.

## Load Research Samples

Any IDRs installed as part of the Load Research Program, i.e. in support of ERCOT load profiling or TDSP cost allocation/rate design, are not subject to the IDR requirements stated in ERCOT Protocols Section 18.6.1, “Interval Data Recorder Installation and Use in Settlement.” These Load Research IDRs may be moved as needed.

ERCOT has the responsibility to monitor and evaluate current Load Research Samples in the field.  ERCOT may request additions, deletions, or a wholesale removal and installation of the monitoring recorders.  The process will follow the Load Profiling Operating Guides Section 14, “Load Research Samples.”

## Supplemental

CRs may want Supplemental Load Profiling (i.e. DLC, TOU, etc.) to improve Load Profile definitions.   The CR shall follow procedures in Section 15 for Supplemental Load Profiling.  Metering for Supplemental Load Profiling shall be in accordance with ERCOT Protocol Section 10, “Metering” and ERCOT Protocol Section 18, “Load Profiling”.

All IDR installations for Supplemental Load Profiling shall be consistent with IDR metering requirements in Protocols Section 10.9.2 “TDSP Metering Entities”.  Additionally, any TOU metering for Supplemental Load Profiling must be able to collect and record meter data into specified time-of-use periods approved by the Public Utility Commission of Texas (PUCT).