Attachment A:	 ERCOT Data Sets

The following table describes the data sets that ERCOT requires to operate the ERCOT energy network.

| ACTION |  | DATA QUANTITY |  |  | PURPOSE |  |  | SUPPLIED BY |  |  | RECEIVED BY |  | FREQUENCY |  |  | MODE OF COMM. |  | FORMAT |  |  | PROTOCOLS REFERENCE / COMMENTS |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DAY-AHEAD SCHEDULING PROCESS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Extend Day-Ahead Scheduling Process |  | Notification to extend Day-Ahead Scheduling Process to Two-Days-Ahead |  |  | Emergency Notification |  |  | ERCOT |  |  | QSE |  | Demand |  |  | API |  | XML |  |  | Protocols Section 4.4.19, Decision to Extend Day-Ahead Scheduling Process to Two-Day-Ahead Scheduling Process |  |  |  |
| Day Ahead Notification |  | Transmission Outages |  |  | Day-Ahead Planning |  |  | ERCOT |  |  | QSE |  | Daily before 0600 |  |  | API |  | XML |  |  | Protocols Section 4.4.2, Advance Publication of Forecasted System Conditions |  |  |  |
|  |  | Total Transmission Capacity |  |  | Day-Ahead Planning |  |  | ERCOT |  |  | QSE |  | Daily before 0600 |  |  | API |  | XML |  |  | Protocols Section 4.4.2, Advance Publication of Forecasted System Conditions |  |  |  |
|  |  | Weather assumptions used by ERCOT to forecast system conditions |  |  | Day-Ahead Planning |  |  | ERCOT |  |  | QSE |  | Daily before 0600 |  |  | API |  | XML |  |  | Protocols Section 4.4.2, Advance Publication of Forecasted System Conditions |  |  |  |
| Day-Ahead Notification |  | Ancillary Services (AS) Plan |  |  | Day-Ahead Planning |  |  | ERCOT |  |  | QSE |  | Daily before 0600 |  |  | API |  | XML |  |  | Protocols Section 4.4.3, Notification to the Market of ERCOT's Day-Ahead Ancillary Services Plan |  |  |  |
|  |  | Ancillary Service Obligations |  |  | Day-Ahead Planning |  |  | ERCOT |  |  | QSE |  | Daily by 0600 |  |  | API |  | XML |  |  | Protocols Section 4.4.4, Notification to QSEs of Ancillary Service Obligations |  |  |  |
|  |  | Mandatory Balancing Energy Service Down Bid Percentage requirements |  |  | Day-Ahead Planning |  |  | ERCOT |  |  | QSE |  | Daily by 0600 |  |  | API |  | XML |  |  | Protocols Section 4.4.5, Notification to QSEs of Mandatory Balancing Energy Service down Bid Percentage Requirements |  |  |  |
|  |  | Transmission and Distribution Loss Estimates |  |  | Day-Ahead Planning |  |  | ERCOT |  |  | QSE |  | Daily by 0600 |  |  | API |  | XML |  |  | Protocols Section 4.4.6, Notification to QSEs of Transmission Loss Estimates |  |  |  |
|  |  | Update of Forecasted System Conditions (total forecast system load and load forecasts in each congestion Zone) |  |  | Day-Ahead Planning |  |  | ERCOT |  |  | QSE |  | Daily @ 0600 |  |  | API |  | XML |  |  | Protocols Section 4.4.7, Update of Forecasted System |  |  |  |
| QSEs Submit Balanced Schedules of Obligations and Supply |  | Balanced Schedules |  |  | CSC Congestion and Capacity Insufficiency |  |  | QSE |  |  | ERCOT |  | Daily before 1100, @ 1300, then as needed |  |  | API |  | XML |  |  | Protocols Section 4.4.8 & 4.4.10, Submittal of Day-Ahead Schedules and Ancillary Services Schedules |  |  |  |
| Self-Arranged Ancillary Services |  | Self Provided Regulation Up |  |  | AS (Rolled up from REP to QSE) |  |  | QSE |  |  | ERCOT |  | Daily before 1100, @ 1300, then as needed |  |  | API |  | XML |  |  | Protocols Section 4.4.8 & 4.4.10, Submittal of Day-Ahead Schedules and Ancillary Services Schedules |  |  |  |
|  |  | Self Provided Regulation Down |  |  | AS (Rolled up from REP to QSE) |  |  | QSE |  |  | ERCOT |  | Daily before 1100, @ 1300, then as needed |  |  | API |  | XML |  |  | Protocols Section 4.4.8 & 4.4.10, Submittal of Day-Ahead Schedules and Ancillary Services Schedules |  |  |  |
|  |  | Self Provided Responsive Reserve |  |  | AS (By Portfolio) |  |  | QSE |  |  | ERCOT |  | Daily before 1100, @ 1300, then as needed |  |  | API |  | XML |  |  | Protocols Section 4.4.8 & 4.4.10, Submittal of Day-Ahead Schedules and Ancillary Services Schedules |  |  |  |
|  |  | Self Provided Non-Spinning Reserve |  |  | AS (By Portfolio, By Zone) |  |  | QSE |  |  | ERCOT |  | Daily before 1100, @ 1300, then as needed |  |  | API |  | XML |  |  | Protocols Section 4.4.8 & 4.4.10, Submittal of Day-Ahead Schedules and Ancillary Services Schedules |  |  |  |
| Validate Schedules and Bids |  | Notification of invalid or mismatched schedules |  |  | Validate Schedules |  |  | ERCOT |  |  | QSE |  | Daily @ 1100 & 1300 |  |  | API |  | XML |  |  | Protocols Section 4.4.12, ERCOT Validation of Qualified Scheduling Entity's Day-Ahead Schedules and Bids |  |  |  |
|  |  | Corrected Schedules |  |  | Submit corrected information |  |  | QSE |  |  | ERCOT |  | Daily @ 1115 & 1315 |  |  | API |  | XML |  |  | Protocols Section 4.4.12, ERCOT Validation of Qualified Scheduling Entity's Day-Ahead Schedules and Bids |  |  |  |
|  |  | Notification of CSC Congestion and/or Capacity Insufficiency |  |  | Update Schedules |  |  | ERCOT |  |  | QSE |  | Daily by 1115 |  |  | API |  | XML |  |  | Protocols Section 4.4.9, ERCOT Notification of CSC Congestion and/or Capacity Insufficiency based on 1100 |  |  |  |
| Procure Ancillary Services |  | AS Bids to ERCOT |  |  | Ancillary Services Market |  |  | QSE |  |  | ERCOT |  | Daily by 1300 |  |  | API |  | XML |  |  | Protocols Section 4.4.11, Ancillary Services Bid |  |  |  |
|  |  | Purchase Regulation Up |  |  | Complete Day-Ahead Ancillary Services Plan |  |  | ERCOT |  |  | QSE |  | Daily by 1330 |  |  | API |  | XML |  |  | Protocols Section 4.4.13, ERCOT Day-Ahead Ancillary Services Procurement Process |  |  |  |
|  |  | Purchase Regulation Down |  |  | Complete Day-Ahead Ancillary Services Plan |  |  | ERCOT |  |  | QSE |  | Daily by 1330 |  |  | API |  | XML |  |  | Protocols Section 4.4.13, ERCOT Day-Ahead Ancillary Services Procurement Process |  |  |  |
|  |  | Purchase Responsive Reserves |  |  | Complete Day-Ahead Ancillary Services Plan |  |  | ERCOT |  |  | QSE |  | Daily by 1330 |  |  | API |  | XML |  |  | Protocols Section 4.4.13, ERCOT Day-Ahead Ancillary Services Procurement Process |  |  |  |
|  |  | Purchase Non-Spinning Reserves |  |  | Complete Day-Ahead Ancillary Services Plan |  |  | ERCOT |  |  | QSE |  | Daily by 1330 |  |  | API |  | XML |  |  | Protocols Section 4.4.13, ERCOT Day-Ahead Ancillary Services Procurement Process |  |  |  |
|  |  | Ancillary Services Schedules |  |  | Notification of Selected Ancillary Services Bids |  |  | ERCOT |  |  | QSE |  | Daily by 1330 |  |  | API |  | XML |  |  | Protocols Section 4.4.14, ERCOT Day-Ahead Ancillary Services Procurement Process |  |  |  |
|  |  | Publish Day-Ahead MCPs for Capacity (MCPCs) for each Day-Ahead Ancillary Service |  |  | Complete ERCOT's Day-Ahead Ancillary Services Plan |  |  | ERCOT |  |  | QSE |  | Daily @ 1330 |  |  | API |  | XML |  |  | Protocols Section 4.4.13, ERCOT Day-Ahead Ancillary Services Procurement Process |  |  |  |
|  |  | Final Ancillary Services Schedules |  |  | Updated Day Ahead Ancillary Services Schedule |  |  | QSE |  |  | ERCOT |  | Daily @ 1500 |  |  | API |  | XML |  |  | Protocols Section 4.4.14, ERCOT Day-Ahead Ancillary Services Procurement Process |  |  |  |
| QSE Present Resource Plan |  | Unit Availability Status |  |  | Generation Planning, AS Verification |  |  | QSE |  |  | ERCOT |  | Daily @ 1600, then as needed |  |  | API |  | XML |  |  | Protocols Section 4.4.15, QSE Resource Plans |  |  |  |
|  |  | Unit Planned Net Hourly Generation MW |  |  | Generation Planning, AS Verification |  |  | QSE |  |  | ERCOT |  | Daily @ 1600, then as needed |  |  | API |  | XML |  |  | Protocols Section 4.4.15, QSE Resource Plans |  |  |  |
|  |  | Unit Planned High Net Hourly Operating Limit MW |  |  | Generation Planning, AS Verification |  |  | QSE |  |  | ERCOT |  | Daily @ 16000, then as needed |  |  | API |  | XML |  |  | Protocols Section 4.4.15, QSE Resource |  |  |  |
|  |  | Unit Planned Low Net Hourly Operating Limit MW |  |  | Generation Planning, AS Verification |  |  | QSE |  |  | ERCOT |  | Daily @ 1600, then as needed |  |  | API |  | XML |  |  | Protocols Section 4.4.15, QSE Resource Plans |  |  |  |
|  |  | Load acting as a Resource |  |  | Generation Planning, AS Verification |  |  | QSE |  |  | ERCOT |  | Daily @ 1600, then as needed |  |  | API |  | XML |  |  | Protocols Section 4.4.15, QSE Resource Plans |  |  |  |
|  |  | Notification to QSE for insufficient resources |  |  | Generation Planning, AS Verification |  |  | ERCOT |  |  | QSE |  | Daily by 1600 |  |  | API |  | XML |  |  | Protocols Section 4.1.1, QSE Resource Plans |  |  |  |
| Procure Replacement Reserve Service |  | Unit-Specific Replacement Reserve Service (RPRS) Bids |  |  | QSEs may submit Replacement Reserve Service (RPRS) Bids |  |  | QSE |  |  | ERCOT |  | Daily by 1600, then as needed |  |  | API |  | XML |  |  | Protocols Section 4.4.16, ERCOT Receipt of Replacement Reserve Service Bids |  |  |  |
|  |  | Resource Specific Premiums |  |  | QSEs may submit hourly Resource specific premiums |  |  | QSE |  |  | ERCOT |  | Daily by 1430, then as needed |  |  | API |  | XML |  |  | Protocols Section 4.5.3, ERCOT Receipt of Resource Specific Premiums for Operational Congestion Management |  |  |  |
|  |  | Procurement of Replacement Reserve |  |  | ERCOT purchase RPRS |  |  | ERCOT |  |  | QSE |  | Daily by 1800 |  |  | API |  | XML |  |  | Protocols Section 4.4.17, ERCOT Procurement of Replacement Reserve Service As Needed |  |  |  |
|  |  | Adjustments to schedules of Reliability Must-Run (RMR) resources |  |  | Adjustments to the QSE's schedules |  |  | ERCOT |  |  | QSE |  | Demand |  |  | API |  | XML |  |  | Protocols Section 4.5.11 , ERCOT Deployment of Reliability Must-Run Generators |  |  |  |
| ADJUSTMENT PERIOD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | QSE's can make schedule adjustments and ERCOT can request additional bids. |  |  |  |  |  |  |  | Time T is the start of the Operating Period |  |  |  |  |  |  |  | The Adjustment Period (AP) begins following the close of the Day-Ahead market and continues until one (1) hour prior to the beginning of the Operating Hour. |  |  |  |
| Adjustment Period Schedule Changes |  | Submit or change Energy Schedule and Ancillary Services Schedules |  |  | Adjustment Period Scheduling Process |  |  | QSE |  |  | ERCOT |  | T minus 60 minutes |  |  | API |  | XML |  |  | Protocols Section 4.5.1, Receipt of Adjustment Period Schedule Changes |  |  |  |
|  |  | Submit, change, or remove Balancing Energy Service Up and Down Bids |  |  | Adjustment Period Scheduling Process |  |  | QSE |  |  | ERCOT |  | T minus 60 minutes |  |  | API |  | XML |  |  | Protocols Section 4.5.1, Receipt of Adjustment Period Schedule Changes |  |  |  |
|  |  | Submit, change, or remove RPRS Bids |  |  | Adjustment Period Scheduling Process |  |  | QSE |  |  | ERCOT |  | T minus 60 minutes |  |  | API |  | XML |  |  | Protocols Section 4.5.1, Receipt of Adjustment Period Schedule Changes |  |  |  |
|  |  | Require mandatory Balancing Energy Service Down Bid Curves |  |  | Adjustment Period Scheduling Process |  |  | ERCOT |  |  | QSE |  | T minus 60 minutes |  |  | API |  | XML |  |  | Protocols Section 4.5.2, Receipt of QSE's Balancing Energy Bid Curves |  |  |  |
|  |  | Submit Balancing Energy Service Up and Down Bid Curves |  |  | Adjustment Period Scheduling Process |  |  | QSE |  |  | ERCOT |  | T minus 60 minutes |  |  | API |  | XML |  |  | Protocols Section 4.5.2, Receipt of QSE's Balancing Energy Bid Curves |  |  |  |
|  |  | Announce need to procure RPRS |  |  | Procure Submitted Bids |  |  | ERCOT |  |  | QSE |  | T minus 60 minutes |  |  | API |  | XML |  |  | Protocols Section 4.5.6, ERCOT Notice of Need to Procure Replacement Reserve Service Resources |  |  |  |
|  |  | Announce need to procure additional Ancillary Services |  |  | Procure Submitted Bids |  |  | ERCOT |  |  | QSE |  | T minus 60 minutes |  |  | API |  | XML |  |  | Protocols Section 4.5.8, ERCOT Notice of Need to Procure Additional Ancillary Services |  |  |  |
|  |  | QSE Update their Resource Plan |  |  | Update prior to close of the Adjustment Period |  |  | QSE |  |  | ERCOT |  | T minus 60 minutes |  |  | API |  | XML |  |  | Protocols Section 4.5.10, Updated Resource Plans |  |  |  |
|  |  | QSE Update their Balanced Schedules |  |  | Adjustment Period Scheduling Process |  |  | QSE |  |  | ERCOT |  | T |  |  | API |  | XML |  |  | Protocols Section 4.5.6 & 4.5.8, Updated Balanced Schedules |  |  |  |
|  |  | QSE Resubmit Corrected Balanced Schedule |  |  | Adjustment Period Scheduling Process |  |  | QSE |  |  | ERCOT |  | T plus 15 minutes |  |  | API |  | XML |  |  | Protocols Section 4.5.6 & 4.5.8, Corrected Balanced Schedule |  |  |  |
|  |  | Procure RPRS |  |  | ERCOT purchase RPRS |  |  | ERCOT |  |  | QSE |  | T plus 30 minutes |  |  | API |  | XML |  |  | Protocols Section 4.5.6, |  |  |  |
|  |  | Procure additional Ancillary Services |  |  | ERCOT purchase additional ancillary service |  |  | ERCOT |  |  | QSE |  | T plus 30 minutes |  |  | API |  | XML |  |  | Protocols Section 4.5.8, |  |  |  |
| OPERATING PERIOD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | DNP 3.0 protocol will be used for new ports to RTUs or SCADA systems. |  |  |  |
| FCS Inputs |  | Frequency |  |  | Real-Time Monitoring and Control |  |  | ERCOT/QSE |  |  | ERCOT |  | 2 sec |  |  | RTU |  | DNP 3.0 |  |  | Protocols Section 6.10.5, QSE Real Power Performance Criteria |  |  |  |
| Unit Monitoring |  | Generating Unit MW |  |  | Real-Time Monitoring & Security Analysis |  |  | QSE |  |  | ERCOT |  | 2 sec |  |  | RTU |  | DNP 3.0 |  |  | Protocols Section 6.5.1.1, Requirements for Operating Period Data for System Reliability and Ancillary Service Provision Net Generation is preferred. If gross MW is supplied, aux load should also be provided. |  |  |  |
|  |  | Generating Unit Mvar |  |  | Real-Time Monitoring & Security Analysis |  |  | QSE |  |  | ERCOT |  | 10 sec |  |  | RTU /ICCP |  | DNP 3.0 |  |  | Protocols Section 6.5.1.1, Requirements for Operating Period Data for System Reliability and Ancillary Service Provision Net Generation is preferred. If gross MVar is supplied, aux load should also be provided. |  |  |  |
|  |  | Private Network Net Interchange to ERCOT |  |  | Real-Time Monitoring & Security Analysis |  |  | QSE |  |  | ERCOT |  | 2 Sec |  |  | RTU |  | DNP 3.0 |  |  | For private networks, the net interchange shall be provided along with gross MW and MVar per generating unit. |  |  |  |
|  |  | Private Network Net Interchange High Limit |  |  | Real-Time Monitoring & Security Analysis |  |  | QSE |  |  | ERCOT |  | 10 sec |  |  | RTU/ ICCP |  | DNP 3.0 |  |  | For private networks, the high operating limit will reflect the maximum generation available at the point of interconnection to ERCOT. |  |  |  |
|  |  | Private Network Net Interchange Low Limit |  |  | Real-Time Monitoring & Security Analysis |  |  | QSE |  |  | ERCOT |  | 10 sec |  |  | RTU/ ICCP |  | DNP 3.0 |  |  | For private networks, the low operating limit will reflect the minimum generation available at the point of interconnection to ERCOT. |  |  |  |
|  |  | Status of switching devices in the plant switchyard affecting flows on the ERCOT system |  |  | Real-Time Monitoring & Security Analysis |  |  | QSE |  |  | ERCOT |  | 10 sec |  |  | RTU/ ICCP |  | DNP 3.0 |  |  | Protocols Section 6.5.1.1, Requirements for Operating Period Data for System Reliability and Ancillary Service Provision |  |  |  |
|  |  | Generating Unit Breaker Status |  |  | Real-Time Monitoring & Security Analysis |  |  | QSE |  |  | ERCOT |  | 10 sec |  |  | RTU/ ICCP |  | DNP 3.0 |  |  | Protocols Section 6.5.1.1, Requirements for Operating Period Data for System Reliability and Ancillary Service Provision |  |  |  |
|  |  | Generating Unit High Operating Limit |  |  | Real-Time Monitoring & Security Analysis |  |  | QSE |  |  | ERCOT |  | 10 sec |  |  | RTU/ ICCP |  | DNP 3.0 |  |  | Protocols Section 6.5.1.1, Requirements for Operating Period Data for System Reliability and Ancillary Service Provision |  |  |  |
|  |  | Generating Unit Low Operating Limit |  |  | Real-Time Monitoring & Security Analysis |  |  | QSE |  |  | ERCOT |  | 10 sec |  |  | RTU/ ICCP |  | DNP 3.0 |  |  | Protocols Section 6.5.1.1, Requirements for Operating Period Data for System Reliability and Ancillary Service Provision |  |  |  |
| NOTE: Operating Period data from QSE units can be made available to the PGCs host TDSP on request. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Load used as a Resource MW |  |  | Real-Time Monitoring |  |  | QSE |  |  | ERCOT |  | 2 sec |  |  | RTU |  | DNP 3.0 |  |  | Protocols Section 6.5.1.1, Requirements for Operating Period Data for System Reliability and Ancillary Service Provision |  |  |  |
|  |  | Load Resource Breaker Status |  |  | Real-Time Monitoring |  |  | QSE |  |  | ERCOT |  | 10 sec |  |  | RTU/ ICCP |  | DNP 3.0 |  |  | Protocols Section 6.5.1.1, Requirements for Operating Period Data for System Reliability and Ancillary Service Provision |  |  |  |
|  |  | Load Resource (providing Responsive Reserve) High-Set Underfrequency Relay Status |  |  | Real-Time Monitoring |  |  | QSE |  |  | ERCOT |  | 10 sec |  |  | RTU/ ICCP |  | DNP 3.0 |  |  | Protocols Section 6.5.1.1, Requirements for Operating Period Data for System Reliability and Ancillary Service Provision |  |  |  |
|  |  | QSE Frequency Bias |  |  | Real-Time Monitoring |  |  | QSE |  |  | ERCOT |  | 2 sec |  |  | RTU |  | DNP 3.0 |  |  | Protocols Section 6.5.1.1 Frequency Bias of Portfolio Generation under QSE Operation |  |  |  |
|  |  | Dynamic Resource Power Schedules |  |  | Real-Time Monitoring |  |  | QSE |  |  | ERCOT |  | 2 sec |  |  | RTU |  | DNP 3.0 |  |  | Protocols Sections 4.9 Implementation Principles for Dynamic Load Schedules and 6.10.4.3 - Schedule Control Error |  |  |  |
|  |  | SCE |  |  | Real-Time Monitoring and Control (control only for regulation providers) |  |  | QSE |  |  | ERCOT |  | 2 sec |  |  | RTU |  | DNP 3.0 |  |  | Feedback for QSE regulation calculation. Protocols Section 6.10.5, QSE Real Power Performance Criteria |  |  |  |
|  |  | Regulation Feedback (loop-back) |  |  | Real-Time Monitoring |  |  | QSE |  |  | ERCOT |  | 2 sec |  |  | RTU |  | DNP 3.0 |  |  | Protocols Section 6.5.1.1 Requirement for Operating Period Data for System Reliability and Ancillary Service Provision & 6.7.2(8), Regulation Service - Feedback the amount of RGS power (MW) being provided each control cycle |  |  |  |
|  |  | Responsive Reserve Availability MW |  |  | Real-Time Monitoring |  |  | QSE |  |  | ERCOT |  | 10 sec |  |  | RTU/ ICCP |  | DNP 3.0 |  |  | Feedback to Responsive Reserve Deployment. Protocols Section 6.7.3 (6), Deployment of Responsive Reserve Service |  |  |  |
|  |  | Responsive Reserve Deployment MW |  |  | Real-Time Monitoring |  |  | QSE |  |  | ERCOT |  | 10 sec |  |  | RTU/ ICCP |  | DNP 3.0 |  |  | Feedback to Responsive Reserve Deployment. Protocols Section 6.7.3 (6), Deployment of Responsive Reserve Service |  |  |  |
|  |  | Non-Spinning Reserve Availability MW |  |  | Real-Time Monitoring |  |  | QSE |  |  | ERCOT |  | 10 sec |  |  | RTU/ ICCP |  | DNP 3.0 |  |  | Feedback to Non-Spinning Reserve Deployment. Protocols Section 6.7.4 (7), Deployment of Non-Spinning Reserve Service |  |  |  |
|  |  | Non-Spinning Reserve Deployment MW |  |  | Real-Time Monitoring |  |  | QSE |  |  | ERCOT |  | 10 sec |  |  | RTU/ ICCP |  | DNP 3.0 |  |  | Feedback to Non-Spinning Reserve Deployment. Protocols Section 6.7.4 (7), Deployment of Non-Spinning Reserve Service |  |  |  |
| Deploy Ancillary Services |  | Instructed Regulation Up / Down MW |  |  | FCS Instruction |  |  | ERCOT |  |  | QSE |  | 4 sec |  |  | RTU |  | DNP 3.0 |  |  | Protocols Section 6.5.1.1, Requirement for Operating Period Data for System Reliability and Ancillary Service Provision & 6.7.2,(1) Regulation Service |  |  |  |
|  |  | Instructed Responsive Reserve MW |  |  | FCS Instruction |  |  | ERCOT |  |  | QSE |  | 4 sec |  |  | RTU |  | DNP 3.0 |  |  | Protocols Section 6.5.1.1, Requirement for Operating Period Data for System Reliability and Ancillary Service Provision & 6.7.3 (1), Responsive Reserve Service |  |  |  |
|  |  | SCE |  |  | Performance Monitoring |  |  | ERCOT |  |  | QSE |  | 10 sec |  |  | RTU |  | DNP 3.0 |  |  | SCE Comparison for real-time monitoring and analysis by the ERCOT and QSE. Protocols Section 6.10.4.4, Schedule Control Error |  |  |  |
|  |  | Governor Response |  |  | Monitoring |  |  | ERCOT |  |  | QSE |  | 10 sec |  |  | RTU |  | DNP 3.0 |  |  | ERCOT calculated governor response from QSE frequency bias. Protocols Section 6.10.4.4, Schedule Control Error |  |  |  |
|  |  | Generating Unit Governor Response |  |  | Real-Time Monitoring |  |  | ERCOT |  |  | QSE |  | 10 sec. |  |  | RTU |  | DNP 3.0 |  |  | Protocols Section 6.5.1.1 Requirements for Operating Period Data for System Reliability and Ancillary Service Provision. |  |  |  |
| FCS Control Instructions |  | Instructed Non-Spinning Reserve MW |  |  | Instruction |  |  | ERCOT |  |  | QSE |  | Every 15 minutes |  |  | API |  | XML |  |  | Protocols Section 6.7.4(6), Non-Spinning Reserve Service |  |  |  |
|  |  | Instructed Balancing Energy MW (Up / Down) |  |  | Instruction |  |  | ERCOT |  |  | QSE |  | Every 15 minutes with 10 minutes notice |  |  | API |  | XML |  |  | Protocols Section 6.5.2, & 6.7.1.1(8) Balancing Energy Service |  |  |  |
|  |  | Unit High Side Bus Voltage kV |  |  | Real-Time Monitoring |  |  | QSE |  |  | ERCOT |  | 10 sec |  |  | RTU/ ICCP |  | DNP 3.0 |  |  | Protocols Section 6.5.7, Voltage Support Service, May be supplied by the TDSP. |  |  |  |
|  |  | Bus Voltage at Private Network connection to ERCOT |  |  | Real-Time Monitoring |  |  | QSE |  |  | ERCOT |  | 10 sec |  |  | RTU |  | DNP 3.0 |  |  | This voltage may be supplied by the TDSP. |  |  |  |
| Reliability Instructions |  | Instructed Replacement Reserve |  |  | Instruction |  |  | ERCOT |  |  | QSE |  | As needed |  |  | API |  | XML |  |  | Protocols Section 6.5.6, Replacement Reserve Service |  |  |  |
|  |  | Instructed Voltage Support |  |  | Instruction |  |  | ERCOT |  |  | QSE |  | As needed |  |  | API |  | XML |  |  | Protocols Section 6.5.7, Voltage Support Service |  |  |  |
|  |  | Instructed Black Start |  |  | Instruction |  |  | ERCOT |  |  | QSE |  | As needed |  |  | API |  | XML |  |  | Protocols Section 6.5.8, Black Start Service |  |  |  |
|  |  | Instructed Reliability Must-Run |  |  | Instruction |  |  | ERCOT |  |  | QSE |  | As needed |  |  | API |  | XML |  |  | Protocols Section 6.5.9, Reliability Must-Run Service |  |  |  |
|  |  | Instructed Out-of-Merit Capacity |  |  | Instruction |  |  | ERCOT |  |  | QSE |  | As needed |  |  | API |  | XML |  |  | Protocols Section 6.5.10, Out-of-Merit Capacity and Out-of-Merit Energy Services |  |  |  |
|  |  | Instructed Out-of-Merit Energy |  |  | Instruction |  |  | ERCOT |  |  | QSE |  | As needed |  |  | API |  | XML |  |  | Protocols Section 6.5.10, Out-of-Merit Capacity and Out-of-Merit Energy Services |  |  |  |
| Security Analysis This section describes the data sets requested for security analysis. The TDSP shall supply all data available at its control center to ERCOT in the data sets described below. Data not currently available at the TDSP’s control center that ERCOT has determined is necessary to provide accurate security analysis results, will be made available after ERCOT and the TDSP agrees that the request can be achieved in a reasonable timeframe and cost. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Generation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Generating data is provided by QSEs as defined above under Operating Period Data |  |  |  |
| Bus |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Bus Voltage kV |  |  | Real-Time Monitoring |  |  | TDSP |  |  | ERCOT |  | 10 sec |  |  | ICCP |  | Blocks 1&2 |  |  |  |  |  |  |
|  |  | Bus Frequency Hz |  |  | Real-Time Monitoring |  |  | TDSP |  |  | ERCOT |  | 10 sec |  |  | ICCP |  | Blocks 1&2 |  |  |  |  |  |  |
| Transformer |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Transformer Flow MW |  |  | Real-Time Monitoring |  |  | TDSP |  |  | ERCOT |  | 10 sec |  |  | ICCP |  | Blocks 1&2 |  |  | Positive flow is into the transformer |  |  |  |
|  |  | Transformer Flow Mvar |  |  | Real-Time Monitoring |  |  | TDSP |  |  | ERCOT |  | 10 sec |  |  | ICCP |  | Blocks 1&2 |  |  | Positive flow is into the transformer |  |  |  |
|  |  | LTC Tap Position |  |  | Real-Time Monitoring |  |  | TDSP |  |  | ERCOT |  | 10 sec |  |  | ICCP |  | Blocks 1&2 |  |  |  |  |  |  |
|  |  | Transformer Status |  |  | Real-Time Monitoring |  |  | TDSP |  |  | ERCOT |  | 10 sec |  |  | ICCP |  | Blocks 1&2 |  |  | Used if the computer(s) have a Network Topology Processor |  |  |  |
| Line |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Circuit Breaker data may be used instead of line data. |  |  |  |
|  |  | Line Flow MW |  |  | Real-Time Monitoring |  |  | TDSP |  |  | ERCOT |  | 10 sec |  |  | ICCP |  | Blocks 1&2 |  |  | Positive flow is out of the station |  |  |  |
|  |  | Line Flow Mvar |  |  | Real-Time Monitoring |  |  | TDSP |  |  | ERCOT |  | 10 sec |  |  | ICCP |  | Blocks 1&2 |  |  | Positive flow is out of the station |  |  |  |
|  |  | Circuit Status |  |  | Real-Time Monitoring |  |  | TDSP |  |  | ERCOT |  | 10 sec |  |  | ICCP |  | Blocks 1&2 |  |  | Used if the computer(s) have a Network Topology Processor |  |  |  |
| Shunt |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Reactive Support from Shunt Mvar |  |  | Real-Time Monitoring |  |  | TDSP |  |  | ERCOT |  | 10 sec |  |  | ICCP |  | Blocks 1&2 |  |  | Positive for supplying VAR Negative for absorbing VAR |  |  |  |
|  |  | Bank Status |  |  | Real-Time Monitoring |  |  | TDSP |  |  | ERCOT |  | 10 sec |  |  | ICCP |  | Blocks 1&2 |  |  | Used if the computer(s) have a Network Topology Processor |  |  |  |
| Circuit Breaker |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Includes: Circuit Breaker (CB), Line Switch (LS), and Disconnect Switch (DCS) (where applicable) Line data may be used instead of circuit breaker data. |  |  |  |
|  |  | CB MW flow |  |  | Real-Time Monitoring |  |  | TDSP |  |  | ERCOT |  | 10 sec |  |  | ICCP |  | Blocks 1&2 |  |  | Positive flow is from bus section to equipment (optional) |  |  |  |
|  |  | CB Mvar |  |  | Real-Time Monitoring |  |  | TDSP |  |  | ERCOT |  | 10 sec |  |  | ICCP |  | Blocks 1&2 |  |  | Positive flow is from bus section to equipment (optional) |  |  |  |
|  |  | Switch Position |  |  | Real-Time Monitoring |  |  | TDSP |  |  | ERCOT |  | 10 sec |  |  | ICCP |  | Blocks 1&2 |  |  |  |  |  |  |
| Load |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Load metering is preferred from the high side of the transformation point of the substation distribution transformer. |  |  |  |
|  |  | Load in MW |  |  | Real-Time Monitoring |  |  | TDSP |  |  | ERCOT |  | 10 sec |  |  | ICCP |  | Blocks 1&2 |  |  | Negative for Load MW |  |  |  |
|  |  | Load in Mvar |  |  | Real-Time Monitoring |  |  | TDSP |  |  | ERCOT |  | 10 sec |  |  | ICCP |  | Blocks 1&2 |  |  | Negative for Load Mvar |  |  |  |
| DC Injection |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | DC Injection in MW |  |  | Real-Time Monitoring |  |  | TDSP |  |  | ERCOT |  | 10 sec |  |  | ICCP |  | Blocks 1&2 |  |  | Positive flow is out of the station |  |  |  |
|  |  | DC Injection in Mvar |  |  | Real-Time Monitoring |  |  | TDSP |  |  | ERCOT |  | 10 sec |  |  | ICCP |  | Blocks 1&2 |  |  | Positive flow is out of the station |  |  |  |
|  |  | DC Tie Status |  |  | Real-Time Monitoring |  |  | TDSP |  |  | ERCOT |  | 10 sec |  |  | ICCP |  | Blocks 1&2 |  |  | Used if the Computer(s) have a Network Topology Processor |  |  |  |
| Weather Zone Tie Lines |  | Line Flow MW |  |  | Real-Time Monitoring |  |  | TDSP |  |  | ERCOT |  | 10 sec |  |  | ICCP DNP 3.0 |  | Blocks 1&2 |  |  | TDSP’s responsible for supplying weather zone tie line data must provide a redudant path from their normal ICCP link. This second path may be a second ICCP association to ERCOT or an RTU or other device using DNP 3.0 protocol. |  |  |  |
| NOTE: Real-Time data for Security Analysis can be provided to TDSPs on request. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| LEGEND: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Supplied by/Received by |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ERCOT - ERCOT Independent System Operator |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| QSE - Qualified Scheduling Entity |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TDSP - Transmission, Distribution Service Provider |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PGC - Power Generating Company |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Mode of Communication |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Web Portal - Online application available to market participants via the World Wide Web |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TCH - Transaction Clearinghouse. The mechanism used to send/receive EDI transactions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| API - Programmatic API. A message protocol used to automate data exchange between ERCOT and market participants |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ICCP - Inter-Control Center Communication Protocol. A standard protocol used to automate data exchange real-time control information between the ERCOT and participants |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RTU - Remote Terminal Unit. Used to gather real-time control/monitoring information. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Format |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EDI - Electronic Data Exchange format. EDI formats are specified by the Texas SET working group and are based on standard EDI transaction protocols. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| XML - eXtensible Markup Language. XML is a self-describing meta language used to facilitate the exchange of data. XML will format requests and receive replies using the API. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CSV - Comma Separated Values. A generic file format where elements of data are separated by commas within a file. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Office Document - One of the common Microsoft Office file formats such as MS Word or MS Excel. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |