| PRR Number | 253PRR | PRR Title | EPS Meter Clock Time Maintenance |
| --- | --- | --- | --- |
| ###### Status |  | Approved by the Board on 10/16/2001. |  |
| Protocol Section Requiring Revision |  | 10.9.1 (7) and 10.9.2 (5) |  |
| Requested Resolution |  | ## Wording Change |  |
| Revision Description |  | The language as it currently exists in Protocols requires all TDSP and EPS Meter clocks to be maintained on Central Prevailing Time. The proposed language allows TDSP and EPS Meters to stay on another time reference with the meter data adjusted to Central Prevailing Time. |  |
| ###### Reason for Revision |  | The way the protocols currently read, market participants are prohibited from using several vendors meters that were originally recommended by ERCOT as acceptable for EPS meters. The change will allow the use of those meters. The problem is due to the way the meters actually handle daylight savings time vs. how the protocols currently state that the meters should handle daylight savings time. |  |
| ## Timeline |  |  |  |
| Date Received |  | 7/5/01 |  |
| Date Posted |  | 7/6/01 |  |
| Comments Due |  | 7/27/01 |  |
| PRS Review Date |  | 8/03/2001 |  |
| PRS Recommendation |  | Approved |  |
| TAC Consideration |  | 10/04/2001 – Approved. |  |
| Board Consideration |  | 10/16/2001 – Approved. |  |

Proposed Protocol Language Revision

The Meter Working Group reviewed protocol sections 10.9.1 (7) and 10.9.2 (5) and reached consensus that a change is needed.  The recommended change is detailed below.

- 10.9.1 ERCOT Polled Settlement Meters

Language as it currently exists in Protocols that requires all EPS Meter clocks to be maintained on Central Prevailing Time.

(7) Meter clocks shall be maintained on the Central Prevailing Time, adjusted for standard or daylight Savings Time, and must be synchronized to within +/- 1% of the Settlement Interval when compared with the National Institute of Standards and Technology (NIST) Atomic Clock.  ERCOT will perform the time synchronization for meters at the time of the interrogation if the meter is outside tolerance; and

Proposed language that allows EPS Meters to stay on another time reference with the meter data adjusted to Central Prevailing Time.

(7) Meter clocks shall be maintained on a time reference standard that enables ERCOT MDAS to maintain the IDR data on the Central Prevailing Time.  The meter clock shall be synchronized to within +/- 1% of the Settlement Interval when compared with the National Institute of Standards and Technology (NIST) Atomic Clock.  ERCOT will perform the time synchronization for meters at the time of the interrogation if the meter is outside tolerance; and

### 10.9.2 TDSP Metered Entities

Language as it currently exists in Protocols that requires all TDSP IDR meter clocks to be maintained on Central Prevailing Time.

(5) Meter clocks used within ERCOT shall be maintained on the Central Prevailing Time (adjusted for standard or daylight savings time) standard and must be synchronized to within at least +/- 5% of the settlement interval when compared to the National Institute of Standards and Technology (NIST) Atomic Clock;

Proposed language that allows TDSP IDR meter clocks to stay on another time reference with the meter data adjusted to Central Prevailing Time.

(5) Meter clocks used within ERCOT shall be maintained on a time reference that enables the TDSP to submit data on the Central Prevailing Time.  The meter clock shall be synchronized to within at least +/- 5% of the settlement interval when compared to the National Institute of Standards and Technology (NIST) Atomic Clock;

| ## Sponsor |  |
| --- | --- |
| Name | Darrell Klimitchek |
| E-mail Address | darrell@stec.org |
| Company | South Texas Electric Cooperative |
| Company Address | P.O. Box 119 FM 447 Nursery, TX 77976 |
| Phone Number | (361) 485-6275 |
| Fax Number | (361) 575-2786 |
| ## ERCOT Impact Analysis |  |
| Date |  |
| Financial Impact |  |
| Analysis |  |
| ## TAC Action |  |
| TAC Vote Date | 10/04/2001 |
| TAC Proposed Implementation Date | 11/01/2001 |
| TAC Vote Results | Approved |
| ## ERCOT Board Action |  |
| ERCOT Board Vote Date | 10/16/2001 |
| ERCOT Board Implementation Date | 11/01/2001 |
| ERCOT Board Results | Approved |
| ## Appeal |  |
| Appeal Date |  |
| Entity Initiating Appeal |  |
| Nature of Appeal |  |