| PRR Number | 253PRR | PRR Title | EPS Meter Clock Time Maintenance |
| --- | --- | --- | --- |
| ###### Date |  | 07/23/01 |  |
| ## Submitter’s Information |  |  |  |
| Name |  | Brenda Harris |  |
| E-mail Address |  | bharris@reliant.com |  |
| Company |  | Reliant Energy |  |
| Company Address |  | 1111 Louisiana Houston, Texas 77002 |  |
| Phone Number |  | 713-207-7345 |  |
| Fax Number |  | 713-207-9815 |  |

Comments

To ensure that there will not be an error when synchronizing to the Central Standard time we would like to add the following language:

(10.9.1)

(7) Meter clocks shall be maintained on a time reference standard that enables ERCOT MDAS to maintain the IDR data on the Central Prevailing Time.  The reporting from the TDSP’s time reference to Central Prevailing Time should be instantaneous so that no error is created due to the conversion or reporting itself. The meter clock shall be synchronized to within +/- 1% of the Settlement Interval when compared with the National Institute of Standards and Technology (NIST) Atomic Clock.  ERCOT will perform the time synchronization for meters at the time of the interrogation if the meter is outside tolerance; and

(10.9.2)

5) Meter clocks used within ERCOT shall be maintained on a time reference that enables the TDSP to submit data on the Central Prevailing Time. The reporting from the TDSP’s time reference to Central Prevailing Time should be instantaneous so that no error is created due to the conversion or reporting itself. The meter clock shall be synchronized to within at least +/- 5% of the settlement interval when compared to the National Institute of Standards and Technology (NIST) Atomic Clock;