| PRR Number | 279PRR | PRR Title | Responsibility Transfers for Capacity Auction Products |
| --- | --- | --- | --- |
| ###### Status |  | Posted |  |
| Protocol Section Requiring Revision |  | 4.9.5 |  |
| Requested Resolution |  | ## Urgent |  |
| Revision Description |  | The PUCT has required IOUs to sell at auction 15% of their capacity in 25 mw blocks. One of the products from these blocks is balancing energy through the use of a Responsibility Transfer. Over 200 possible RspT will place a burden on ERCOT as well as the seller. This is a simpler method for the buyer to bid in the balancing energy market but have the seller receive a message rather than a continuous signal. |  |
| ###### Reason for Revision |  | Simplify Capacity Auction product implementation |  |
| ## Timeline |  |  |  |
| Date Received |  | 8/28/2001 |  |
| Date Posted |  | 8/28/2001 |  |
| Comments Due |  |  |  |
| PRS Review Date |  |  |  |
| PRS Recommendation |  |  |  |
| TAC Consideration |  |  |  |
| Board Consideration |  |  |  |

Proposed Protocol Language Revision

### 4.9.5 Responsibility Transfers for PUCT Required Capacity Auctions (RspTCA)

RsPT may be used to shift responsibility for Supply from one QSE to another QSE within the same Congestion Zone for settlement purposes. The total energy scheduled from each Supply, added together, will not change. As the RspT Dynamic Schedule of one QSE changes there must be an equal and opposite change in the RspT schedule of the other QSE.

Only one of the QSEs will act as the controller of the Real Time Dynamic Schedule and will send a Real Time signal to ERCOT that represents that QSE’s current power commitment to the other QSE. The Dynamic Schedule of the other QSE will be adjusted correspondingly by ERCOT such that the total scheduled amount of Supply from the two QSEs equals the total energy scheduled by the RspT schedules prior to Real Time.

Each QSE must enter an Inter-QSE Trade to represent a RspT prior to the Operating Period. RspTs will meet the telemetry requirements of ERCOT and must be updated at a rate compatible with ERCOT frequency control system.

Capacity Auction products allow the entitlement holder (Buyer) to sell balancing energy as well as other ancillary services.  To facilitate the sale of balancing energy from capacity auction products, a simplified RspTCA will allow the Buyer’s QSE to enter the bid into the ERCOT system and for ERCOT to notify the entitlement Seller’s QSE of the deployment.

The two QSEs would submit a proposal of the RspTCA to ERCOT for approval identifying the quantity, time duration, and the CSC zone in which the product will be delivered. When approved, the Buyer’s QSE would be able to submit a separate bid curve flagging that it was a RspTCA bid for a certain QSE in a certain zone. If ERCOT deploys energy from this bid, the deployment signal would be sent to the entitlement seller’s QSE rather than the biding QSE. The deployment signal would include an identifier of the bidder’s QSE for accounting purposes. This RspTCA would not allow an Inter-QSE Trade since it is for purposes of balancing energy only. All other energy or ancillary service schedules or deployments for these capacity auction entitlements would be through normal processes in these protocols.

| ## Sponsor |  |
| --- | --- |
| Name | Jerry Ward |
| E-mail Address | Gward1@txu.com |
| Company | TXU |
| Company Address | 1601 Bryan Street Dallas, Texas 75201 |
| Phone Number | 214-875-9254 |
| Fax Number | 214-875-9066 |
| ## ERCOT Impact Analysis |  |
| Date |  |
| Financial Impact |  |
| Analysis |  |
| ## TAC Action |  |
| TAC Vote Date |  |
| TAC Proposed Implementation Date |  |
| TAC Vote Results |  |
| ## ERCOT Board Action |  |
| ERCOT Board Vote Date |  |
| ERCOT Board Implementation Date |  |
| ERCOT Board Results |  |
| ## Appeal |  |
| Appeal Date |  |
| Entity Initiating Appeal |  |
| Nature of Appeal |  |