| PRR Number | 289PRR | PRR Title | Block Load Transfer |
| --- | --- | --- | --- |
| ###### Status |  |  |  |
| Protocol Section Requiring Revision |  | 5.7 Block Load Transfers between ERCOT and Non-ERCOT Control Areas |  |
| Requested Resolution |  | ## Urgent |  |
| Revision Description |  | Simplify the way ERCOT protocols accommodate Block Load Transfers and make the treatment of ERCOT and Non-ERCOT loads similar. Removed boxed text (pip 209) as the language should be capable of being implemented immediately with no changes to the system design. The present/old language treats Non-ERCOT load as any other ERCOT Load and subjects it to all charges that would apply to any ERCOT load. However, ERCOT load being served by a Non-ERCOT entity is treated through an inadvertent or pay back account. Under the changed/new language, BLTs where Non-ERCOT load is moved into ERCOT will be assigned an ESI ID & be treated as any other load. BLTs where ERCOT load is moved into a Non-ERCOT Control Area will be treated as a “resource” in the ERCOT settlement system. Such resources would only be dispatched by ERCOT under emergency or contingency situations and such dispatch instructions will be treated as OOME instructions. |  |
| ###### Reason for Revision |  | Changes requested by ERCOT, CFE and AEP to simplify the implementation of block load transfers. |  |
| ## Timeline |  |  |  |
| Date Received |  | 11/05/2001 |  |
| Date Posted |  | 11/05/2001 |  |
| Comments Due |  | 11/26/2001 |  |
| PRS Review Date |  | 11/30/2001 |  |
| PRS Recommendation |  |  |  |
| TAC Consideration |  |  |  |
| Board Consideration |  |  |  |

Proposed Protocol Language Revision

## 5.7 	Block Load Transfers between ERCOT and Non-ERCOT Control Areas

Under Alert or EECP conditions it may be necessary for ERCOT to request the implementation Block Load Transfers (BLT) schemes which will transfer loads normally located in the ERCOT Control Area to a Non-ERCOT Control Area.  Similarly, when Non-ERCOT Control Areas experience certain transmission contingency or short supply conditions, it may be necessary for ERCOT to agree to the implementation of BLT schemes which will transfer loads normally located in a Non-ERCOT Control Area to the ERCOT Control Area.  BLTs will be restricted to the following conditions:

(1) 	BLTs shall only occur under a specific Dispatch Instruction from ERCOT.

(2) 	BLTs that are looped systems may be tied to the other power pool’s electrical system through multiple interconnection points at the same time.  Transfers of looped configurations will only be permitted if all interconnection points are netted under a single ESI ID.

(3) 	BLTs of load to the ERCOT Control Area  will be treated as Load in the ERCOT settlement system and will only be permitted if such transfers will not jeopardize the reliability of the ERCOT System.  Under an ERCOT Emergency Notice, BLTs that have been implemented may be curtailed or terminated by ERCOT in order to maintain the reliability of the ERCOT system.

(4) 	BLTs of load from the ERCOT Control Area will be treated as a Resource in the ERCOT settlement system and will only be instructed with the permission of the affected Non-ERCOT Control Area.  Under emergency conditions, BLTs that have been implemented may be curtailed or terminated by the Non-ERCOT Control Area in order to maintain the reliability of the Non-ERCOT system.

(5) 	Restoration of service to outage customers using BLTs will be accomplished, as quickly as possible if the transfers will not jeopardize the reliability of the ERCOT System and only under specific Dispatch Instruction from ERCOT.

- The necessary Market Participant agreements, metering, and ERCOT settlement systems are in place prior to the implementation of any BLT.

- Installation of any BLT metering point or any metering point for transfer of Load from another control area into ERCOT requires the approval of ERCOT.  BLT metering shall conform to the following requirements:

  - Revinue Quality

  - Four (4) channel bi-directional Kwh/Kvarh, 15min. Interval Data Recording (IDR).

  - Remote interigation.

5.7.1  BLT Data Model in ERCOT Systems

Each BLT point which will transfer loads normally located in the ERCOT control area to a Non-ERCOT control area will be established as a pseudo generation facility and designated as such in ERCOT systems. Each BLT point which will transfer loads normally located in a Non-ERCOT control area to the ERCOT control area will have an ESI ID associated with it and will be treated as load in the ERCOT systems.

5.7.2 Registration of BLT Points

BLTs that block ERCOT load into a Non-ERCOT control area will be treated as a Resource by ERCOT and assigned a Resource ID.  An ERCOT registered Resource Entity, with an ERCOT registered QSE affiliation, must complete the appropriate Asset Registration form.  This Asset registration form along with the Metering Design Documentation will be the basis for establishing the ERCOT Data Model.

BLTs that block Non-ERCOT load into the ERCOT control area will be treated as a load by ERCOT and assigned an ESID.  An ERCOT registered LSE, with an ERCOT registered QSE affiliation, must be assigned to the ESI ID associated to BLT Point.

5.7.3 Scheduling & Operation of BLTs

The QSE of the Resource associated with a BLT Point will include that Resource in its Resource Plan.  The QSE will not be required to provide the real time data to ERCOT normally provided for Resources.  The Resource Plan shall reflect the availability of the resource, but ERCOT shall confirm its availability with the Non-ERCOT control area prior to issuing any dispatch instructions to the QSE and the TDSP.  Any energy delivered under such a dispatch instruction shall be treated as an OOME instruction to the QSE.

The QSE of the LSE assigned to the ESI ID associated with a BLT Point will include that load in its Balanced Schedules.

5.7.4 Settlement of BLT Points

When ERCOT load is blocked into a Non-ERCOT control area, the energy delivered to ERCOT load through a BLT will be treated as generation into the Congestion Zone in the settlement system.  The generation and dispatch instruction will be attributed to the Resource registered for that BLT point.   ERCOT customers, customer’s REPs, and REP’s QSEs served by a Non-ERCOT control area through a BLT will not be impacted by the switch to a Non-ERCOT resource.  ERCOT will settle with all parties through the ERCOT settlement process.

When a Non-ERCOT control area load is blocked into ERCOT, the energy delivered to the Non-ERCOT control area through a BLT will be treated as load in the Congestion Zone in the settlement system. The QSE of the LSE assigned to the ESI ID associated with the BLT point load is responsible for any applicable fees and charges as described in Section 9 Settlement and Billing.

| ## Sponsor |  |
| --- | --- |
| Name | Richard Ross |
| E-mail Address | rross@aep.com |
| Company | AEP |
| Company Address | 2488 E 81st Street Tulsa, OK 74137 |
| Phone Number | 918-493-9544 |
| Fax Number | 866-947-1796 |
| ## ERCOT Impact Analysis |  |
| Date |  |
| Financial Impact |  |
| Analysis |  |
| ## TAC Action |  |
| TAC Vote Date |  |
| TAC Proposed Implementation Date |  |
| TAC Vote Results |  |
| ## ERCOT Board Action |  |
| ERCOT Board Vote Date |  |
| ERCOT Board Implementation Date |  |
| ERCOT Board Results |  |
| ## Appeal |  |
| Appeal Date |  |
| Entity Initiating Appeal |  |
| Nature of Appeal |  |