| PRR Number | 292PRR | PRR Title | Revise Resource Specific payment formula |
| --- | --- | --- | --- |
| ###### Status |  | Posted |  |
| Protocol Section Requiring Revision |  | 7.4.3.1 and 7.4.3.2 |  |
| Requested Resolution |  | ## Urgent |  |
| Revision Description |  | Revise the formulas to reflect the language. |  |
| ###### Reason for Revision |  | The current formulas are not correct. Proposed formula changes were presented to WMS on September17, 2001. |  |
| ## Timeline |  |  |  |
| Date Received |  | 10/10/2001 |  |
| Date Posted |  | 10/12/2001 |  |
| Comments Due |  | 10/30/2001 |  |
| PRS Review Date |  | 10/31/2001 |  |
| PRS Recommendation |  |  |  |
| TAC Consideration |  | 11/08/2001 |  |
| Board Consideration |  | 11/20/2001 |  |

Proposed Protocol Language Revision

[PIP 120: Local Balancing Premiums. The section following needs to be inserted once the system has an equation to settle local congestion separately from other Balancing Energy costs/payments]

7.4.3.1 Balancing Energy Up from a Specific Resource

In accordance with section 6.7.1.2, Deployment of Balancing Energy when Congestion Occurs, if a Market Solution exists, whenever an incremental specific Resource is deployed to solve Local Congestion, the QSE will be paid the incremental premium specified.

LPCRSUqiz 	= 	-1 * Σ((PMqiuz –MCPEiz )* Max(0,  Min(MRqiuz -OLquiz  ,IOLqiuz - OLqiuz)))

PMqiuz		=	Max (BPMqiuz, BPMqiuz + MCPEiz )

Where:

LPCRSUqiz:	Payment to QSE for providing Balancing Energy Up from Specific Resource to solve local congestion per QSE per interval per zone

PMqiuz:	Incremental Premium Specified for that specific Resource per interval

BPMqiuz:	Incremental bid premium for that specific Resource per interval

OLquiz:	 Resource plan output level submitted by the QSE for the specific Resource given the resource specific  instruction for that interval

IOLquiz:	The Instructed output level for the resource given resource specific instruction

MRqiuz:

Meter reading per unit per interval.

MCPEiz:	Market clearing price of Energy per interval per zone

7.4.3.2 Balancing Energy Down from a Specific Resource

In accordance with section 6.7.1.2, if a Market Solution exists, whenever a decremental specific Resource is deployed to resolve the Local Congestion, the QSE will be paid the difference of MCPE in the commercial zone, which the specific Resource is in, and the decremental bid premium specified.

LPCRSDqiz 	= 	-1 * Σ( (MCPEiz - BPMqiuz ) * Max(0,

Min(OLqiuz -MRquiz  , OLqiuz - IOLqiuz)))u

Where:

LPCRSDqiz:	Payment to QSE for providing Balancing Energy Down from specific Resource to solve local congestion per QSE per interval per zone

BPMqiuz:	Decremental bid Premium Specified for that specific Resource per interval

OLquiz:	Resource plan output level submitted by the QSE for the specific Resource given the resource specific  instruction for that interval

IOLquiz:	The Instructed output level for the resource given resource specific instruction

MRqiuz:	Meter reading per unit per interval.

MCPEiz:	Market clearing price of Energy per interval per zone

| ## Sponsor |  |
| --- | --- |
| Name | Jun Yu |
| E-mail Address | jyu@ercot.com |
| Company | ERCOT |
| Company Address | 2705 W. Lake Drive, Taylor, Texas 76574 |
| Phone Number | 512-248-3969 |
| Fax Number | 512-248-3082 |
| ## ERCOT Impact Analysis |  |
| Date |  |
| Financial Impact |  |
| Analysis |  |
| ## TAC Action |  |
| TAC Vote Date |  |
| TAC Proposed Implementation Date |  |
| TAC Vote Results |  |
| ## ERCOT Board Action |  |
| ERCOT Board Vote Date |  |
| ERCOT Board Implementation Date |  |
| ERCOT Board Results |  |
| ## Appeal |  |
| Appeal Date |  |
| Entity Initiating Appeal |  |
| Nature of Appeal |  |