| PRR Number | 300PRR | PRR Title | TCR Congestion Management |
| --- | --- | --- | --- |
| ###### Status |  |  |  |
| Protocol Section Requiring Revision |  | Section 6: Ancillary Services Section 7: Congestion Management Section 16.8: Registration of TCR Account Holders |  |
| Requested Resolution |  | ## Urgent |  |
| Revision Description |  | Establish Congestion Management guidelines and requirements for February 15, 2002. Strike System Congestion Fund upon conversion to direct assignment of Zonal Congestion costs. Strike Zonal Congestion Tracking upon conversion to direct assignment of Zonal Congestion costs. |  |
| ###### Reason for Revision |  | Have reached the twenty million dollar ($20,000,000) threshold. |  |
| ## Timeline |  |  |  |
| Date Received |  |  |  |
| Date Posted |  |  |  |
| PRS Review Date |  |  |  |
| PRS Recommendation |  |  |  |

Proposed Protocol Language Revision

# 7 CONGESTION MANAGEMENT

## 7.1 Overview of ERCOT Congestion Management

ERCOT will implement a Zonal Congestion management scheme on February 15, 2002 that is flow-based, whereby the ERCOT Transmission Grid, including attached Generation Resources and Loads, will be divided into a predetermined number of Congestion Zones.  Each Congestion Zone is defined such that each Generation Resource or Load within the Congestion Zone boundaries has a similar effect on the loading (Shift Factor) of Transmission Facilities between Congestion Zones.  For purposes of solving Zonal Congestion the Shift Factor will be assumed the same for all Generation Resources and Loads within a Congestion Zone.  Therefore any imbalance between Loads and Generation Resources in a Congestion Zone will be deemed to have the same impact on a given loading between Congestion Zones.

This Congestion management scheme applies Shift Factors, determined by ERCOT, on the zonal QSE schedules to predict potential Congestion on CSCs under the known topology of the ERCOT System.  The Shift Factors determined by ERCOT should most closely represent the effect of Generation Resources and Loads in the Congestion Zones on the CSCs with the current topology of ERCOT System.  This scheme is used in the Day Ahead and Adjustment Periods to evaluate potential Congestion and notify the market accordingly.  The ERCOT also uses this scheme, along with other factors, to determine if Replacement Reserve Service should be purchased in a Congestion Zone to provide additional Balancing Energy Service to provide a solution for expected Congestion.

ERCOT will manage transmission congestion and will categorize the cost of congestion management as either Zonal Congestion management costs or Local Congestion Management Costs.  Zonal Congestion management costs are those costs that are attributable to managing congestion on -CSCs or predefined Closely Related Elements (CRE).  The costs of managing Zonal Congestion will be directly assigned to QSEs based on each QSE’s impact on CSCs or CREs. All other congestion management costs are considered Local Congestion management costs.

ERCOT will use the Zonal Congestion management model with Shift Factors of Generation Resources and Loads on CSCs and zonal Balancing Energy Service MCPEs  to determine the Shadow Prices  of energy across corresponding CSCs. In addition to the Shadow Price, other costs, such as Replacement Reserve Service costs, may also be assessed. These Shadow Prices will be used in settlement to directly assign the cost of managing Zonal Congestion to QSEs.   In addition, the costs of any Replacement Reserve Service procured on a zonal basis for congestion management purposes will also be direct assigned to QSEs.

The Local Congestion management scheme relies on a more detailed model to determine how each particular Resource or Load impacts the transmission system.  This model does not use portfolios and it does use the current known topology of the transmission system.

The cost to solve Local Congestion will be uplifted pro rata to each QSE based on the Load Ratio Share by QSE.

ERCOT will manage Congestion by:

- Evaluating the levels of Zonal Congestion and any other Congestion during the Day Ahead, the Adjustment Period and the Operating Period using appropriate models of the ERCOT Transmission Grid;

- Examining the impacts of QSE energy schedules on CSCs;

- Posting on the MIS, the total megawatt quantity impacts on every CSC, and allowing QSEs to adjust schedules to mitigate potential Congestion on the CSCs;

- Procuring during the Adjustment Period, as needed, Replacement Reserve Service (RPRS) to use with other Resources for which QSEs have submitted Balancing Energy bids to provide sufficient capacity for Balancing Energy flows in the Operating Hour while respecting operational limits of the ERCOT Transmission Grid;

- Determining settlement for QSEs providing RPRS procured to manage Congestion;

- Determining settlement for QSEs providing Balancing Energy associated with resolving Zonal Congestion;

- Allocating the costs of Zonal Congestion management to QSEs in accordance with the applicable interim period Load Ratio Share allocation method through February 14, 2002;

- Allocating the costs of Zonal Congestion management to QSEs in accordance with the direct assignment  method beginning on February 15, 2002;

- The cost to solve Local Congestion will be uplifted pro rata to each QSE based on the Load Ratio Share by QSE.

ERCOT will carry out these steps in accordance with this Section and the Scheduling and Ancillary Service Scheduling and Selection requirements in Sections 4 and 6, respectively.

## 7.2 CSC Zone Determination

### 7.2.1 Principles for Determination and Modification and/or Elimination of CSC Congestion Zones

#### 7.2.1.1 Developing the Commercial Model

ERCOT will reassess CSCs annually, based on changes to the ERCOT System topology, and will identify new Congestion Zones based on the reassessed CSC’s by November 1 of each year.  ERCOT will use the following process to identify CSC’s:

- By October 1, of each year, ERCOT will complete an analysis of load flow data and expected system additions and will determine expected operating limits and constraints to be used in the designation of CSCs for the upcoming calendar year.

- Determine candidates for CSC designation: A directional transmission path or interface will be considered as a candidate for CSC designation if the actual annual Congestion costs from the prior year including the cost of any zonal BES deployment, RMR, OOMC, OOME, or unit specific deployments used to resolve Congestion on that particular transmission path in that particular direction during the prior year and/or projected annual Congestion costs for the CSC designation year exceeds a threshold amount determined by the appropriate subcommittee.  ERCOT will take into account generation and transmission system changes in designating candidates for CSC.

- Designate CSCs using the following steps:

- Establish Study Zones.  Study Zones are groupings of buses used to test transfer limits using power system simulation tools.  These zones should be reasonable approximations of the final Congestion Zones.  The following steps are performed to establish the Study Zones:

  - If no new candidate CSCs are considered other than prior year’s CSCs, then use the Congestion Zones from the previous year as Study Zones.

  - If new candidate CSCs are considered, then calculate shift factors and perform cluster analysis based on the candidate CSCs to create new Study Zones.

- Perform system simulation studies with transfers between the study zones and find constraints on the transmission system to determine if candidate CSCs are appropriate.

- Determine if the candidate CSCs qualify for CSC designation based, in part, on the criterion that there must be a sufficiently competitive market (at a minimum, apparent Market Solutions readily identifiable) to resolve Congestion on the transmission path to be considered for CSC designation.

- CSC approval process: The appropriate ERCOT Technical Advisory Committee (TAC) Subcommittee will review the process followed above to determine if the list of constraints should be recommended for approval to the TAC and the Board.

- 7.2.1.2 Congestion Zone & Zonal Average Shift Factor Determination Methodology

ERCOT will determine Congestion Zones and Zonal Average Shift Factors by:

- Developing Shift Factors using a linearized (DC) model to identify the impact of each transmission bus on each CSC relative to an ERCOT reference bus.

- Using statistical clustering to aggregate transmission busses into zones based upon similar Shift Factors relative to all CSCs.  To determine the appropriate number of zones, the clustering must meet the following criteria: (i) each CSC must straddle a zonal boundary (however, not every zonal boundary need be straddled by a CSC); (ii) the R2 (coefficient of determination) from the cluster analysis should be greater than a threshold level set by the appropriate subcommittee; and (iii) the maximum difference between the Zonal Average Shift Factor and the Shift Factor of any generation bus within the zone should not exceed a limit set by the same subcommittee. The limits will be set to balance the competing goals of minimizing the number of Congestion Zones and maximizing the accuracy of the Commercial Model’s representation of the ERCOT System

- Creating Zonal Average Shift Factors for each CSC by averaging the individual bus Shift Factors within each Congestion  Zone for the corresponding CSC, weighted by the Generation Resource megawatt levels on the bus in the load flow base case used to determine CSC limits.

- Congestion Zone approval process: The appropriate ERCOT Technical Advisory Committee (TAC) Subcommittee will review the process followed above to determine if the Congestion Zones should be recommended for approval to the TAC and the Board.

#### 7.2.1.3 Determining Closely Related Elements (CRE)

The appropriate ERCOT Technical Advisory Committee (TAC) Subcommittee will review the CRE identification process used to determine the CREs that should be recommended for approval to the TAC and the Board.    ERCOT may determine CREs beginning with the year 2003 as follows:

- Determine the Zonal Average Shift Factor for a particular CSC (Xz) for each Zone z.

- Determine the zonal average shift factor for the candidate CRE (Yz) for each Zone z using the same generation weighting as in (1) but ignoring Boundary Generation Resource busses that would cluster into a different Congestion Zone with respect to the CRE.

- Determine positive “a” applying least-square curve fitting to the following equation:

Yz = a (Xz) + bz  for all Zones z.

- Using “a” from (3), determine the maximum absolute value of bz.

- Also determine the total capacity (MW) of Boundary Generation Resources that would cluster into a different Congestion Zone.

- If the maximum absolute value of bz is less than a threshold set by the appropriate TAC subcommittee, not to exceed 0.2, and the total capacity of Boundary Generation Resources that would cluster into a different Congestion Zone is less than 1,500 MW, then the element is a CRE for the particular CSC.

By November 1 of each year, the appropriate ERCOT subcommittee will report to the TAC and ERCOT Board with recommended CSC designations,  resulting Congestion Zone boundaries,  CRE designations and associated Boundary Generation Resources for ERCOT Board review and approval.

Immediately following ERCOT Board approval changes in the CSCs and the resulting Congestion Zone boundaries and CRE designations will be posted on the MIS.  This posting will include a bus-by-bus identification of each Congestion Zone.

## 7.3 Congestion Management  Procedure

### 7.3.1 Determination of Zonal Congestion

ERCOT will analyze energy schedules to determine the existence and extent of Zonal Congestion as part of the Day Ahead Scheduling Process.

### 7.3.2 Resolution of  Congestion and its Implications for BES Prices

ERCOT will resolve  Congestion by the following means:

- Using adjusted Balanced Schedules received from QSEs after ERCOT’s posting of CSC impacts greater than the CSC Limit, ERCOT will reassess the resulting level of Zonal Congestion.  ERCOT will take no further Zonal Congestion actions if the adjusted Balanced Schedules resolve initial Zonal Congestion.

- If Zonal Congestion still exists following receipt of adjusted Balanced Schedules, ERCOT may procure RPRS in accordance with Section 6.6.3.2. ERCOT will then balance the energy within the ERCOT System in the Operating Period respecting all operational limitations of the ERCOT System.

- Zonal Balancing Energy can only be deployed for Congestion Management purposes to resolve flow limit violations on CSCs and CREs.

- Congestion is resolved in real-time as follows:

- Calculate zonal BES deployments needed to maintain flows within CSC Flow Limits.  If the congestion that needs to be resolved is on a CRE, then  Boundary Generation Resources unique to that CRE   will be instructed to operate at the respective Resource Plan levels (i.e., this is not an instructed deviation from schedule or Resource Plan).

- Based on (i), deploy unit specific instructions to manage Local Congestion.

- In the event of a contingency, deploy BES to maintain flows within pre-second contingency CSC flow limits and deploy unit-specific instructions to manage any resulting Local Congestion.

The result of Congestion Management should ensure that:

- Balancing Energy MCPEs  are the same in all zones if no CSC or CRE is constrained.

- Balancing Energy MCPEs  will be different in two or more  Congestion Zones only if one or more CSCs or CREs are constrained.

- The Shadow Price of a constrained CSC or CRE will be calculated by taking the difference in Balancing Energy MCPEs  between any two zones divided by the difference in Zonal Average Shift Factors on the corresponding CSC or CRE of the two zones even in the event of an outage of the CSC or CRE.

### 7.3.3 Interim Period - Settlement /Zonal Congestion

This Subsection, 7.3.3, shall expire and terminate upon ERCOT’s conversion to direct assignment of Zonal Congestion costs.

ERCOT will use an interim Postage Stamp Allocation method for settlement of Zonal Congestion costs to QSEs in Section 7.3.3.1 as described below beginning on June 1, 2001.  ERCOT will convert to a direct assignment of cost methodology of Zonal Congestion (with pre-assigned TCRs) starting on February 15, 2002.Settlement will be in accordance with Section 6.9, Settlement for ERCOT-Provided Ancillary Services, and Section 7.3, Congestion Management for CSCs/Zonal Congestion.

#### 7.3.3.1 System Congestion Fund

This Subsection, 7.3.3.1, shall expire and terminate upon ERCOT’s conversion to direct assignment of Zonal Congestion costs.

ERCOT will maintain a System Congestion Fund (SCF) for purposes of collecting and paying for CSC/Zonal Congestion Management costs during the period when the Postage Stamp Allocation method of settlement is used.

ERCOT will manage the SCF as follows:

- Annually, ERCOT will estimate the total monthly costs to relieve CSC/Zonal Congestion for the next year.  The sum of these monthly costs will be the total expected CSC/Zonal Congestion costs for the next year.

- By December 1 of each year, the ERCOT Board will review and approve a SCF fee to be collected from all QSEs representing Load based on the annual expected CSC/Zonal Congestion cost divided by the total annual estimated MWh ERCOT System Load for the next year.  ERCOT will reevaluate the SCF fee to assure that the SCF collects as close to the expected CSC/Zonal Congestion costs as possible without over or under collecting.  Any funds remaining in the SCF at the end of the year will go to offset the next year’s system Congestion costs. Any shortages, not covered by ERCOT’s line of credit, will be uplifted to all QSEs representing Load on a Load Ratio Share.

- ERCOT will make payments from the SCF to QSEs representing Resources used to provide Replacement Reserves and Balancing Energy to solve CSC/Zonal Congestion in accordance with Section 6.8, Compensation for Services Provided.

- Each QSE will be charged, on a daily basis, an amount equal to the SCF fee multiplied by the actual megawatt hour of Load represented by that QSE in accordance with this Section.

- Any over collection into the SCF will be kept in an interest bearing account to be used to offset future CSC/Zonal cost and or returned on a QSE Load Ratio Share upon moving to a direct assignment of cost methodology of CSC/Zonal Congestion.

- The System Congestion Fund calculation will be as follows:

SCFqi		=	FFi	* 	AMLqi

Where:

- SCFqi	System Congestion Fund costs per interval per QSE

- FFi	The Forecasted Fee per megawatt hour fee per interval, in accordance with this Section 7.3.3.1.

- AMLqi	The Adjusted Metered Load in that interval for a given QSE

#### 7.3.3.2 Tracking Balancing Energy Zonal Congestion Cost

This Subsection, 7.3.3.2, shall expire and terminate upon ERCOT’s conversion to direct assignment of Zonal Congestion costs.

The calculation for tracking Balancing Energy Zonal Congestion costs is the following:

ZCBEzci            =   SPZCi * Max(0, IZCi – TTCZCi)

IZCi	= SUM((QRSiz - SOiz) * SFzZC )z

The total Balancing Energy CSC costs per interval for the entire market is calculated as follows:

ZCBEi		= 	SUM(ZCBEzci)ZC

Where:

i	Interval being calculated

z	Congestion Zone

ZC	 Zonal Congestion Constraints

ZCBEi 	The total Balancing Energy ZC costs per interval for the entire market

ZCBEzci 	Zonal Congestion Energy Related Congestion Charge per interval

SPZCi    	Shadow Price per Zonal Congestion, per interval from the Zonal Congestion calculation of Balancing Energy deployment for the interval, as provided by the ERCOT MOS/POS System

IZCi    	Scheduled MW Impact per ZC, per interval

TTCZCi    	Actual Total Transfer Capability on a given constraint per Zonal Congestion per interval

SFzZC	Commercial Model Shift Factor per Zonal Congestion, per zone

QRSiz	QSE Resource Schedule per interval, per zone

- SOiz	Scheduled Obligation per interval, per zone

#### 7.3.3.3 Replacement Reserve Service CSC/Zonal Congestion Cost

This Subsection, 7.3.3.3, shall expire and terminate upon ERCOT’s conversion to direct assignment of Zonal Congestion costs.

During the interim period, the Replacement Reserve CSC/Zonal Congestion Capacity Cost will be tracked against the twenty million dollars ($20,000,000.00) threshold in accordance with Section 7.3.3.1, System Congestion Fund.

The Shadow Price and Zonal Congestion Impact are determined by ERCOT in accordance with Section 6.6.3.2.1 of these Protocols.  During the interim period, Replacement Reserve Zonal Congestion costs are tracked against the SCF as follows:

A QSE’s Obligation for Replacement Reserves (including OOMC and RMR Units providing RPRS) procured for Local Congestion, as determined by the mathematical optimization process, will be shared by the QSE’s in relation to their Load share of the total ERCOT Load for the interval.

The cost of Replacement capacity that is not assigned in the mathematical optimization process will be shared by all QSEs in relation to their Load Ratio Share of the total ERCOT Load for the interval.

7.3.3.3.1 Interim Period

This Subsection, 7.3.3.3.1, shall expire and terminate upon ERCOT’s conversion to direct assignment of Zonal Congestion costs.

The formula used to calculate Replacement Reserve Zonal Congestion costs during the interim period, for tracking purposes is as follows:

ZCRPi 	= 	SPCZCi* Max(0, (IZCi  - TTCzci))

i		Interval being calculated

ZCRPi	Replacement Reserve Service Zonal Congestion Capacity Cost ($) per interval per Zonal Congestion

SPCZCi	Shadow Price ($/MW) of Zonal Congestion Capacity per interval

Izci	Scheduled Impact (MW) per Zonal Congestion per interval at the time of procurement.

TTCZCi	Minimum Forecasted Total Transfer Capacity in MW in that interval on the  Zonal Congestion used to determine capacity needed to solve Zonal Congestion

7.3.4 Settlement of Zonal Congestion

#### 7.3.4.1 Balancing Energy Zonal Congestion Charge

After the interim period, when direct assignment cost methodology for Zonal Congestion is in effect, QSEs will pay for the cost of clearing ZonalCongestion in accordance with Section 6.9 of these Protocols. The calculation for the Zonal Congestion Management Charge is as follows:

ICSCiq	= Σ ((QRSiqz - SOiqz) * SFzcsc )z

If ICSCiq > 0 then

CSCBECSCiq	=	SPCSCi	* max(0,  ICSCiq – PTCRCSCiq)

Else (counterflow)

CSCBECSCiq	=	SPCSCi	* ICSCiq

End

CSCBEiq	=	SUM(CSCBECSCiq)CSC

i	Interval being calculated

q	QSE

z	Congestion Zone

CSC	Commercially Significant Constraint

CSCBECSCiq 	CSC Energy Related Congestion Charge, per interval, per CSC, per QSE

CSCBEiq 	CSC Energy Related Congestion Charge, per interval of that QSE

SPCSCi    	Shadow Price per CSC, per interval

ICSCiq    	Actual MW Impact per CSC, per interval, per QSE

PTCRCSCiq    	Pre-assigned Transmission Congestion Rights per CSC, per interval, per QSE

SFzcsc	Zonal Average Shift Factor per CSC, per zone

QRSiqz	QSE Resource Schedule per interval, per zone, per QSE

SOiqz	Scheduled Obligation (prior to real-time) per interval, per zone, per QSE

The Zonal Average Shift Factors for settlement purposes will be computed using actual operational Shift Factors of Generators within the zone weighted by the generation level in the monthly base case.  The monthly base case will be the most recent base case used by ERCOT but no less current than the base case used by ERCOT to determine the number of TCRs available for sale for the corresponding month.

#### 7.3.4.2 Replacement Reserve Service Zonal Congestion Charge

Replacement Reserve Zonal Congestion costs will be allocated directly to the QSE impacting the CSC.   The Replacement Reserve Zonal Congestion costs per interval per QSE will be determined by multiplying the Shadow Price of CSC capacity by the difference of the QSE’s CSC impact and any QSE pre-assigned TCR’s in that interval.

The CSC impact is determined by taking for each Congestion Zone, the maximum of either the difference between each QSE’s Resource schedules and the Load schedules at the time of each round of Replacement Reserve capacity procurement or the difference of their actual Resource and their Adjusted Metered Load within that Congestion Zone multiplied by the Zonal Average Shift Factor.  The calculation  of the Replacement Reserve Service Zonal Congestion Charge will be as follows:

CSCRPiq 	= 		SPCCSCi* max(0, ICSCiq  - min(ICSCiq , PTCRcsciq))

Icsciq                 =	     max [0,

Σ (QRSTOPiqz – CLTOPiqz) * SFzcsc)z,

Σ (AMRiqz - AMLiqz) * SFzcsc)z]

i	Interval being calculated

q	QSE

z	Congestion Zone

CSC	Commercially Significant Constraint

CSCRPiq 	CSC Replacement Reserve related Congestion charge, per interval,  per QSE

SPCCSCi    	Capacity Shadow Price per CSC, per interval

ICSCiq    	Actual MW Impact per CSC, per interval, per QSE

PTCRCSCiq    	Pre-assigned Transmission Congestion Rights per CSC, per interval, per QSE

SFzcsc	Zonal Average Shift Factor per CSC, per zone

QRSTOPiqz	QSE Resource Schedule per interval, per zone, at the time of purchase, per QSE

CLTOPiqz	Current Load at the time of purchase per zone per interval per QSE

AMLiqz	Adjusted Metered Load in that Congestion Zone per interval of that QSE

AMRiqz	Actual Metered Resource values in that Congestion Zone per interval of that QSE

## 7.4 Congestion Management for Local Congestion

### 7.4.1 Determination of Local Congestion

ERCOT will use the Operational Model of the ERCOT System in combination with QSE energy  schedules and ERCOT forecasted Load to determine the extent of Local Congestion.

### 7.4.2 Resolution of Local Congestion

ERCOT will procure RPRS if necessary to provide sufficient capacity to resolve forecast Local Congestion. ERCOT will purchase RPRS from the Resource(s) having the lowest cost per megawatt based on each Resource’s bid price and its ability to resolve the Congestion given its Shift Factor on the Local Congestion. Bid quantity will not be taken into account in the decision to purchase RPRS.

ERCOT will instruct and dispatch Resources in Real Time in a manner that does not exceed the operational limits known to ERCOT. The Resources that have been instructed to operate within a range to resolve Local Congestion are paid or pay a price, in accordance with Section 6.8, that depends on whether a Market Solution exists for resolving the Local Congestion.

The Local Congestion Costs are embedded in each service deployed to resolve Local Congestion such as OOMC, OOME, and unit specific premiums.

The formula to calculate Local Congestion Replacement Reserve payment can be found in Section 6.8.1.11 of these Protocols.

### 7.4.3 Settlement of Local Congestion Costs

Net Local Congestion costs will be settled in the Balancing Energy settlement process.

[PIP 120: Local Balancing Premiums. The section following needs to be inserted once the system has an equation to settle local congestion separately from other Balancing Energy costs/payments]

7.4.3.1 Balancing Energy Up from a Specific Resource

In accordance with section 6.7.1.2, Deployment of Balancing Energy when Congestion Occurs, if a Market Solution exists, whenever an incremental specific Resource is deployed to solve Local Congestion, the QSE will be paid the incremental premium specified.

LPCRSUqiz 	= 	-1 * Σ((PMqiuz –MCPEiz )* Max(0, OLquiz - ALqiuz))

PMqiuz		=	Max (BPMqiuz, BPMqiuz + MCPEiz )

Where:

LPCRSUqiz:	Payment to QSE for providing Balancing Energy Up from Specific Resource to solve local congestion per QSE per interval per zone

PMqiuz:	Incremental Premium Specified for that specific Resource per interval

BPMqiuz:	Incremental bid premium for that specific Resource per interval

OLquiz:	The resource’s current output level

ALqiuz:	The minimum of the allowed range specified in the Dispatch Instruction at the time of receipt of the Dispatch Instruction

MCPEiz:	Market clearing price of Energy per interval per zone

7.4.3.2 Balancing Energy Down from a Specific Resource

In accordance with section 6.7.1.2, if a Market Solution exists, whenever a decremental specific Resource is deployed to resolve the Local Congestion, the QSE will be paid the difference of MCPE in the commercial zone, which the specific Resource is in, and the decremental bid premium specified.

LPCRSDqiz 	= 	-1 * Σ( (MCPEiz - BPMqiuz ) * LCqiuz)u

Where:

LPCRSDqiz:	Payment to QSE for providing Balancing Energy Down from specific Resource to solve local congestion per QSE per interval per zone

BPMqiuz:	Decremental bid Premium Specified for that specific Resource per interval

LCqiuz:	Balancing Energy Down instruction for that unit to solve local congestion per interval

MCPEiz:	Market clearing price of Energy per interval per zone

7.4.3.3 Balancing Energy Charge to Solve Local Congestion

In accordance with this section, all the cost associated with Balancing Energy from a specific resource to solve Local Congestion will be allocated on a QSE Load Ratio Share.

LCCiq = - 1* Σ ((LPCRSUqiz + LPCRSDqiz) * LRSiq)

Where:

LPCRSUqiz:	Payment to QSE for providing Balancing Energy Up from specific Resource to solve local congestion per QSE per interval per zone

LPCRSDqiz:	Payment to QSE for providing Balancing Energy Down from specific Resource to solve local congestion per QSE per interval per zone

LCCiq:	Local Congestion load allocation charge per QSE per interval

LRSqi 	Load Ratio Share

## 7.4.4  Direct Assignment of Local Congestion Costs

ERCOT shall directly assign Local Congestion costs beginning six months after ERCOT’s incurred costs of clearing Local Congestion during a rolling twelve month period reaches 20 million dollars.

## 7.5 Transmission Congestion Rights

Pre-assigned TCRs are treated differently from the TCRs addressed in this section.  The function and settlement of Pre-assigned TCRs are addressed elsewhere in this Section.

### 7.5.1 Function of Transmission Congestion Rights

TCRs function as financial hedges (similar to financial options with zero strike price) against the marginal costs to resolve Zonal Congestion.  The total costs of Zonal Congestion comprise those portions of RPRS costs and the Balancing Energy Service costs associated with managing Zonal Congestion.  The TCR holder will receive an amount equal to the directly assigned congestion costs for an equivalent quantity of scheduled flow.  TCRs are not deratable once they are sold in the auction(s).

A TCR is a financial right on a specified directional CSC for a particular hour that entitles the holder of record to receive remuneration equal to the sum of non-negative BES and RPRS Shadow Prices ($/MW/15-minute) over all 15-minute intervals within the hour for the corresponding CSC multiplied by 1 MW.

### 7.5.2	Procedure for Computation of Transmission Congestion Right Quantities

ERCOT will model the ERCOT System with guidance and assistance from the appropriate ERCOT TAC Subcommittee using steady-state and dynamic power system simulation software and the CSC interfaces determined in accordance with Section 7.2.  The capacity of the CSCs will be fully used and there will be no reservation of capacity for any particular segment of Load.

ERCOT will determine CSC Flow Limits as follows:

- Determine the pre-contingency flow limit on the directional CSC to withstand any single contingency of any directional CRE corresponding to the CSC, including the CSC, irrespective of the post-contingency limiting element due to the contingency, for all zone-to-zone transfer studies.

- Determine the pre-contingency flow limit on the CSC to withstand any single contingency that results in any CRE corresponding to the CSC to become the post-contingency limiting element for all zone-to-zone transfer studies.

- The lower of the CSC flow limits from (1) and (2) above sets the CSC Flow Limit.

7.5.2.1	Annual TCR Quantities

Annually determined summer ERCOT peak flow limits for CSCs will provide the starting basis for determining the number of TCRs to be auctioned.  The flow limit established for each CSC on an annual basis may vary from time to time based on changes in system conditions.  The ultimate number of TCRs to be auctioned for the CSCs will take into account forecasted changes in system conditions.

ERCOT will determine the total quantity of TCRs for the annual distribution by adjusting the annual summer peak base case CSC Flow Limits by the following amount: for each CSC, the sum over all nodes of the product of the expected level of load at each node from the annual summer peak base case multiplied by the difference between the corresponding Shift Factor for that node and CSC and the Zonal Average Shift Factor.

7.5.2.2	Monthly TCR Quantities

ERCOT will determine the total quantity of TCRs for the monthly distribution by adjusting the corresponding modified seasonal base case CSC Flow Limits by the following amount: for each CSC, the sum over all nodes of the product of the expected level of load at each node from the modified seasonal base case multiplied by the difference between the corresponding Shift Factor for that node and CSC and the Zonal Average Shift Factor.  The modified seasonal base case for a particular month will be based on the most limiting situation within the corresponding month.  This analysis takes into account all transmission outages of significant duration and impact with the objective of collecting sufficient revenues from direct assigned congestion costs to pay all TCR holders.

ERCOT may distribute TCRs in amounts less than the total quantities based on expectations of the frequency and magnitude of potential CSC de-rates that could occur during any particular month.

7.5.2.3  ERCOT Posting of TCR Quantities

Upon review and approval of the TCR amount, ERCOT will determine and post on the MIS the total quantity, the amount pre-assigned, and the remaining available quantity of TCRs for each CSC.  The remaining available quantity of TCRs for auction will be the total amount of TCRs less the amount pre-assigned.

### 7.5.3 Annual and Monthly Auction of Transmission Congestion Rights

#### 7.5.3.1 Annual and Monthly Auction Summary

Sixty percent (60%) of the annual total quantity of TCRs less Pre-assigned TCRs for any given CSC will be awarded  to Market Participants based on the results of an initial annual auction. These TCRs will be made available in the annual auction for each hour of the year Each annual auction will be completed no later than the 15th of December, on a date set by ERCOT.

Prior to each monthly auction, ERCOT will determine the total amount of TCRs available for each month based upon forecasted system conditions for the month.  The amount of TCRs that will be auctioned in the monthly auction will be the total amount of TCRs, less any Pre-assigned TCRs, and less any TCRs for that month sold in the annual auction.  However, in determining the amount of TCRs available in each monthly auction, ERCOT shall seek to achieve revenue neutrality and avoid a revenue shortfall due to payment to TCR holders for the month. The TCRs auctioned in the monthly auction will be allocated to Market Participants based on the results of monthly auctions. These TCRs will be made available in the monthly auction for all hours of each day of each month for which the auction is being held. .  Each monthly auction will be completed no later than the 15th of each month prior to the month that the TCRs are effective, on a date set by ERCOT.

#### 7.5.3.2 Auction Procedures

ERCOT will develop a procedure for authenticating ownership for each TCR.

ERCOT, or its designee, will conduct simple, single round, single-clearing-price auction for each of the CSCs.  The auction awards TCRs from the highest price to the lowest price until one hundred percent (100%) have been awarded, unless otherwise indicated herein.  The lowest awarded price becomes the market-clearing price for all awarded  TCRs in the annual and monthly auctions.  ERCOT shall implement a single round, simultaneous combinatorial auction prior to  the annual auction for TCRs effective in 2003.

The auction process will be open to any qualified bidder except TDSPs. To participate in the auctions, a bidder must meet financial security requirements established by ERCOT and must have access to the computer hardware, software and communications equipment required to participate.

##### 7.5.4.2.1 Notice for Annual and Monthly Auction

Thirty days (30) prior to the annual auction and ten (10) days prior to the monthly auction, ERCOT will post:

- The Zonal Average Shift Factor matrix and the zone-to-zone impact matrix (to determine the megawatt impact on each CSC);

- The number of TCRs to be issued for each period (annual, monthly) in that auction;

- Deadline for bidders to satisfy financial requirements;

- Specifications for the equipment necessary to participate in the auction;

- The date and time by which bids must be submitted;

- The bid format;

- CSCs and  closely related elements (CRE);

- Any other information of commercial significance to bidders.

##### 7.5.4.2.2 Auction

The auction will be a simple, single round, single-clearing-price auction with the following steps:

- The auction will start with the CSC that has the largest amount of capacity available to be auctioned and end with the CSC that has the smallest amount of capacity available to be auctioned.

- Bidders will submit bids containing price/quantity pairs.  Prices are bid in dollars (with no more than two decimal points) per megawatt hour, and must be non-negative.  Quantities must be bid in whole megawatt quantities, with a minimum bid quantity of one megawatt.

- The auction system will check the possible award amount of the bid against the credit posted by the bidder.  Any bids submitted that violate ERCOT-approved credit limits shall be rejected.

- The auction system will compile all bids submitted by auction participants into a bid stack starting with the price quantity bid at the highest price and descending to the lowest price.

- The auction system will award bids starting with the highest priced bid.  The auction system will then award to the next highest priced bid and continue this process until all of the  available TCRs for the particular CSC is awarded.

- If all of the annual bids are awarded and there are TCRs remaining, the remaining TCRs will be included in the monthly auction.

- If all of the monthly bids are awarded and there are TCRs remaining, the remaining TCRs will be not be sold.

- The last price/quantity pair awarded will set the price for all TCRs on that particular CSC called the TCR Clearing Price.

- In the event that there are multiple bids with prices equal to the TCR Clearing Price, the remaining TCRs will be awarded on a pro-rata basis.   The remainder of TCRs that cannot be distributed in whole integers will be awarded to the bidder who bid for the largest amount of TCRs at the clearing price.

- The auctioneer shall announce the results of the auction simultaneously to all TCR bidders, by posting the results on the MIS. For each CSC, the total quantity of TCRs awarded, the clearing price, the compiled bid curve, and the number of awardees will be posted.  The identity of the awardees will not be posted.

- (11) The total value of the TCRs awarded that are not yet paid for will be deducted from the available credit line for the next auction for each auction participant.

7.5.4.3 TCR Auction Charge

TCR auction proceeds will be collected in the following manner:

TCRCRMPp = TCRCSCRMPp * TCR HOURSp * MCPTTCRp

Where:

p     	Effective TCR period, could be a month or a year.

TCRCRMPp 	TCR Charge per RMP for that period

TCRCSCRMPp	TCR quantity awarded in MW to the RMP (by flow direction) of that period

TCR HOURSp	Total number of hours in the effective TCR period (p)

MCPTTCRp 	TCR Market Clearing Price for the TCR quantity for that period

RMP	Registered Market Participant

7.5.5 Allocation Method and Timing for Distributing TCR Auction Revenues

The auction revenues from each annual auction will be divided into monthly revenues in the same proportion as ERCOT’s monthly energy forecast.  This amount will be added to the monthly auction revenue.  The total monthly revenues from the auctions will be credited to QSEs once per month on a  monthly Load Ratio Share for the ERCOT peak operating interval during the month basis.  These revenues will be paid to QSEs no later than the 15th of the following month.

Any difference between the direct assigned congestion costs charged by ERCOT and Zonal Congestion Credits paid to TCR holders for any 15-minute interval will be credited or charged to all QSEs on a Load Ratio Share basis for the same 15-minute interval.  ERCOT shall provide a report to be published on its MIS that details, by interval, any zonal congestion revenue shortfalls or surpluses.  The report shall be published on a monthly basis within seven (7) Business Days of the end of a month for the prior month’s activity.  The report shall provide a description of the methodology and billing determinants used to determine and assess any revenue shortfalls or surpluses, and shall include any billing determinants necessary to replicate the results in the report, to the extent such billing determinants are not already available.

#### 7.5.6 TCR Settlement

To participate in ERCOT’s TCR settlements, a TCR holder need not be a QSE nor be represented by a QSE, but must be registered as a TCR account holder with ERCOT and have the ability to make and accept electronic transfer of funds.

Congestion Credits, calculated as described below, will be determined based on the record of ownership in ERCOT’s database, at the end of the day for which the TCRs are effective, for the relevant CSC, based on the published Shadow Price(s) for the same.  The TCR holders will be paid Congestion Credits in accordance with their TCR ownership in accordance with the settlement calendar in Section 9.

TCRPAYhp                 =	Σ (TCRCSChp * (Σ (SPCSCi /4)i – SPCCSCh))CSC

h	Hour being calculated

i	Intervals within the hour

p	TCR owner of record

CSC	Commercially Significant Constraint

CSCRPiq 	CSC Replacement Reserve related Congestion charge, per interval,  per QSE

TCRCSChp    	Total number of TCRs owned per hour, per TCR owner of record

SPCSCi    	BES Shadow Price per CSC, per interval

SPCCSCh    	RPRS Capacity Shadow Price per CSC, per hour

TCRPAYhp    	TCR payment per hour, per TCR owner of record

#### 7.5.7  Allocation of Pre-assigned TCRs

Upon the zonal method being implemented, MOUs and ECs which own or have a long-term (greater than five years) contractual commitment for annual capacity and energy from a specific remote Generation Resource, and that commitment was entered into prior to September 1, 1999, are eligible for pre-assigned TCRs.  The pre-assigned TCRs will be allocated as follows:

- The pre-assigned TCRs will extend three years after a MOUs or ECs enters into Customer Choice or the effective date of the removal of Federal Private Use Act restrictions that may be applicable to specific Generation Resources.

- The pre-assigned TCRs cost shall be a flat rate that is equal to the three year average (1997-99) of the Entity’s Load Ratio Share of the total ERCOT re-dispatch costs for planned transactions.

- The allocation of pre-assigned TCRs will be sufficient to isolate the MOUs and ECs from any direct-assigned costs associated with transporting the output of their remote Generation Resources to their native Load.

- Pre-assigned TCRs may only be used to schedule the eligible Generation Resources.

- Pre-assigned TCRs may not be traded in the secondary market.

- Holders of pre-assigned TCRs shall not be precluded from participating in the market to purchase additional TCRs

- Pre-assigned TCRs are subject to change or elimination at any time, upon ERCOT Board of Directors approval or PUCT order.

### 7.5.8 Secondary Market Exchange of Transmission Congestion Rights

With the exception of pre-assigned TCRs, Market Participants can exchange TCRs in any secondary market. ERCOT will initially populate  a database of hourly TCR holders with the annual and monthly first-purchasers of TCRs.  Subsequent holders or owners (other than first-purchasers) of the TCRs who want the change in ownership to be recorded in the ERCOT TCR ownership database will be responsible for assuring that the sellers report the transactions to ERCOT’s satisfaction and that the sellers authorize the transaction within the ERCOT database in accordance to ERCOT’s procedures.

### 7.5.9 Transmission Congestion Right Value Under Physical Curtailment or De-rating

TCRs will retain their full value even in the event of physical curtailment or flow limitation below the megawatt quantity for which TCRs were defined on any given CSC path.

### 7.6 Incorporation of Reliability Must-Run and Out of Merit Order Resources

RMR and OOM Resources will be used for Congestion Management as described in Sections 4, Scheduling, Section 5, Dispatch and Section 6, Ancillary Services. RMR and OOM Resources will be used for Congestion Management only in the absence of Market Solutions to alleviate Local Congestion .  RMR and OOM Resources will be compensated in accordance with Section 6.8

## 7.7 Remedies to Congestion Through Transmission Expansion

ERCOT will review the cost of correcting localized transmission limitations through new construction or other means, and will compare this to the costs incurred to correct those problems.  ERCOT shall publish the average cost of the constraint by period for each transmission limitation subtotaled by TDSP.  This information will be included in ERCOT’s  annual transmission planning report. If the projected or actual cost of ERCOT directed	 Congestion Management including all energy and capacity tools, is greater than the cost of correcting the transmission limitations through construction of Facilities, then ERCOT may recommend an upgrade of Facilities to the TDSP.

## 16.8 Registration of TCR Account Holders

Any Entity wishing to participate in the auction and registration of TCRs shall register with ERCOT and execute a TCR Account Holder Agreement prior to any participation in such program.  In order to participate in the TCR auction, TCR bidders must meet the financial requirements approved by the ERCOT Board.

## 16.8.1 Qualification of a TCR Account Holder

### 16.8.1.1 Criteria for Qualification as a TCR Account Holder

To be qualified as a TCR Account Holder, an Entity must:

(1) Execute a TCR Account Holder Agreement;

(2) Demonstrate to ERCOT’s reasonable satisfaction that the Entity is capable of performing the functions of a TCR Account Holder;

(3) Demonstrate to ERCOT’s reasonable satisfaction that the Entity is capable of complying with the requirements of all relevant ERCOT Protocols and guidelines; and

(4) Satisfy ERCOT’s creditworthiness requirements as set forth in this Section if applying to bid in ERCOT’s TCR Auction.

TCR Account Holders shall promptly notify ERCOT of any change that materially affects the Entity’s ability to satisfy the criteria set forth above.  If a TCR Account Holder fails to inform ERCOT within one (1) calendar day of a material change in the information provided which may affect the reliability or safety of the ERCOT System or the financial security of ERCOT, ERCOT may refuse to accept bids or TCR transfers from the TCR Account Holder and take any other action deemed appropriate.

Continued qualification, as a TCR Account Holder is contingent upon adherence to all requirements set forth in these Protocols. ERCOT shall suspend the TCR Account Holder’s rights as an Account Holder at such time as ERCOT reasonably determines that the Entity does not satisfy the criteria set forth above.

### 16.8.1.2 TCR Account Holder Qualification Requirements

- To meet the minimum requirements for qualification by ERCOT, a TCR Account Holder must:

- (1) Submit an application for qualification, including any applicable fee.

- (2) Execute any required agreements relating to use of the ERCOT network, software and systems.

- (3) Designate a representative who shall be responsible for operational communications and who shall have sufficient authority to commit and bind the TCR Account Holder.

- (4) Be financially responsible for payment of any applicable settlement charges as set forth in Sections 6, 7 and 9 of these Protocols.

- (5) Demonstrate a working functional interface with any applicable ERCOT systems as defined by these Protocols; and

- (6) Provide all necessary bank account information and arrange for Fed-Wire System transfers for two-way confirmation.

- (7)

### 16.8.1.3 Application Process for TCR Account Holder Qualification

To qualify as a TCR Account Holder, a TCR Account Holder applicant must submit to ERCOT a TCR Account Holder application, TCR credit application which may specify that the applicant does not intend to bid in which case no credit is required, and any applicable fee.  ERCOT shall post on the ERCOT web site the form in which TCR Account Holder application must be submitted, all materials that must be provided with the TCR Account Holder application, the TCR Account Holder Agreement, and the fee schedule, if any, applicable to TCR Account Holder applications and TCR credit application.  The TCR Account Holder application shall be attested to by a duly authorized officer or agent of the TCR Account Holder applicant.  The TCR Account Holder applicant shall promptly notify ERCOT of any material changes affecting a pending TCR Account Holder application using the appropriate form posted on the ERCOT web site.  The application must be submitted at least ten (10) days prior to the proposed date of commencement of service.

#### 16.8.1.3.1 Notice of Receipt of TCR Account Holder Application

Within three (3) Business Days after receiving each TCR Account Holder application and TRC Account Holder credit application, ERCOT shall send the TCR Account Holder applicant a written confirmation that ERCOT has received the TCR Account Holder application and TCR credit application.  ERCOT shall return without review any TCR Account Holder application that does not include the proper application fee.  The remainder of this subsection shall not apply to any TCR Account Holder application returned for failure to include the proper application fee.

#### 16.8.1.3.2 Sufficiency of Information Provided by TCR Account Holder Applicant

Within ten (10) Business Days after receipt of a TCR Account Holder application and TCR credit application, ERCOT shall notify the TCR Account Holder applicant in writing if the TCR Account Holder application is incomplete.  Within ten (10) Business Days after receipt of a TCR Account Holder application, if ERCOT does not notify an applicant that the application is incomplete, the application shall be deemed complete as of the date of receipt by ERCOT.

If ERCOT determines that a TCR Account Holder application and TCR credit application is not complete, ERCOT’s notification shall explain the deficiencies and stipulate the additional information necessary to make the TCR Account Holder application complete. The TCR Account Holder applicant shall then have five (5) Business Days from the receipt of ERCOT’s notification, or such longer period as ERCOT may allow, providing the additional required information.  If the TCR Account Holder applicant responds to the ERCOT notification within the allotted time, then the TCR Account Holder application shall be deemed complete on the date that ERCOT receives the complete additional information from the applicant.

If the TCR Account Holder applicant does not respond to ERCOT’s notification within the time allotted, ERCOT will reject the application and will notify the applicant according to the procedures below.

#### 16.8.1.3.3 ERCOT Acceptance or Rejection of TCR Account Holder Application

ERCOT shall approve or reject each TCR Account Holder application and TCR credit application not more than ten (10) Business Days after the TCR Account Holder application and TCR credit application has been deemed complete in accordance with this subsection.  Upon reasonable notice to the TCR Account Holder applicant, ERCOT may conduct a site visit as part of its evaluation of a TCR Account Holder application.  ERCOT may approve a TCR Account Holder application on the condition that the TCR Account Holder applicant agrees to a limitation(s) with respect to the types of transactions that it may conduct with ERCOT.

If ERCOT approves a TCR Account Holder application, it shall send an approval letter to the TCR Account Holder applicant, along with a TCR Account Holder Agreement and any other required agreements relating to use of the ERCOT network, software and systems for the TCR Account Holder applicant’s signature.

If ERCOT rejects a TCR Account Holder application, ERCOT shall send the TCR Account Holder applicant a rejection letter explaining the grounds upon which ERCOT has rejected the TCR Account Holder application. Appropriate grounds for rejecting a TCR Account Holder application include:

- (1) Required information not provided to ERCOT in the allotted time,

- (2) Non-compliance with technical requirements; and/or

- (3) Non-compliance with other specific eligibility requirements set forth in this Section or in any other part of these Protocols.

Not later than ten (10) Business Days after receipt of a rejection letter, the TCR Account Holder applicant may challenge the rejection of its TCR Account Holder application utilizing the dispute resolution procedures set forth in Section 20, Alternative Dispute Resolution Procedures.  Regardless of whether or not the TCR Account Holder applicant challenges the rejection, the applicant may submit a new TCR Account Holder application and fee and ERCOT shall process the new TCR Account Holder application in accordance with this subsection.

### 16.8.1.4 Remaining Steps for TCR Account Holder Qualification

An applicant must coordinate or perform the following:

- (1) Return the executed TCR Account Holder Agreement and other related agreements to ERCOT,

- (2) Coordinate with ERCOT and other Entities as necessary, to test all communications necessary to participate in the ERCOT TCR market,

- (3) Submit a Service Filing

- (4) Demonstrate compliance with security requirements.

- (5) Return the executed TCR credit agreement

#### 16.8.1.4.1 TCR Account Holder Application

Not less than fifteen (15) calendar days prior to commencement of any TCR bidding or TCR transfer activities with ERCOT, each TCR Account Holder must submit a complete application, including declaration on any Subordinate TCR Account Holders. ERCOT shall post on the ERCOT web sight the forms and procedures to be used by TCR Account Holders to submit an application.  The application shall include:

- (1) Proof of credit for ERCOT security amount, as detailed below; the security amount will increase or decrease with the addition or discontinuance of represented Account Holders and/or their respective market activity;

Not more than three (3) Business Days after receiving each application, ERCOT shall send a written notification to the TCR Account Holder that it has received the application. If the application is not complete, ERCOT shall notify the TCR Account Holder by telephone, by email, and certified mail with an explanation of the additional information necessary to make the application complete.

Not more than ten (10) calendar days after a complete application (either an application that is initially complete or one that has been supplemented pursuant to the above procedures) is received by ERCOT, ERCOT shall either notify the TCR Account Holder it may begin TCR bidding and TCR transfers upon its proposed commencement date or that ERCOT has reasonably determined that the TCR Account Holder’s application is insufficient.

Not later than ten (10) Business Days after receipt of a notice of insufficiency, the TCR Account Holder may challenge the notice of insufficiency utilizing the dispute resolution procedures set forth in Section 20, Alternative Dispute Resolution Procedures. Regardless of whether or not the TCR Account Holder challenges the notice of insufficiency, the TCR Account Holder may submit a new application and ERCOT shall process the new application in accordance with this subsection.

### 16.8.1.5 TCR Account Holder Financial Security

#### 16.8.1.5.1 ERCOT Creditworthiness Requirements for TCR Account Holders

TCR Account Holder’s must meet ERCOT’s creditworthiness requirements and maintain any minimum security amount required, as provided herein and demonstrated in a form acceptable to ERCOT.  If the TCR Account Holder’s creditworthiness rating falls below the level established by the ERCOT Board described below, ERCOT will suspend the TCR Account Holder’s TCR bidding activities under these Protocols until the TCR Account Holder submits another form of security in accordance with this Section.

##### 16.8.1.5.1.1 Requirements for Establishing Creditworthiness Rating

Minimum short term and long-term debt ratings, minimum average ties/interest earning ratio (TIER) and debt service coverage (DSC) ratios, and minimum equity ratios to establish credit worthiness will be adopted by the ERCOT Board of Directors.

A TCR Account Holder may meet ERCOT’s credit requirements, and is not initially required to post security, if the TCR Account Holder meets one of the following requirements:

(a) Has a Entity Short-Term or Long-Term Debt Rating that meets or exceeds the Minimum Short-Term and Long-Term Debt Ratings adopted by the Board;

(b) Is an Electric Cooperative, and (i) is a Rural Utilities Service (“RUS”) power supply borrower, and (ii) achieves the average TIER and DSC coverage ratios to be approved by the ERCOT Board, as such terms are defined in the Code of Federal Regulations Chapter XVII, and (iii) maintain an equity level approved by the ERCOT Board;

(c) Is a privately held company without Short Term or Long Term Debt Ratings, and (i) has equity in the amount equal to or greater than the minimum equity level approved by the ERCOT Board for a TCR Account Holder under this subsection, which may be adjusted by the Board from time to time in its discretion; and, (ii) provides its most recent audited and/or unaudited financial statements and other information as requested by ERCOT on a basis approved by the ERCOT Board (such as annually, quarterly, or monthly).

##### 16.8.1.5.1.2 Alternative Means of Satisfying ERCOT Creditworthiness Requirements

If a TCR Account Holder does not meet the creditworthiness qualifications above, the TCR Account Holder may satisfy ERCOT’s creditworthiness requirements through one of the following means:

(1) Another Entity that meets or exceeds the Minimum Debt Ratings, as established by the ERCOT Board, may provide a corporate guarantee of the TCR Account Holder’s liability to ERCOT in an amount equal to the TCR Account Holder’s Total Estimated Liability (TEL) or Estimated Aggregate Liability (EAL), whichever is applicable; provided that the corporate guarantee issuer shall provide annual audited financial statements not later than thirty (30) days after the close of each of the issuer’s fiscal year.

(2) The TCR Account Holder may provide an irrevocable letter of credit naming ERCOT beneficiary in an amount equal to or greater than the TCR Account Holder’s Total Estimated Liability (TEL) or Estimated Aggregate Liability (EAL), whichever is applicable.  ERCOT may reject the letter of credit if the issuing bank is unacceptable to ERCOT or if the conditions under which ERCOT can draw against the letter of credit are unacceptable to ERCOT.

(3) The TCR Account Holder may deposit cash in an account designated by ERCOT with the understanding that ERCOT may draw part or all of the cash so deposited to satisfy any overdue payments owed by the TCR Account Holder to ERCOT.  The account may bear interest payable directly to the TCR Account Holder, provided, however, any such arrangements shall not restrict ERCOT’s immediate access to the funds.  The cash so deposited shall be in an amount equal to or greater than the TCR Account Holder’s TEL or EAL, whichever is applicable.

### 16.8.1.6 Monitoring of Creditworthiness by ERCOT

ERCOT shall monitor the creditworthiness of each TCR Account Holder.  With respect to a TCR Account Holder that meets ERCOT creditworthiness requirements pursuant to 16.8.1.5.1, Requirements for Establishing Creditworthiness Rating, such TCR Account Holder shall inform ERCOT within three (3) Business Days if it has experienced a material change in its ability to satisfy the credit requirements of subsection 16.8.1.5.1.2, Alternative Means of Satisfying ERCOT Creditworthiness Requirements.  If the TCR Account Holder fails to promptly satisfy ERCOT creditworthiness requirements, then ERCOT may suspend the TCR Account Holder’s right to bid for TCRs.

With respect to TCR Account Holders meeting creditworthiness requirements using an alternative means provided in subsection 16.8.5.1, Alternative Means of Satisfying ERCOT Creditworthiness Requirements, each TCR Account Holder is responsible, at all times, for maintaining security in an amount at or above its EAL or TEL, as applicable.  ERCOT shall promptly notify each TCR Account Holder of changes to its TEL or EAL and allow the TCR Account Holder a reasonable time to provide additional security if necessary to maintain compliance with subsection 16.8.1.5. TCR Account Holder Financial Security.  If a TCR Account Holder fails to provide additional security within the time allowed by ERCOT, ERCOT may refuse to accept TCR bids from the TCR Account Holder and take any other action deemed appropriate.

ERCOT shall issue notifications to the Account Holder when the TCR bid liability is greater than the TCR Account Holder’s posted security.

### 16.8.1.7 Late Payments by Account Holders

It is the sole responsibility of each TCR Account Holder to ensure that amounts due to ERCOT, by such TCR Account Holder and any Subordinate TCR Account Holders it has designated, are submitted on a timely basis.  Each Subordinate TCR Account Holder will receive a separate invoice.  Netting of the amounts due by Subordinate TCR Account Holders is not allowed.  The amount due on the separate invoices for each Subordinate TCR Account Holder must be submitted by the due date and time due.  If a Subordinate TCR Account Holder does not submit the full amount due by the due date and time due (net remittances by 10 AM on the due date), ERCOT shall deduct the amount due by that Subordinate TCR Account Holder from the TCR Account Holder and/or any other Subordinate TCR Account Holder of that TCR Account Holder to the extent of the amount due and not paid by the late paying Subordinate TCR Account Holder before calculating short payments to other ERCOT Market Participants.

Regardless of reason, if the payment is not received by ERCOT’s financial institution by the due date and time, the payment will be considered to be a “Late Payment”.  An incomplete payment shall be considered a Late Payment.   In the event of a Late Payment, ERCOT will immediately contact a Senior Representative of the TCR Account Holder telephonically and demand payment of the past due amount.

This procedure does not preclude ERCOT from drawing upon a TCR Account Holder’s security any time when a TCR Account Holder’s payment is late.  For the Late Payment in any rolling 12-month period, ERCOT shall review the circumstances and reason for the Late Payment, and shall, in its sole discretion, determine whether it should take action against the TCR Account Holder.  ERCOT will send a notification to a Senior Representative of the Account Holder advising the TCR Account Holder whether or not ERCOT is taking action, and advising the TCR Account Holder of the action.

##### 16.8.1.7.1 Enforcement

ERCOT shall make reasonable efforts to meet with Senior Representative(s) of the TCR Account Holder to discuss the Late Payment occurrences.  ERCOT shall take the following actions:

Terminate the TCR Account Holder’s Agreement with ERCOT, as provided in these Protocols or in the TCR Account Holder’s Agreement.

### 16.8.1.8 Release of Account Holder’s Security Requirement

ERCOT shall, following the termination of a TCR Account Holder Agreement and within thirty (30) days of being satisfied that no sums remain owing by the TCR Account Holder under these Protocols, return or release to the TCR Account Holder, as appropriate, any security provided by the TCR Account Holder under this Section.  The TCR Account Holder or any of its Affiliates will not be allowed to participate in future TCR auctions or ERCOT recognized TCR transfers.

[>>>>>SECTION 6 REVISIONS HAVE NOT BEEN REVIEWED BY CMWG<<<<<]

##### 6.6.3.2.1	Specific Procurement Process Requirements for Replacement Reserve Service in the Adjustment Period

(9)	QSEs whose schedules have impacts on CSCs according to the Commercial Model (using Zonal Average Shift Factors at the time of RPRS procurement for each Zone) at the rate of their impact less their scheduled pre-assigned TCRs shall be charged congestion costs associated with the impact.

- The costs of resolving Local Congestion are based on the amount of capacity required to solve Local Congestion.  This cost will be allocated to all QSEs based on the Load Ratio Share for the relevant period.  This cost will be tracked by specific constraint to aid the determination of the potential addition to the constraint as a CSC.

- If all of the cost of RPRS is not allocated by one of the above methods then the allocation will be uplifted to all QSEs based on the Load Ratio Share for the relevant period. If ERCOT collects more RPRS costs in this manner than is necessary, the excess funds collected by ERCOT will be credited to all QSEs based on the LoadRatio Share for the relevant period.

**************

### 6.7.1	Deployment of Balancing Energy

#### 6.7.1.1	Creation of the Balancing Energy Bid Stack

(7)	ERCOT’s System Operator will not have access to individual bid prices or the expected MCPE if the next energy bid is selected.  Rather, the Operator will deploy all or a portion of a bid, moving up and down the deployment energy stack. All bids will remain in one stack and the MCPE will be posted, unless there is Congestion.  If energy stacks must be separated by Congestion Zone, because of Zonal Congestion, the MCPE of each zone will be posted.

##### 6.9.2.1.2	Replacement Reserve Uplift Charge

- Prior to direct assignment of Zonal Congestion costs,  the QSE’s Obligation for Replacement Reserve procured for CSC Congestion will be recovered as part of the System Congestion Fund described in Section 7.3.3.1, System Congestion Fund.

(2)	The calculation for Replacement Reserve Uplift Charge will be as follows:

UCRPiq 	= 	-1 * (Σ (PCRPiQSE + LPCRPiq) + Σ (USRPiQSE) + Σ (CSCRPiQSE)) * LRSqi

Where:

i:		Interval being calculated.

z:		CSC zone being settled.

UCRPi:		Replacement Reserve Service Uplift Charge ($)

PCRPiQSE:	Service Cost ($) per interval for all QSEs of Procured Capacity of Replacement Reserve procured to resolve Zonal Congestion and Capacity Insufficiency per single Resource

LPCRPiQSE:	Service Cost ($) per interval for all QSEs of Procured Capacity of Replacement Reserve procured to resolve Local Congestion per single Resource

USRPiQSE:	Replacement Reserve Service Under Scheduled Charge ($) per interval for all QSEs

CSCRPiQSE:	Replacement Reserve Service CSC Impact Capacity Charge ($) per interval for all QSEs

LRSqi:	Load Ratio Share (Factor)  (Adjusted Metered Load / Total System Load) per hourly interval of that QSE

#### *****************

- [The following was approved in concept but the language was not voted on by WMS.]

- 7.x.x	Secondary Market Posting and Offer Requirement

- There is no limit to the number or percentage of TCRs that can be held by any TCR Holder at any time.  However, if a TCR Holder and its Affiliate(s) are the owner of record of TCRs in a particular month, subsequent to the close of that corresponding month’s auction, of more than twenty-percent (20%) of any particular CSC for any hour within the month, it shall, at that time, post to the ERCOT TCR bulletin board those TCRs owned in excess of the 20% and offer them for sale in secondary markets at or less than the corresponding monthly clearing price for the TCRs associated with the CSC.

- [The following proposed definitions were not voted on by WMS but are included for commenting.]

## Section 2: Definitions and Acronyms

Boundary Generation Resources

Those Resources, identified prior to ERCOT’s annual TCR auction, on a unit-specific basis, that are related to a combination of CSC and CRE, and whose impacts on the CRE are opposite to those on the related CSC.

Closely Related Elements (CREs)

Those transmission facilities that have shift factor impacts similar to those associated with a particular Commercially Significant Constraint (CSC), and for which there exists a limited amount of Boundary Generation Resources between it and the particular CSC, so that the zonal deployment of Balancing Energy Service is effective in mitigating Zonal Congestion.

Local Congestion

Any Congestion that cannot be resolved by deployment of Balancing Energy Service by Congestion Zone.  Local Congestion will include those actions, and related costs, associated with mitigating overloads on CSCs or CREs beyond that which can be accomplished by the zonal deployment of BES.

Pre-Assigned Congestion Rights (PCRs)

Congestion Rights allocated prior to the annual TCR auctions to MOUs and ECs which own or have a long-term (greater than five years) contractual commitments, entered into prior to September 1, 1999, for annual capacity and energy from a specific remote Generation Resource.

Transmission Congestion Right Holder (TCR Holder)

A Registered Market Participant that is qualified to bid for and own TCRs in ERCOT’s annual or monthly auctions, or that has acquired such rights in secondary markets for purposes of participating in ERCOT’s financial settlements for Congestion Credits.  [A TCR Holder is a Registered Market Participant whose record of ownership appears in ERCOT’s databases of auction results or financial settlements for Zonal Congestion Credits].

Zonal Congestion

Congestion on CSCs or CREs that can be resolved by zonal deployment of Balancing Energy Services and RPRS.

Zonal Congestion Credits

Payments equal to the directly assigned costs of BES and RPRS that were incurred in managing Zonal Congestion.  Zonal Congestion Credits are paid to TCR Holders, appearing in ERCOT’s database, as the owner of record for the hour of relevant Settlement Intervals.

| ## Sponsor |  |
| --- | --- |
| Name | Michael R Davis |
| E-mail Address | michael.r.davis@dynegy.com |
| Company | Dynegy Marketing and Trade |
| Company Address | 1000 Louisiana Street, Houston, Texas 77002 |
| Phone Number | 713-767-4523 |
| Fax Number | 713-767-5986 |
| ## ERCOT Impact Analysis |  |
| Date |  |
| Financial Impact |  |
| Analysis |  |
| ## TAC Action |  |
| TAC Vote Date |  |
| TAC Proposed Implementation Date |  |
| TAC Vote Results |  |
| ## ERCOT Board Action |  |
| ERCOT Board Vote Date |  |
| ERCOT Board Implementation Date |  |
| ERCOT Board Results |  |
| ## Appeal |  |
| Appeal Date |  |
| Entity Initiating Appeal |  |
| Nature of Appeal |  |