| PRR Number | 305PRR | PRR Title | Accounting for Inter-QSE trades under direct assignment congestion cost allocation |
| --- | --- | --- | --- |
| ###### Recommended Action |  | Approve |  |
| System Change |  |  |  |
| Recommended Effective Date |  | March 1, 2002 |  |
| ###### PRR Background |  | ## Clarifies language and formulae in Section 7.3.4 to indicate the recognition of Inter-QSE trades (note: Although intended, these revisions have not yet been verified for accuracy by ERCOT Staff. Thus, some cleanup may be required.) |  |
| Protocol Revision Subcommittee Recommendation |  | PRS recommends approval of PRR305 as modified by PRS. |  |

Recommended Language

### 7.3.4	Settlement of Zonal Congestion

#### 7.3.4.1	Balancing Energy CSC Congestion Charge

The calculation for Zonal Congestion Management Charge is as follows:

ICSCiq	= Σ ((QSSiqz - SOiqz) * SFzcsc )z

If ICSCiq > 0 then,

CSCBECSCiq	=	SPCSCi	* MAX(0,  ICSCiq – PCRCSCiq)

Else (counterflow)

CSCBECSCiq	=	SPCSCi	* ICSCiq

End

CSCBEiq	=	SUM(CSCBECSCiq)CSC

i	Interval being calculated

q	QSE

z	Congestion Zone

CSC	Commercially Significant Constraint

CSCBECSCiq 	CSC Energy Related Congestion Charge, per interval, per CSC, per QSE

CSCBEiq 	CSC Energy Related Congestion Charge, per interval of that QSE

SPCSCi    	Shadow Price per CSC, per interval

ICSCiq    	Actual MW Impact per CSC, per interval, per QSE

PCRCSCiq    	Pre-assigned Congestion Rights per CSC, per interval, per QSE

SFzcsc	Commercial Model Shift Factor per CSC, per zone

QSSiqz	QSE Supply Schedule per interval, per zone, per QSE

SOiqz	Scheduled Obligation (prior to real-time) per interval, per zone, per QSE

The Zonal Shift Factors for settlement purposes will be computed using operational Shift Factors of Generators within the zone weighted by the generation level in the monthly base case.  The monthly base case will be the most recent base case used by ERCOT but no less current than the base case used by ERCOT to determine the number of TCRs available for sale for the corresponding month.

#### 7.3.4.2	Replacement Reserve Service Zonal Congestion Charge

Replacement Reserve Zonal Congestion costs will be allocated directly to the QSE impacting the CSC.  The Replacement Reserve Zonal Congestion costs per interval per QSE will be determined by multiplying the Shadow Price of CSC capacity by the difference of the QSE’s CSC impact and any QSE PCRs in that interval.

The CSC impact is determined by taking for each Congestion Zone, the maximum of either the difference between each QSE’s Supply schedules and the Obligation schedules at the time of each round of Replacement Reserve capacity procurement or the difference of their actual Supply and their actual Obligation within that Congestion Zone multiplied by the Zonal Shift Factor.  The calculation of the Replacement Reserve Service Zonal Congestion Charge will be as follows:

CSCRPhq = SPCCSCh* MAX(0, ICSChq  - MIN(ICSChq , PCRcschq))

ICSChq          =   max [0,MAX[ Σ (MAX(QSSTOPiqz – COTOPiqz) * SFzcsc)z ]n, Σ (ASiqz - AOiqz) * SFzcsc)z ]i

h	hour being calculated

i	Interval being calculated

q	QSE

z	Congestion Zone

CSC	Commercially Significant Constraint

CSCRPiq 	CSC Replacement Reserve related Congestion charge, per interval, per QSE

SPCCSCi    	Capacity Shadow Price per CSC, per interval

ICSCiq    	Actual MW Impact per CSC, per interval, per QSE

PCRCSCiq    	Pre-assigned Congestion Rights per CSC, per interval, per QSE

SFzcsc	Zonal Shift Factor per CSC, per zone

QSSTOPiqz	QSE Supply Schedule per interval, per zone, at the time of purchase, per QSE

COTOPiqz	Current Obligation at the time of purchase per zone per interval per QSE

ALOiqz	Actual Obligation in that Congestion Zone per interval of that QSE, which equals the QSE’s Adjusted Metered Load plus energy exports and energy sales to another QSE.

ASiqz	Actual Supply in that Congestion Zone per interval of that QSE, which equals the QSE’s metered Resource values plus energy imports and energy purchases from another QSE.

| PRR Evaluation |  |
| --- | --- |
| Non-ERCOT Market Comparison | No Comparison conducted. |
| Comments Author | Comments Summary |
|  | None submitted. |