| PRR Number | 311PRR | PRR Title | To transition residential and commercial direct load control programs into BULs |
| --- | --- | --- | --- |
| ###### Recommended Action |  | Approve |  |
| Recommended Effective Date |  | Concurrent with the implementation of DLC. |  |
| System Impact |  | Yes |  |
| PRR Background |  | This change will permit smaller energy consumers (those under 1 MW) an opportunity to participate as a BUL, without the need to install IDRs on the entire population of load control program participants. Requiring IDRs on all program participants would be prohibitively expensive and would create a barrier to entry for direct load control programs involving smaller energy consumers. ## This change also provides a more reasonable baseline for the calculation of the BUL resource provided by smaller energy consumers (those under 1 MW). Load control or load management programs such as air conditioner cycling are temperature sensitive. Calculation of a bas1eline based on historical data, which will be based on different temperature levels, will not accurately represent customer Base Load at the time of operation of the program. Lagged load profiles reflect temperature sensitivity and should be used directly. |  |
| Protocol Revision Subcommittee Recommendation |  | PRS recommends approval of language as a boxed item until the implementation of DLC. |  |

Recommended Language

Add to Section 6.10.5.2 (under PIP112, after the second sentence):

While IDRs must normally be present on all Loads providing BUL Resources, IDRs may be installed and read on a sample of the Loads qualified by a QSE as a BUL Resource, provided the Loads are sampled in accordance with Section 18.7.2.

Add to 6.10.2, PIP112, 4th paragraph, after sentence “Load used to provide BUL resources shall be qualified for correct operation by comparing the average energy  . . . .”

While this qualification shall normally require IDRs to be present on all Loads providing the BUL resource, IDRs may be installed and read on a sample of the Loads qualified by a QSE as a BUL Resource, provided the Loads are sampled in accordance with Section 18.7.2.

Add to Section 6.8.1.14 (under PIP112, after “Given:”):

IF Small Customer Load Management THEN

BULiq 	= 	AIMLLMiq – BMRLMiq

Add to Section 6.8.1.14 (at end of definitions):

AIMLLMiq 	QSE aggregate baseline load for small customers who participate in load management program assuming the program is not in operation from the appropriate load profile.

BMRLMiq 	QSE Aggregate participating customer load during operation of load management program from the appropriate load profile.

Add to 6.10.5.2, PIP112, at the end of 2nd paragraph:

For small customer load management programs, Base Load for each interval is determined from the aggregated appropriate baseline load profile for participating customers assuming the load management program is not in operation.

Add to 6.10.5.2, PIP112, in the 3rd paragraph after the second sentence ending “each interval from the Base Load as determined above.”:

For small customer load management programs during each interval, the expected total reduction of Load shall be determined by subtracting the aggregated participating customer load during operation of the load management program, which is calculated from the appropriate load profile, from the Base Load for these customers as calculated above.

| PRR Evaluation |  |
| --- | --- |
| Non-ERCOT Market Comparison | No Comparison conducted. |
| Comments Author | Comments Summary |
|  | None submitted. |