| PRR Number | 318PRR | PRR Title | Resolving Transmission Busses Allocated to Two Different Congestion Zones |
| --- | --- | --- | --- |
| ###### Date |  | February 22, 2002 |  |
| ## Submitter’s Information |  |  |  |
| Name |  | John Shaw |  |
| E-mail Address |  |  |  |
| Company |  | Entergy Solutions |  |
| Company Address |  |  |  |
| Phone Number |  |  |  |
| Fax Number |  |  |  |

Comments

#### 7.2.1.2	  Congestion Zone & Zonal Shift Factor Determination Methodology

ERCOT will determine Congestion Zones and Zonal Shift Factors by:

- Developing Shift Factors using a linearized (DC) model to identify the impact of each transmission bus on each CSC relative to an ERCOT reference bus.  Substation shift factors will be calculated based upon the average transmission bus shift factor assigned to each substation.

- Using statistical clustering to aggregate substation shift factors into zones based upon similar Shift Factors relative to all CSCs.  To determine the appropriate number of zones, the clustering must meet the following criteria: (i) each CSC must straddle a zonal boundary (however, not every zonal boundary need be straddled by a CSC); (ii) the R2 (coefficient of determination) from the cluster analysis should be greater than a threshold level set by the appropriate subcommittee;  (iii) station IDs as provided by TDSPs in Protocol Section 15.1.2.5 can only be assigned into one Congestion zone; and (iv) the maximum difference between the Zonal Shift Factor and the Shift Factor of any generation bus within the zone should not exceed a limit set by the same subcommittee. The limits will be set to balance the competing goals of minimizing the number of Congestion Zones and maximizing the accuracy of the Commercial Model’s representation of the ERCOT System.

- Creating Zonal Shift Factors for each CSC by averaging the individual substation bus Shift Factors within each Congestion Zone for the corresponding CSC, weighted by the Generation Resource megawatt levels on the bus in the load flow base case used to determine CSC limits.

- Congestion Zone approval process: The appropriate ERCOT Technical Advisory Committee (TAC) Subcommittee will review the process followed above to determine if the Congestion Zones should be recommended for approval to the TAC and the Board.