| PRR Number | 318PRR | PRR Title | Resolving Transmission Busses Allocated to Two Different Congestion Zones |
| --- | --- | --- | --- |
| ###### Status |  | Posted |  |
| Protocol Section Requiring Revision |  | 7.2.1.2 Congestion Zone & Zonal Shift Factor Determination Methodology |  |
| Requested Resolution |  | ## Normal |  |
| Revision Description |  | Add methodology to alleviate the situation of one station having transmission busses assigned to two different transmission congestion zones. |  |
| ###### Reason for Revision |  | Current protocol provides no resolution to the situation where the clustering process results in transmission busses within a single station being assigned to different transmission congestion zones. |  |
| ## Timeline |  |  |  |
| Date Received |  | 02/22/2002 |  |
| Date Posted |  | 02/22/2002 |  |
| Comments Due |  | 03/15/2002 |  |
| PRS Review Date |  | 03/--/2002 |  |
| PRS Recommendation |  |  |  |
| TAC Consideration |  | 05/09/2002 |  |
| Board Consideration |  | 05/21/2002 |  |

Proposed Protocol Language Revision

#### 7.2.1.2	  Congestion Zone & Zonal Shift Factor Determination Methodology

ERCOT will determine Congestion Zones and Zonal Shift Factors by:

- Developing Shift Factors using a linearized (DC) model to identify the impact of each transmission bus on each CSC relative to an ERCOT reference bus.

- Using statistical clustering to aggregate transmission busses into zones based upon similar Shift Factors relative to all CSCs.  To determine the appropriate number of zones, the clustering must meet the following criteria: (i) each CSC must straddle a zonal boundary (however, not every zonal boundary need be straddled by a CSC); (ii) the R2 (coefficient of determination) from the cluster analysis should be greater than a threshold level set by the appropriate subcommittee;  (iii) station IDs as provided by TDSPs in Protocol Section 15.1.2.5 can only be assigned into one Congestion zone; and (iv) the maximum difference between the Zonal Shift Factor and the Shift Factor of any generation bus within the zone should not exceed a limit set by the same subcommittee. The limits will be set to balance the competing goals of minimizing the number of Congestion Zones and maximizing the accuracy of the Commercial Model’s representation of the ERCOT System.

- If the clustering process described in (2) results in the splitting of two or more busses within any single station into different zones, then the following process will be used to adjust the zone selection to place all busses within a single station in the same zone:

- For generating plant stations, determine which zone contains the largest amount of generating capacity of the station and assign all busses of that station to that zone;

- For all other stations, determine which zone is assigned the largest total amount of customer load of the busses in the station and assign all busses of that station to that zone;

- If there is no load or generating capacity in the station, or if the amount of generating capacity and/or load assigned to individual station busses are equal, then the load or generating capacity shall be assigned to the zone containing the lowest bus number of the station.

- Creating Zonal Shift Factors for each CSC by averaging the individual bus Shift Factors within each Congestion Zone for the corresponding CSC, weighted by the Generation Resource megawatt levels on the bus in the load flow base case used to determine CSC limits.

- Congestion Zone approval process: The appropriate ERCOT Technical Advisory Committee (TAC) Subcommittee will review the process followed above to determine if the Congestion Zones should be recommended for approval to the TAC and the Board.

| ## Sponsor |  |
| --- | --- |
| Name | Henry Durrwachter |
| E-mail Address | hdurrwachter@txu.com |
| Company | TXU Energy Trading |
| Company Address | 1717 Main Street, Dallas TX 75201 |
| Phone Number | (214) 875-9285 |
| Fax Number | (214) 875-9246 |
| ## ERCOT Impact Analysis |  |
| Date |  |
| Financial Impact |  |
| Analysis |  |
| ## TAC Action |  |
| TAC Vote Date |  |
| TAC Proposed Implementation Date |  |
| TAC Vote Results |  |
| ## ERCOT Board Action |  |
| ERCOT Board Vote Date |  |
| ERCOT Board Implementation Date |  |
| ERCOT Board Results |  |
| ## Appeal |  |
| Appeal Date |  |
| Entity Initiating Appeal |  |
| Nature of Appeal |  |