| PRR Number | PRR342 | PRR Title | Simultaneous selection of Regulation Up, Responsive Reserve and Non-Spinning Reserve |
| --- | --- | --- | --- |
| ###### Impact Analysis Date |  | November 22nd, 2002 |  |
| ###### Cost/Budgetary Impact |  | Budget approved |  |
| System Change |  | The ERCOT ancillary service market is cleared sequentially before the implementation of the simultaneous selection of Regulation Up, Responsive Reserve and Non-Spinning Reserve. To implement the simultaneous optimization procurement process, the ERCOT market operations system will be impacted in the following areas: - Package I XML API and Portal The XML API and Portal will be modified so that those ancillary services bids that do not follow the bidding rules as specified in this PRR will be disregarded and the corresponding Qualified Scheduling Entity (QSE) will be notified. - Ancillary Service Market Clearing Algorithm The ancillary service market clearing algorithm with the optimization engine needs to be developed, so that the cost can be minimized during the simultaneous procurement process. - Loader (Importer and Exporter) The Loader needs to be developed to transfer the ancillary service bids information from the market database to the Ancillary Service Clearing Engine. - Market Operations Interface (MOI) and market database MOI and market database will be impacted. - Related System Functions The impact on the P1->P2 interface, P2->P1 interface, the existing Scheduling, Pricing and Dispatch (SPD) functions and Power Operations System will be small. |  |
| Implementation Timeline |  | Approximately 9 months |  |
| ERCOT Staffing Impact |  | System design and development, acceptance testing, integration testing and procedure updates, operations support and contracting |  |
| Business Processes Impact |  | During the implementation of PRR342, the following business processes will be impacted: - The MCPC determination mechanism under simultaneous procurement - Market Clearing mechanism with insufficient bids and price determination process under this market condition - Design the market clearing mechanism with Load Participation - Second market clearing mechanism with simultaneous procurement implementation - Work out the procedure to handle price reversal under simultaneous procurement |  |
| Comments |  |  |  |