Texas

Standard

Electronic

Transaction

814_22:

##### Continuous Service Agreement (CSA) CR Move In Request

Electronic Data Interchange

ANSI ASC X12 Ver/Rel 004010

Transaction Set 814

Texas 814_22:

CONTINUOUS SERVICE AGREEMENT (CSA)

CR MOVE IN REQUEST

This transaction set, from ERCOT to CSA CR, is used to start CSA service of the ESI ID.

Document Flow:

- ERCOT to CSA CR

|  |  | # Summary of Changes |
| --- | --- | --- |
| July 3, 2000 Version 1.0 |  | Initial Release |
| August 23, 2000 Version 1.1 |  | The following changes were made: |
|  |  | - Changed Life Support (REF~SU) to remove “Required” and add the following note, “We are not sure of the requirements of this information at this time. When the rules are finalized we will revisit whether the life support indicator will be sent.” |
|  |  | - Corrected Scenarios |
| December 12, 2000 Version 1.2 |  | The following changes were made: |
|  |  | - Change the term Utility to Transmission Distribution Service Provider (TDSP) to match ERCOT Protocols. |
|  |  | - Change the term Retail Electric Provider (REP) to Competitive Retailer (CR) to match ERCOT Protocols. |
|  |  | - Change description page transaction purpose to read the same as ERCOT Protocols. |
|  |  | - Insert the “How to use this Implementation Guide” page. |
|  |  | - Remove “.” between X and 12 in all references to ANSI ASC X12. |
|  |  | - Add BGN08 Element to indicate the TX SET Transaction Number per Change Control #2000-015. |
|  |  | - Add examples to end of transaction. |
|  |  | - Add Time of Use code 71-- Summer Super On-Peak to REF04 of REF~4P and REF~IX Segments in the NM1 loop per Change Control #2000-18. |
| March 9, 2001 Version 1.3 |  | The following changes were made: |
|  |  | - Removed Scenario Names from Transaction Description page |
|  |  | - Corrected the How to Use this Implementation Guide page |
|  |  | - Changed RA/Clearinghouse to ERCOT |
|  |  | - Corrected BGN06 to “Must Use” |
|  |  | - Removed second gray box example on N1~AY (ERCOT) |
|  |  | - Added decimal gray box example to REF~4P Meter Multiplier |
| August 3, 2001 Version 1.4 |  | The following changes were made: |
|  |  | - Added REF~PRT Unmetered Service Type. Ref. 2001-041 & 2001-129 |
|  |  | - Added REF~SPL Unmetered Service Type Station ID. Ref. 2001-041 & 2001-129 |
|  |  | - Added REF~AQ Unmetered Service Type Distribution Loss Factor. Ref. 2001-041 & 2001-129 |
|  |  | - Modify gray box in REF~4P. Ref. 2001-118 |
|  |  | - Modify gray box in REF~LO. Ref. 2001-116 |
|  |  | - Modify examples to show real load profiles and station id’s. Ref. 2001-116 |
|  |  | - Modify gray box on REF01 of REF~LO. Ref. 2001-116 |
|  |  | - Modified example in REF~IX. Changed from K1MON to KHMON |
|  |  | - Fixed example in gray box of REF~MT from “REF~IX~6.0~TU^51” to “REF~IX~6.0~KHMON~TU^51” |
|  |  | - Add new unmetered service types. Ref 2001-047. (Change was previously implemented. Documenting reference to change control in the summary of changes 2/26/02) |
|  |  | Added REDLINES from CHANGE CONTROLS: 2001-206 11/14/01 |
|  |  | 2002-277 – 02/27/02 – Updated consistency on REF~TZ and DTM~313 |

How to Use this Implementation Guide

This section is used to show the X12 Rules for this segment.  You must look further into the grayboxes below for Texas Rules.

Segment:	REF Reference Identification (ESI ID)

Position:	030

Loop:	LIN        Optional

Level:	Detail

Usage:	Optional

Max Use:	>1

Purpose:	To specify identifying information

Syntax Notes:	1	At least one of REF02 or REF03 is required.

2	If either C04003 or C04004 is present, then the other is required.

3	If either C04005 or C04006 is present, then the other is required.

Semantic Notes:	1	REF04 contains data relating to the value cited in REF02.

Comments:

This section is used to show the Texas Rules for implementation of this segment.

| Notes: |  | Required |
| --- | --- | --- |
|  |  | REF~Q5~~10111111234567890ABCDEFGHIJKLMNOPQRS |

Data Element Summary

One or more examples.

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | REF01 | 128 | Reference Identification Qualifier |  |  |  | M |  | ID 2/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code qualifying the Reference Identification |  |  |  |  |  |  |  |  |
|  |  |  |  | Q5 |  | Property Control Number |  |  |  |  |  |
|  |  |  |  |  |  | Electric Service Identifier (ESI ID) |  |  |  |  |  |
| Must Use | REF03 | 352 | Description |  |  |  | X |  | AN 1/80 |  |  |
|  |  |  | A free-form description to clarify the related data elements and their content |  |  |  |  |  |  |  |  |
|  |  |  | ESI ID |  |  |  |  |  |  |  |  |

These are X12 code descriptions, which often do not relate to the Texas descriptions.

X12 cannot keep up with Texas needs, thus, Texas often changes the meaning of existing codes.  See the corresponding graybox for the Texas definitions.

This column shows the X12 attributes for each data element.

M = Mandatory

O= Optional

X = Conditional

AN = Alphanumeric

N# = Implied Decimal at position #

ID = Identification

R = Real

1/30 = Minimum 1, Maximum 30

This column shows the Texas use of each data element.

814 General Request, Response or Confirmation

ANSI ASC X12 Structure

Functional Group ID=GE

Introduction:

This Draft Standard for Trial Use contains the format and establishes the data contents of the General Request, Response or Confirmation Transaction Set (814) for use within the context of an Electronic Data Interchange (EDI) environment. This standard can be used to request actions to be performed, to respond to a request for actions to be performed or to confirm information related to actions performed.

Heading:

Pos.	Seg.		Req.		Loop	Notes and

No.	ID	Name	Des.	Max.Use	Repeat	Comments

| M | 010 | ST | Transaction Set Header | M | 1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| M | 020 | BGN | Beginning Segment | M | 1 |  |  |  |  |  |  |  |  |
|  |  |  | LOOP ID - N1 |  |  | >1 |  |  |  |  |  |  |  |
|  | 040 | N1 | Name | O | 1 |  | n1 |  |  |  |  |  |  |

Detail:

Pos.	Seg.		Req.		Loop	Notes and

No.	ID	Name	Des.	Max.Use	Repeat	Comments

|  |  |  | LOOP ID - LIN |  |  | >1 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 010 | LIN | Item Identification | O | 1 |  |  |  |  |  |  |  |  |
|  | 020 | ASI | Action or Status Indicator | O | 1 |  |  |  |  |  |  |  |  |
|  | 030 | REF | Reference Identification | O | >1 |  |  |  |  |  |  |  |  |
|  | 040 | DTM | Date/Time Reference | O | >1 |  |  |  |  |  |  |  |  |
|  |  |  | LOOP ID - NM1 |  |  | >1 |  |  |  |  |  |  |  |
|  | 080 | NM1 | Individual or Organizational Name | O | 1 |  | n2 |  |  |  |  |  |  |
|  | 130 | REF | Reference Identification | O | >1 |  |  |  |  |  |  |  |  |
|  | 140 | DTM | Date/Time Reference | O | >1 |  |  |  |  |  |  |  |  |
| M | 150 | SE | Transaction Set Trailer | M | 1 |  |  |  |  |  |  |  |  |

Transaction Set Notes

1.	The N1 loop is used to identify the transaction sender and receiver.

2.	The NM1 loop is used to identify the parties associated with the individual line item (LIN), such as an individual consumer in a consolidated third party Consumer Service Provider transaction.

Segment:	ST Transaction Set Header

Position:	010

Loop:

Level:	Heading

Usage:	Mandatory

Max Use:	1

Purpose:	To indicate the start of a transaction set and to assign a control number

Syntax Notes:

Semantic Notes:	1	The transaction set identifier (ST01) is used by the translation routines of the interchange partners to select the appropriate transaction set definition (e.g., 810 selects the Invoice Transaction Set).

Comments:

| Notes: |  | Required |
| --- | --- | --- |
|  |  | ST~814~000000001 |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | ST01 | 143 | Transaction Set Identifier Code |  |  |  | M |  | ID 3/3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code uniquely identifying a Transaction Set |  |  |  |  |  |  |  |
|  |  |  |  | 814 |  | General Request, Response or Confirmation |  |  |  |  |
| Must Use | ST02 | 329 | Transaction Set Control Number |  |  |  | M |  | AN 4/9 |  |
|  |  |  | Identifying control number that must be unique within the transaction set functional group assigned by the originator for a transaction set |  |  |  |  |  |  |  |

Segment:	BGN Beginning Segment

Position:	020

Loop:

Level:	Heading

Usage:	Mandatory

Max Use:	1

Purpose:	To indicate the beginning of a transaction set

Syntax Notes:	1	If BGN05 is present, then BGN04 is required.

Semantic Notes:	1	BGN02 is the transaction set reference number.

2	BGN03 is the transaction set date.

3	BGN04 is the transaction set time.

4	BGN05 is the transaction set time qualifier.

5	BGN06 is the transaction set reference number of a previously sent transaction affected by the current transaction.

Comments:

| Notes: |  | Required |
| --- | --- | --- |
|  |  | BGN~13~200104021200719~20010402~~~200104011956531~~22 |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | BGN01 | 353 | Transaction Set Purpose Code |  |  |  | M |  | ID 2/2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code identifying purpose of transaction set |  |  |  |  |  |  |  |  |
|  |  |  |  | 13 |  | Request |  |  |  |  |  |
| Must Use | BGN02 | 127 | Reference Identification |  |  |  | M |  | AN 1/30 |  |  |
|  |  |  | Reference information as defined for a particular Transaction Set or as specified by the Reference Identification Qualifier |  |  |  |  |  |  |  |  |
|  |  |  | A unique transaction identification number assigned by the originator of this transaction. This number must be unique over time. Transaction Reference numbers will only contain uppercase letters (A to Z) and digits (0 to 9). Note that punctuation (spaces, dashes, etc.) must be excluded. |  |  |  |  |  |  |  |  |
| Must Use | BGN03 | 373 | Date |  |  |  | M |  | DT 8/8 |  |  |
|  |  |  | Date expressed as CCYYMMDD |  |  |  |  |  |  |  |  |
|  |  |  | The transaction creation date - the date that the data was processed by the sender's application system. |  |  |  |  |  |  |  |  |
| Must Use | BGN06 | 127 | Reference Identification |  |  |  | O |  | AN 1/30 |  |  |
|  |  |  | Reference information as defined for a particular Transaction Set or as specified by the Reference Identification Qualifier |  |  |  |  |  |  |  |  |
|  |  |  | Refers to the BGN02 of the Move Out Request (814_24). This number will be tracked in the BGN06 through the lifecycle of the Move Out Process. |  |  |  |  |  |  |  |  |
| Must Use | BGN08 | 306 | Action Code |  |  |  | O |  | ID 1/2 |  |  |
|  |  |  | Code indicating type of action |  |  |  |  |  |  |  |  |
|  |  |  | This segment is used to initially identify the type of 814 that is being sent or received. Ignore the ANSI X12 definition of the code. |  |  |  |  |  |  |  |  |
|  |  |  |  | 22 |  | Dissolve |  |  |  |  |  |
|  |  |  |  |  |  | Indicates Texas SET Transaction 814_22 |  |  |  |  |  |

Segment:	N1 Name (Transmission Distribution Service Provider)

Position:	040

Loop:	N1        Optional

Level:	Heading

Usage:	Optional

Max Use:	1

Purpose:	To identify a party by type of organization, name, and code

Syntax Notes:	1	At least one of N102 or N103 is required.

2	If either N103 or N104 is present, then the other is required.

Semantic Notes:

Comments:	1	This segment, used alone, provides the most efficient method of providing organizational identification. To obtain this efficiency the "ID Code" (N104) must provide a key to the table maintained by the transaction processing party.

2	N105 and N106 further define the type of entity in N101.

| Notes: |  | Required |
| --- | --- | --- |
|  |  | N1~8S~TDSP COMPANY~1~007909411 |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | N101 | 98 | Entity Identifier Code |  |  |  | M |  | ID 2/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code identifying an organizational entity, a physical location, property or an individual |  |  |  |  |  |  |  |  |
|  |  |  |  | 8S |  | Consumer Service Provider (CSP) |  |  |  |  |  |
|  |  |  |  |  |  | Transmission Distribution Service Provider (TDSP) |  |  |  |  |  |
| Must Use | N102 | 93 | Name |  |  |  | X |  | AN 1/60 |  |  |
|  |  |  | Free-form name |  |  |  |  |  |  |  |  |
|  |  |  | TDSP Name |  |  |  |  |  |  |  |  |
| Must Use | N103 | 66 | Identification Code Qualifier |  |  |  | X |  | ID 1/2 |  |  |
|  |  |  | Code designating the system/method of code structure used for Identification Code (67) |  |  |  |  |  |  |  |  |
|  |  |  |  | 1 |  | D-U-N-S Number, Dun & Bradstreet |  |  |  |  |  |
|  |  |  |  | 9 |  | D-U-N-S+4, D-U-N-S Number with Four Character Suffix |  |  |  |  |  |
| Must Use | N104 | 67 | Identification Code |  |  |  | X |  | AN 2/80 |  |  |
|  |  |  | Code identifying a party or other code |  |  |  |  |  |  |  |  |
|  |  |  | TDSP D-U-N-S Number or D-U-N-S + 4 Number |  |  |  |  |  |  |  |  |

Segment:	N1 Name (ERCOT)

Position:	040

Loop:	N1        Optional

Level:	Heading

Usage:	Optional

Max Use:	1

Purpose:	To identify a party by type of organization, name, and code

Syntax Notes:	1	At least one of N102 or N103 is required.

2	If either N103 or N104 is present, then the other is required.

Semantic Notes:

Comments:	1	This segment, used alone, provides the most efficient method of providing organizational identification. To obtain this efficiency the "ID Code" (N104) must provide a key to the table maintained by the transaction processing party.

2	N105 and N106 further define the type of entity in N101.

| Notes: |  | Required |
| --- | --- | --- |
|  |  | N1~AY~ERCOT~1~183529049~~41 (ERCOT to CSA CR) |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | N101 | 98 | Entity Identifier Code |  |  |  | M |  | ID 2/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code identifying an organizational entity, a physical location, property or an individual |  |  |  |  |  |  |  |  |
|  |  |  |  | AY |  | Clearinghouse |  |  |  |  |  |
|  |  |  |  |  |  | ERCOT |  |  |  |  |  |
| Must Use | N102 | 93 | Name |  |  |  | X |  | AN 1/60 |  |  |
|  |  |  | Free-form name |  |  |  |  |  |  |  |  |
|  |  |  | ERCOT Name |  |  |  |  |  |  |  |  |
| Must Use | N103 | 66 | Identification Code Qualifier |  |  |  | X |  | ID 1/2 |  |  |
|  |  |  | Code designating the system/method of code structure used for Identification Code (67) |  |  |  |  |  |  |  |  |
|  |  |  |  | 1 |  | D-U-N-S Number, Dun & Bradstreet |  |  |  |  |  |
|  |  |  |  | 9 |  | D-U-N-S+4, D-U-N-S Number with Four Character Suffix |  |  |  |  |  |
| Must Use | N104 | 67 | Identification Code |  |  |  | X |  | AN 2/80 |  |  |
|  |  |  | Code identifying a party or other code |  |  |  |  |  |  |  |  |
|  |  |  | ERCOT D-U-N-S Number or D-U-N-S + 4 Number |  |  |  |  |  |  |  |  |
| Must Use | N106 | 98 | Entity Identifier Code |  |  |  | O |  | ID 2/3 |  |  |
|  |  |  | Code identifying an organizational entity, a physical location, property or an individual |  |  |  |  |  |  |  |  |
|  |  |  |  | 41 |  | Submitter |  |  |  |  |  |

Segment:	N1 Name (Competitive Retailer)

Position:	040

Loop:	N1        Optional

Level:	Heading

Usage:	Optional

Max Use:	1

Purpose:	To identify a party by type of organization, name, and code

Syntax Notes:	1	At least one of N102 or N103 is required.

2	If either N103 or N104 is present, then the other is required.

Semantic Notes:

Comments:	1	This segment, used alone, provides the most efficient method of providing organizational identification. To obtain this efficiency the "ID Code" (N104) must provide a key to the table maintained by the transaction processing party.

2	N105 and N106 further define the type of entity in N101.

| Notes: |  | Required |
| --- | --- | --- |
|  |  | N1~SJ~CSA CR NAME~1~007909422~~40 |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | N101 | 98 | Entity Identifier Code |  |  |  | M |  | ID 2/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code identifying an organizational entity, a physical location, property or an individual |  |  |  |  |  |  |  |  |
|  |  |  |  | SJ |  | Service Provider |  |  |  |  |  |
|  |  |  |  |  |  | Continuous Service Agreement (CSA) Competitive Retailer (CR) |  |  |  |  |  |
| Must Use | N102 | 93 | Name |  |  |  | X |  | AN 1/60 |  |  |
|  |  |  | Free-form name |  |  |  |  |  |  |  |  |
|  |  |  | CSA CR Name |  |  |  |  |  |  |  |  |
| Must Use | N103 | 66 | Identification Code Qualifier |  |  |  | X |  | ID 1/2 |  |  |
|  |  |  | Code designating the system/method of code structure used for Identification Code (67) |  |  |  |  |  |  |  |  |
|  |  |  |  | 1 |  | D-U-N-S Number, Dun & Bradstreet |  |  |  |  |  |
|  |  |  |  | 9 |  | D-U-N-S+4, D-U-N-S Number with Four Character Suffix |  |  |  |  |  |
| Must Use | N104 | 67 | Identification Code |  |  |  | X |  | AN 2/80 |  |  |
|  |  |  | Code identifying a party or other code |  |  |  |  |  |  |  |  |
|  |  |  | CSA CR D-U-N-S Number or D-U-N-S + 4 Number |  |  |  |  |  |  |  |  |
| Must Use | N106 | 98 | Entity Identifier Code |  |  |  | O |  | ID 2/3 |  |  |
|  |  |  | Code identifying an organizational entity, a physical location, property or an individual |  |  |  |  |  |  |  |  |
|  |  |  |  | 40 |  | Receiver |  |  |  |  |  |

Segment:	LIN Item Identification

Position:	010

Loop:	LIN        Optional

Level:	Detail

Usage:	Optional

Max Use:	1

Purpose:	To specify basic item identification data

Syntax Notes:	1	If either LIN04 or LIN05 is present, then the other is required.

2	If either LIN06 or LIN07 is present, then the other is required.

3	If either LIN08 or LIN09 is present, then the other is required.

4	If either LIN10 or LIN11 is present, then the other is required.

5	If either LIN12 or LIN13 is present, then the other is required.

6	If either LIN14 or LIN15 is present, then the other is required.

7	If either LIN16 or LIN17 is present, then the other is required.

8	If either LIN18 or LIN19 is present, then the other is required.

9	If either LIN20 or LIN21 is present, then the other is required.

10	If either LIN22 or LIN23 is present, then the other is required.

11	If either LIN24 or LIN25 is present, then the other is required.

12	If either LIN26 or LIN27 is present, then the other is required.

13	If either LIN28 or LIN29 is present, then the other is required.

14	If either LIN30 or LIN31 is present, then the other is required.

Semantic Notes:	1	LIN01 is the line item identification

Comments:	1	See the Data Dictionary for a complete list of IDs.

2	LIN02 through LIN31 provide for fifteen different product/service IDs for each item. For example: Case, Color, Drawing No., U.P.C. No., ISBN No., Model No., or SKU.

| Notes: |  | Required |
| --- | --- | --- |
|  |  | LIN~1~SH~EL~SH~CE~SH~MVI (Move In for CSA CR) |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | LIN01 | 350 | Assigned Identification |  |  |  | O |  | AN 1/20 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Alphanumeric characters assigned for differentiation within a transaction set |  |  |  |  |  |  |  |  |
| Must Use | LIN02 | 235 | Product/Service ID Qualifier |  |  |  | M |  | ID 2/2 |  |  |
|  |  |  | Code identifying the type/source of the descriptive number used in Product/Service ID (234) |  |  |  |  |  |  |  |  |
|  |  |  |  | SH |  | Service Requested |  |  |  |  |  |
| Must Use | LIN03 | 234 | Product/Service ID |  |  |  | M |  | AN 1/48 |  |  |
|  |  |  | Identifying number for a product or service |  |  |  |  |  |  |  |  |
|  |  |  | Commodity |  |  |  |  |  |  |  |  |
|  |  |  |  | EL |  | Electric Service |  |  |  |  |  |
| Must Use | LIN04 | 235 | Product/Service ID Qualifier |  |  |  | X |  | ID 2/2 |  |  |
|  |  |  | Code identifying the type/source of the descriptive number used in Product/Service ID (234) |  |  |  |  |  |  |  |  |
|  |  |  |  | SH |  | Service Requested |  |  |  |  |  |
| Must Use | LIN05 | 234 | Product/Service ID |  |  |  | X |  | AN 1/48 |  |  |
|  |  |  | Identifying number for a product or service |  |  |  |  |  |  |  |  |
|  |  |  |  | CE |  | Energy Services |  |  |  |  |  |
| Must Use | LIN06 | 235 | Product/Service ID Qualifier |  |  |  | X |  | ID 2/2 |  |  |
|  |  |  | Code identifying the type/source of the descriptive number used in Product/Service ID (234) |  |  |  |  |  |  |  |  |
|  |  |  |  | SH |  | Service Requested |  |  |  |  |  |
| Must Use | LIN07 | 234 | Product/Service ID |  |  |  | X |  | AN 1/48 |  |  |
|  |  |  | Identifying number for a product or service |  |  |  |  |  |  |  |  |
|  |  |  |  | MVI |  | Move In |  |  |  |  |  |
|  |  |  |  |  |  | Request for a customer enrollment when a new customer moves into a premise. In this case the CSA is "moving in" to the premise. |  |  |  |  |  |

Segment:	ASI Action or Status Indicator

Position:	020

Loop:	LIN        Optional

Level:	Detail

Usage:	Optional

Max Use:	1

Purpose:	To indicate the action to be taken with the information provided or the status of the entity described

Syntax Notes:

Semantic Notes:

Comments:

| Notes: |  | Identifies the action to be taken or the status of a requested action for the service identified in the LIN segment. |
| --- | --- | --- |
|  |  | Required |
|  |  | ASI~7~021 |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | ASI01 | 306 | Action Code |  |  |  | M |  | ID 1/2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code indicating type of action |  |  |  |  |  |  |  |
|  |  |  |  | 7 |  | Request |  |  |  |  |
| Must Use | ASI02 | 875 | Maintenance Type Code |  |  |  | M |  | ID 3/3 |  |
|  |  |  | Code identifying the specific type of item maintenance |  |  |  |  |  |  |  |
|  |  |  |  | 021 |  | Addition |  |  |  |  |

Segment:	REF Reference Identification (Distribution Loss Factor Code)

Position:	030

Loop:	LIN        Optional

Level:	Detail

Usage:	Optional

Max Use:	>1

Purpose:	To specify identifying information

Syntax Notes:	1	At least one of REF02 or REF03 is required.

2	If either C04003 or C04004 is present, then the other is required.

3	If either C04005 or C04006 is present, then the other is required.

Semantic Notes:	1	REF04 contains data relating to the value cited in REF02.

Comments:

| Notes: |  | The Distribution Loss Factor Code is used to identify the related loss factor value that is used in settlement. The code must be the contained in a loss factor table that the TDSP has provided to ERCOT. It is usually related to the delivery voltage or rate class. It is not the same as the transformer loss factor used when the TDSP meters service at a different point than it delivers service. |
| --- | --- | --- |
|  |  | Accept Response: Required Reject Response: Not Used |
|  |  | REF~AQ~T |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | REF01 | 128 | Reference Identification Qualifier |  |  |  | M |  | ID 2/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code qualifying the Reference Identification |  |  |  |  |  |  |  |  |
|  |  |  |  | AQ |  | Access Code |  |  |  |  |  |
|  |  |  |  |  |  | Distribution Loss Factor Code |  |  |  |  |  |
| Must Use | REF02 | 127 | Reference Identification |  |  |  | X |  | AN 1/30 |  |  |
|  |  |  | Reference information as defined for a particular Transaction Set or as specified by the Reference Identification Qualifier |  |  |  |  |  |  |  |  |
|  |  |  | With the exception of code "T", each Distribution Company will post what each of the next 5 codes represent and how they are applied. |  |  |  |  |  |  |  |  |
|  |  |  |  | A |  |  |  |  |  |  |  |
|  |  |  |  | B |  |  |  |  |  |  |  |
|  |  |  |  | C |  |  |  |  |  |  |  |
|  |  |  |  | D |  |  |  |  |  |  |  |
|  |  |  |  | E |  |  |  |  |  |  |  |
|  |  |  |  | T |  | Transmission Only |  |  |  |  |  |

Segment:	REF Reference Identification (ESI ID)

Position:	030

Loop:	LIN        Optional

Level:	Detail

Usage:	Optional

Max Use:	>1

Purpose:	To specify identifying information

Syntax Notes:	1	At least one of REF02 or REF03 is required.

2	If either C04003 or C04004 is present, then the other is required.

3	If either C04005 or C04006 is present, then the other is required.

Semantic Notes:	1	REF04 contains data relating to the value cited in REF02.

Comments:

| Notes: |  | Required |
| --- | --- | --- |
|  |  | REF~Q5~~10111111234567890ABCDEFGHIJKLMNOPQRS |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | REF01 | 128 | Reference Identification Qualifier |  |  |  | M |  | ID 2/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code qualifying the Reference Identification |  |  |  |  |  |  |  |  |
|  |  |  |  | Q5 |  | Property Control Number |  |  |  |  |  |
|  |  |  |  |  |  | Electric Service Identifier (ESI ID) |  |  |  |  |  |
| Must Use | REF03 | 352 | Description |  |  |  | X |  | AN 1/80 |  |  |
|  |  |  | A free-form description to clarify the related data elements and their content |  |  |  |  |  |  |  |  |
|  |  |  | ESI ID |  |  |  |  |  |  |  |  |

Segment:	REF Reference Identification (Station ID)

Position:	030

Loop:	LIN        Optional

Level:	Detail

Usage:	Optional

Max Use:	>1

Purpose:	To specify identifying information

Syntax Notes:	1	At least one of REF02 or REF03 is required.

2	If either C04003 or C04004 is present, then the other is required.

3	If either C04005 or C04006 is present, then the other is required.

Semantic Notes:	1	REF04 contains data relating to the value cited in REF02.

Comments:

| Notes: |  | The value shall correspond to an ERCOT-defined Station ID that is published on the ERCOT website. |
| --- | --- | --- |
|  |  | Accept Response: Required Reject Response: Not Used |
|  |  | REF~SPL~~WEST |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | REF01 | 128 | Reference Identification Qualifier |  |  |  | M |  | ID 2/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code qualifying the Reference Identification |  |  |  |  |  |  |  |  |
|  |  |  |  | SPL |  | Standard Point Location Code (SPLC) |  |  |  |  |  |
|  |  |  |  |  |  | Substation at which the TDSP's distribution system is connected to the transmission grid for the Service Delivery Point (SDP). The value shall correspond to an ERCOT-defined Station ID that is published on the ERCOT website. |  |  |  |  |  |
| Must Use | REF03 | 352 | Description |  |  |  | X |  | AN 1/80 |  |  |
|  |  |  | A free-form description to clarify the related data elements and their content |  |  |  |  |  |  |  |  |
|  |  |  | Station ID |  |  |  |  |  |  |  |  |

Segment:	REF Reference Identification (Life Support Indicator)

Position:	030

Loop:	LIN        Optional

Level:	Detail

Usage:	Optional

Max Use:	>1

Purpose:	To specify identifying information

Syntax Notes:	1	At least one of REF02 or REF03 is required.

2	If either C04003 or C04004 is present, then the other is required.

3	If either C04005 or C04006 is present, then the other is required.

Semantic Notes:	1	REF04 contains data relating to the value cited in REF02.

Comments:

| Notes: |  | We are not sure of the requirements of this information at this time. When the rules are finalized we will revisit whether the life support indicator will be sent. |
| --- | --- | --- |
|  |  | REF~SU~Y |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | REF01 | 128 | Reference Identification Qualifier |  |  |  | M |  | ID 2/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code qualifying the Reference Identification |  |  |  |  |  |  |  |  |
|  |  |  |  | SU |  | Special Processing Code |  |  |  |  |  |
|  |  |  |  |  |  | Life Support Indicator |  |  |  |  |  |
| Must Use | REF02 | 127 | Reference Identification |  |  |  | X |  | AN 1/30 |  |  |
|  |  |  | Reference information as defined for a particular Transaction Set or as specified by the Reference Identification Qualifier |  |  |  |  |  |  |  |  |
|  |  |  |  | I |  | Investigating |  |  |  |  |  |
|  |  |  |  |  |  | Investigating whether life support is required |  |  |  |  |  |
|  |  |  |  | N |  | No |  |  |  |  |  |
|  |  |  |  |  |  | Life support is not required |  |  |  |  |  |
|  |  |  |  | Y |  | Yes |  |  |  |  |  |
|  |  |  |  |  |  | Customer is on Life Support |  |  |  |  |  |

Segment:	DTM Date/Time Reference (Service Period Start)

Position:	040

Loop:	LIN        Optional

Level:	Detail

Usage:	Optional

Max Use:	>1

Purpose:	To specify pertinent dates and times

Syntax Notes:	1	At least one of DTM02 DTM03 or DTM05 is required.

2	If DTM04 is present, then DTM03 is required.

3	If either DTM05 or DTM06 is present, then the other is required.

Semantic Notes:

Comments:

| Notes: |  | Required |
| --- | --- | --- |
|  |  | DTM~150~20010415 |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | DTM01 | 374 | Date/Time Qualifier |  |  |  | M |  | ID 3/3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code specifying type of date or time, or both date and time |  |  |  |  |  |  |  |
|  |  |  |  | 150 |  | Service Period Start |  |  |  |  |
| Must Use | DTM02 | 373 | Date |  |  |  | X |  | DT 8/8 |  |
|  |  |  | Date expressed as CCYYMMDD |  |  |  |  |  |  |  |

Segment:	NM1 Individual or Organizational Name (Meter Level Information)

Position:	080

Loop:	NM1        Optional

Level:	Detail

Usage:	Optional

Max Use:	1

Purpose:	To supply the full name of an individual or organizational entity

Syntax Notes:	1	If either NM108 or NM109 is present, then the other is required.

2	If NM111 is present, then NM110 is required.

Semantic Notes:	1	NM102 qualifies NM103.

Comments:	1	NM110 and NM111 further define the type of entity in NM101.

| Notes: |  | Required |
| --- | --- | --- |
|  |  | NM1~MQ~3~~~~~~32~GE10349811 NM1~MQ~3~~~~~~93~UNMETERED |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | NM101 | 98 | Entity Identifier Code |  |  |  | M |  | ID 2/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code identifying an organizational entity, a physical location, property or an individual |  |  |  |  |  |  |  |  |
|  |  |  |  | MQ |  | Metering Location |  |  |  |  |  |
| Must Use | NM102 | 1065 | Entity Type Qualifier |  |  |  | M |  | ID 1/1 |  |  |
|  |  |  | Code qualifying the type of entity |  |  |  |  |  |  |  |  |
|  |  |  |  | 3 |  | Unknown |  |  |  |  |  |
| Must Use | NM108 | 66 | Identification Code Qualifier |  |  |  | X |  | ID 1/2 |  |  |
|  |  |  | Code designating the system/method of code structure used for Identification Code (67) |  |  |  |  |  |  |  |  |
|  |  |  |  | 32 |  | Assigned by Property Operator |  |  |  |  |  |
|  |  |  |  |  |  | Meter Number |  |  |  |  |  |
|  |  |  |  | 93 |  | Code assigned by the organization originating the transaction set |  |  |  |  |  |
|  |  |  |  |  |  | This code used when a Meter Number is NOT provided in NM109 (i.e., when NM109 contains "UNMETERED"). |  |  |  |  |  |
| Must Use | NM109 | 67 | Identification Code |  |  |  | X |  | AN 2/80 |  |  |
|  |  |  | Code identifying a party or other code |  |  |  |  |  |  |  |  |
|  |  |  | Meter Number or, UNMETERED - for Unmetered Services Meter numbers will only contain uppercase letters (A to Z) and digits (0 to 9). Note that punctuation (spaces, dashes, etc.) must be excluded, and leading and trailing zeros that are part of the meter number must be present. |  |  |  |  |  |  |  |  |

Segment:	REF Reference Identification (Meter Multiplier)

Position:	130

Loop:	NM1        Optional

Level:	Detail

Usage:	Optional

Max Use:	>1

Purpose:	To specify identifying information

Syntax Notes:	1	At least one of REF02 or REF03 is required.

2	If either C04003 or C04004 is present, then the other is required.

3	If either C04005 or C04006 is present, then the other is required.

Semantic Notes:	1	REF04 contains data relating to the value cited in REF02.

Comments:

| Notes: |  | There will be one REF~4P segment for each Meter Type, creating the potential for more than one REF~4P segment per NM1 Loop. For instance, if the meter has multiple meter types, and therefore the REF~MT segment is REF~MT~COMBO, there will be multiple REF~4P segments, one to indicate the meter multiplier for each type. For example, NM1~MQ~3~~~~~~32~1234568MG REF~MT~COMBO REF~4P~1200~K1015~TU^41 REF~4P~1~KH015~TU^41 Note: Format of the multiplier could be in the form of a fraction (e.g. 48.78432) Also note that since the Meter Multiplier is required for each Time of Use, this segment is also used to determine which Time of Use will be sent on the 867 transaction. |
| --- | --- | --- |
|  |  | Required for each meter type for each meter that is used for billing purposes. This segment will not be provided in the NM1 loop for UNMETERED services. |
|  |  | REF~4P~10~KHMON~TU^41 REF~4P~1~K1MON~TU^41 REF~4P~1.5~K1MON~TU^41 |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | REF01 | 128 | Reference Identification Qualifier |  |  |  | M |  | ID 2/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code qualifying the Reference Identification |  |  |  |  |  |  |  |  |
|  |  |  |  | 4P |  | Affiliation Number |  |  |  |  |  |
|  |  |  |  |  |  | Meter Multiplier (Ending Reading - Beginning Reading) * Meter Multiplier = Billed Usage |  |  |  |  |  |
| Must Use | REF02 | 127 | Reference Identification |  |  |  | X |  | AN 1/30 |  |  |
|  |  |  | Reference information as defined for a particular Transaction Set or as specified by the Reference Identification Qualifier |  |  |  |  |  |  |  |  |
|  |  |  | Meter Multiplier |  |  |  |  |  |  |  |  |
| Must Use | REF03 | 352 | Description |  |  |  | X |  | AN 1/80 |  |  |
|  |  |  | A free-form description to clarify the related data elements and their content |  |  |  |  |  |  |  |  |
|  |  |  | Meter Type (see REF~MT for valid codes). "COMBO" is not a valid code for this element. |  |  |  |  |  |  |  |  |
| Must Use | REF04 | C040 | Reference Identifier |  |  |  | O |  |  |  |  |
|  |  |  | To identify one or more reference numbers or identification numbers as specified by the Reference Qualifier |  |  |  |  |  |  |  |  |
|  |  |  | Note this is a composite data element. Populate C04001 & C04002. |  |  |  |  |  |  |  |  |
| Must Use | C04001 | 128 | Reference Identification Qualifier |  |  |  | M |  | ID 2/3 |  |  |
|  |  |  | Code qualifying the Reference Identification |  |  |  |  |  |  |  |  |
|  |  |  |  | TU |  | Trial Location Code |  |  |  |  |  |
|  |  |  |  |  |  | Time of Use |  |  |  |  |  |
| Must Use | C04002 | 127 | Reference Identification |  |  |  | M |  | AN 1/30 |  |  |
|  |  |  | Reference information as defined for a particular Transaction Set or as specified by the Reference Identification Qualifier |  |  |  |  |  |  |  |  |
|  |  |  |  | 41 |  | Off Peak |  |  |  |  |  |
|  |  |  |  | 42 |  | On Peak |  |  |  |  |  |
|  |  |  |  | 43 |  | Intermediate Peak |  |  |  |  |  |
|  |  |  |  | 51 |  | Totalizer |  |  |  |  |  |
|  |  |  |  |  |  | Totalizer/Total/Max (Demand) |  |  |  |  |  |
|  |  |  |  | 71 |  | Summer Super On Peak |  |  |  |  |  |

Segment:	REF Reference Identification (Number of Dials)

Position:	130

Loop:	NM1        Optional

Level:	Detail

Usage:	Optional

Max Use:	>1

Purpose:	To specify identifying information

Syntax Notes:	1	At least one of REF02 or REF03 is required.

2	If either C04003 or C04004 is present, then the other is required.

3	If either C04005 or C04006 is present, then the other is required.

Semantic Notes:	1	REF04 contains data relating to the value cited in REF02.

Comments:

| Notes: |  | There will be one REF~IX segment for each applicable Meter Type, creating the potential for more than one REF~IX segment per NM1 Loop. For instance, if the meter has multiple types, and therefore the REF~MT segment is REF~MT~COMBO, there may be multiple REF~IX segments, one to indicate the number of dials for each type that has dials. For example, NM1~MQ~3~~~~~~32~1234568MG REF~MT~COMBO REF~IX~6.0~KHMON~TU^51 REF~IX~5.1~KHMON~TU^51 |
| --- | --- | --- |
|  |  | Required for each meter type for each meter that is used for billing purposes that has dials. This segment will not be provided in the NM1 loop for UNMETERED services or for demand. |
|  |  | REF~IX~6.0~KHMON~TU^51 |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | REF01 | 128 | Reference Identification Qualifier |  |  |  | M |  | ID 2/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code qualifying the Reference Identification |  |  |  |  |  |  |  |  |
|  |  |  |  | IX |  | Item Number |  |  |  |  |  |
|  |  |  |  |  |  | Number of Dials on the Meter displayed as X.Y. The notation X.Y means that the meter has X dials to the left of the decimal point, and Y dials to the right. |  |  |  |  |  |
| Must Use | REF02 | 127 | Reference Identification |  |  |  | X |  | AN 1/30 |  |  |
|  |  |  | Reference information as defined for a particular Transaction Set or as specified by the Reference Identification Qualifier |  |  |  |  |  |  |  |  |
|  |  |  | Number of Dials |  |  |  |  |  |  |  |  |
| Must Use | REF03 | 352 | Description |  |  |  | X |  | AN 1/80 |  |  |
|  |  |  | A free-form description to clarify the related data elements and their content |  |  |  |  |  |  |  |  |
|  |  |  | Meter Type (see REF~MT for valid codes). "COMBO" is not a valid code for this element. |  |  |  |  |  |  |  |  |
| Must Use | REF04 | C040 | Reference Identifier |  |  |  | O |  |  |  |  |
|  |  |  | To identify one or more reference numbers or identification numbers as specified by the Reference Qualifier |  |  |  |  |  |  |  |  |
|  |  |  | Note that this is a composite data element. Populate C04001 and C04002. |  |  |  |  |  |  |  |  |
| Must Use | C04001 | 128 | Reference Identification Qualifier |  |  |  | M |  | ID 2/3 |  |  |
|  |  |  | Code qualifying the Reference Identification |  |  |  |  |  |  |  |  |
|  |  |  |  | TU |  | Trial Location Code |  |  |  |  |  |
|  |  |  |  |  |  | Time of Use |  |  |  |  |  |
| Must Use | C04002 | 127 | Reference Identification |  |  |  | M |  | AN 1/30 |  |  |
|  |  |  | Reference information as defined for a particular Transaction Set or as specified by the Reference Identification Qualifier |  |  |  |  |  |  |  |  |
|  |  |  |  | 41 |  | Off Peak |  |  |  |  |  |
|  |  |  |  | 42 |  | On Peak |  |  |  |  |  |
|  |  |  |  | 43 |  | Intermediate Peak |  |  |  |  |  |
|  |  |  |  | 51 |  | Totalizer |  |  |  |  |  |
|  |  |  |  |  |  | Totalizer/Total/Max (Demand) |  |  |  |  |  |
|  |  |  |  | 71 |  | Summer Super On Peak |  |  |  |  |  |

Segment:	REF Reference Identification (Load Profile)

Position:	130

Loop:	NM1        Optional

Level:	Detail

Usage:	Optional

Max Use:	>1

Purpose:	To specify identifying information

Syntax Notes:	1	At least one of REF02 or REF03 is required.

2	If either C04003 or C04004 is present, then the other is required.

3	If either C04005 or C04006 is present, then the other is required.

Semantic Notes:	1	REF04 contains data relating to the value cited in REF02.

Comments:

| Notes: |  | The load profile is made up of the following codes, with mandatory underscores between each: Profile Group and Segment Code, Weather Zone Code, Meter Data Type Code, Weather Sensitivity Code, TOU Schedule Code Required |
| --- | --- | --- |
|  |  | REF~LO~RESLOWR_WEST_NIDR_NWS_NOTOU |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | REF01 | 128 | Reference Identification Qualifier |  |  |  | M |  | ID 2/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code qualifying the Reference Identification |  |  |  |  |  |  |  |  |
|  |  |  |  | LO |  | Load Planning Number |  |  |  |  |  |
|  |  |  |  |  |  | Load profile |  |  |  |  |  |
| Must Use | REF02 | 127 | Reference Identification |  |  |  | X |  | AN 1/30 |  |  |
|  |  |  | Reference information as defined for a particular Transaction Set or as specified by the Reference Identification Qualifier |  |  |  |  |  |  |  |  |
|  |  |  | Load Profile |  |  |  |  |  |  |  |  |

Segment:	REF Reference Identification (Meter Type)

Position:	130

Loop:	NM1        Optional

Level:	Detail

Usage:	Optional

Max Use:	>1

Purpose:	To specify identifying information

Syntax Notes:	1	At least one of REF02 or REF03 is required.

2	If either C04003 or C04004 is present, then the other is required.

3	If either C04005 or C04006 is present, then the other is required.

Semantic Notes:	1	REF04 contains data relating to the value cited in REF02.

Comments:

| Notes: |  | There will only be one REF~MT segment for each NM1 loop. If there are multiple meter types on this meter, only one REF~MT will be sent. It will contain the code COMBO. The specific meter types will be identified in REF03 for the attribute segments contained within this loop. For example, REF~IX~6.0~KHMON~TU^51. |
| --- | --- | --- |
|  |  | Required for each meter that is used for billing purposes. This segment will not be provided in the NM1 loop for UNMETERED services. |
|  |  | REF~MT~KHMON REF~MT~COMBO |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | REF01 | 128 | Reference Identification Qualifier |  |  |  | M |  | ID 2/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code qualifying the Reference Identification |  |  |  |  |  |  |  |  |
|  |  |  |  | MT |  | Meter Ticket Number |  |  |  |  |  |
|  |  |  |  |  |  | Meter Type |  |  |  |  |  |
| Must Use | REF02 | 127 | Reference Identification |  |  |  | X |  | AN 1/30 |  |  |
|  |  |  | Reference information as defined for a particular Transaction Set or as specified by the Reference Identification Qualifier |  |  |  |  |  |  |  |  |
|  |  |  | When REF01 is MT, the meter type is expressed as a five-character field. The first two characters are the type of consumption, the last three characters are the metering interval reported for billing purposes. "COMBO" is used for a meter that records more than one measurement. Valid values can be a combination of the following values: Type of Consumption K1 Kilowatt Demand (kW) K2 Kilovolt Amperes Reactive Demand (kVAR) K3 Kilovolt Amperes Reactive Hour (kVARH) K4 Kilovolt Amperes (kVA) KH Kilowatt Hour (kWh) Metering Interval Reported for Billing Purposes nnn Number of minutes from 001 to 999 DAY Daily MON Monthly QTR Quarterly For Example: KHMON Kilowatt Hours Per Month K1015 Kilowatt Demand per 15 minute interval Other Valid Codes COMBO This code is used to indicate that the meter has multiple measurements, e.g., one meter that measures both kWh and Demand. |  |  |  |  |  |  |  |  |

Segment:	REF Reference Identification (TDSP Rate Class)

Position:	130

Loop:	NM1        Optional

Level:	Detail

Usage:	Optional

Max Use:	>1

Purpose:	To specify identifying information

Syntax Notes:	1	At least one of REF02 or REF03 is required.

2	If either C04003 or C04004 is present, then the other is required.

3	If either C04005 or C04006 is present, then the other is required.

Semantic Notes:	1	REF04 contains data relating to the value cited in REF02.

Comments:

| Notes: |  | Required for each meter that is used for billing purposes. This segment must also be sent when account has UNMETERED services available. |
| --- | --- | --- |
|  |  | REF~NH~RS1 |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | REF01 | 128 | Reference Identification Qualifier |  |  |  | M |  | ID 2/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code qualifying the Reference Identification |  |  |  |  |  |  |  |  |
|  |  |  |  | NH |  | Rate Card Number |  |  |  |  |  |
|  |  |  |  |  |  | TDSP Rate Class or Tariff |  |  |  |  |  |
| Must Use | REF02 | 127 | Reference Identification |  |  |  | X |  | AN 1/30 |  |  |
|  |  |  | Reference information as defined for a particular Transaction Set or as specified by the Reference Identification Qualifier |  |  |  |  |  |  |  |  |
|  |  |  | TDSP Rate Class or Tariff |  |  |  |  |  |  |  |  |

Segment:	REF Reference Identification (TDSP Rate Subclass)

Position:	130

Loop:	NM1        Optional

Level:	Detail

Usage:	Optional

Max Use:	>1

Purpose:	To specify identifying information

Syntax Notes:	1	At least one of REF02 or REF03 is required.

2	If either C04003 or C04004 is present, then the other is required.

3	If either C04005 or C04006 is present, then the other is required.

Semantic Notes:	1	REF04 contains data relating to the value cited in REF02.

Comments:

| Notes: |  | Required if maintained by TDSP, must be sent for each meter that is used for billing purposes. This segment must also be sent when account has UNMETERED services available. |
| --- | --- | --- |
|  |  | REF~PR~123 |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | REF01 | 128 | Reference Identification Qualifier |  |  |  | M |  | ID 2/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code qualifying the Reference Identification |  |  |  |  |  |  |  |  |
|  |  |  |  | PR |  | Price Quote Number |  |  |  |  |  |
|  |  |  |  |  |  | TDSP Rate Subclass - Used to provide further classification of a rate. |  |  |  |  |  |
| Must Use | REF02 | 127 | Reference Identification |  |  |  | X |  | AN 1/30 |  |  |
|  |  |  | Reference information as defined for a particular Transaction Set or as specified by the Reference Identification Qualifier |  |  |  |  |  |  |  |  |
|  |  |  | TDSP Rate Subclass |  |  |  |  |  |  |  |  |

Segment:	REF Reference Identification (Unmetered Service Type)

Position:	130

Loop:	NM1        Optional

Level:	Detail

Usage:	Optional

Max Use:	>1

Purpose:	To specify identifying information

Syntax Notes:	1	At least one of REF02 or REF03 is required.

2	If either C04003 or C04004 is present, then the other is required.

3	If either C04005 or C04006 is present, then the other is required.

Semantic Notes:	1	REF04 contains data relating to the value cited in REF02.

Comments:

| Notes: |  | There will be one REF~PRT segment for each applicable Unmetered Service Meter Type, creating the potential for more than one REF~PRT segment per NM1 Loop. |
| --- | --- | --- |
|  |  | Accept Response: Required for each unmetered service type used for billing purposes. Reject Response: Not Used |
|  |  | REF~PRT~MV~750~QQ^100 |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | REF01 | 128 | Reference Identification Qualifier |  |  |  | M |  | ID 2/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code qualifying the Reference Identification |  |  |  |  |  |  |  |  |
|  |  |  |  | PRT |  | Product Type |  |  |  |  |  |
|  |  |  |  |  |  | Defined Unmetered Service Type |  |  |  |  |  |
| Must Use | REF02 | 127 | Reference Identification |  |  |  | X |  | AN 1/30 |  |  |
|  |  |  | Reference information as defined for a particular Transaction Set or as specified by the Reference Identification Qualifier |  |  |  |  |  |  |  |  |
|  |  |  | If a new code is needed, it must be requested via the Change Control process. |  |  |  |  |  |  |  |  |
|  |  |  |  | AN |  | Antenna |  |  |  |  |  |
|  |  |  |  | AR |  | Argon |  |  |  |  |  |
|  |  |  |  | BB |  | Bill Boards |  |  |  |  |  |
|  |  |  |  | BS |  | Bus Shelters |  |  |  |  |  |
|  |  |  |  | CU |  | Cat Unit |  |  |  |  |  |
|  |  |  |  | ED |  | Electronic Devise |  |  |  |  |  |
|  |  |  |  | FL |  | Fluorescent |  |  |  |  |  |
|  |  |  |  | HA |  | Historical/Antique |  |  |  |  |  |
|  |  |  |  | IN |  | Incandescent |  |  |  |  |  |
|  |  |  |  | LV |  | Levys |  |  |  |  |  |
|  |  |  |  | MH |  | Metal Halide |  |  |  |  |  |
|  |  |  |  | MV |  | Mercury Vapor |  |  |  |  |  |
|  |  |  |  | OT |  | Other Unmetered |  |  |  |  |  |
|  |  |  |  | PA |  | Power Analog node |  |  |  |  |  |
|  |  |  |  | PB |  | Phone Booth |  |  |  |  |  |
|  |  |  |  | PO |  | Phone Outlet |  |  |  |  |  |
|  |  |  |  | PS |  | Pump Station |  |  |  |  |  |
|  |  |  |  | RR |  | Rail Road Crossings |  |  |  |  |  |
|  |  |  |  | SD |  | Sodium |  |  |  |  |  |
|  |  |  |  | TL |  | Traffic Lights |  |  |  |  |  |
|  |  |  |  | TR |  | Transceiver |  |  |  |  |  |
|  |  |  |  | WM |  | Wallpacked Mercury Vapor |  |  |  |  |  |
|  |  |  |  | WS |  | Warning Sirens |  |  |  |  |  |
| Dep | REF03 | 352 | Description |  |  |  | X |  | AN 1/80 |  |  |
|  |  |  | A free-form description to clarify the related data elements and their content |  |  |  |  |  |  |  |  |
|  |  |  | Wattage/Lumens when applicable |  |  |  |  |  |  |  |  |
| Must Use | REF04 | C040 | Reference Identifier |  |  |  | O |  |  |  |  |
|  |  |  | To identify one or more reference numbers or identification numbers as specified by the Reference Qualifier |  |  |  |  |  |  |  |  |
|  |  |  | Note that this is a composite data element. Populate C04001 and C04002. |  |  |  |  |  |  |  |  |
| Must Use | C04001 | 128 | Reference Identification Qualifier |  |  |  | M |  | ID 2/3 |  |  |
|  |  |  | Code qualifying the Reference Identification |  |  |  |  |  |  |  |  |
|  |  |  |  | QQ |  | Unit Number |  |  |  |  |  |
|  |  |  |  |  |  | Number of devices |  |  |  |  |  |
| Must Use | C04002 | 127 | Reference Identification |  |  |  | M |  | AN 1/30 |  |  |
|  |  |  | Reference information as defined for a particular Transaction Set or as specified by the Reference Identification Qualifier |  |  |  |  |  |  |  |  |
|  |  |  | Number of unmetered devices of this type. |  |  |  |  |  |  |  |  |

Segment:	REF Reference Identification (Meter Reading Cycle)

Position:	130

Loop:	NM1        Optional

Level:	Detail

Usage:	Optional

Max Use:	>1

Purpose:	To specify identifying information

Syntax Notes:	1	At least one of REF02 or REF03 is required.

2	If either C04003 or C04004 is present, then the other is required.

3	If either C04005 or C04006 is present, then the other is required.

Semantic Notes:	1	REF04 contains data relating to the value cited in REF02.

Comments:

| Notes: |  | Meter cycles will be provided on the TDSP's website. |
| --- | --- | --- |
|  |  | Required when the meter read is done by cycle. Required for unmetered services. One of the either the REF~TZ or DTM~313 is required for metered services but not both. |
|  |  | REF~TZ~15 REF~TZ~01 |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | REF01 | 128 | Reference Identification Qualifier |  |  |  | M |  | ID 2/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code qualifying the Reference Identification |  |  |  |  |  |  |  |  |
|  |  |  |  | TZ |  | Total Cycle Number |  |  |  |  |  |
|  |  |  |  |  |  | Cycle number when the meter will be read. |  |  |  |  |  |
| Must Use | REF02 | 127 | Reference Identification |  |  |  | X |  | AN 1/30 |  |  |
|  |  |  | Reference information as defined for a particular Transaction Set or as specified by the Reference Identification Qualifier |  |  |  |  |  |  |  |  |
|  |  |  | Metered: Meter Reading Cycle Unmetered: Billing Cycle Must be numeric and only two digits. |  |  |  |  |  |  |  |  |

Segment:	DTM Date/Time Reference (Meter Cycle by Day of the Month)

Position:	140

Loop:	NM1        Optional

Level:	Detail

Usage:	Optional

Max Use:	>1

Purpose:	To specify pertinent dates and times

Syntax Notes:	1	At least one of DTM02 DTM03 or DTM05 is required.

2	If DTM04 is present, then DTM03 is required.

3	If either DTM05 or DTM06 is present, then the other is required.

Semantic Notes:

Comments:

| Notes: |  | Required on metered services if meter read based on a day of the month. Not used for unmetered services. One of the either the REF~TZ or the DTM~313 is required for metered services but not both. |
| --- | --- | --- |
|  |  | DTM~313~~~~DD~01 |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | DTM01 | 374 | Date/Time Qualifier |  |  |  | M |  | ID 3/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code specifying type of date or time, or both date and time |  |  |  |  |  |  |  |  |
|  |  |  |  | 313 |  | Cycle |  |  |  |  |  |
|  |  |  |  |  |  | Meter Cycle by Day of the Month |  |  |  |  |  |
| Must Use | DTM05 | 1250 | Date Time Period Format Qualifier |  |  |  | X |  | ID 2/3 |  |  |
|  |  |  | Code indicating the date format, time format, or date and time format |  |  |  |  |  |  |  |  |
|  |  |  |  | DD |  | Day of Month in Numeric Format |  |  |  |  |  |
| Must Use | DTM06 | 1251 | Date Time Period |  |  |  | X |  | AN 1/35 |  |  |
|  |  |  | Expression of a date, a time, or range of dates, times or dates and times |  |  |  |  |  |  |  |  |
|  |  |  | Day of month that the meter will be read, in DD format (e.g., 01 - 28). |  |  |  |  |  |  |  |  |

Segment:	SE Transaction Set Trailer

Position:	150

Loop:

Level:	Detail

Usage:	Mandatory

Max Use:	1

Purpose:	To indicate the end of the transaction set and provide the count of the transmitted segments (including the beginning (ST) and ending (SE) segments)

Syntax Notes:

Semantic Notes:

Comments:	1	SE is the last segment of each transaction set.

| Notes: |  | Required |
| --- | --- | --- |
|  |  | SE~10~000000001 |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | SE01 | 96 | Number of Included Segments | M |  | N0 1/10 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Total number of segments included in a transaction set including ST and SE segments |  |  |  |  |
| Must Use | SE02 | 329 | Transaction Set Control Number | M |  | AN 4/9 |  |
|  |  |  | Identifying control number that must be unique within the transaction set functional group assigned by the originator for a transaction set |  |  |  |  |

814_22 Example #1 of 1

Tenant Move Out – Revert to CSA CR Request – ERCOT to CSA CR

| ERCOT to CSA CR Tenant Initiated Customer Move Out Used to start service of the ESI ID with the CSA CR |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ST~814~000000001 |  |  | Transaction Type, Control Number |  |  |
| BGN~13~2001060223404~20010602~~~2001060181424~~22 |  |  | Request, Unique Transaction Number, Transaction Date, SET Transaction Number |  |  |
| N1~8S~TDSP COMPANY~1~007909411 |  |  | TDSP Name and DUNS Number |  |  |
| N1~AY~ERCOT~1~183529049~~41 |  |  | ERCOT Name and DUNS Number, Sender |  |  |
| N1~SJ~CSA CR NAME~9~007909422CSA1~~40 |  |  | CSA CR Name and DUNS Number, Receiver |  |  |
| LIN~1~SH~EL~SH~CE~SH~MVI |  |  | Continuous Service Agreement Move In Request for CSA CR |  |  |
|  | ASI~7~021 |  | Request for Move In |  |  |
|  | REF~AQ~T |  |  | Distribution Loss Factor Code |  |
|  | REF~Q5~~101234500000000000000000000001000041 |  | ESI ID |  |  |
|  | REF~SPL~~WEST |  |  | Station ID |  |
|  | REF~SU~N |  | Life Support – Unsure whether this will be required |  |  |
|  | DTM~150~20010604 |  | Service Period Start |  |  |
|  | NM1~MQ~3~~~~~~32~W123456789 |  | Beginning of Meter Loop, Meter Number |  |  |
|  |  | REF~4P~1~KHMON~TU^51 | Meter Multiplier |  |  |
|  |  | REF~IX~5.0~KHMON~TU^51 | Number of Dials |  |  |
|  |  | REF~LO~RESLOWR_WEST_NIDR_NWS_NOTOU | Load Profile |  |  |
|  |  | REF~PRT~MV~750~QQ^100 | Unmetered Service Type, Wattage/Lumens, Number of Devices of this type. |  |  |
|  |  | REF~MT~KHMON | Meter Type |  |  |
|  |  | REF~NH~RES | TDSP Rate Class or Tariff |  |  |
|  |  | REF~PR~T209 | TDSP Rate Subclass |  |  |
|  |  | DTM~313~~~~DD~01 | Meter Cycle by Day of the Month |  |  |
| SE~22~000000001 |  |  | Number of Segments, Control Number |  |  |