Texas

Standard

Electronic

Transaction

814_21:

##### Create/Maintain/Retire ESI ID Response

Electronic Data Interchange

ANSI ASC X12 Ver/Rel 004010

Transaction Set 814

Texas 814_21:

CREATE/MAINTAIN/RETIRE ESI ID RESPONSE

This transaction set:

... from ERCOT to TDSP, is used to acknowledge receipt of the 814_20 Create/Maintain/Retire ESI ID Request.

...  from the current CR, new CR and CSA CR to ERCOT, is used to acknowledge receipt of the 814_20 Create/Maintain/Retire ESI ID Request.

Document Flow:

- ERCOT to TDSP

- Current CR to ERCOT

- New CR to ERCOT

- CSA CR to ERCOT

|  |  | # Summary of Changes |
| --- | --- | --- |
| July 3, 2000 Version 1.0 |  | Initial Release |
| August 23, 2000 Version 1.1 |  | The following changes were made: |
|  |  | - Added LIN code “IN” for Initial Conversion/Opt In |
|  |  | - Added rejection reason codes “A83” and “API” in REF~7G |
| December 12, 2000 Version 1.2 |  | The following changes were made: |
|  |  | - Change the term Utility to Transmission Distribution Service Provider (TDSP) to match ERCOT Protocols. |
|  |  | - Change the term Retail Electric Provider (REP) to Competitive Retailer (CR) to match ERCOT Protocols. |
|  |  | - Change description page transaction purpose to read the same as ERCOT Protocols. |
|  |  | - Insert the “How to use this Implementation Guide” page. |
|  |  | - Remove “.” between X and 12 in all references to ANSI ASC X12. |
|  |  | - Add BGN08 Element to indicate the TX SET Transaction Number per Change Control #2000-015. |
|  |  | - Add examples to end of transaction. |
| March 16, 2001 Version 1.3 |  | The following changes were made: |
|  |  | - Removed Scenario Names from Transaction Description page |
|  |  | - Corrected the How to Use this Implementation Guide page |
|  |  | - Changed RA/Clearinghouse to ERCOT |
| August 3, 2001 Version 1.4 |  | The following changes were made: |
|  |  | - Modify gray box in REF~7G. Ref. 2001-119 |
|  |  | - Modify gray box in BGN06. Ref. 2001-139 |
|  |  | - 2002-287 – 04/12/02 – enhanced Zip Code Reject Reason gray box in REF~7G segment. |

How to Use this Implementation Guide

This section is used to show the X12 Rules for this segment.  You must look further into the grayboxes below for Texas Rules.

Segment:	REF Reference Identification (ESI ID)

Position:	030

Loop:	LIN        Optional

Level:	Detail

Usage:	Optional

Max Use:	>1

Purpose:	To specify identifying information

Syntax Notes:	1	At least one of REF02 or REF03 is required.

2	If either C04003 or C04004 is present, then the other is required.

3	If either C04005 or C04006 is present, then the other is required.

Semantic Notes:	1	REF04 contains data relating to the value cited in REF02.

Comments:

This section is used to show the Texas Rules for implementation of this segment.

| Notes: |  | Required |
| --- | --- | --- |
|  |  | REF~Q5~~10111111234567890ABCDEFGHIJKLMNOPQRS |

Data Element Summary

One or more examples.

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | REF01 | 128 | Reference Identification Qualifier |  |  |  | M |  | ID 2/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code qualifying the Reference Identification |  |  |  |  |  |  |  |  |
|  |  |  |  | Q5 |  | Property Control Number |  |  |  |  |  |
|  |  |  |  |  |  | Electric Service Identifier (ESI ID) |  |  |  |  |  |
| Must Use | REF03 | 352 | Description |  |  |  | X |  | AN 1/80 |  |  |
|  |  |  | A free-form description to clarify the related data elements and their content |  |  |  |  |  |  |  |  |
|  |  |  | ESI ID |  |  |  |  |  |  |  |  |

These are X12 code descriptions, which often do not relate to the Texas descriptions.

X12 cannot keep up with Texas needs, thus, Texas often changes the meaning of existing codes.  See the corresponding graybox for the Texas definitions.

This column shows the X12 attributes for each data element.

M = Mandatory

O= Optional

X = Conditional

AN = Alphanumeric

N# = Implied Decimal at position #

ID = Identification

R = Real

1/30 = Minimum 1, Maximum 30

This column shows the Texas use of each data element.

814 General Request, Response or Confirmation

ANSI ASC X12 Structure

Functional Group ID=GE

Introduction:

This Draft Standard for Trial Use contains the format and establishes the data contents of the General Request, Response or Confirmation Transaction Set (814) for use within the context of an Electronic Data Interchange (EDI) environment. This standard can be used to request actions to be performed, to respond to a request for actions to be performed or to confirm information related to actions performed.

Heading:

Pos.	Seg.		Req.		Loop	Notes and

No.	ID	Name	Des.	Max.Use	Repeat	Comments

| M | 010 | ST | Transaction Set Header | M | 1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| M | 020 | BGN | Beginning Segment | M | 1 |  |  |  |  |  |  |  |  |
|  |  |  | LOOP ID - N1 |  |  | >1 |  |  |  |  |  |  |  |
|  | 040 | N1 | Name | O | 1 |  | n1 |  |  |  |  |  |  |

Detail:

Pos.	Seg.		Req.		Loop	Notes and

No.	ID	Name	Des.	Max.Use	Repeat	Comments

|  |  |  | LOOP ID - LIN |  |  | >1 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 010 | LIN | Item Identification | O | 1 |  |  |  |  |  |  |  |  |
|  | 020 | ASI | Action or Status Indicator | O | 1 |  |  |  |  |  |  |  |  |
|  | 030 | REF | Reference Identification | O | >1 |  |  |  |  |  |  |  |  |
| M | 150 | SE | Transaction Set Trailer | M | 1 |  |  |  |  |  |  |  |  |

Transaction Set Notes

1.	The N1 loop is used to identify the transaction sender and receiver.

Segment:	ST Transaction Set Header

Position:	010

Loop:

Level:	Heading

Usage:	Mandatory

Max Use:	1

Purpose:	To indicate the start of a transaction set and to assign a control number

Syntax Notes:

Semantic Notes:	1	The transaction set identifier (ST01) is used by the translation routines of the interchange partners to select the appropriate transaction set definition (e.g., 810 selects the Invoice Transaction Set).

Comments:

| Notes: |  | Required |
| --- | --- | --- |
|  |  | ST~814~000000001 |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | ST01 | 143 | Transaction Set Identifier Code |  |  |  |  | M |  | ID 3/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Code uniquely identifying a Transaction Set |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 814 |  | General Request, Response or Confirmation |  |  |  |  |  |
| Must Use | ST02 | 329 | Transaction Set Control Number |  |  |  |  | M |  | AN 4/9 |  |  |
|  |  |  |  | Identifying control number that must be unique within the transaction set functional group assigned by the originator for a transaction set |  |  |  |  |  |  |  |  |

Segment:	BGN Beginning Segment

Position:	020

Loop:

Level:	Heading

Usage:	Mandatory

Max Use:	1

Purpose:	To indicate the beginning of a transaction set

Syntax Notes:	1	If BGN05 is present, then BGN04 is required.

Semantic Notes:	1	BGN02 is the transaction set reference number.

2	BGN03 is the transaction set date.

3	BGN04 is the transaction set time.

4	BGN05 is the transaction set time qualifier.

5	BGN06 is the transaction set reference number of a previously sent transaction affected by the current transaction.

Comments:

| Notes: |  | Required |
| --- | --- | --- |
|  |  | BGN~11~200104021200719~20010402~~~200104011956531~~21 |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | BGN01 | 353 | Transaction Set Purpose Code |  |  |  |  |  | M |  | ID 2/2 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Code identifying purpose of transaction set |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 11 |  | Response |  |  |  |  |  |  |  |
| Must Use | BGN02 | 127 | Reference Identification |  |  |  |  |  | M |  | AN 1/30 |  |  |  |
|  |  |  |  | Reference information as defined for a particular Transaction Set or as specified by the Reference Identification Qualifier |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | A unique transaction identification number assigned by the originator of this transaction. This number must be unique over time. Transaction Reference numbers will only contain uppercase letters (A to Z) and digits (0 to 9). Note that punctuation (spaces, dashes, etc.) must be excluded. |  |  |  |  |  |  |  |  |  |  |
| Must Use | BGN03 | 373 | Date |  |  |  |  |  | M |  | DT 8/8 |  |  |  |
|  |  |  |  | Date expressed as CCYYMMDD |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | The transaction creation date - the date that the data was processed by the sender's application system. |  |  |  |  |  |  |  |  |  |  |
| Must Use | BGN06 | 127 | Reference Identification |  |  |  |  |  | O |  | AN 1/30 |  |  |  |
|  |  |  |  | Reference information as defined for a particular Transaction Set or as specified by the Reference Identification Qualifier |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Refers to the BGN02 of the Create/Maintain/Retire ESID Request (814_20) sent from the TDSP. This number will be tracked in the BGN06 through the lifecycle of the Create/Maintain/Retire ESID process. From ERCOT to the TDSP, use the BGN02 of the 814_20 from the TDSP. From the current CR to ERCOT, use the BGN06 of the 814_20 from ERCOT, which refers to the BGN02 of the 814_20 from the TDSP. |  |  |  |  |  |  |  |  |  |  |
| Must Use | BGN08 | 306 | Action Code |  |  |  |  |  | O |  | ID 1/2 |  |  |  |
|  |  |  |  | Code indicating type of action |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | This segment is used to initially identify the type of 814 that is being sent or received. Ignore the ANSI X12 definition of the code. |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 21 |  | Disapprove |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | Indicates Texas SET Transaction 814_21 |  |  |  |  |  |  |

Segment:	N1 Name (Transmission Distribution Service Provider)

Position:	040

Loop:	N1        Optional

Level:	Heading

Usage:	Optional

Max Use:	1

Purpose:	To identify a party by type of organization, name, and code

Syntax Notes:	1	At least one of N102 or N103 is required.

2	If either N103 or N104 is present, then the other is required.

Semantic Notes:

Comments:	1	This segment, used alone, provides the most efficient method of providing organizational identification. To obtain this efficiency the "ID Code" (N104) must provide a key to the table maintained by the transaction processing party.

2	N105 and N106 further define the type of entity in N101.

| Notes: |  | Required if TDSP is the Receiver, otherwise not used. |
| --- | --- | --- |
|  |  | N1~8S~TDSP COMPANY~1~007909411~~40 |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | N101 | 98 | Entity Identifier Code |  |  |  |  |  | M |  | ID 2/3 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Code identifying an organizational entity, a physical location, property or an individual |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 8S |  | Consumer Service Provider (CSP) |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | Transmission Distribution Service Provider (TDSP) |  |  |  |  |  |  |
| Must Use | N102 | 93 | Name |  |  |  |  |  | X |  | AN 1/60 |  |  |  |
|  |  |  |  | Free-form name |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | TDSP Name |  |  |  |  |  |  |  |  |  |  |
| Must Use | N103 | 66 | Identification Code Qualifier |  |  |  |  |  | X |  | ID 1/2 |  |  |  |
|  |  |  |  | Code designating the system/method of code structure used for Identification Code (67) |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 1 |  | D-U-N-S Number, Dun & Bradstreet |  |  |  |  |  |  |  |
|  |  |  |  |  | 9 |  | D-U-N-S+4, D-U-N-S Number with Four Character Suffix |  |  |  |  |  |  |  |
| Must Use | N104 | 67 | Identification Code |  |  |  |  |  | X |  | AN 2/80 |  |  |  |
|  |  |  |  | Code identifying a party or other code |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | TDSP D-U-N-S Number or D-U-N-S + 4 Number |  |  |  |  |  |  |  |  |  |  |
| Must Use | N106 | 98 | Entity Identifier Code |  |  |  |  |  | O |  | ID 2/3 |  |  |  |
|  |  |  |  | Code identifying an organizational entity, a physical location, property or an individual |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 40 |  | Receiver |  |  |  |  |  |  |  |

Segment:	N1 Name (ERCOT)

Position:	040

Loop:	N1        Optional

Level:	Heading

Usage:	Optional

Max Use:	1

Purpose:	To identify a party by type of organization, name, and code

Syntax Notes:	1	At least one of N102 or N103 is required.

2	If either N103 or N104 is present, then the other is required.

Semantic Notes:

Comments:	1	This segment, used alone, provides the most efficient method of providing organizational identification. To obtain this efficiency the "ID Code" (N104) must provide a key to the table maintained by the transaction processing party.

2	N105 and N106 further define the type of entity in N101.

| Notes: |  | Required |
| --- | --- | --- |
|  |  | N1~AY~ERCOT~1~183529049~~41 N1~AY~ERCOT~1~183529049~~40 |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | N101 | 98 | Entity Identifier Code |  |  |  |  |  | M |  | ID 2/3 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Code identifying an organizational entity, a physical location, property or an individual |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | AY |  | Clearinghouse |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | ERCOT |  |  |  |  |  |  |
| Must Use | N102 | 93 | Name |  |  |  |  |  | X |  | AN 1/60 |  |  |  |
|  |  |  |  | Free-form name |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | ERCOT Name |  |  |  |  |  |  |  |  |  |  |
| Must Use | N103 | 66 | Identification Code Qualifier |  |  |  |  |  | X |  | ID 1/2 |  |  |  |
|  |  |  |  | Code designating the system/method of code structure used for Identification Code (67) |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 1 |  | D-U-N-S Number, Dun & Bradstreet |  |  |  |  |  |  |  |
|  |  |  |  |  | 9 |  | D-U-N-S+4, D-U-N-S Number with Four Character Suffix |  |  |  |  |  |  |  |
| Must Use | N104 | 67 | Identification Code |  |  |  |  |  | X |  | AN 2/80 |  |  |  |
|  |  |  |  | Code identifying a party or other code |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | ERCOT D-U-N-S Number or D-U-N-S + 4 Number |  |  |  |  |  |  |  |  |  |  |
| Must Use | N106 | 98 | Entity Identifier Code |  |  |  |  |  | O |  | ID 2/3 |  |  |  |
|  |  |  |  | Code identifying an organizational entity, a physical location, property or an individual |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 40 |  | Receiver |  |  |  |  |  |  |  |
|  |  |  |  |  | 41 |  | Submitter |  |  |  |  |  |  |  |

Segment:	N1 Name (Competitive Retailer)

Position:	040

Loop:	N1        Optional

Level:	Heading

Usage:	Optional

Max Use:	1

Purpose:	To identify a party by type of organization, name, and code

Syntax Notes:	1	At least one of N102 or N103 is required.

2	If either N103 or N104 is present, then the other is required.

Semantic Notes:

Comments:	1	This segment, used alone, provides the most efficient method of providing organizational identification. To obtain this efficiency the "ID Code" (N104) must provide a key to the table maintained by the transaction processing party.

2	N105 and N106 further define the type of entity in N101.

| Notes: |  | Required if CR is the Sender, otherwise not used. |
| --- | --- | --- |
|  |  | N1~SJ~CR NAME~1~007909422~~41 |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | N101 | 98 | Entity Identifier Code |  |  |  |  |  | M |  | ID 2/3 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Code identifying an organizational entity, a physical location, property or an individual |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | SJ |  | Service Provider |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | Competitive Retailer (CR) |  |  |  |  |  |  |
| Must Use | N102 | 93 | Name |  |  |  |  |  | X |  | AN 1/60 |  |  |  |
|  |  |  |  | Free-form name |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | CR Name |  |  |  |  |  |  |  |  |  |  |
| Must Use | N103 | 66 | Identification Code Qualifier |  |  |  |  |  | X |  | ID 1/2 |  |  |  |
|  |  |  |  | Code designating the system/method of code structure used for Identification Code (67) |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 1 |  | D-U-N-S Number, Dun & Bradstreet |  |  |  |  |  |  |  |
|  |  |  |  |  | 9 |  | D-U-N-S+4, D-U-N-S Number with Four Character Suffix |  |  |  |  |  |  |  |
| Must Use | N104 | 67 | Identification Code |  |  |  |  |  | X |  | AN 2/80 |  |  |  |
|  |  |  |  | Code identifying a party or other code |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | CR D-U-N-S Number or D-U-N-S + 4 Number |  |  |  |  |  |  |  |  |  |  |
| Must Use | N106 | 98 | Entity Identifier Code |  |  |  |  |  | O |  | ID 2/3 |  |  |  |
|  |  |  |  | Code identifying an organizational entity, a physical location, property or an individual |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 41 |  | Submitter |  |  |  |  |  |  |  |

Segment:	LIN Item Identification

Position:	010

Loop:	LIN        Optional

Level:	Detail

Usage:	Optional

Max Use:	1

Purpose:	To specify basic item identification data

Syntax Notes:	1	If either LIN04 or LIN05 is present, then the other is required.

2	If either LIN06 or LIN07 is present, then the other is required.

3	If either LIN08 or LIN09 is present, then the other is required.

4	If either LIN10 or LIN11 is present, then the other is required.

5	If either LIN12 or LIN13 is present, then the other is required.

6	If either LIN14 or LIN15 is present, then the other is required.

7	If either LIN16 or LIN17 is present, then the other is required.

8	If either LIN18 or LIN19 is present, then the other is required.

9	If either LIN20 or LIN21 is present, then the other is required.

10	If either LIN22 or LIN23 is present, then the other is required.

11	If either LIN24 or LIN25 is present, then the other is required.

12	If either LIN26 or LIN27 is present, then the other is required.

13	If either LIN28 or LIN29 is present, then the other is required.

14	If either LIN30 or LIN31 is present, then the other is required.

Semantic Notes:	1	LIN01 is the line item identification

Comments:	1	See the Data Dictionary for a complete list of IDs.

2	LIN02 through LIN31 provide for fifteen different product/service IDs for each item. For example: Case, Color, Drawing No., U.P.C. No., ISBN No., Model No., or SKU.

| Notes: |  | Required |
| --- | --- | --- |
|  |  | LIN~1~SH~EL~SH~MP |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | LIN01 | 350 | Assigned Identification |  |  |  |  |  | O |  | AN 1/20 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Alphanumeric characters assigned for differentiation within a transaction set |  |  |  |  |  |  |  |  |  |  |
| Must Use | LIN02 | 235 | Product/Service ID Qualifier |  |  |  |  |  | M |  | ID 2/2 |  |  |  |
|  |  |  |  | Code identifying the type/source of the descriptive number used in Product/Service ID (234) |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | SH |  | Service Requested |  |  |  |  |  |  |  |
| Must Use | LIN03 | 234 | Product/Service ID |  |  |  |  |  | M |  | AN 1/48 |  |  |  |
|  |  |  |  | Identifying number for a product or service |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Commodity |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | EL |  | Electric Service |  |  |  |  |  |  |  |
| Must Use | LIN04 | 235 | Product/Service ID Qualifier |  |  |  |  |  | X |  | ID 2/2 |  |  |  |
|  |  |  |  | Code identifying the type/source of the descriptive number used in Product/Service ID (234) |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | SH |  | Service Requested |  |  |  |  |  |  |  |
| Must Use | LIN05 | 234 | Product/Service ID |  |  |  |  |  | X |  | AN 1/48 |  |  |  |
|  |  |  |  | Identifying number for a product or service |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | IN |  | Initial Conversion or Opt-In |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | Submission of premise data to ERCOT for initial conversion or opt in. |  |  |  |  |  |  |
|  |  |  |  |  | MP |  | Maintain Premise |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | Indicates this transaction relates to the establishment, maintenance, and termination of premise data. |  |  |  |  |  |  |

Segment:	ASI Action or Status Indicator

Position:	020

Loop:	LIN        Optional

Level:	Detail

Usage:	Optional

Max Use:	1

Purpose:	To indicate the action to be taken with the information provided or the status of the entity described

Syntax Notes:

Semantic Notes:

Comments:

| Notes: |  | Identifies the action to be taken or the status of a requested action for the service identified in the LIN segment. |
| --- | --- | --- |
|  |  | Required |
|  |  | ASI~WQ~001 |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | ASI01 | 306 | Action Code |  |  |  |  | M |  | ID 1/2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Code indicating type of action |  |  |  |  |  |  |  |  |
|  |  |  |  |  | U |  | Reject |  |  |  |  |  |
|  |  |  |  |  | WQ |  | Accept |  |  |  |  |  |
| Must Use | ASI02 | 875 | Maintenance Type Code |  |  |  |  | M |  | ID 3/3 |  |  |
|  |  |  |  | Code identifying the specific type of item maintenance |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 001 |  | Change |  |  |  |  |  |
|  |  |  |  |  | 002 |  | Delete |  |  |  |  |  |
|  |  |  |  |  | 021 |  | Addition |  |  |  |  |  |

Segment:	REF Reference Identification (Rejection Reason)

Position:	030

Loop:	LIN        Optional

Level:	Detail

Usage:	Optional

Max Use:	>1

Purpose:	To specify identifying information

Syntax Notes:	1	At least one of REF02 or REF03 is required.

2	If either C04003 or C04004 is present, then the other is required.

3	If either C04005 or C04006 is present, then the other is required.

Semantic Notes:	1	REF04 contains data relating to the value cited in REF02.

Comments:

| Notes: |  | This transaction allows multiple change request on a single 814. If any single change request is invalid, the transaction is rejected in its entirety with appropriate reject reasons. |
| --- | --- | --- |
|  |  | More than one rejection reason code may be sent by repeating the REF~7G segment. Accept Response: Not Used Reject Response: Required |
|  |  | REF~7G~A13~ADDITIONAL REASON TEXT HERE |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | REF01 | 128 | Reference Identification Qualifier |  |  |  |  |  | M |  | ID 2/3 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Code qualifying the Reference Identification |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 7G |  | Data Quality Reject Reason |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | Reject reasons associated with a reject status notification. |  |  |  |  |  |  |
| Must Use | REF02 | 127 | Reference Identification |  |  |  |  |  | X |  | AN 1/30 |  |  |  |
|  |  |  |  | Reference information as defined for a particular Transaction Set or as specified by the Reference Identification Qualifier |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 008 |  | ESI ID exists but is not active |  |  |  |  |  |  |  |
|  |  |  |  |  | A13 |  | Other |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | Explanation Required in REF03. |  |  |  |  |  |  |
|  |  |  |  |  | A76 |  | ESI ID Invalid or Not Found |  |  |  |  |  |  |  |
|  |  |  |  |  | A83 |  | Invalid or Unauthorized Action |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | Information provided was not supported in the Texas SET Standards. |  |  |  |  |  |  |
|  |  |  |  |  | ACI |  | Action Code (ASI01) Invalid |  |  |  |  |  |  |  |
|  |  |  |  |  | ANK |  | Invalid Source of Information |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | Receiver of request does not provide requested service and/or does not have requested information. |  |  |  |  |  |  |
|  |  |  |  |  | API |  | Required information missing |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | Explanation Required in REF03. May not be used in place of other, more specific error codes. |  |  |  |  |  |  |
|  |  |  |  |  | D76 |  | DUNS Number Invalid or Not Found |  |  |  |  |  |  |  |
|  |  |  |  |  | DIV |  | Date Invalid |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | Use REF03 to further describe the invalid date |  |  |  |  |  |  |
|  |  |  |  |  | LPI |  | Load Profile Invalid |  |  |  |  |  |  |  |
|  |  |  |  |  | MTI |  | Maintenance Type Code (ASI02) Invalid |  |  |  |  |  |  |  |
|  |  |  |  |  | ZIP |  | Invalid Zip Code |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | Zip Code does not contain only 5 or 9 characters |  |  |  |  |  |  |
| Dep | REF03 | 352 | Description |  |  |  |  |  | X |  | AN 1/80 |  |  |  |
|  |  |  |  | A free-form description to clarify the related data elements and their content |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Used to further describe the status reason code sent in REF02. Code "A13" requires a text explanation in this element. |  |  |  |  |  |  |  |  |  |  |

Segment:	REF Reference Identification (ESI ID)

Position:	030

Loop:	LIN        Optional

Level:	Detail

Usage:	Optional

Max Use:	>1

Purpose:	To specify identifying information

Syntax Notes:	1	At least one of REF02 or REF03 is required.

2	If either C04003 or C04004 is present, then the other is required.

3	If either C04005 or C04006 is present, then the other is required.

Semantic Notes:	1	REF04 contains data relating to the value cited in REF02.

Comments:

| Notes: |  | Required |
| --- | --- | --- |
|  |  | REF~Q5~~10111111234567890ABCDEFGHIJKLMNOPQRS |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | REF01 | 128 | Reference Identification Qualifier |  |  |  |  |  | M |  | ID 2/3 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Code qualifying the Reference Identification |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Q5 |  | Property Control Number |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | Electric Service Identifier (ESI ID) |  |  |  |  |  |  |
| Must Use | REF03 | 352 | Description |  |  |  |  |  | X |  | AN 1/80 |  |  |  |
|  |  |  |  | A free-form description to clarify the related data elements and their content |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | ESI ID |  |  |  |  |  |  |  |  |  |  |

Segment:	SE Transaction Set Trailer

Position:	150

Loop:

Level:	Detail

Usage:	Mandatory

Max Use:	1

Purpose:	To indicate the end of the transaction set and provide the count of the transmitted segments (including the beginning (ST) and ending (SE) segments)

Syntax Notes:

Semantic Notes:

Comments:	1	SE is the last segment of each transaction set.

| Notes: |  | Required |
| --- | --- | --- |
|  |  | SE~11~000000001 |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | SE01 | 96 | Number of Included Segments |  | M |  | N0 1/10 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Total number of segments included in a transaction set including ST and SE segments |  |  |  |  |
| Must Use | SE02 | 329 | Transaction Set Control Number |  | M |  | AN 4/9 |  |
|  |  |  |  | Identifying control number that must be unique within the transaction set functional group assigned by the originator for a transaction set |  |  |  |  |

814_21 Example #1 of 8

Create ESI ID Accept Response – ERCOT to TDSP

| ERCOT to TDSP Create ESI ID Request is Accepted |  |  |
| --- | --- | --- |
| ST~814~000000001 |  | Transaction Type, Control Number |
| BGN~11~2001060223401~20010602~~~2001060112301~~21 |  | Request, Unique Transaction Number, Transaction Date, SET Transaction Number |
| N1~8S~TDSP COMPANY~1~007909411~~40 |  | TDSP Name and DUNS Number, Receiver |
| N1~AY~ERCOT~1~183529049~~41 |  | ERCOT and DUNS Number, Sender |
| LIN~1~SH~EL~SH~MP |  | Maintain Premise |
|  | ASI~WQ~021 | Accept Addition |
|  | REF~Q5~~101234500000000000000000000001000011 | ESI ID |
| SE~8~000000001 |  | Number of Segments, Control Number |

814_21 Example #2 of 8

Create ESI ID Reject Response – ERCOT to TDSP

| ERCOT to TDSP Create ESI ID Request is Rejected |  |  |
| --- | --- | --- |
| ST~814~000000001 |  | Transaction Type, Control Number |
| BGN~11~2001060223401~20010602~~~2001060112301~~21 |  | Request, Unique Transaction Number, Transaction Date, SET Transaction Number |
| N1~8S~TDSP COMPANY~1~007909411~~40 |  | TDSP Name and DUNS Number, Receiver |
| N1~AY~ERCOT~1~183529049~~41 |  | ERCOT and DUNS Number, Sender |
| LIN~1~SH~EL~SH~MP |  | Maintain Premise |
|  | ASI~U~021 | Reject Addition |
|  | REF~7G~LPI~LOAD PROFILE INVALID | Reason Code and Text |
|  | REF~Q5~~101234500000000000000000000001000011 | ESI ID |
| SE~9~000000001 |  | Number of Segments, Control Number |

814_21 Example #3 of 8

Maintain ESI ID Notification Accept Response – Current CR to ERCOT

| CR to Current ERCOT Change ESI ID information – Notification of a Change in Read Cycle CR Accepts Change in Read Cycle forwarded from ERCOT |  |  |
| --- | --- | --- |
| ST~814~000000001 |  | Transaction Type, Control Number |
| BGN~11~2001060234501~20010602~~~2001060123401~~21 |  | Request, Unique Transaction Number, Transaction Date, SET Transaction Number |
| N1~AY~ERCOT~1~183529049~~40 |  | ERCOT Name and DUNS Number, Receiver |
| N1~SJ~CR NAME~9~007909422CRN1~~41 |  | Current CR Name and DUNS Number, Sender |
| LIN~1~SH~EL~SH~MP |  | Maintain Premise |
|  | ASI~WQ~001 | Accept Addition |
|  | REF~Q5~~101234500000000000000000000001000011 | ESI ID |
| SE~8~000000001 |  | Number of Segments, Control Number |

814_21 Example #4 of 8

Maintenance ESI ID Notification Reject Response – Current CR to ERCOT

| Current CR to ERCOT Change ESI ID information – Notification of a Change in Read Cycle CR Reject Change in Read Cycle forwarded from ERCOT |  |  |
| --- | --- | --- |
| ST~814~000000001 |  | Transaction Type, Control Number |
| BGN~11~2001060234501~20010602~~~2001060123401~~21 |  | Request, Unique Transaction Number, Transaction Date, SET Transaction Number |
| N1~AY~ERCOT~1~183529049~~40 |  | ERCOT Name and DUNS Number, Receiver |
| N1~SJ~CR NAME~9~007909422CRN1~~41 |  | Current CR Name and DUNS Number, Sender |
| LIN~1~SH~EL~SH~MP |  | Maintain Premise |
|  | ASI~U~001 | Reject Addition |
|  | REF~7G~A13~REASON TEXT | Reason Code and Text |
|  | REF~Q5~~101234500000000000000000000001000011 | ESI ID |
| SE~9~000000001 |  | Number of Segments, Control Number |

814_21 Example #5 of 8

Retire ESI ID Accept Response –ERCOT to TDSP

| ERCOT to the TDSP Accept Request to Retire ESI ID |  |  |
| --- | --- | --- |
| ST~814~000000001 |  | Transaction Type, Control Number |
| BGN~11~2001060223402~20010602~~~2001060112302~~21 |  | Request, Unique Transaction Number, Transaction Date, SET Transaction Number |
| N1~8S~TDSP COMPANY~1~007909411~~40 |  | TDSP Name and DUNS Number, Receiver |
| N1~AY~ERCOT~1~183529049~~41 |  | ERCOT and DUNS Number, Sender |
| LIN~1~SH~EL~SH~MP |  | Termination Request an Echo from LIN in 814_20 |
|  | ASI~WQ~002 | Accept Retire ESI ID |
|  | REF~Q5~~101234500000000000000000000001000021 | ESI ID |
| SE~8~000000001 |  | Number of Segments, Control Number |

814_21 Example #6 of 8

Retire ESI ID Reject Response –ERCOT to TDSP

| ERCOT to the TDSP Reject Response to Retire ESI ID |  |  |
| --- | --- | --- |
| ST~814~000000001 |  | Transaction Type, Control Number |
| BGN~11~2001060223402~20010602~~~2001060112302~~21 |  | Request, Unique Transaction Number, Transaction Date, SET Transaction Number |
| N1~8S~TDSP COMPANY~1~007909411~~40 |  | TDSP Name and DUNS Number, Receiver |
| N1~AY~ERCOT~1~183529049~~41 |  | ERCOT Name and DUNS Number, Sender |
| LIN~1~SH~EL~SH~MP |  | Maintain Premise |
|  | ASI~U~002 | Reject Retire ESI ID |
|  | REF~7G~MTI~Maintenance Type Code (ASI02) Invalid | Reject Code and Text |
|  | REF~Q5~~101234500000000000000000000001000021 | ESI ID |
| SE~9~000000001 |  | Number of Segments, Control Number |

814_21 Example #7 of 8

Maintain ESI ID Accept Response – ERCOT to TDSP

| ERCOT to TDSP Change ESI ID information – Notification of a Change in Read Cycle CR Accept Change in Read Cycle |  |  |
| --- | --- | --- |
| ST~814~000000001 |  | Transaction Type, Control Number |
| BGN~11~2001060223403~20010602~~~2001060112303~~21 |  | Request, Unique Transaction Number, Transaction Date, SET Transaction Number |
| N1~8S~TDSP COMPANY~1~007909411~~40 |  | TDSP Name and DUNS Number, Receiver |
| N1~AY~ERCOT~1~183529049~~41 |  | ERCOT and DUNS Number, Sender |
| LIN~1~SH~EL~SH~MP |  | Maintain Premise |
|  | ASI~WQ~001 | Accept Change |
|  | REF~Q5~~101234500000000000000000000001000031 | ESI ID |
| SE~8~000000001 |  | Number of Segments, Control Number |

814_21 Example #8 of 8

Maintain ESI ID Reject Response – ERCOT to TDSP

| ERCOT to TDSP Change ESI ID information – Notification of a Change in Read Cycle CR Reject Change in Read Cycle forwarded from ERCOT |  |  |
| --- | --- | --- |
| ST~814~000000001 |  | Transaction Type, Control Number |
| BGN~11~2001060223403~20010602~~~2001060112303~~21 |  | Request, Unique Transaction Number, Transaction Date, SET Transaction Number |
| N1~8S~TDSP COMPANY~1~007909411~~40 |  | TDSP Name and DUNS Number, Receiver |
| N1~AY~ERCOT~1~183529049~~41 |  | ERCOT and DUNS Number, Sender |
| LIN~1~SH~EL~SH~MP |  | Maintain Premise |
|  | ASI~U~001 | Reject Change |
|  | REF~7G~A13~METER CYCLE INVALID | Reject Code and Text |
|  | REF~Q5~~101234500000000000000000000001000031 | ESI ID |
| SE~9~000000001 |  | Number of Segments, Control Number |