Texas

Standard

Electronic

Transaction

867_04:

##### Initial Meter Read Notification

Electronic Data Interchange

ANSI ASC X12 Ver/Rel 004010

Transaction Set 867

Texas 867_04:

INITIAL METER READ NOTIFICATION

This transaction set...

...  from the TDSP to ERCOT is used to report the initial read associated with a Switch and/or a Move-In.

...  from the ERCOT to CR is used to report the initial read associated with a Switch and/or a Move-In.

Document Flow:

- TDSP to ERCOT

- ERCOT to CR

|  |  | # Summary of Changes |
| --- | --- | --- |
| August 23, 2000 Version 1.1 |  | Initial Release |
| December 12, 2000 Version 1.2 |  | The following changes were made: |
|  |  | - Change the term Utility to Transmission Distribution Service Provider (TDSP) to match ERCOT Protocols. |
|  |  | - Change the term Retail Electric Provider (REP) to Competitive Retailer (CR) to match ERCOT Protocols. |
|  |  | - Change description page transaction purpose to read the same as ERCOT Protocols. |
|  |  | - Insert the “How to use this Implementation Guide” page. |
|  |  | - Remove “.” between X and 12 in all references to ANSI ASC X12. |
|  |  | - Add Structure Page |
|  |  | - Add examples to end of transaction. |
|  |  | - Add Time of Use code 71-- Summer Super On-Peak to MEA07 of PTD~SU loop per Change Control #2000-18. |
|  |  | - Change PTD04 and PTD05 to dependent and add gray box to state, “Required for metered services, otherwise not used.” |
|  |  | - Add gray box stating “Required for metered services, otherwise not used” to the PTD~BJ loop, QTY and MEA segments. |
| March 27, 2001 Version 1.3 |  | The following changes were made: |
|  |  | - Remove scenarios from cover page |
|  |  | - Correct examples for unmetered loop |
|  |  | - Replace RA, Settlement, Registration Agent with ERCOT |
|  |  | - Added REF segment for Original Transaction Reference Number |
| August, 2001 Version1.4 |  | The following changes were made: |
|  |  | - Change language on page 2 ‘This transaction set…’ ref. 2001-106 |
|  |  | - Change language in gray box of REF~TN ref. 2001-115 |
|  |  | - Change language in gray box of REF02 of REF~TN ref. 2001-115 |
|  |  | Added REDLINES from Change Controls: 2001-180 |

How to Use this Implementation Guide

This section is used to show the X12 Rules for this segment.  You must look further into the grayboxes below for Texas Rules.

This section is used to show the X12 Rules for this segment.  You must look further into the grayboxes below for Texas Rules.

Segment:	REF Reference Identification (ESI ID)

Position:	030

Loop:	LIN        Optional

Level:	Detail

Usage:	Optional

Max Use:	>1

Purpose:	To specify identifying information

Syntax Notes:	1	At least one of REF02 or REF03 is required.

2	If either C04003 or C04004 is present, then the other is required.

3	If either C04005 or C04006 is present, then the other is required.

Semantic Notes:	1	REF04 contains data relating to the value cited in REF02.

Comments:

This section is used to show the Texas Rules for implementation of this segment.

This section is used to show the Texas Rules for implementation of this segment.

One or more examples.

| Notes: |  | Required |
| --- | --- | --- |
|  |  | REF~Q5~~10111111234567890ABCDEFGHIJKLMNOPQRS |

One or more examples.

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| # Must Use | REF01 | 128 | Reference Identification Qualifier |  |  |  | M |  | ID 2/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code qualifying the Reference Identification |  |  |  |  |  |  |  |  |
| This column shows the Texas use of each data element. This column shows the X12 attributes for each data element. M = Mandatory, O= Optional, X = Conditional AN = Alphanumeric, N# = Implied Decimal, ID = Identification, R = Real 1/30 = Minimum 1, Maximum 30 These are X12 code descriptions, which often do not relate to the Texas descriptions. X12 cannot keep up with Texas needs, thus, Texas often changes the meaning of existing codes. See the corresponding graybox for the Texas definitions. |  |  |  | Q5 |  | Property Control Number |  |  |  |  |  |
|  |  |  |  |  |  | Electric Service Identifier (ESI ID) |  |  |  |  |  |
| # Must Use | REF03 | 352 | Description |  |  |  | X |  | AN 1/80 |  |  |
|  |  |  | A free-form description to clarify the related data elements and their content |  |  |  |  |  |  |  |  |
|  |  |  | ESI ID |  |  |  |  |  |  |  |  |

This column shows the Texas use of each data element.

This column shows the X12 attributes for each data element.

M = Mandatory

O= Optional

X = Conditional

AN = Alphanumeric

N# = Implied Decimal at position #

ID = Identification

R = Real

1/30 = Minimum 1, Maximum 30

These are X12 code descriptions, which often do not relate to the Texas descriptions.

X12 cannot keep up with Texas needs, thus, Texas often changes the meaning of existing codes.  See the corresponding graybox for the Texas definitions.

867 Product Transfer and Resale Report

Functional Group ID=PT

Introduction:

This Draft Standard for Trial Use contains the format and establishes the data contents of the Product Transfer and Resale Report Transaction Set (867) for use within the context of an Electronic Data Interchange (EDI) environment. The transaction set can be used to: (1) report information about product that has been transferred from one location to another; (2) report sales of product from one or more locations to an end customer; or (3) report sales of a product from one or more locations to an end customer, and demand beyond actual sales (lost orders). Report may be issued by either buyer or seller.

Heading:

Pos.	Seg.		Req.		Loop	Notes and

No.	ID	Name	Des.	Max.Use	Repeat	Comments

| M | 010 | ST | Transaction Set Header | M | 1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| M | 020 | BPT | Beginning Segment for Product Transfer and Resale | M | 1 |  |  |  |  |  |  |  |  |
|  | 060 | REF | Reference Identification | O | 12 |  |  |  |  |  |  |  |  |
|  |  |  | LOOP ID - N1 |  |  | 5 |  |  |  |  |  |  |  |
|  | 080 | N1 | Name | O | 1 |  |  |  |  |  |  |  |  |

Detail:

Pos.	Seg.		Req.		Loop	Notes and

No.	ID	Name	Des.	Max.Use	Repeat	Comments

|  |  |  | LOOP ID - PTD |  |  | >1 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| M | 010 | PTD | Product Transfer and Resale Detail | M | 1 |  |  |  |  |  |  |  |  |
|  | 020 | DTM | Date/Time Reference | O | 10 |  |  |  |  |  |  |  |  |
|  |  |  | LOOP ID - QTY |  |  | >1 |  |  |  |  |  |  |  |
|  | 110 | QTY | Quantity | O | 1 |  |  |  |  |  |  |  |  |
|  | 160 | MEA | Measurements | O | 40 |  |  |  |  |  |  |  |  |

Summary:

Pos.	Seg.		Req.		Loop	Notes and

No.	ID	Name	Des.	Max.Use	Repeat	Comments

| M | 030 | SE | Transaction Set Trailer | M | 1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Segment:	ST Transaction Set Header

Position:	010

Loop:

Level:	Heading

Usage:	Mandatory

Max Use:	1

Purpose:	To indicate the start of a transaction set and to assign a control number

Syntax Notes:

Semantic Notes:	1	The transaction set identifier (ST01) is used by the translation routines of the interchange partners to select the appropriate transaction set definition (e.g., 810 selects the Invoice Transaction Set).

Comments:

| Notes: |  | Required |
| --- | --- | --- |
|  |  | ST~867~000000001 |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | ST01 | 143 | Transaction Set Identifier Code |  |  |  | M |  | ID 3/3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code uniquely identifying a Transaction Set |  |  |  |  |  |  |  |
|  |  |  |  | 867 |  | Product Transfer and Resale Report |  |  |  |  |
| Must Use | ST02 | 329 | Transaction Set Control Number |  |  |  | M |  | AN 4/9 |  |
|  |  |  | Identifying control number that must be unique within the transaction set functional group assigned by the originator for a transaction set |  |  |  |  |  |  |  |

Segment:	BPT Beginning Segment for Product Transfer and Resale

Position:	020

Loop:

Level:	Heading

Usage:	Mandatory

Max Use:	1

Purpose:	To indicate the beginning of the Product Transfer and Resale Report Transaction Set and transmit identifying data

Syntax Notes:	1	If either BPT05 or BPT06 is present, then the other is required.

Semantic Notes:	1	BPT02 identifies the transfer/resale number.

2	BPT03 identifies the transfer/resale date.

3	BPT08 identifies the transfer/resale time.

4	BPT09 is used when it is necessary to reference a Previous Report Number.

Comments:

| Notes: |  | Required |
| --- | --- | --- |
|  |  | BPT~SU~200107310001~20010731 |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | BPT01 | 353 | Transaction Set Purpose Code |  |  |  | M |  | ID 2/2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code identifying purpose of transaction set |  |  |  |  |  |  |  |
|  |  |  |  | SU |  | Status Update |  |  |  |  |
| Must Use | BPT02 | 127 | Reference Identification |  |  |  | O |  | AN 1/30 |  |
|  |  |  | Reference information as defined for a particular Transaction Set or as specified by the Reference Identification Qualifier |  |  |  |  |  |  |  |
|  |  |  | A unique transaction identification number assigned by the originator of this transaction. This number must be unique over time. Transaction Reference numbers will only contain uppercase letters (A to Z) and digits (0 to 9). Note that punctuation (spaces, dashes, etc.) must be excluded. |  |  |  |  |  |  |  |
| Must Use | BPT03 | 373 | Date |  |  |  | M |  | DT 8/8 |  |
|  |  |  | Date expressed as CCYYMMDD |  |  |  |  |  |  |  |
|  |  |  | The transaction creation date - the date that the data was processed by the sender's application system. |  |  |  |  |  |  |  |

Segment:	REF Reference Identification (ESI ID)

Position:	060

Loop:

Level:	Heading

Usage:	Optional

Max Use:	12

Purpose:	To specify identifying information

Syntax Notes:	1	At least one of REF02 or REF03 is required.

2	If either C04003 or C04004 is present, then the other is required.

3	If either C04005 or C04006 is present, then the other is required.

Semantic Notes:	1	REF04 contains data relating to the value cited in REF02.

Comments:

| Notes: |  | Required |
| --- | --- | --- |
|  |  | REF~Q5~~10111111234567890ABCDEFGHIJKLMNOPQRS |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | REF01 | 128 | Reference Identification Qualifier |  |  |  | M |  | ID 2/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code qualifying the Reference Identification |  |  |  |  |  |  |  |  |
|  |  |  |  | Q5 |  | Property Control Number |  |  |  |  |  |
|  |  |  |  |  |  | Electric Service Identifier |  |  |  |  |  |
| Must Use | REF03 | 352 | Description |  |  |  | X |  | AN 1/80 |  |  |
|  |  |  | A free-form description to clarify the related data elements and their content |  |  |  |  |  |  |  |  |
|  |  |  | ESI ID |  |  |  |  |  |  |  |  |
|  |  |  | This free-form text can not contain any characters that may be used as element delimiters, sub-element delimiters, segment terminators, or field separators(This includes: asterisk *, pipes ¦, tabs, linefeeds, carets ^, angle brackets < >, and tildes ~). |  |  |  |  |  |  |  |  |

Segment:	REF Reference Identification(Original Transaction ID)

Position:	060

Loop:

Level:	Heading

Usage:	Optional

Max Use:	12

Purpose:	To specify identifying information

Syntax Notes:	1	At least one of REF02 or REF03 is required.

2	If either C04003 or C04004 is present, then the other is required.

3	If either C04005 or C04006 is present, then the other is required.

Semantic Notes:	1	REF04 contains data relating to the value cited in REF02.

Comments:

| Notes: |  | The source of this value will be the BGN06 of the 814_03. |
| --- | --- | --- |
|  |  | Required |
|  |  | REF~TN~1234567820010620 |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | REF01 | 128 | Reference Identification Qualifier |  |  |  | M |  | ID 2/3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code qualifying the Reference Identification |  |  |  |  |  |  |  |
|  |  |  |  | TN |  | Transaction Reference Number |  |  |  |  |
| Must Use | REF02 | 127 | Reference Identification |  |  |  | X |  | AN 1/30 |  |
|  |  |  | Reference information as defined for a particular Transaction Set or as specified by the Reference Identification Qualifier |  |  |  |  |  |  |  |
|  |  |  | Original Transaction Reference Number from the BGN06 of the 814_03. |  |  |  |  |  |  |  |

Segment:	N1 Name (Transmission Distribution Service Provider)

Position:	080

Loop:	N1        Optional

Level:	Heading

Usage:	Optional

Max Use:	1

Purpose:	To identify a party by type of organization, name, and code

Syntax Notes:	1	At least one of N102 or N103 is required.

2	If either N103 or N104 is present, then the other is required.

Semantic Notes:

Comments:	1	This segment, used alone, provides the most efficient method of providing organizational identification. To obtain this efficiency the "ID Code" (N104) must provide a key to the table maintained by the transaction processing party.

2	N105 and N106 further define the type of entity in N101.

| Notes: |  | Required |
| --- | --- | --- |
|  |  | N1~8S~TDSP COMPANY~1~007909411~~41 (TDSP to ERCOT) N1~8S~TDSP COMPANY~1~007909411 (ERCOT to CR) |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | N101 | 98 | Entity Identifier Code |  |  |  | M |  | ID 2/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code identifying an organizational entity, a physical location, property or an individual |  |  |  |  |  |  |  |  |
|  |  |  |  | 8S |  | Consumer Service Provider (CSP) |  |  |  |  |  |
|  |  |  |  |  |  | Transmission Distribution Service Provider (TDSP) |  |  |  |  |  |
| Must Use | N102 | 93 | Name |  |  |  | X |  | AN 1/60 |  |  |
|  |  |  | Free-form name |  |  |  |  |  |  |  |  |
|  |  |  | Utility Name |  |  |  |  |  |  |  |  |
|  |  |  | This free-form text can not contain any characters that may be used as element delimiters, sub-element delimiters, segment terminators, or field separators(This includes: asterisk *, pipes ¦, tabs, linefeeds, carets ^, angle brackets < >, and tildes ~). |  |  |  |  |  |  |  |  |
| Must Use | N103 | 66 | Identification Code Qualifier |  |  |  | X |  | ID 1/2 |  |  |
|  |  |  | Code designating the system/method of code structure used for Identification Code (67) |  |  |  |  |  |  |  |  |
|  |  |  |  | 1 |  | D-U-N-S Number, Dun & Bradstreet |  |  |  |  |  |
|  |  |  |  | 9 |  | D-U-N-S+4, D-U-N-S Number with Four Character Suffix |  |  |  |  |  |
| Must Use | N104 | 67 | Identification Code |  |  |  | X |  | AN 2/80 |  |  |
|  |  |  | Code identifying a party or other code |  |  |  |  |  |  |  |  |
|  |  |  | TDSP D-U-N-S Number or D-U-N-S + 4 Number |  |  |  |  |  |  |  |  |
| Dep | N106 | 98 | Entity Identifier Code |  |  |  | O |  | ID 2/3 |  |  |
|  |  |  | Code identifying an organizational entity, a physical location, property or an individual |  |  |  |  |  |  |  |  |
|  |  |  | TDSP to ERCOT: Required ERCOT to CR: Not Used |  |  |  |  |  |  |  |  |
|  |  |  |  | 41 |  | Submitter |  |  |  |  |  |

Segment:	N1 Name (ERCOT)

Position:	080

Loop:	N1        Optional

Level:	Heading

Usage:	Optional

Max Use:	1

Purpose:	To identify a party by type of organization, name, and code

Syntax Notes:	1	At least one of N102 or N103 is required.

2	If either N103 or N104 is present, then the other is required.

Semantic Notes:

Comments:	1	This segment, used alone, provides the most efficient method of providing organizational identification. To obtain this efficiency the "ID Code" (N104) must provide a key to the table maintained by the transaction processing party.

2	N105 and N106 further define the type of entity in N101.

| Notes: |  | Required |
| --- | --- | --- |
|  |  | N1~AY~ERCOT~1~183529049~~40 (TDSP to ERCOT) N1~AY~ERCOT~1~183529049~~41 (ERCOT to CR) |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | N101 | 98 | Entity Identifier Code |  |  |  | M |  | ID 2/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code identifying an organizational entity, a physical location, property or an individual |  |  |  |  |  |  |  |  |
|  |  |  |  | AY |  | Clearinghouse |  |  |  |  |  |
|  |  |  |  |  |  | ERCOT |  |  |  |  |  |
| Must Use | N102 | 93 | Name |  |  |  | X |  | AN 1/60 |  |  |
|  |  |  | Free-form name |  |  |  |  |  |  |  |  |
|  |  |  | ERCOT Name |  |  |  |  |  |  |  |  |
|  |  |  | This free-form text can not contain any characters that may be used as element delimiters, sub-element delimiters, segment terminators, or field separators(This includes: asterisk *, pipes ¦, tabs, linefeeds, carets ^, angle brackets < >, and tildes ~). |  |  |  |  |  |  |  |  |
| Must Use | N103 | 66 | Identification Code Qualifier |  |  |  | X |  | ID 1/2 |  |  |
|  |  |  | Code designating the system/method of code structure used for Identification Code (67) |  |  |  |  |  |  |  |  |
|  |  |  |  | 1 |  | D-U-N-S Number, Dun & Bradstreet |  |  |  |  |  |
|  |  |  |  | 9 |  | D-U-N-S+4, D-U-N-S Number with Four Character Suffix |  |  |  |  |  |
| Must Use | N104 | 67 | Identification Code |  |  |  | X |  | AN 2/80 |  |  |
|  |  |  | Code identifying a party or other code |  |  |  |  |  |  |  |  |
|  |  |  | ERCOT D-U-N-S Number or D-U-N-S + 4 Number |  |  |  |  |  |  |  |  |
| Must Use | N106 | 98 | Entity Identifier Code |  |  |  | O |  | ID 2/3 |  |  |
|  |  |  | Code identifying an organizational entity, a physical location, property or an individual |  |  |  |  |  |  |  |  |
|  |  |  |  | 40 |  | Receiver |  |  |  |  |  |
|  |  |  |  | 41 |  | Submitter |  |  |  |  |  |

Segment:	N1 Name (Competitive Retailer)

Position:	080

Loop:	N1        Optional

Level:	Heading

Usage:	Optional

Max Use:	1

Purpose:	To identify a party by type of organization, name, and code

Syntax Notes:	1	At least one of N102 or N103 is required.

2	If either N103 or N104 is present, then the other is required.

Semantic Notes:

Comments:	1	This segment, used alone, provides the most efficient method of providing organizational identification. To obtain this efficiency the "ID Code" (N104) must provide a key to the table maintained by the transaction processing party.

2	N105 and N106 further define the type of entity in N101.

| Notes: |  | Required |
| --- | --- | --- |
|  |  | N1~SJ~CR~9~007909422CR1 (TDSP to ERCOT) N1~SJ~CR~9~007909422CRP1~~40 (ERCOT to CR) |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | N101 | 98 | Entity Identifier Code |  |  |  | M |  | ID 2/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code identifying an organizational entity, a physical location, property or an individual |  |  |  |  |  |  |  |  |
|  |  |  |  | SJ |  | Service Provider |  |  |  |  |  |
|  |  |  |  |  |  | Competitive Retailer (CR) |  |  |  |  |  |
| Must Use | N102 | 93 | Name |  |  |  | X |  | AN 1/60 |  |  |
|  |  |  | Free-form name |  |  |  |  |  |  |  |  |
|  |  |  | CR Name |  |  |  |  |  |  |  |  |
|  |  |  | This free-form text can not contain any characters that may be used as element delimiters, sub-element delimiters, segment terminators, or field separators(This includes: asterisk *, pipes ¦, tabs, linefeeds, carets ^, angle brackets < >, and tildes ~). |  |  |  |  |  |  |  |  |
| Must Use | N103 | 66 | Identification Code Qualifier |  |  |  | X |  | ID 1/2 |  |  |
|  |  |  | Code designating the system/method of code structure used for Identification Code (67) |  |  |  |  |  |  |  |  |
|  |  |  |  | 1 |  | D-U-N-S Number, Dun & Bradstreet |  |  |  |  |  |
|  |  |  |  | 9 |  | D-U-N-S+4, D-U-N-S Number with Four Character Suffix |  |  |  |  |  |
| Must Use | N104 | 67 | Identification Code |  |  |  | X |  | AN 2/80 |  |  |
|  |  |  | Code identifying a party or other code |  |  |  |  |  |  |  |  |
|  |  |  | CR D-U-N-S Number or D-U-N-S + 4 Number |  |  |  |  |  |  |  |  |
| Dep | N106 | 98 | Entity Identifier Code |  |  |  | O |  | ID 2/3 |  |  |
|  |  |  | Code identifying an organizational entity, a physical location, property or an individual |  |  |  |  |  |  |  |  |
|  |  |  | TDSP to ERCOT: Not Used ERCOT to CR: Required |  |  |  |  |  |  |  |  |
|  |  |  |  | 40 |  | Receiver |  |  |  |  |  |

Segment:	PTD Product Transfer and Resale Detail (Switch Date/Read)

Position:	010

Loop:	PTD        Mandatory

Level:	Detail

Usage:	Mandatory

Max Use:	1

Purpose:	To indicate the start of detail information relating to the transfer/resale of a product and provide identifying data

Syntax Notes:	1	If either PTD02 or PTD03 is present, then the other is required.

2	If either PTD04 or PTD05 is present, then the other is required.

Semantic Notes:

Comments:

| Notes: |  | One PTD loop is required for each meter. |
| --- | --- | --- |
|  |  | PTD~BJ (unmetered device) PTD~BJ~~~MG~1234568MG (metered device) |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | PTD01 | 521 | Product Transfer Type Code |  |  |  | M |  | ID 2/2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code identifying the type of product transfer |  |  |  |  |  |  |  |  |
|  |  |  |  | BJ |  | Relocation |  |  |  |  |  |
|  |  |  |  |  |  | Switch Date/Read |  |  |  |  |  |
| Dep | PTD04 | 128 | Reference Identification Qualifier |  |  |  | X |  | ID 2/3 |  |  |
|  |  |  | Code qualifying the Reference Identification |  |  |  |  |  |  |  |  |
|  |  |  | Required for metered service, otherwise not used. |  |  |  |  |  |  |  |  |
|  |  |  |  | MG |  | Meter Number |  |  |  |  |  |
| Dep | PTD05 | 127 | Reference Identification |  |  |  | X |  | AN 1/30 |  |  |
|  |  |  | Reference information as defined for a particular Transaction Set or as specified by the Reference Identification Qualifier |  |  |  |  |  |  |  |  |
|  |  |  | Meter Number Meter numbers will contain only uppercase letters (A to Z) and digits (0 to 9). Note that punctuation (spaces, dashes, etc.) must be excluded, and significant leading and trailing zeros that are part of the meter number must be present. |  |  |  |  |  |  |  |  |
|  |  |  | Required for metered service, otherwise not used. |  |  |  |  |  |  |  |  |

Segment:	DTM Date/Time Reference (Actual Switch Date)

Position:	020

Loop:	PTD        Mandatory

Level:	Detail

Usage:	Optional

Max Use:	10

Purpose:	To specify pertinent dates and times

Syntax Notes:	1	At least one of DTM02 DTM03 or DTM05 is required.

2	If DTM04 is present, then DTM03 is required.

3	If either DTM05 or DTM06 is present, then the other is required.

Semantic Notes:

Comments:

| Notes: |  | Required |
| --- | --- | --- |
|  |  | DTM~140~20010731 |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | DTM01 | 374 | Date/Time Qualifier |  |  |  | M |  | ID 3/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code specifying type of date or time, or both date and time |  |  |  |  |  |  |  |  |
|  |  |  |  | 140 |  | Actual |  |  |  |  |  |
|  |  |  |  |  |  | Actual Switch Date |  |  |  |  |  |
| Must Use | DTM02 | 373 | Date |  |  |  | X |  | DT 8/8 |  |  |
|  |  |  | Date expressed as CCYYMMDD |  |  |  |  |  |  |  |  |

Segment:	QTY Quantity

Position:	110

Loop:	QTY        Optional

Level:	Detail

Usage:	Optional

Max Use:	1

Purpose:	To specify quantity information

Syntax Notes:	1	At least one of QTY02 or QTY04 is required.

2	Only one of QTY02 or QTY04 may be present.

Semantic Notes:	1	QTY04 is used when the quantity is non-numeric.

Comments:

| Notes: |  | Required for metered service, otherwise not used. Each unit of measure requires a separate QTY loop. Only one unit of measure can be used for each QTY loop. For a read with two different unit of measures the following structure should be followed: PTD QTY MEA QTY MEA |
| --- | --- | --- |
|  |  | QTY~QD~~~NV |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | QTY01 | 673 | Quantity Qualifier |  |  |  | M |  | ID 2/2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code specifying the type of quantity |  |  |  |  |  |  |  |  |
|  |  |  |  | KA |  | Estimated |  |  |  |  |  |
|  |  |  |  |  |  | Read is estimated |  |  |  |  |  |
|  |  |  |  | QD |  | Quantity Delivered |  |  |  |  |  |
|  |  |  |  |  |  | Read is actual |  |  |  |  |  |
| Must Use | QTY04 | 61 | Free-Form Message |  |  |  | X |  | AN 1/30 |  |  |
|  |  |  | Free-form information |  |  |  |  |  |  |  |  |
|  |  |  |  | NV |  | No Value |  |  |  |  |  |

Segment:	MEA Measurements (Meter Reads)

Position:	160

Loop:	QTY        Optional

Level:	Detail

Usage:	Optional

Max Use:	40

Purpose:	To specify physical measurements or counts, including dimensions, tolerances, variances, and weights  (See Figures Appendix for example of use of C001)

Syntax Notes:	1	At least one of MEA03 MEA05 MEA06 or MEA08 is required.

2	If MEA05 is present, then MEA04 is required.

3	If MEA06 is present, then MEA04 is required.

4	If MEA07 is present, then at least one of MEA03 MEA05 or MEA06 is required.

5	Only one of MEA08 or MEA03 may be present.

Semantic Notes:	1	MEA04 defines the unit of measure for MEA03, MEA05, and MEA06.

Comments:	1	When citing dimensional tolerances, any measurement requiring a sign (+ or -), or any measurement where a positive (+) value cannot be assumed, use MEA05 as the negative (-) value and MEA06 as the positive (+) value.

| Notes: |  | Required for metered service, otherwise not used. |
| --- | --- | --- |
|  |  | MEA~~~~KH~~11005~51 |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | MEA04 | C001 | Composite Unit of Measure |  |  |  | X |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | To identify a composite unit of measure (See Figures Appendix for examples of use) |  |  |  |  |  |  |  |  |
|  |  |  | Please note that this is a composite data element, populate C00101. |  |  |  |  |  |  |  |  |
| Must Use | C00101 | 355 | Unit or Basis for Measurement Code |  |  |  | M |  | ID 2/2 |  |  |
|  |  |  | Code specifying the units in which a value is being expressed, or manner in which a measurement has been taken |  |  |  |  |  |  |  |  |
|  |  |  |  | K3 |  | Kilovolt Amperes Reactive Hour |  |  |  |  |  |
|  |  |  |  |  |  | kVARH |  |  |  |  |  |
|  |  |  |  | KH |  | Kilowatt Hour |  |  |  |  |  |
|  |  |  |  |  |  | kWh |  |  |  |  |  |
| Must Use | MEA06 | 741 | Range Maximum |  |  |  | X |  | R 1/20 |  |  |
|  |  |  | The value specifying the maximum of the measurement range |  |  |  |  |  |  |  |  |
|  |  |  | Switch Read New Customer, New Meter, installed during the Off-Peak consumption period measurement could be zero. |  |  |  |  |  |  |  |  |
| Must Use | MEA07 | 935 | Measurement Significance Code |  |  |  | O |  | ID 2/2 |  |  |
|  |  |  | Code used to benchmark, qualify or further define a measurement value |  |  |  |  |  |  |  |  |
|  |  |  |  | 41 |  | Off Peak |  |  |  |  |  |
|  |  |  |  | 42 |  | On Peak |  |  |  |  |  |
|  |  |  |  | 43 |  | Intermediate |  |  |  |  |  |
|  |  |  |  |  |  | Mid-Peak |  |  |  |  |  |
|  |  |  |  | 51 |  | Total |  |  |  |  |  |
|  |  |  |  |  |  | Totalizer/Total/Max (Demand) |  |  |  |  |  |
|  |  |  |  | 71 |  | Low |  |  |  |  |  |
|  |  |  |  |  |  | Summer Super On-Peak |  |  |  |  |  |

Segment:	SE Transaction Set Trailer

Position:	030

Loop:

Level:	Summary

Usage:	Mandatory

Max Use:	1

Purpose:	To indicate the end of the transaction set and provide the count of the transmitted segments (including the beginning (ST) and ending (SE) segments)

Syntax Notes:

Semantic Notes:

Comments:	1	SE is the last segment of each transaction set.

| Notes: |  | Required |
| --- | --- | --- |
|  |  | SE~11~000000001 |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | SE01 | 96 | Number of Included Segments | M |  | N0 1/10 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Total number of segments included in a transaction set including ST and SE segments |  |  |  |  |
| Must Use | SE02 | 329 | Transaction Set Control Number | M |  | AN 4/9 |  |
|  |  |  | Identifying control number that must be unique within the transaction set functional group assigned by the originator for a transaction set |  |  |  |  |

867_04 Example #1 of 5

Initial Meter Read Notification METERED SERVICES

| TDSP to ERCOT Used to send the initial/start read and date upon completion of a Cycle or Off-Cycle Switch or Move-In Essentially a pass through from the TDSP to the CR |  |  |  |
| --- | --- | --- | --- |
| ST~867~000000001 |  |  | Transaction Type, Control Number |
| BPT~SU~20010731X0001~20010731 |  |  | Status Update, Reference Number, Date |
| REF~Q5~~10111111234567890ABCDEFGHIJKLMNOPQRS |  |  | ESI ID |
| N1~8S~TDSP COMPANY~1~007909411~~41 |  |  | TDSP, Name, DUNs Number, Sender |
| N1~AY~ERCOT~1~183529049~~40 |  |  | ERCOT, Name, DUNs Number, Receiver |
| N1~SJ~CR~9~007909422CRN1 |  |  | CR, Name, DUNs Number |
| PTD~BJ~~~MG~1234568MG |  |  | Meter Number |
|  | DTM~140~20010731 |  | Actual Switch Date, Date |
|  | QTY~QD~~~NV |  | Quantity Delivered, No Value |
|  |  | MEA~~~~KH~~11005~51 | Unit of Measurement, Measurement, Totalizer |
| SE~11~000000001 |  |  | Number of Segments, Control Number |

867_04 Example #2 of 5

Initial Meter Read Notification METERED SERVICES – 3 METERS (INCLUDING A TIME-OF-USE, ALL REGISTER READINGS) – Off –Cycle Switch

| TDSP to ERCOT Used to send the initial/start read and date upon completion of a Cycle or Off-Cycle Switch or Move-In Essentially a pass through from the TDSP to the CR |  |  |  |
| --- | --- | --- | --- |
| ST~867~000000001 |  |  | Transaction Type, Control Number |
| BPT~SU~20010731X0001~20010731 |  |  | Status Update, Reference Number, Date |
| REF~Q5~~10111111234567890ABCDEFGHIJKLMNOPQRS |  |  | ESI ID |
| N1~8S~TDSP COMPANY~1~007909411~~41 |  |  | TDSP, Name, DUNs Number, Sender |
| N1~AY~ERCOT~1~183529049~~40 |  |  | ERCOT, Name, DUNs Number, Receiver |
| N1~SJ~CR~9~007909422CRN1 |  |  | CR, Name, DUNs Number |
| PTD~BJ~~~MG~1234568MG |  |  | Meter Number |
|  | DTM~140~20010830 |  | Actual Switch Date, Date |
|  | QTY~QD~~~NV |  | Quantity Delivered, No Value |
|  |  | MEA~~~~KH~~29876~51 | Unit of Measurement, Measurement, Totalizer |
| PTD~BJ~~~MG~1256567MG |  |  | Meter Number |
|  | DTM~140~20010830 |  | Actual Switch Date, Date |
|  | QTY~QD~~~NV |  | Quantity Delivered, No Value |
|  |  | MEA~~~~KH~~34532~51 | Unit of Measurement, Measurement, Totalizer |
| PTD~BJ~~~MG~14455656MG |  |  | Meter Number |
|  | DTM~140~20010731 |  | Actual Switch Date, Date |
|  | QTY~QD~~~NV |  | Quantity Delivered, No Value |
|  |  | MEA~~~~KH~~28789~41 | Unit of Measurement, Measurement, Off Peak |
|  |  | MEA~~~~KH~~18789~42 | Unit of Measurement, Measurement, On Peak |
|  |  | MEA~~~~KH~~34589~43 | Unit of Measurement, Measurement, Mid Peak |
|  |  | MEA~~~~KH~~24579~71 | Unit of Measurement, Measurement, Summer Super Peak |
|  |  | MEA~~~~KH~~22229~51 | Unit of Measurement, Measurement, Totalizer |
| SE~23~000000001 |  |  | Number of Segments, Control Number |

867_04 Example #3 of 5

Initial Meter Read Notification METERED SERVICES – 2 METERS – NON-INTERVAL AND TIME-OF-USE  METER– Move-In (New Customer, New Meter, installed during the Off-Peak consumption period)

| TDSP to ERCOT Used to send the initial/start read and date upon completion of a Cycle or Off-Cycle Switch or Move-In Essentially a pass through from the TDSP to the CR |  |  |  |
| --- | --- | --- | --- |
| ST~867~000000001 |  |  | Transaction Type, Control Number |
| BPT~SU~20010731X0001~20010731 |  |  | Status Update, Reference Number, Date |
| REF~Q5~~10111111234567890ABCDEFGHIJKLMNOPQRS |  |  | ESI ID |
| N1~8S~TDSP COMPANY~1~007909411~~41 |  |  | TDSP, Name, DUNs Number, Sender |
| N1~AY~ERCOT~1~183529049~~40 |  |  | ERCOT, Name, DUNs Number, Receiver |
| N1~SJ~CR~9~007909422CRN1 |  |  | CR, Name, DUNs Number |
| PTD~BJ~~~MG~14455656MG |  |  | Meter Number |
|  | DTM~140~20010731 |  | Actual Switch Date, Date |
|  | QTY~QD~~~NV |  | Quantity Delivered, No Value |
|  |  | MEA~~~~KH~~0~41 | Unit of Measurement, Measurement, Off Peak |
|  |  | MEA~~~~KH~~0~51 | Unit of Measurement, Measurement, Totalizer |
| PTD~BJ~~~MG~1234568MG |  |  | Meter Number |
|  | DTM~140~20010830 |  | Actual Switch Date, Date |
|  | QTY~QD~~~NV |  | Quantity Delivered, No Value |
|  |  | MEA~~~~KH~~0~51 | Unit of Measurement, Measurement, Totalizer |
| SE~16~000000001 |  |  | Number of Segments, Control Number |

867_04 Example #4 of 5

Initial Meter Read Notification UNMETERED SERVICES

| TDSP to ERCOT Used to send the initial/start read and date upon completion of a Cycle or Off-Cycle Switch or Move-In Essentially a pass through from the TDSP to the CR |  |  |
| --- | --- | --- |
| ST~867~000000001 |  | Transaction Type, Control Number |
| BPT~SU~20010731X0001~20010731 |  | Status Update, Reference Number, Date |
| REF~Q5~~10111111234567890ABCDEFGHIJKLMNOPQRS |  | ESI ID |
| N1~8S~TDSP COMPANY~1~007909411~~41 |  | TDSP, Name, DUNs Number, Sender |
| N1~AY~ERCOT~1~183529049~~40 |  | ERCOT, Name, DUNs Number, Receiver |
| N1~SJ~CR~9~007909422CR1 |  | CR, Name, DUNs Number |
| PTD~BJ |  | Unmetered Services |
|  | DTM~140~20010731 | Actual Switch Date, Date |
| SE~9~000000001 |  | Number of Segments, Control Number |

867_04 Example #5 of 5

Initial Meter Read Notification METERED SERVICES – INTERVAL METER- CYCLE SWITCH

| TDSP to ERCOT Used to send the initial/start read and date upon completion of a Cycle or Off-Cycle Switch or Move-In Essentially a pass through from the TDSP to the CR |  |  |  |
| --- | --- | --- | --- |
| ST~867~000000001 |  |  | Transaction Type, Control Number |
| BPT~SU~20010731X0001~20010731 |  |  | Status Update, Reference Number, Date |
| REF~Q5~~10111111234567890ABCDEFGHIJKLMNOPQRS |  |  | ESI ID |
| N1~8S~TDSP COMPANY~1~007909411~~41 |  |  | TDSP, Name, DUNs Number, Sender |
| N1~AY~ERCOT~1~183529049~~40 |  |  | ERCOT, Name, DUNs Number, Receiver |
| N1~SJ~CR~9~007909422CRN1 |  |  | CR, Name, DUNs Number |
| PTD~BJ~~~MG~14455656MG |  |  | Meter Number |
|  | DTM~140~20010731 |  | Actual Switch Date, Date |
|  | QTY~QD~~~NV |  | Quantity Delivered, No Value |
|  |  | MEA~~~~KH~~23456~51 | Unit of Measurement, Measurement, Totalizer |
| SE~11~000000001 |  |  | Number of Segments, Control Number |