| PRR Number | 332PRR | PRR Title | Unit-specific Dispatch Instructions and Resources Undergoing Required Testing |
| --- | --- | --- | --- |
| ###### Recommended Action |  | Approve |  |
| Proposed Effective Date |  | September 1, 2002, implementation of the system change pending ERCOT scheduling. |  |
| System Impact |  | May require a revision to the Operating Guides for implementation. |  |
| Benefit |  | Allows QSEs to notify ERCOT when units are in test status. |  |
| PRR Background |  | ## This revision addresses operating practices by prohibiting the ISO from issuing resource-specific dispatch instructions while the resource is in the process of pre-COD testing, required seasonal real and reactive testing, and testing necessary prior to and returning from annual planned maintenance (example: turbine trip tests prior to or after an annual turnaround). This PRR is submitted to ensure that the noted Protocol sections reflect recently approved ISO internal procedural steps in dealing with units undergoing testing. |  |
| Protocol Revision Subcommittee Recommendation |  | PRS recommends approval as revised. |  |
| ###### TAC Recommendation |  | TAC recommends approval as recommended by PRS. |  |

Recommended Language

### 4.5.2	Receipt of QSE’s Balancing Energy Bid Curves

ERCOT will receive QSEs Balancing Energy bid curves by the end of each Adjustment Period for an Operating Hour.  QSEs may voluntarily submit Balancing Energy Service Up and Down bid curves to ERCOT for use in the Operating Period.

QSEs may also be required to submit mandatory Balancing Energy Service Down bid curves if called on to do so by ERCOT.  If so, the QSEs will submit an amount of Balancing Energy Service Down bids sufficient to meet ERCOT’s required percentage of Balancing Energy Service Down bids as posted at 0600 of the Day Ahead.  Balancing Energy Service Down bids shall be supplied for at least the percentage requirement posted by ERCOT multiplied by the QSE’s scheduled energy resources in each Congestion Zone, less any QSE trades, averaged over the hour.

---------------------------------------------------------------------------------------------------------------------

### Dispatch Instruction Procedures

The procedures for issuing and responding to Dispatch Instructions are as follows:

- All Dispatch Instructions to Resources — whether for dispatch of Ancillary Services, System Emergencies or any other reason — shall be directed to the QSE responsible for the affected Resource.  ERCOT shall refrain from issuing dispatch instructions to generating units and other resources undergoing testing with the exception of,

    - dispatch instructions that are a part of the testing; or

    - during conditions when the generating unit is the only alternative for solving a transmission constraint; or .

    - During Force Majeure Events that threaten the reliability of the system.

---------------------------------------------------------------------------------------------------------------------

### EECP STEPS

Step 1 — Maintain 2300 MW of ERCOT’s reserve service Resources comprised of: (1) undeployed Responsive Reserve Service; and (2) that amount of Non-Spinning Reserve Service which has been deployed but has not ramped to its specified output level. ERCOT will:

- Use those available Ancillary Services that can be deployed to increase Responsive Reserves.

- Provide Dispatch Instructions to QSEs to start all Resources that are available in the time frame of the emergency (OOMC). Similarly, ERCOT will provide Dispatch Instructions to QSEs to suspend any ongoing generating unit or resource performance testing and maximize Resource deployment (OOME) to increase Responsive Reserve levels on other Resources. QSEs intending to provide OOMC and OOME will comply as soon as practicable

- Use available DC Tie capacity that is not already being used by the market.

- Start RMR Units available in the time frame of the Emergency. RMR Units should be loaded to full capability.

- --------------------------------------------------------------------------------------------------------------

### 6.10.2	General Capacity Testing Requirements

QSEs shall provide ERCOT a list identifying each Generation Resource unit that is expected to operate more than 168 hours in a Season as a provider of energy and/or Ancillary Services. ERCOT shall evaluate during each Season of expected operation, the Net Dependable Capability of each unit expected to operate more than 168 hours except for any Generation Resources used solely for energy services and whose capacity is less than 10 MW. Prior to the beginning of each Season, QSEs shall identify the Generation Resources to be tested during the Season and the specific week of the test if known. This schedule may be modified by the QSE (including retests) during the Season. QSEs not identifying a specific week for a Generation Resource unit test must test the unit within the first 168 hours of run time during the Season or operate with a net dependable capability equal to the highest integrated hourly MWh output demonstrated during the first 168 hours of run time. QSEs do not have to bring units on-line or shut down solely for the purpose of the seasonal verification.

------------------------------------------------------------------------------------------------------------------

### Ancillary Services Qualification Criteria and Portfolio Test Methods

Only QSEs that have been qualified and tested may be used to provide Ancillary Services.  ERCOT shall develop and operate its qualification and testing program to meet the following requirements for each Ancillary Service.

| PRR Evaluation |  |
| --- | --- |
| Non-ERCOT Market Comparison | No Comparison conducted. |
| Comments Author | Comments Summary |
| ERCOT | Commented on potential costs to implement and requested clarification. |