# Texas SET Change Control

Texas SET Change Request

Change Control #2001-143

This TX SET Change Request can be found on the ERCOT website at www.ercot.com  .

| Requester’s Name: Shirley Whyte, Jill Prince & Sharon Pollard on behalf of RMS | Company Name: ITPTA, TXU & AEP | Phone #: 713-906-5224 214-875-1545 |
| --- | --- | --- |
| Date of Request: 7/06/01 | Affected EDI Transaction Set #(s): 867-03 | E-Mail Address: swhyte@mindspring.com jprince1@txu.com |
| Requested Priority: Emergency | Requested Implementation Date: | Status: |

Brief Explanation (This will be copied into the description in the Change Control Summary Spreadsheet):

RMS Metering Workshop determined that the PTD~PP Loop in the 867_03 will  be sent by Non-ERCOT TDSPs to provide net interval usage when the Master Meter is on one ESI ID and the additive and/or subtractive meters are on ESI IDs that are different than the master Meter.

Reason for Request (Explain why this change is needed. For business or technical purposes?):

The changes are requested to meet the business needs defined by RMS.

Detail Explanation  (Exactly what change is required? To which TX SET Standards? Why?):

PTD~PP –  867_03

.

- In PTD~PP, Change designation to DEP. (dependent

Change language in the PTD~PP Gray box  from:  This loop is required for  ERCOT Participants only. to: This loop is always required for  ERCOT TDSPs.  Non-ERCOT TDPS will send this loop only when reporting Master Meter netted interval usage

- Add  Gray Box to PTD~PP

ERCOT TDSPs must report 15-minute intervals in the PTD~PP.  Non-ERCOT TDSPs are not bound by this requirement and may report in other interval increments.

For Additive and Subtractive Metering Only:

Non-ERCOT TDSPs must only transmit this loop to provide netted interval usage for Master Meters.  ERCOT TDSPs always transmit this loop with netted interval usage regardless of whether the meter is a Master Meter.  In order to provide an easy method for flagging the necessity to pick up netted intervals in the PTD~PP loop, an indicator is sent in the PTD~BO.  If the PTD06 in the PTD~BO is equal to “AI” or “AO” the receiver knows that netted interval usage for Master/Additive/Subtractive metering is being provided in the PTD~PP.

Missing or Abundance of Consumption:

Not reported for Interval Meters.

ERCOT uses this loop for settlement purposes.  ERCOT TDSPs provide the value of “ERCOT” in the Power Region in the REF~SR in the Header.  ERCOT uses the Power Region designation to determine if the data in the PTD~PP is intended to be loaded into their system for settlement.   Non-ERCOT TDSPs must choose the appropriate Power Region (not “ERCOT”) so that settlement is not affected.

867_03 Redlines:

Segment:	PTD Product Transfer and Resale Detail (Interval Usage Summary Across Meters)

Position:	010

Loop:	PTD        Mandatory

Level:	Detail

Usage:	Mandatory

Max Use:	1

Purpose:	To indicate the start of detail information relating to the transfer/resale of a product and provide identifying data

Syntax Notes:	1	If either PTD02 or PTD03 is present, then the other is required.

2	If either PTD04 or PTD05 is present, then the other is required.

Semantic Notes:

Comments:

| Notes: |  | PTD = PP (Interval Usage Summary Across Meters) This PTD Loop is used to provide usage data, for interval meters, summarized across meters for each interval. There will be one PTD loop for each unit of measure. For example, if there are three meters measuring kWh, the three kWh values will be added together and one quantity will be provided for each interval. ERCOT TDSPs must report 15-minute intervals in the PTD~PP. Non-ERCOT TDSPs are not bound by this requirement and may report in other interval increments. For Additive and Subtractive Metering Only: Non-ERCOT TDSPs must only transmit this loop to provide netted interval usage for Master Meters. ERCOT TDSPs always transmit this loop with netted interval usage regardless of whether the meter is a Master Meter. In order to provide an easy method for flagging the necessity to pick up netted intervals in the PTD~PP loop, an indicator is sent in the PTD~BO. If the PTD06 in the PTD~BO is equal to “AI” or “AO” the receiver knows that netted interval usage is being provided for Master/Additive/Subtractive metering in the PTD~PP. Missing or Abundance of Consumption: Not reported for Interval Meters. ERCOT uses this loop for settlement purposes. ERCOT TDSPs provide the value of “ERCOT” in the Power Region in the REF~SR in the Header. ERCOT uses the Power Region designation to determine if the data in the PTD~PP is intended to be loaded into their system for settlement. Non-ERCOT TDSPs must choose the appropriate Power Region (not “ERCOT”) so that settlement is not affected. |
| --- | --- | --- |
|  |  | This loop is always required for ERCOT TDSPs. Non-ERCOT TDPS will send this loop only when reporting Master Meter netted interval usage. |
| Dep |  | PTD~PP |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | PTD01 | 521 | Product Transfer Type Code |  |  |  | M |  | ID 2/2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code identifying the type of product transfer |  |  |  |  |  |  |  |  |
|  |  |  |  | PP |  | Pool Level Movement and/or Sale |  |  |  |  |  |
|  |  |  |  |  |  | Interval Level Usage Summary |  |  |  |  |  |

Examples:

Monthly Usage INTERVAL – MASTER METER – INCLUDING MASTER/SUBTRACTIVE METERING  WHEN THE MASTER METER IS ON A DIFFERENT ESI ID THAN THE SUBTRACT METER- - CONSUMPTION TO ERCOT CONTAINS ADJUSTMENT

| TDSP to ERCOT Original Monthly Detailed and Summarized Usage Interval Meter that is measured at the Master meter and includes the subtractive meters (totalized across all subtract meters (1-##)) |  |  |  |  |
| --- | --- | --- | --- | --- |
| ST~867~000000001 |  |  |  | Transaction Type, Control Number |
| BPT~00~2001072343K01~20010731~C1 |  |  |  | Original, Reference Number, Date, Report Type Code (Interval meters Summary Report) |
| REF~Q5~~10111111234567890ABCDEFGHIJKLMNOPQRS |  |  |  | Electric Service Identifier, ESI ID |
| N1~8S~TDSP COMPANY~1~007909411 |  |  |  | TDSP, Name, DUNs Number |
| N1~AY~ERCOT~1~183529049~~41 |  |  |  | ERCOT, Name, DUNs Number, Sender |
| N1~SJ~CR~9~007909422CRN1~~40 |  |  |  | CR, Name, DUNs Number, Receiver |
| PTD~BO~~~MG~1234568MG |  |  |  | Interval Summary Information (across intervals), Meter Number Master Meter |
|  | DTM~150~20010630 |  |  | Service Start, Date |
|  | DTM~151~20010731 |  |  | Service End, Date |
|  | REF~JH~A |  |  | Meter Role, Additive |
|  | REF~MT~KH015 |  |  | Meter Type, Kilowatt Hours per Interval |
|  | QTY~QD~12591 |  | Quantity Delivered, Actual, Quantity Amount equals the consumption of master meter |  |
|  |  | MEA~AF~~~KH~23336~35680~51 |  | Actual/Actual, Measurement Code, Unit of Measurement, Range Minimum, Range Maximum, Measurement Significance Code – Totalizer |
|  |  | MEA~~CO~1.02 |  | Transformer Loss Factor |
|  |  | MEA~~ZA~.95 |  | Power Factor |
| PTD~BO~~~~~AO |  |  |  | Subtractive Metering Code for reporting the total of one or more Subtract meters. Subtractive Metering Code indicates the net interval usage for the Master Meter can be found in the PTD~PP Loop.) |
|  | DTM~150~20010630 |  |  | Service Start, Date |
|  | DTM~151~20010731 |  |  | Service End, Date |
|  | REF~JH~S |  |  | Meter Role, Subtractive |
|  | REF~MT~KH015 |  |  | Meter Type, Kilowatt Hours per Interval |
|  | QTY~QD~2745 |  | Quantity Delivered, Actual, Quantity Amount equals the sum of all subtract meters |  |
| PTD~PM~~~MG~1234568MG |  |  | Interval Detail Information, Master Meter Number |  |
|  | DTM~150~20010630 |  |  | Service Start, Date |
|  | DTM~151~20010731 |  |  | Service End, Date |
|  | REF~6W~1 |  |  | Channel Number |
|  | REF~MT~KH015 |  |  | Meter Type, Kilowatt Hours per Interval |
|  | REF~JH~A |  |  | Meter Role, Additive |
|  | QTY~QD~220 |  |  | Quantity Delivered, Actual, Quantity Amount |
|  |  | DTM~194~20010630~0015 |  | Service End, Date |
|  | QTY~QD~190 |  |  | Quantity Delivered, Actual, Quantity Amount |
|  |  | DTM~194~20010630~0030 |  | Service End, Date |
|  | QTY~QD~180 |  |  | Quantity Delivered, Actual, Quantity Amount |
|  |  | DTM~194~20010630~0045 |  | Service End, Date |
|  | QTY~QD~196 |  |  | Quantity Delivered, Actual, Quantity Amount |
|  |  | DTM~194~20010630~0100 |  | Service End, Date |
|  | QTY~QD~188 |  |  | Quantity Delivered, Actual, Quantity Amount |
|  |  | DTM~194~20010630~0115 |  | Service End, Date |
|  |  |  |  | …Interval-reads repeat at 15-minute increments until they reach 2359 which is the last reading of each day… |
|  | QTY~QD~208 |  |  | Quantity Delivered, Actual, Quantity Amount |
|  |  | DTM~194~20010731~2359 |  | Service End, Date |
| PTD~PP |  |  |  | Interval Detail Information summarized adjusted consumption for ERCOT reporting. Because the PTD06 is “AO” in the PTD~BO. |
|  | DTM~150~20010630~0000 |  |  | Service Start, Date |
|  | DTM~151~20010731~2359 |  |  | Service End, Date |
|  | REF~JH~A |  |  | Meter Role |
|  | REF~MT~KH015 |  |  | Meter Type, Kilowatt Hours per Interval |
|  | QTY~QD~172 |  |  | Quantity Delivered, Actual, Quantity Amount equals the netted sum of this interval for the master meter (ESI ID). |
|  |  | DTM~194~20010630~0015 |  | Service End, Date |
|  | QTY~QD~148 |  |  | Quantity Delivered, Actual, Quantity Amount equals the netted sum of this interval for the master meter (ESI ID). |
|  |  | DTM~194~20010630~0030 |  | Service End, Date |
|  | QTY~QD~141 |  |  | Quantity Delivered, Actual, Quantity Amount equals the netted sum of this interval for the master meter (ESI ID). |
|  |  | DTM~194~20010630~0045 |  | Service End, Date |
|  | QTY~QD~153 |  |  | Quantity Delivered, Actual, Quantity Amount equals the netted sum of this interval for the master meter (ESI ID). |
|  |  | DTM~194~20010630~0100 |  | Service End, Date |
|  | QTY~QD~147 |  |  | Quantity Delivered, Actual, Quantity Amount equals the netted sum of this interval for the master meter (ESI ID). |
|  |  | DTM~194~20010630~0115 |  | Service End, Date |
|  |  |  |  | …Interval-reads repeat at 15-minute increments until they reach 2359 which is the last reading of each day… |
|  | QTY~QD~163 |  |  | Quantity Delivered, Actual, Quantity Amount equals the netted sum of this interval for the master meter. |
|  |  | DTM~194~20010731~2359 |  | Service End, Date |
| SE~2507~000000001 |  |  |  | Number of Segments, Control Number |

For Change Control Manager Use Only:

| Date of TX SET Discussion: | Expected Implementation Date: |  |
| --- | --- | --- |

TX SET Discussion and Resolution:

Priority Classifications

| Emergency Priority | Updated as soon as possible or otherwise directed by TX SET |
| --- | --- |
| High Priority | Changes / Enhancements updated by the next release, or as otherwise directed by TX SET |
| Low Priority | Changes / Enhancements updated no earlier than the next release, or as otherwise directed by TX SET |

Please submit this form via e-mail to txsetchangecontrol@ercot.com .

Your request will be evaluated and prioritized at an upcoming TX SET meeting or conference call.