# Texas SET Change Control Form

Texas SET Change Request

Change Control #2001-172

This TX SET Change Request can be found on the ERCOT website at www.ercot.com  .

| Requester’s Name: Kim Wall/Robert Hill | Company Name: Energy Services Group (ESG) | Phone #: 610-799-2740 |
| --- | --- | --- |
| Date of Request: 9/5/01 | Affected EDI Transaction Set #(s): 867_02 | E-Mail Address: kwall@energyservicesgroup.net |
| Emergency Priority* (Y/N): Y | Requested Implementation Date: Nov 1,2001. | Production Implementation Date: Jan 1, 2002 |
| Testing Required for this CC (Y/N): (Defer to Test Plan Team) | Testing Flight Number: Flight 1001 | Status: |

Brief Explanation (This will be copied into the description in the Change Control Summary Spreadsheet):

Change control #153 attempted to remove meter reads from the historical usage. In doing so, it also removed the ability to distinguish between On Peak, Off Peak, Total, Etc.  This ability must be returned in order for the transaction to be useful for Time of Use meters.

Reason for Request (Explain why this change is needed. For business or technical purposes?):

My colleague Robert Hill was analyzing the changes to the Texas SET Version 1.4 and noticed that the MEA~~PRQ segment was removed from both the PTD~BO (Inveral Summary) and PTD~PL (Non-Interval Detail) loops according to Change Control #153.  Change Control #153 is not appearing on the website so I can't check to see what the intention was.

I'm okay with removing the MEA~~PRQ from the Interval Summary loop because Intervals will not have time of use and I'm not concerned about receiving the meter reads on a historical basis.

I am concerned, however, about removing this segment from the Non-Interval Detail loop (PTD~PL).  There is a statement on the QTY that says, "There will be one QTY loop for each Time of Use (i.e., one for On Peak kWh, one for Off Peak kWh, one for Total kWh, etc.)"

By removing the MEA~~PRQ, you are removing the differentiator between the loops.  Without this segment, multiple QTY loops will be sent, one for each time of use, but there no way to tell which loop is for which time of use.

.

Detail Explanation  (Exactly what change is required? To which TX SET Standards? Why?):

“Un-remove” or “add back” the MEA~~PRQ to the PTD~PL loop in the 867_02 with some changes to it to remove the requirement for providing meter reads. Instead of providing all the elements that are required in version 1.3, only require MEA02, MEA03 and MEA07 (see below).

In Version 1.3, TDSPs are currently providing the MEA segment as defined on page 37 (I’ve copied that page below and I’m showing redlines of how to alter the page).  Note that MEA01, 05 and 06 is what we are trying to get rid of. MEA04 is not necessary because the measurement is included in the Meter Type (and according to PTD segment, there will be one PTD per unit of measure).

For Change Control Manager Use Only:

| Date of TX SET Discussion: | Expected Implementation Date: |  |
| --- | --- | --- |

TX SET Discussion and Resolution:

| *Emergency Priority | Used for Change Controls that require immediate implementation. |
| --- | --- |

Please submit this form via e-mail to txsetchangecontrol@ercot.com .

Your request will be evaluated and prioritized at an upcoming TX SET meeting or conference call.

COPIED FROM VERSION 1.3:

Segment:	MEA Measurements (Meter Reads)

Position:	160

Loop:	QTY        Optional

Level:	Detail

Usage:	Optional

Max Use:	40

Purpose:	To specify physical measurements or counts, including dimensions, tolerances, variances, and weights  (See Figures Appendix for example of use of C001)

Syntax Notes:	1	At least one of MEA03 MEA05 MEA06 or MEA08 is required.

2	If MEA05 is present, then MEA04 is required.

3	If MEA06 is present, then MEA04 is required.

4	If MEA07 is present, then at least one of MEA03 MEA05 or MEA06 is required.

5	Only one of MEA08 or MEA03 may be present.

Semantic Notes:	1	MEA04 defines the unit of measure for MEA03, MEA05, and MEA06.

Comments:	1	When citing dimensional tolerances, any measurement requiring a sign (+ or -), or any measurement where a positive (+) value cannot be assumed, use MEA05 as the negative (-) value and MEA06 as the positive (+) value.

| Notes: |  | PTD = PL (Non-Interval Detail) |
| --- | --- | --- |
|  |  | Required |
|  |  | MEA~~PRQ~1000~~~~51 |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | MEA02 | 738 | Measurement Qualifier |  |  |  | O |  | ID 1/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code identifying a specific product or process characteristic to which a measurement applies |  |  |  |  |  |  |  |  |
|  |  |  |  | PRQ |  | Product Reportable Quantity |  |  |  |  |  |
|  |  |  |  |  |  | Consumption |  |  |  |  |  |
| Must Use | MEA03 | 739 | Measurement Value |  |  |  | X |  | R 1/20 |  |  |
|  |  |  | The value of the measurement |  |  |  |  |  |  |  |  |
|  |  |  | Total consumption after multiplier is applied. This must match QTY02 and is only being populated |  |  |  |  |  |  |  |  |
| Must Use | MEA07 | 935 | Measurement Significance Code |  |  |  | O |  | ID 2/2 |  |  |
|  |  |  | Code used to benchmark, qualify or further define a measurement value |  |  |  |  |  |  |  |  |
|  |  |  |  | 41 |  | Off Peak |  |  |  |  |  |
|  |  |  |  | 42 |  | On Peak |  |  |  |  |  |
|  |  |  |  | 43 |  | Intermediate |  |  |  |  |  |
|  |  |  |  |  |  | Mid-Peak |  |  |  |  |  |
|  |  |  |  | 51 |  | Total |  |  |  |  |  |
|  |  |  |  |  |  | Totalizer/Total/Max (Demand) |  |  |  |  |  |
|  |  |  |  | 71 |  | Low |  |  |  |  |  |
|  |  |  |  |  |  | Summer Super On-Peak |  |  |  |  |  |

COPIED FROM VERSION 1.4

Notice that the MEA~~PRQ was left in the first example.

867_02 Example #1 of 4

Historical Usage NON-INTERVAL METERS

| ERCOT to CR Summarized Historical Usage for Non-Interval Metered Services Months of July and August |  |  |  |
| --- | --- | --- | --- |
| ST~867~000000001 |  |  | Transaction Type, Control Number |
| BPT~52~20010831X0005~20010831~DD~~~~~987987987 |  |  | Request, Reference Number, Date, Report Type Code (Non-interval meters), Reference Identification from the BGN06 of the Historical Usage Request (814_03 or 814_26) |
| REF~Q5~~10111111234567890ABCDEFGHIJKLMNOPQRS |  |  | ESI ID |
| N1~8S~TDSP COMPANY~1~007909411 |  |  | TDSP, Name, DUNs Number |
| N1~AY~ERCOT~1~183529049~~41 |  |  | ERCOT, Name, DUNs Number, Sender |
| N1~SJ~CR~9~007909422CRN1~~40 |  |  | CR, Name, DUNs Number, Receiver |
| PTD~PL~~~MG~1234568G |  |  | Non-interval Summary |
|  | DTM~150~20010630 |  | Service Start, Date |
|  | DTM~151~20010731 |  | Service End, Date |
|  | REF~JH~A |  | Meter Role, Additive |
|  | REF~MT~KHMON |  | Meter Type, Kilowatt Hours per Month |
|  | QTY~QD~1000 |  | Quantity Delivered, Actual, Quantity Amount |
|  |  | MEA~~PRQ~1000~~~~51 | , Consumption, Consumption Value, Measurement Significance Code – Totalizer |
|  |  | MEA~~CO~1.02 | Transformer Loss Factor |
|  |  | MEA~~MU~10 | Multiplier |
|  |  | MEA~~ZA~.95 | Power Factor |
| PTD~PL~~~MG~1234568G |  |  | Non-interval Summary |
|  | DTM~150~20010731 |  | Service Start, Date |
|  | DTM~151~20010830 |  | Service End, Date |
|  | REF~JH~A |  | Meter Role, Additive |
|  | REF~MT~KHMON |  | Meter Type, Kilowatt Hours per Month |
|  | QTY~QD~940 |  | Quantity Delivered, Actual, Quantity Amount |
|  |  | MEA~~PRQ~940~~~~51 | Consumption, Consumption Value, Measurement Significance Code – Totalizer |
|  |  | MEA~~CO~1.02 | Transformer Loss Factor |
|  |  | MEA~~MU~10 | Multiplier |
|  |  | MEA~~ZA~.95 | Power Factor |
| SE~27~000000001 |  |  | Number of Segments, Control Number |

867_02 Historical Usage Example #4 of 4

NON-INTERVAL METERS, METER CHANGE OUT – MIXED VALUES

| ERCOT to CR Historical Usage for (one month) a Partial Month of Non-Interval Metered Services Meter Exchange to an Interval Meter and the remainder of the Month Reporting Interval Usage |  |  |  |
| --- | --- | --- | --- |
| ST~867~000000001 |  |  | Transaction Type, Control Number |
| BPT~52~20010731X0008~20010731~DR~~~~~987987987 |  |  | Historical Usage Indicator, Reference Number, Date, Report Type Code (Non-interval meters and interval meters ), Reference Identification from the BGN06 of the Historical Usage Request (814_03 or 814_26) |
| REF~Q5~~10111111234567890ABCDEFGHIJKLMNOPQRS |  |  | ESI ID |
| N1~8S~TDSP COMPANY~1~007909411 |  |  | TDSP, Name, DUNs Number |
| N1~AY~ERCOT~1~183529049~~41 |  |  | ERCOT, Name, DUNs Number, Sender |
| N1~SJ~CR~9~007909422CRN1~~40 |  |  | CR, Name, DUNs Number, Receiver |
| PTD~BO~~~MG~8654321H |  |  | Interval Summary Information, Meter Number |
|  | DTM~514~20010717 |  | Meter Exchange, Date |
|  | DTM~151~20010731 |  | Service End, Date |
|  | REF~JH~A |  | Meter Role, Additive |
|  | REF~MT~KH015 |  | Meter Type, Kilowatt Hours per Interval |
|  | QTY~QD~274500 |  | Quantity Delivered, Actual, Quantity Amount |
|  |  | MEA~~CO~1.02 | Transformer Loss Factor |
|  |  | MEA~~ZA~.95 | Power Factor |
| PTD~BO~~~MG~8654321H |  |  | Interval Summary Information, Meter Number |
|  | DTM~150~20010731 |  | Meter Exchange, Date |
|  | DTM~151~20010830 |  | Service End, Date |
|  | REF~JH~A |  | Meter Role, Additive |
|  | REF~MT~KH015 |  | Meter Type, Kilowatt Hours per Interval |
|  | QTY~QD~633600 |  | Quantity Delivered, Actual, Quantity Amount |
|  |  | MEA~~CO~1.02 | Transformer Loss Factor |
|  |  | MEA~~ZA~.95 | Power Factor |
| PTD~PL~~~MG~1234568G |  |  | Non-interval Summary, Meter Number |
|  | DTM~150~20010630 |  | Service Start, Date |
|  | DTM~514~20010717 |  | Exchange Date, Date |
|  | REF~JH~A |  | Meter Role, Additive |
|  | REF~MT~KHMON |  | Meter Type, Kilowatt Hours per Month |
|  | QTY~QD~1000 |  | Quantity Delivered, Actual, Quantity Amount |
|  |  | MEA~~PRQ~1000~~~~51 | Consumption, Consumption Value, Measurement Significance Code – Totalizer |
|  |  | MEA~~CO~1.02 | Transformer Loss Factor |
|  |  | MEA~~MU~10 | Multiplier |
|  |  | MEA~~ZA~.95 | Power Factor |
| PTD~PM~~~MG~8654321H |  |  | Interval Summary, Meter Number |
|  | DTM~514~20010717~0000 |  | Exchange Date, Date |
|  | DTM~151~20010731 |  | Service End, Date |
|  | REF~6W~2 |  | Channel Number |
|  | REF~MT~KH015 |  | Meter Type, Kilowatt Hours per Interval |
|  | REF~JH~A |  | Meter Role, Additive |
|  | QTY~QD~220 |  | Quantity Delivered, Actual, Quantity Amount |
|  | DTM~194~20010717~0015 |  | Service End, Date |
|  | QTY~QD~190 |  | Quantity Delivered, Actual, Quantity Amount |
|  | DTM~194~20010717~0030 |  | Service End, Date |
|  | QTY~QD~180 |  | Quantity Delivered, Actual, Quantity Amount |
|  | DTM~194~20010717~0045 |  | Service End, Date |
|  | QTY~QD~196 |  | Quantity Delivered, Actual, Quantity Amount |
|  | DTM~194~20010717~0100 |  | Service End, Date |
|  | QTY~QD~188 |  | Quantity Delivered, Actual, Quantity Amount |
|  | DTM~194~20010717~0115 |  | Service End, Date |
|  |  |  | …Interval-reads repeat at 15-minute increments until they reach 2359 which is the last reading of each day… |
|  | QTY~QD~208 |  | Quantity Delivered, Actual, Quantity Amount |
|  | DTM~194~20010731~2359 |  | Service End, Date |
| PTD~PM~ ~~MG~8654321H |  |  | Interval Summary, Meter Number |
|  | DTM~514~20010731~0000 |  | Exchange Date, Date |
|  | DTM~151~20010830 |  | Service End, Date |
|  | REF~6W~2 |  | Channel Number |
|  | REF~MT~KH015 |  | Meter Type, Kilowatt Hours per Interval |
|  | REF~JH~A |  | Meter Role, Additive |
|  | QTY~QD~220 |  | Quantity Delivered, Actual, Quantity Amount |
|  | DTM~194~20010731~0015 |  | Service End, Date |
|  | QTY~QD~190 |  | Quantity Delivered, Actual, Quantity Amount |
|  | DTM~194~20010731~0030 |  | Service End, Date |
|  | QTY~QD~180 |  | Quantity Delivered, Actual, Quantity Amount |
|  | DTM~194~20010731~0045 |  | Service End, Date |
|  | QTY~QD~196 |  | Quantity Delivered, Actual, Quantity Amount |
|  | DTM~194~20010731~0100 |  | Service End, Date |
|  | QTY~QD~188 |  | Quantity Delivered, Actual, Quantity Amount |
|  | DTM~194~20010731~0115 |  | Service End, Date |
|  |  |  | …Interval-reads repeat at 15-minute increments until they reach 2359 which is the last reading of each day… |
|  | QTY~QD~208 |  | Quantity Delivered, Actual, Quantity Amount |
|  | DTM~194~20010830~2359 |  | Service End, Date |
| SE~4225~000000001 |  |  | Number of Segments, Control Number |

867_02 Historical Usage Example #4 of 4

NON-INTERVAL METERS, METER CHANGE OUT – MIXED VALUES

| ERCOT to CR Historical Usage for (one month) a Partial Month of Non-Interval Metered Services Meter Exchange to an Interval Meter and the remainder of the Month Reporting Interval Usage |  |  |  |
| --- | --- | --- | --- |
| ST~867~000000001 |  |  | Transaction Type, Control Number |
| BPT~52~20010731X0008~20010731~DR~~~~~987987987 |  |  | Historical Usage Indicator, Reference Number, Date, Report Type Code (Non-interval meters and interval meters ), Reference Identification from the BGN06 of the Historical Usage Request (814_03 or 814_26) |
| REF~Q5~~10111111234567890ABCDEFGHIJKLMNOPQRS |  |  | ESI ID |
| N1~8S~TDSP COMPANY~1~007909411 |  |  | TDSP, Name, DUNs Number |
| N1~AY~ERCOT~1~183529049~~41 |  |  | ERCOT, Name, DUNs Number, Sender |
| N1~SJ~CR~9~007909422CRN1~~40 |  |  | CR, Name, DUNs Number, Receiver |
| PTD~BO~~~MG~8654321H |  |  | Interval Summary Information, Meter Number |
|  | DTM~514~20010717 |  | Meter Exchange, Date |
|  | DTM~151~20010731 |  | Service End, Date |
|  | REF~JH~A |  | Meter Role, Additive |
|  | REF~MT~KH015 |  | Meter Type, Kilowatt Hours per Interval |
|  | QTY~QD~274500 |  | Quantity Delivered, Actual, Quantity Amount |
|  |  | MEA~~CO~1.02 | Transformer Loss Factor |
|  |  | MEA~~ZA~.95 | Power Factor |
| PTD~BO~~~MG~8654321H |  |  | Interval Summary Information, Meter Number |
|  | DTM~150~20010731 |  | Meter Exchange, Date |
|  | DTM~151~20010830 |  | Service End, Date |
|  | REF~JH~A |  | Meter Role, Additive |
|  | REF~MT~KH015 |  | Meter Type, Kilowatt Hours per Interval |
|  | QTY~QD~633600 |  | Quantity Delivered, Actual, Quantity Amount |
|  |  | MEA~~CO~1.02 | Transformer Loss Factor |
|  |  | MEA~~ZA~.95 | Power Factor |
| PTD~PL~~~MG~1234568G |  |  | Non-interval Summary, Meter Number |
|  | DTM~150~20010630 |  | Service Start, Date |
|  | DTM~514~20010717 |  | Exchange Date, Date |
|  | REF~JH~A |  | Meter Role, Additive |
|  | REF~MT~KHMON |  | Meter Type, Kilowatt Hours per Month |
|  | QTY~QD~1000 |  | Quantity Delivered, Actual, Quantity Amount |
|  | MEA~~CO~1.02 |  | Transformer Loss Factor |
|  | MEA~~MU~10 |  | Multiplier |
|  | MEA~~ZA~.95 |  | Power Factor |
| PTD~PM~~~MG~8654321H |  |  | Interval Summary, Meter Number |
|  | DTM~514~20010717~0000 |  | Exchange Date, Date |
|  | DTM~151~20010731 |  | Service End, Date |
|  | REF~6W~2 |  | Channel Number |
|  | REF~MT~KH015 |  | Meter Type, Kilowatt Hours per Interval |
|  | REF~JH~A |  | Meter Role, Additive |
|  | QTY~QD~220 |  | Quantity Delivered, Actual, Quantity Amount |
|  | DTM~194~20010717~0015 |  | Service End, Date |
|  | QTY~QD~190 |  | Quantity Delivered, Actual, Quantity Amount |
|  | DTM~194~20010717~0030 |  | Service End, Date |
|  | QTY~QD~180 |  | Quantity Delivered, Actual, Quantity Amount |
|  | DTM~194~20010717~0045 |  | Service End, Date |
|  | QTY~QD~196 |  | Quantity Delivered, Actual, Quantity Amount |
|  | DTM~194~20010717~0100 |  | Service End, Date |
|  | QTY~QD~188 |  | Quantity Delivered, Actual, Quantity Amount |
|  | DTM~194~20010717~0115 |  | Service End, Date |
|  |  |  | …Interval-reads repeat at 15-minute increments until they reach 2359 which is the last reading of each day… |
|  | QTY~QD~208 |  | Quantity Delivered, Actual, Quantity Amount |
|  | DTM~194~20010731~2359 |  | Service End, Date |
| PTD~PM~ ~~MG~8654321H |  |  | Interval Summary, Meter Number |
|  | DTM~514~20010731~0000 |  | Exchange Date, Date |
|  | DTM~151~20010830 |  | Service End, Date |
|  | REF~6W~2 |  | Channel Number |
|  | REF~MT~KH015 |  | Meter Type, Kilowatt Hours per Interval |
|  | REF~JH~A |  | Meter Role, Additive |
|  | QTY~QD~220 |  | Quantity Delivered, Actual, Quantity Amount |
|  | DTM~194~20010731~0015 |  | Service End, Date |
|  | QTY~QD~190 |  | Quantity Delivered, Actual, Quantity Amount |
|  | DTM~194~20010731~0030 |  | Service End, Date |
|  | QTY~QD~180 |  | Quantity Delivered, Actual, Quantity Amount |
|  | DTM~194~20010731~0045 |  | Service End, Date |
|  | QTY~QD~196 |  | Quantity Delivered, Actual, Quantity Amount |
|  | DTM~194~20010731~0100 |  | Service End, Date |
|  | QTY~QD~188 |  | Quantity Delivered, Actual, Quantity Amount |
|  | DTM~194~20010731~0115 |  | Service End, Date |
|  |  |  | …Interval-reads repeat at 15-minute increments until they reach 2359 which is the last reading of each day… |
|  | QTY~QD~208 |  | Quantity Delivered, Actual, Quantity Amount |
|  | DTM~194~20010830~2359 |  | Service End, Date |
| SE~4225~000000001 |  |  | Number of Segments, Control Number |