Texas SET Change Control

Texas SET Change Request

Change Control #2001-145

This TX SET Change Request can be found on the ERCOT website at www.ercot.com  .

| Requester’s Name: Shirley Whyte, Jill Prince, Sharon Pollard on behalf of RMS | Company Name: ITPTA, TXU & AEP | Phone #: 713-906-5224 214-875-1545 |
| --- | --- | --- |
| Date of Request: 7/06/01 | Affected EDI Transaction Set #(s): 867-03 | E-Mail Address: swhyte@mindspring.com jprince1@txu.com |
| Requested Priority: Emergency | Requested Implementation Date: | Status: |

Brief Explanation (This will be copied into the description in the Change Control Summary Spreadsheet):

RMS Issue #61

RMS Metering Workshop determined that a new loop is required to provide net interval usage at the ESI ID level.

Reason for Request (Explain why this change is needed. For business or technical purposes?):

. This new net usage loop applies only to interval meters and the data elements for this new loop are defined below. (Non-interval loop PTD~SU already addresses net usage).The changes are requested to meet the business needs defined by RMS.

Detail Explanation  (Exactly what change is required? To which TX SET Standards? Why?):

- Add PTD~IA – 867_03

Placement of the PTD~IA is to be after the PTD~BD and before the PTD~BO.

New loop must contain the following data elements

  - Date range

  - Net billable consumption

  - Meter role = ignore

  - Meter type (unit of measure; kWh/kVArh only)

  - PTD Flag to indicate that this is the net usage loop.

The new loop will not repeat the listing of following data elements as they are already provided in the BO loop (IDR), PL Loop (non-IDR), PM Loop (IDR):

  - Transformer loss (included in usage, but factor communicated so it can be backed out at CR discretion)

  - Power factor

  - Meter number

  - Meter reads

Segment:	PTD Product Transfer and Resale Detail (Net Interval Usage Summary)

Position:	010

Loop:	PTD        Mandatory

Level:	Detail

Usage:	Mandatory

Max Use:	1

Purpose:	To indicate the start of detail information relating to the transfer/resale of a product and provide identifying data

Syntax Notes:	1	If either PTD02 or PTD03 is present, then the other is required.

2	If either PTD04 or PTD05 is present, then the other is required.

Semantic Notes:

Comments:

| Notes: |  | PTD = IA (Net Interval Usage Summary) One PTD loop is required for each unit of measure at the ESI ID. This loop is required to provide summarized net interval usage at the ESI ID level. This net usage loop applies only to interval meters. The PTD~IA Loop provides the net of the Interval summary (PTD~BO) loops and also provides a control total for the total of the intervals reported in the Interval Usage Summary Across Meters (PTD~PP) loop. For Additive/Subtractive Metering Only: ERCOT and NON-ERCOT TDSPs provide the PTD~IA for the monthly summarized net interval usage for the Master Meter when the PTD~BO element PTD06 equals “AI” or “AO”. Missing or Abundance of Consumption: Not reported for Interval Meters. |
| --- | --- | --- |
|  |  | PTD~IA |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | PTD01 | 521 | Product Transfer Type Code |  |  |  | M |  | ID 2/2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code identifying the type of product transfer |  |  |  |  |  |  |  |  |
|  |  |  |  | IA |  | Inventory Adjustment |  |  |  |  |  |
|  |  |  |  |  |  | Interval Usage Summary |  |  |  |  |  |

Segment:	REF Reference Identification (Meter Type)

Position:	030

Loop:	PTD        Mandatory

Level:	Detail

Usage:	Optional

Max Use:	20

Purpose:	To specify identifying information

Syntax Notes:	1	At least one of REF02 or REF03 is required.

2	If either C04003 or C04004 is present, then the other is required.

3	If either C04005 or C04006 is present, then the other is required.

Semantic Notes:	1	REF04 contains data relating to the value cited in REF02.

Comments:

| Notes: |  | PTD = IA (Net Interval Usage Summary) This segment is sent to indicate the type of usage that is reported in this PTD loop. |
| --- | --- | --- |
|  |  | Required |
|  |  | REF~MT~KHMON |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | REF01 | 128 | Reference Identification Qualifier |  |  |  | M |  | ID 2/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code qualifying the Reference Identification |  |  |  |  |  |  |  |  |
|  |  |  |  | MT |  | Meter Ticket Number |  |  |  |  |  |
|  |  |  |  |  |  | Meter Type |  |  |  |  |  |
| Must Use | REF02 | 127 | Reference Identification |  |  |  | X |  | AN 1/30 |  |  |
|  |  |  | Reference information as defined for a particular Transaction Set or as specified by the Reference Identification Qualifier |  |  |  |  |  |  |  |  |
|  |  |  | When REF01 is MT, the meter type is expressed as a five-character field. The first two characters are the type of consumption, the last three characters are the metering interval reported by the metering agent. "COMBO" is used for a meter that records more than one measurement, but may not be used for reporting consumption. Valid values can be a combination of the following values: Type of Consumption K3 Kilovolt Amperes Reactive Hour (kVARH) KH Kilowatt Hour (kWh) Metering Interval Reported for Billing Purposes nnn Number of minutes from 001 to 999 For Example: KH060 Kilowatt Hours for hourly interval meter K3015 Kilovolt Amperes Reactive Hours for 15 minute interval meter |  |  |  |  |  |  |  |  |
|  |  |  | "COMBO" can not be used when reporting usage. |  |  |  |  |  |  |  |  |

Segment:	QTY Quantity

Position:	110

Loop:	QTY        Optional

Level:	Detail

Usage:	Optional

Max Use:	1

Purpose:	To specify quantity information

Syntax Notes:	1	At least one of QTY02 or QTY04 is required.

2	Only one of QTY02 or QTY04 may be present.

Semantic Notes:	1	QTY04 is used when the quantity is non-numeric.

Comments:

| Notes: |  | PTD = IA (Net Interval Usage Summary) Note: The billable quantity is the total measured consumption for the billable period including all billable adjustments applicable, e.g., transformer loss factor and the meter multiplier. The consumption in this loop has been netted. Billable quantity in the 867_03 may not match the invoiced quantity in the 810_02 because of TDSP tariff adjustments to the 810_02 data. |
| --- | --- | --- |
|  |  | QTY~QD~22348 |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | QTY01 | 673 | Quantity Qualifier |  |  |  | M |  | ID 2/2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code specifying the type of quantity |  |  |  |  |  |  |  |  |
|  |  |  |  | KA |  | Estimated |  |  |  |  |  |
|  |  |  |  |  |  | Quantity is estimated if the usage from one or more meters was estimated. |  |  |  |  |  |
|  |  |  |  | QD |  | Quantity Delivered |  |  |  |  |  |
|  |  |  |  |  |  | Quantity is actual for all meters |  |  |  |  |  |
| Must Use | QTY02 | 380 | Quantity |  |  |  | X |  | R 1/15 |  |  |
|  |  |  | Numeric value of quantity |  |  |  |  |  |  |  |  |

Segment:	DTM Date/Time Reference (Service Period Start Date)

Position:	210

Loop:	QTY        Optional

Level:	Detail

Usage:	Optional

Max Use:	10

Purpose:	To specify pertinent dates and times

Syntax Notes:	1	At least one of DTM02 DTM03 or DTM05 is required.

2	If DTM04 is present, then DTM03 is required.

3	If either DTM05 or DTM06 is present, then the other is required.

Semantic Notes:

Comments:

| Notes: |  | PTD = IA (Net Interval Usage Summary) Required |
| --- | --- | --- |
|  |  | DTM~150~20010630 |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | DTM01 | 374 | Date/Time Qualifier |  |  |  | M |  | ID 3/3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code specifying type of date or time, or both date and time |  |  |  |  |  |  |  |
|  |  |  |  | 150 |  | Service Period Start |  |  |  |  |
| Must Use | DTM02 | 373 | Date |  |  |  | X |  | DT 8/8 |  |
|  |  |  | Date expressed as CCYYMMDD |  |  |  |  |  |  |  |

Segment:	DTM Date/Time Reference (Service Period End Date)

Position:	210

Loop:	QTY        Optional

Level:	Detail

Usage:	Optional

Max Use:	10

Purpose:	To specify pertinent dates and times

Syntax Notes:	1	At least one of DTM02 DTM03 or DTM05 is required.

2	If DTM04 is present, then DTM03 is required.

3	If either DTM05 or DTM06 is present, then the other is required.

Semantic Notes:

Comments:

| Notes: |  | PTD = IA (Net Interval Usage Summary) Required |
| --- | --- | --- |
|  |  | DTM~151~20010730 |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | DTM01 | 374 | Date/Time Qualifier |  |  |  | M |  | ID 3/3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code specifying type of date or time, or both date and time |  |  |  |  |  |  |  |
|  |  |  |  | 151 |  | Service Period End |  |  |  |  |
| Must Use | DTM02 | 373 | Date |  |  |  | X |  | DT 8/8 |  |
|  |  |  | Date expressed as CCYYMMDD |  |  |  |  |  |  |  |

867_03 Example #6 of  12

Monthly Usage  INTERVAL – ORIGINAL

| ERCOT to CR Interval Summary, Detail, and Summary Across Meters Monthly Usage Report |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ST~867~000000001 |  |  |  |  | Transaction Type, Control Number |
| BPT~00~20010498B0001~20010731~C1 |  |  |  |  | Original, Reference Number, Date, Report Type Code (Interval meters Summary Report) |
| REF~Q5~~10111111234567890ABCDEFGHIJKLMNOPQRS |  |  |  |  | Electric Service Identifier, ESI ID |
| N1~8S~TDSP COMPANY~1~007909411 |  |  |  |  | TDSP, Name, DUNs Number |
| N1~AY~ERCOT~1~183529049~~41 |  |  |  |  | ERCOT, Name, DUNs Number, Sender |
| N1~SJ~CR~9~007909422CRN1~~40 |  |  |  |  | CR, Name, DUNs Number, Receiver |
| PTD~BO~~~MG~1234568MG |  |  |  |  | Interval Summary Information (across intervals), Meter Number |
|  | DTM~150~20010630 |  |  |  | Service Start, Date |
|  | DTM~151~20010731 |  |  |  | Service End, Date |
|  | REF~JH~A |  |  |  | Meter Role, Additive |
|  | REF~MT~KH015 |  |  |  | Meter Type, Kilowatt Hours per Interval |
|  | QTY~QD~232385 |  |  |  | Quantity Delivered, Actual, Quantity Amount equals the sum measured on both channels |
|  |  |  | MEA~AF~~~KH~23336~251164~51 |  | Actual/Actual, Consumption, Consumption Value, Measurement Code, Unit of Measurement, Range Minimum, Range Maximum, Measurement Significance Code – Totalizer |
|  |  |  | MEA~~CO~1.02 |  | Transformer Loss Factor |
|  |  |  | MEA~~ZA~.95 |  | Power Factor |
| PTD~PM~~~MG~1234568MG |  |  |  |  | Interval Detail Information, Meter Number |
|  | DTM~150~20010630 |  |  |  | Service Start, Date |
|  | DTM~151~20010731 |  |  |  | Service End, Date |
|  | REF~6W~1 |  |  |  | Channel Number |
|  | REF~MT~KH015 |  |  |  | Meter Type, Kilowatt Hours per Interval |
|  | REF~JH~A |  |  |  | Meter Role, Additive |
|  | QTY~QD~220 |  |  |  | Quantity Delivered, Actual, Quantity Amount |
|  |  |  | DTM~194~20010630~0015 |  | Service End, Date |
|  | QTY~QD~190 |  |  |  | Quantity Delivered, Actual, Quantity Amount |
|  |  |  | DTM~194~20010630~0030 |  | Service End, Date |
|  | QTY~QD~180 |  |  |  | Quantity Delivered, Actual, Quantity Amount |
|  |  |  | DTM~194~20010630~0045 |  | Service End, Date |
|  | QTY~QD~196 |  |  |  | Quantity Delivered, Actual, Quantity Amount |
|  |  |  | DTM~194~20010630~0100 |  | Service End, Date |
|  | QTY~QD~188 |  |  |  | Quantity Delivered, Actual, Quantity Amount |
|  |  |  | DTM~194~20010630~0115 |  | Service End, Date |
|  |  |  |  |  | …Interval-reads repeat at 15-minute increments until they reach 2359 which is the last reading of each day… |
|  | QTY~QD~208 |  |  |  | Quantity Delivered, Actual, Quantity Amount |
|  |  |  | DTM~194~20010731~2359 |  | Service End, Date |
| PTD~PM~~~MG~1234568MG |  |  |  |  | Interval Detail Information, Meter Number |
|  | DTM~150~20010630 |  |  |  | Service Start, Date |
|  | DTM~151~20010731 |  |  |  | Service End, Date |
|  | REF~6W~2 |  |  |  | Channel Number |
|  | REF~MT~KH015 |  |  |  | Meter Type, Kilowatt Hours per Interval |
|  | REF~JH~A |  |  |  | Meter Role, Additive |
|  | QTY~QD~220 |  |  |  | Quantity Delivered, Actual, Quantity Amount |
|  |  |  | DTM~194~20010630~0015 |  | Service End, Date |
|  | QTY~QD~190 |  |  |  | Quantity Delivered, Actual, Quantity Amount |
|  |  |  | DTM~194~20010630~0030 |  | Service End, Date |
|  | QTY~QD~180 |  |  |  | Quantity Delivered, Actual, Quantity Amount |
|  |  |  | DTM~194~20010630~0045 |  | Service End, Date |
|  | QTY~QD~196 |  |  |  | Quantity Delivered, Actual, Quantity Amount |
|  |  |  | DTM~194~20010630~0100 |  | Service End, Date |
|  | QTY~QD~188 |  |  |  | Quantity Delivered, Actual, Quantity Amount |
|  |  |  | DTM~194~20010630~0115 |  | Service End, Date |
|  |  |  |  |  | …Interval-reads repeat at 15-minute increments until they reach 2359 which is the last reading of each day… |
|  | QTY~QD~208 |  |  |  | Quantity Delivered, Actual, Quantity Amount |
|  |  |  | DTM~194~20010731~2359 |  | Service End, Date |
| PTD~PP |  |  |  |  | Interval Detail Information |
|  | DTM~150~20010630~0000 |  |  |  | Service Start, Date |
|  | DTM~151~20010731~2359 |  |  |  | Service End, Date |
|  | REF~JH~A |  |  |  | Meter Role |
|  | REF~MT~KH015 |  |  |  | Meter Type, Kilowatt Hours per Interval |
|  | QTY~QD~440 |  |  |  | Quantity Delivered, Actual, Quantity Amount equals sum of this interval for both channels |
|  |  |  | DTM~194~20010630~0015 |  | Service End, Date |
|  | QTY~QD~380 |  |  |  | Quantity Delivered, Actual, Quantity Amount equals sum of this interval for both channels |
|  |  |  | DTM~194~20010630~0030 |  | Service End, Date |
|  | QTY~QD~360 |  |  |  | Quantity Delivered, Actual, Quantity Amount equals sum of this interval for both channels |
|  |  |  | DTM~194~20010630~0045 |  | Service End, Date |
|  | QTY~QD~392 |  |  |  | Quantity Delivered, Actual, Quantity Amount equals sum of this interval for both channels |
|  |  |  | DTM~194~20010630~0100 |  | Service End, Date |
|  | QTY~QD~376 |  |  |  | Quantity Delivered, Actual, Quantity Amount equals sum of this interval for both channels |
|  |  |  | DTM~194~20010630~0115 |  | Service End, Date |
|  |  |  |  |  | …Interval-reads repeat at 15-minute increments until they reach 2359 which is the last reading of each day… |
|  | QTY~QD~416 |  |  |  | Quantity Delivered, Actual, Quantity Amount equals sum of this interval for both channels |
|  |  |  | DTM~194~20010731~2359 |  | Service End, Date |
| PTD~IA |  |  |  |  | Net Interval Usage Summary (Netted KWH Usage at ESI ID) |
|  |  | REF~MT~KH015 |  |  | Meter Type, Reference Identification – kWh per monthfor 15 minute interval meter |
|  |  | QTY~QD~232385 |  |  | Quantity delivered, Actual, Actual Quantity |
|  |  |  |  | MEA~~PRQ~232385~~~~51 | Consumption, Actual Measurement Value, Measurement Significance Code - Totalizer |
|  |  |  |  | DTM~150~20010630 | Service Start, Date |
|  |  |  |  | DTM~151~20010731 | Service End, Date |
| SE~2500~000000001 |  |  |  |  | Number of Segments, Control Number |

For Change Control Manager Use Only:

| Date of TX SET Discussion: | Expected Implementation Date: |  |
| --- | --- | --- |

TX SET Discussion and Resolution:

Priority Classifications

| Emergency Priority | Updated as soon as possible or otherwise directed by TX SET |
| --- | --- |
| High Priority | Changes / Enhancements updated by the next release, or as otherwise directed by TX SET |
| Low Priority | Changes / Enhancements updated no earlier than the next release, or as otherwise directed by TX SET |

Please submit this form via e-mail to txsetchangecontrol@ercot.com .