| PRR Number | 367PRR | PRR Title | IDR Installation & Use |
| --- | --- | --- | --- |
| ###### Recommended Action |  | Approve |  |
| Proposed Effective Date |  | February 1, 2003 |  |
| Priority & Rank Assigned |  | N/A |  |
| Summary of Impact Analysis |  | Does not impact the ERCOT computer system. |  |
| Benefit |  | Provides clarification. |  |
| PRR Background |  | 1) The present wording may be misconstrued to imply that a CR can have a TDSP install an IDR for load research purposes, and not be required to use the interval data from that IDR in settlement. To close this gap, the proposed wording limits the exclusion of using the interval data in settlement to the entities for which this paragraph was originally intended to cover. Additionally, IDRs are not installed by ERCOT. However, IDRs may be installed at the request of ERCOT. The proposed wording also addresses this technicality. ## 2) The present wording does not capture the original intent, which was to determine a coincident peak demand. It could be argued that a demand meter can be removed from a Small Non-Residential Premise (up to 1000 kW) and replaced with an IDR that would not have to be utilized for settlement, because the IDR is used to determine (or calculate) the demand. This is contrary to the intent of Section 18.6.1. The proposed wording clarifies the original intent. |  |
| Protocol Revision Subcommittee Recommendation |  | PRS recommends approval as submitted. |  |
| ###### Technical Advisory Committee Recommendation |  | TAC recommends approval of PRR367 as recommended by PRS. |  |

Recommended Language

## 18.6  Mandatory Installation and Use of Interval Data Recorders

### 18.6.1  Interval Data Recorder (IDR) Installation and Use in Settlement

- IDR Requirement:  IDRs shall be installed and utilized for settlement of Premises having either:

  - A peak demand greater than 1000 kW (or 1000 kVa), or

  - Service provided at transmission voltage (above 60 kV).

- A Competitive Retailer may have an IDR installed and used for settlement purposes at any associated Premise outside the IDR Requirement.  Except as stated in item (4) of this subsection, IDRs in place or installed after September 1, 1999 shall be used for settlement.  Once an IDR is installed on a Premise and used for settlement purposes, the given premise shall continue to be settled with its interval data.

- All Non-Metered Loads such as street lighting, regardless of the aggregation level, shall not be required to install IDRs under the IDR Requirement.  These loads shall be settled using Load Profiles.

- For premises not subject to the IDR Requirement in item (1) of this subsection, IDRs installed at the request of ERCOT, a TDSP, a municipal, or a cooperative for load research, rate/tariff design calculation, coincident demand calculation, or Load Profiling purposes shall be exempt from the requirement to use an IDR for settlement purposes.

- For IDR installation procedures reference Section 10.2.2, TDSP Metered Entities.

- TDSPs responsible for any Load transfer schemes between ERCOT and Non-ERCOT regions shall install IDR metering capable of measuring the load served during the period the Load transfer is implemented.

| PRR Evaluation |  |
| --- | --- |
| Non-ERCOT Market Comparison | No Comparison conducted. |
| Comments Author | Comments Summary |
|  | None submitted. |