| PRR Number | 324PRR | PRR Title | EPS Meter Polling Timeline |
| --- | --- | --- | --- |
| ###### Recommended Action |  | Approve |  |
| Recommended Effective Date |  | June 1, 2002 |  |
| System Impact |  | None. |  |
| Benefit |  | Eases meter staffing burden on weekends and holidays for ERCOT and TDSPs. For ERCOT it reduces the need for metering staff to work overtime and reduces the current need for additional meter staff. |  |
| PRR Background |  | ## The Settlement Calendar timeline has been changed to 17-days, from 3-days (by 298PRR), thus making the requirement for daily dialing of EPS meters unnecessary. In addition, the verbiage considers future changes to the Settlement Calendar timeline and will not need revising if the Settlement Calendar timeline changes. These modifications to Protocols will allow ERCOT to discontinue staffing the MV90 operations on weekends thus reducing O&M costs. These changes will not affect availability or quality of data for settlements or to Market Participants. |  |
| Protocol Revision Subcommittee Recommendation |  | PRS recommends approval as submitted. |  |

Recommended Language

Section 10.3.2:

### ERCOT Polled Settlement Meters (EPS)

Each TDSP shall, in accordance with these Protocols and the Operating Guides, provide ERCOT approved metering communication equipment and connection to permit ERCOT access to the TDSP’s EPS Meters.

ERCOT will retrieve meter data electronically and automatically by MDAS on a daily basis on a schedule that ensures quality meter data in support of Settlement Metering timelines.   ERCOT may also collect meter data on demand.

Section 11.1.1

### Overview

ERCOT will collect interval data on a daily basis from all ERCOT Polled Settlement (EPS) metered Entities according to Section 10, Metering of these Protocols.  Collection of data from EPS metered Entities will be done via the Meter Data Acquisition System (MDAS). This data will be validated, edited, estimated, adjusted, netted, loss corrected, split, and aggregated as necessary to provide the required settlement inputs.

Section 11.1.2

EPS meter data acquisition will be performed on the day following the Operating Day.  ERCOT will perform remote interrogation of EPS metered Entities to provide the necessary data for the Settlement process.   Upon initiation of connection with the meter, the MDAS will verify that the meter’s internal IDR protocol (TIM setting) and the device identifier programmed into the IDR match the master file database stored in the MDAS.  If remote-polling fails for any reason, ERCOT will work closely with the TDSP to resolve data collection problems within the time frame defined in Section 10, Metering.

Section 11.1.4

### EPS Meter Data Validation, Editing, and Estimation (VEE)

After EPS time synchronization has been completed and interval meter data has been retrieved, ERCOT will determine if the data is valid.   The validation process will include, but not be limited to the following tests:

- Flagging of intervals with missing data.

- Exception reporting if the total number of zero values for any channel exceeds the tolerance limit.

- Exception reporting if the total number of power outage intervals exceeds the tolerance limit.

- Channel level exception reporting if any single interval breaches the upper or lower threshold of the limit.

- Channel level validation of the percent change between two consecutive intervals being greater than the established tolerance limit.

- Data overlap validation test, which rejects validations when the current interrogation of data overlaps data previously collected.

- Channel level energy tolerance test, which reports exceptions of total energy accumulated from the interval data not being equivalent to the energy calculated from the meter register’s start and stop readings.

- Validation that the number of expected intervals equals the number of actual intervals collected during the interrogation process.

- Validation of data between primary, backup and check meters where available.

ERCOT will perform editing and estimation of EPS meter data according to the Protocols defined in Section 10, Metering.  The Validation, Editing, and Estimation (VEE) process occurs each time data is collected from a meter.

| PRR Evaluation |  |
| --- | --- |
| Non-ERCOT Market Comparison | No Comparison conducted. |
| Comments Author | Comments Summary |
|  | None submitted. |