| PRR Number | 349PRR | PRR Title | Recall and UBES/DBES Deployment Limitations due to Ramp Rates |
| --- | --- | --- | --- |
| Date |  | 09/19/2002 |  |

| ## Submitter’s Information |  |
| --- | --- |
| Name | Srini Sundhararajan |
| E-mail Address | ssundhararajan@ercot.com |
| Company | ERCOT |
| Company Address | 7620 Metro Center Drive, Austin, TX 78744 |
| Phone Number | 512/225-7244 |
| Fax Number | 512/225-7020 |

Proposed Protocol Language Revision

- Balancing Energy Service

- The deployment of power shall be constrained by the bidders’ specified ramp rate except during EECP Operations.

- QSEs are expected to comply with Balancing Energy Service Dispatch Instructions by ramping at a constant ramp rate, as calculated in 6.5.2 (18), during a fixed ramp period starting five (5) minutes prior to the start of the target service interval and ending five (5) minutes after the start of the target service interval.

[

[new language in (17) – (19) replace the need for this.]

- Balancing Energy Service deployment instructions, inclusive of recall instructions, by ERCOT to any QSE are constrained by the amount of energy  that can be deployed by the QSE within the 10-minute ramp period at the ramp rates that are communicated to ERCOT in the QSE’s bid. Expressed in MW/Min, the ramp rates serve as the basis for calculating the maximum change in the amount of energy that can be deployed by the QSE from one 15-minute settlement interval to the next. In calculating these energy amounts, ERCOT will calculate the upper and lower limits of the Balancing Energy Service deployment instruction of the target Settlement Interval as follows:

| Direction of Deployment in Prior Settlement Interval |  | Limits to P1 |
| --- | --- | --- |
| P0 > 0 |  | P0 + 10 x RRU |
| P0 > 0 |  | P0 – min ( P0 / RRU, 10 ) x RRU – [ 10 – min ( P0 / RRU, 10 ) ] x RRD |
| P0 < 0 |  | P0 + min ( -P0 / RRD, 10 ) x RRD + [ 10 – min ( -P0 / RRD, 10 ) ] x RRU |
| P0 < 0 |  | P0 – 10 x RRD |

Where:

P0	MW amount of Balancing Energy Service deployed by ERCOT in the previous Settlement Interval.

P1	MW amount of Balancing Energy Service deployed by ERCOT in the target Settlement Interval.

RRU 	bid ramp rate (always positive) in MW/min associated with the QSE’s Balancing Up bid for the target interval. If RRU is not available for the target interval, ERCOT will use the most recent value of the QSE’s RRU.

RRD 	bid ramp rate (always positive) in MW/min associated with the QSE’s Balancing Down bid for the target interval. If RRD is not available for the target interval, ERCOT will use the most recent value of the QSE’s RRD.

P1 > P0	indicates a deployment of more energy in the up direction and/or less energy in the down direction from the previous Settlement Interval to the target Settlement Interval.

P1 < P0	indicates a deployment of less energy in the up direction and/or more energy in the down direction from the previous Settlement Interval to the target Settlement Interval.

ERCOT will honor the Limits to P1, as calculated above,  in determining P1.

ERCOT will calculate the constant ramp rate for the P1 deployment as [P1-P0]/10.

For settlement and instructed deviation purposes, the P1 instruction along with the constant ramp rate will be used in determining the energy deployed during the ramp period and corresponding previous and target Settlement Intervals.

PIP112 & PRR307: Replace items #17 through #18 with the following once BUL is implemented:

(17) 	With the exception of Balancing Up – Load, [ Replace with the language from (17) above.]

- With the exception of Balancing Up - Load, [ Replace with the language from (18) above.]

- The QSE’s instructed deviation will include the expected energy calculated  using the constant ramp rate, as calculated in 6.5.2 (18), during the 10-minute ramp period when complying with an ERCOT Balancing Energy Service Dispatch Instruction.

[No longer necessary in light of new language in (17) – (19).

- Loads acting as Resources qualified to provide Regulation, Responsive Reserve, Non-Spinning Reserve, or Replacement Reserve Services may also be used by ERCOT to provide Balancing Energy under the OOME instructions and pricing structure. ERCOT shall not use Loads qualified to provide only Balancing Up Load Service under the OOME instructions and pricing structure.

- [(17) and (18) were transferred from here to following (18) above.] (19) is covered through the new (17) – (19) language.

- The minimum number of megawatts of Balancing Energy Service that may be offered to ERCOT is one (1) MW.

- If a QSE’s ramp rate limits its deployment in the target Settlement Interval, it will not be the last incremental deployment scheduled by ERCOT.  The MCPE will be set to the last bid selected to provide the last block of Balancing Energy for that Settlement Interval.