| OGRR Number | 122-OGRR | OGRR Title | Changes due to PRR 332 Unit-specific Dispatch Instructions Undergoing Required Testing |
| --- | --- | --- | --- |
| ###### Status |  | Posted |  |
| Operating Guide Section Requiring Revision |  | Guide 4.5 & 4.5.3.2 EECP Steps |  |
| Protocol Section Requiring Revision, if any. |  | ## N/A |  |
| Requested Resolution |  | ## Normal |  |
| Revision Description |  | Revise OG Sections 4.5 and 4.5.3.2 to update EECP Step 1 to include Resource performance the testing language found in PRR 332 [Protocols Section 5.6.7]. |  |
| ###### Reason for Revision |  | Consistency of Operating Guides with Protocols. |  |
| ## Sponsor |  |  |  |
| Name |  | Rick Keetch |  |
| E-mail Address |  | rkeetch@reliant.com |  |
| Company |  | Reliant Energy |  |
| Company Address |  | 1111 Louisiana |  |
| Phone Number |  | 713-207-2526 |  |
| Fax Number |  | 713-207-0098 |  |
| ## Timeline |  |  |  |
| Date Received |  | 12/11/2002 |  |
| Date Posted |  | 12/11/2002 |  |
| Comments Due |  | 01/02/2002 |  |
| ROS Review Date |  | 02/--/2003 |  |
| ROS Recommendation |  |  |  |
| Other Subcommittee Consideration |  |  |  |
| TAC Consideration |  |  |  |

Proposed Operating Guide Language Revision

### 4.5	 Emergency Electric Curtailment Plain (EECP)

### EECP STEPS

Step 1 — Maintain 2300 MW of ERCOT’s reserve service Resources comprised of: (1) undeployed Responsive Reserve Service; and (2) that amount of Non-Spinning Reserve Service which has been deployed but has not ramped to its specified output level. ERCOT will:

- Use those available Ancillary Services that can be deployed to increase Responsive Reserves.

- Provide Dispatch Instructions to QSEs to start all Resources that are available in the time frame of the emergency (OOMC). Similarly, ERCOT will provide Dispatch Instructions to QSEs to suspend any ongoing generating unit or resource performance testing and maximize Resource deployment (OOME) to increase Responsive Reserve levels on other Resources. QSEs intending to provide OOMC and OOME will comply as soon as practicable

- Use available DC Tie capacity that is not already being used by the market.

- Start RMR Units available in the time frame of the Emergency. RMR Units should be loaded to full capability.

#### 4.5.3.2	  EECP Steps

Step 1 – Maintain ERCOT Undeployed Responsive Reserve Service Resources To 2300 Mw

| OPERATOR | ACTION |
| --- | --- |
| ERCOT | - Start RMR units available in the time frame of the emergency. RMR units should be loaded to full capability. - Issue Dispatch Instructions to QSEs to suspend any ongoing generating unit or Resource performance testing. - Utilize available resources providing Replacement and Non-Spin Reserve Services that can be deployed to increase Responsive Reserves. - Utilize available DC tie capability that is not already being used by the market. - Notify the Southwest Power Pool Security Coordinator. - OOM uncommitted units available within the expected timeframe of the emergency. |
| QSE | - Notify ERCOT of any resources uncommitted but available in the timeframe of the emergency. |

| ## Other TAC Subcommittee Review |  |
| --- | --- |
| PRS Review Date |  |
| PRS Protocols Impact Decision |  |
| WMS Review Date |  |
| RMS Review Date |  |
| ## TAC Action |  |
| TAC Vote Date |  |
| TAC Proposed Implementation Date |  |
| TAC Vote Results |  |
| ## ERCOT Board Action |  |
| ERCOT Board Vote Date |  |
| ERCOT Board Implementation Date |  |
| ERCOT Board Results |  |
| ## Appeal |  |
| Appeal Date |  |
| Entity Initiating Appeal |  |
| Nature of Appeal |  |