COMPLIANCE REPORT TO ROS

FEBRUARY 7, 2003

- Protocol-Based Compliance Enforcement:  ERCOT is still working on an internal process for escalating Protocol compliance problems with market participants that start from the Operating Desks.    Compliance will also begin handling of certain Protocol 6.10 Ancillary Service performance measures in the near future.

- ERCOT Frequency Control Related Issues:

  - Single Control Area CPS 1 Performance (1 minute average deviation) for December improved very slightly to 136.3, up from 135.  It remains above 100 needed for passing.

  - Work continues to fix problems with QSE AS reports.  Some Protocol revisions are needed to update exclusions from AS reporting.

  - PDCWG will be asked to review proposals for new NERC “Balancing” standards now in draft.  These may include a revised measurement that combines DCS disturbance standard with the current 10 minute frequency control measure of CPS2.  ERCOT is exempt from CPS2 as it now stands but may need to comply with a new measure, if it eliminates transmission flow and inadvertent energy issues that don’t apply to ERCOT.

  - It appears that Protocol 350, elimination of the 3 point smoothing algorithm, is scheduled for March 1.

- Transmission Security Issues:

  - ERCOT is participating (Steve Myers) in a NERC group to examine definition issues for transmission security operating limit violations.   Drafting of the new standard is also in progress; Ellis Rankin from Oncor is participating in this group.   Final comments on the request for a new NERC standard on a related subject, facility ratings, were due on January 31st.  Drafting of this standard will likely commence within a couple of months.

  - Performance in December (draft) :  4 violations (over 30 minutes), 3 on South-North, 1 on S-H.  Operations/TO validation needed.   Operations and Compliance will meet in February to develop the list of monitored points which are currently the CSC’s.

- NERC Compliance program for 2003

  - Tabulation for 2002 results will be presented to ROS in March.

  - These measures will be assessed in 2003 that involve Regional procedures:

    - SSWG procedures describing steady state data requirements

    - DWG procedures describing dynamics data requirements

    - Disturbance monitor location requirements from SPWG (in Operating Guides)

- Control Center Backup plans are due to ERCOT on 2/28/03 per new Operating Guides requirement.

- Date for 2003 Underfrequency load shed survey of TDSP’s will be announced next month.