| PRR Number | Draft | PRR Title | Creation of Hubs |
| --- | --- | --- | --- |
| ###### Status |  |  |  |
| Protocol Section Requiring Revision |  |  |  |
| Requested Resolution |  |  |  |
| Revision Description |  | A Hub consists of a fixed set of buses that are treated like a zone in the current ERCOT market and a super node in a nodal market except that QSEs cannot have schedules source from or sink into the Hub. The set of buses that constitute a Hub should be electrically “tight”. Under the current ERCOT market, ERCOT would publish shift factors for the Hubs (which are the average of the set of Hub bus shift factors) and prices for the Hubs and accept schedules into and out of the Hubs. Since there is no Resource or Load in Hubs, all schedules into and out of Hubs would be inter-QSE trades. Therefore all Resources and Loads are assigned to the Congestion Zones through the current process and never assigned to Hubs. In the event that ERCOT decides to adopt a nodal pricing model, then the Hub price is essentially the average nodal price of the set of Hub buses. These Hub prices under the nodal model should very closely mimic the Hub prices under the current ERCOT market as long as localized market power mitigation methods under the nodal model are similar to the current method and the major influence on nodal price differences are the CSCs. Therefore, the Hubs provide continuity even in the event of market design changes – essential stability for long-term contracting. The reason for the continuity of Hub prices is as follows. Under nodal pricing, the price at any node n or bus is given by: Pn = Pr + all CSCs [(SFc,n)x(SPc)] Where, Pn = Price at nodel n Pr = Price at the reference node SFc,n = Shift Factor on CSC c at node n SPc = Shadow Price of CSC c Therefore, the average price of all nodes that constitute the Hub is given by: Average [Pn] = Pr + all CSCs [(Average[SFc,n])x(SPc)] Using the average shift factor of all nodes that constitute the Hub under the current design gives: Hub Price = Pr + all CSCs [(Average[SFc,n])x(SPc)] Thus, the Hub price under the current model is exactly the same as the average of the Hub nodal prices as long as these prices reflect congestion on CSCs only (i.e. localized market power is mitigated in a similar manner to current method). A point to note is that schedules from the North Zone to the North Hub may incur slight congestion charges due to differences in shift factors whenever there’s congestion. It should be obvious that in order for these Hubs to work, ERCOT must publish shift factors and prices as well as allow scheduling to and from Hubs. Hubs will not work without ERCOT facilitation. Initially, in order to keep it simple and test out the concept, three Hubs are proposed: DFW (Dallas area), HOU (Houston area) and SNS (San Antonio ‘n Surrounding area). Hubs can be added later, Hubs cannot be changed or deleted once defined. |  |
| ###### Reason for Revision |  | Create permanent Hubs to provide stability and continuity for transacting in the market and to facilitate forward markets. |  |
| ## Timeline |  |  |  |
| Date Received |  |  |  |
| Date Posted |  |  |  |
| PRS Review Date |  |  |  |
| PRS Recommendation |  |  |  |

Proposed Protocol Language Revision

# Section 2: Definitions and Acronyms

H

Hub

Hub consists of a fixed set of buses and is treated by ERCOT for Scheduling and Settlement purposes the same as a Congestion Zone, except that all schedules into and out of the Hub must be part of an inter-QSE schedule (i.e. a Hub is deemed to have no Resources or Loads within the Hub).

Hub Shift Factor

Hub Shift Factor for a Hub for a particular CSC is equal to the algebraic average of the individual bus Shift Factors for the set of buses that constitute the Hub for the same CSC relative to the same ERCOT reference bus used to calculate Zonal Shift Factors.

### 4.3.2	Schedule Components

Included in each Balanced Schedule is:

- Energy to be produced by Resources that the QSE represents, by Congestion Zone;

- Energy to be consumed by Loads that the QSE represents (including T&D Losses) by Congestion Zone;

- ERCOT allocated Ancillary Services Obligations for the QSE;

- Any energy and Ancillary Services scheduled to or from other QSEs or ERCOT (e.g. Inter-QSE Trades); and

- Any Self-Arranged Ancillary Services.

Only Inter-QSE Trades for Energy may be scheduled through Hubs.

## 7.2	CSC and Congestion Zone Determination and Hub Definition

#### 7.2.2 	Hub Definition

- A Hub consists of a fixed set of buses and is treated by ERCOT for Scheduling and Settlement purposes the same as a Congestion Zone, except that all schedules into and out of the Hub must be part of an Inter-QSE Trade (i.e. a Hub is deemed to have no Resources or Loads within the Hub).

- Since there are no Resources or Loads within Hubs, no Ancillary Services, including Balancing Energy Service, can be offered or provided within Hubs.

- The Hub Shift Factor for a Hub for a particular CSC is equal to the algebraic average of the individual bus Shift Factors for the set of buses that constitute the Hub for the same CSC relative to the same ERCOT reference bus used to calculate Zonal Shift Factors.

- ERCOT shall publish Hub Shift Factors for the all Hubs and Balancing Energy Prices for all Hubs in the same manner as that for Congestion Zones.

- The following are all the Hubs in ERCOT and their fixed set of buses:

- DFW Hub consists of ERCOT bus numbers xxxx, xxxx, ….

- HOU Hub consists of ERCOT bus numbers xxxx, ….

- SNS Hub consists of ERCOT bus numbers xxxx, ….

| ## Sponsor |  |
| --- | --- |
| Name | Shams Siddiqi |
| E-mail Address | Ssiddiqi@lcra.org |
| Company | LCRA |
| Company Address | 3700 Lake Austin Blvd., Austin, Texas 78703 |
| Phone Number | 512-397-6704 |
| Fax Number | 512-473-4026 |
| ## ERCOT Impact Analysis |  |
| Date |  |
| Financial Impact |  |
| Analysis |  |
| ## TAC Action |  |
| TAC Vote Date |  |
| TAC Proposed Implementation Date |  |
| TAC Vote Results |  |
| ## ERCOT Board Action |  |
| ERCOT Board Vote Date |  |
| ERCOT Board Implementation Date |  |
| ERCOT Board Results |  |
| ## Appeal |  |
| Appeal Date |  |
| Entity Initiating Appeal |  |
| Nature of Appeal |  |