| PRR Number | 398PRR | PRR Title | Combined Cycle Units Through Settlement Aggregation |
| --- | --- | --- | --- |
| ###### Status |  | Posted |  |
| Protocol Section Requiring Revision |  | 6.8.2.3 Energy Payments 7.4.3.1 Balancing Energy Up from a Specific Resource 7.4.3.2 Balancing Energy Down from a Specific Resource |  |
| Requested Resolution |  | ## Urgent – the system change is underway, approval of this revision is needed to complete the project. |  |
| Revision Description |  | Amends the equations originally revised by 369PRR - Proposal to Address Dispatching Issues for Combined Cycle Units Through Settlement Aggregation, to clarify how the ERCOT system will process this functionality. |  |
| ###### Reason for Revision |  | The information submitted in this PRR modifies the formulas to detail how ERCOT will settle aggregated units when multiple “types” of instructions are deployed to the same aggregated unit within the same interval. |  |
| ###### Known Benefit for Revision |  | Combined cycle plants will be settled more accurately. |  |
| ## Timeline |  |  |  |
| Date Received |  | 04/10/2003 |  |
| Date Posted |  | 04/11/2003 |  |
| Comments Due |  | 04/22/2003 |  |
| PRS Review Date |  | 04/25/2003 |  |
| PRS Recommendation |  |  |  |
| TAC Consideration |  | 05/08/2003 |  |
| Board Consideration |  | 05/20/2003 |  |

Proposed Protocol Language Revision

#### 6.8.2.3	Energy Payments

- Whenever a Resource is called for OOME Up, the QSE will receive, any Resource Imbalance payment it is entitled to receive as specified in 6.8.1.13.  In addition, an OOME energy payment adjustment will be made such that the resulting total payment to the Resource is equal to the higher of either (a) Resource Category Generic Fuel Cost (RCGFC) for that Resource or the (b) MCPE for the Congestion Zone.

- The calculation for OOME Up is as follows:

PEOOMUPiq 	= SUM (PEOOMUPiuq)uq

Where:

PEOOMUPiq = -1 * EOOMUPiq * [Max [(RCGFCc - MCPEiz), 0]

EOOMUPiuq = 	Max (0, Min ((MRiuq - OLueq), IOOMUPiuq))

The equation below will be used to determine the Total OOM Energy Payments to be allocated to each QSE in Section 6.9.7.2 OOM Energy Charge.

PEOOMUPi 	= SUM (PEOOMUPiu)

Where:

c	Resource category

i	interval

u	unit

z	zone

q	QSE

MCPEiz	Market Clearing Price of Energy for that interval of the zone in which unit resides

PEOOMUPiuq	Out of Merit Energy Up Payment ($) for that interval per unit per QSE

PEOOMUPiq	Out of Merit Energy Up Payment ($) for that interval per QSE

PEOOMUPi	Summation of Out of Merit Energy Up Payment ($) per interval for all QSEs in the market

EOOMUPiuq	OOM Up quantity deployed for that unit per interval per QSE

IOOMUPiu	OOM Energy Up Instructions for that interval for that unit converted to MWh for that interval by dividing by the interval count per hour (4 per hour for 15 minute Settlement Interval)

MRiuq	Meter Reading for that unit for that interval per QSE

OLiuq	Resource Plan output level submitted by the QSE for the specific unit given the OOM instruction for that interval.

RCGFCc	Resource Category Generic Fuel Cost for a specific category.

[PRR369: Replace 6.8.2.3(2) as follows upon system implementation & notice to the Market.]

(2)	The calculation for OOME Up is as follows:

PEOOMUPiq 	= SUM (PEOOMUPiuq)uq+ SUM (PEOOMUPivq)vq

Where:

PEOOMUPiuq 	=	 -1 * EOOMUPiuq * [Max [(RCGFCc - MCPEiz), 0]

EOOMUPiuq 	= 	Max (0, Min ((MRiuq - OLuiq), IOOMUPiuq))

PEOOMUPivq 	=	 -1 * EOOMUPivq * [Max [(RCGFCc - MCPEiz), 0]

EOOMUPivq 	= 	Max (0, Min ((MRivq – OLviq), NETUEQivq)) * OOMAGRivq

The equation below will be used to determine the Total OOM Energy Payments to be allocated to each QSE in Section 6.9.7.2 OOM Energy Charge.

PEOOMUPi 	=	 SUM (PEOOMUPiu)  + SUM (PEOOMUPiv)

NETUEQivq 	=	Max(0,((NETOOMUEQivq + NETLBEUQivq) – (NETOOMDEQivq + NETLBEDQivq))

NETOOMUEQivq 	=	Max(0, (SUM(IOOMUPqui)u – SUM(IOOMDNqui)u))

NETLBEUQivq 	=	Max(0, (SUM(LBEUQqui)u – SUM(LBEDQqui)u))

NETOOMDEQivq 	=	Max(0, (SUM(IOOMDNqui)u – SUM(IOOMUPqui)u))

NETLBEDQivq 	=	Max(0, (SUM(LBEDQqui)u – SUM(LBEUQqui)u))

OOMAGRivq 	=	[SUM(IOOMUPqui)u + SUM(IOOMDNqui)u] / [SUM(LBEUQqui)u + SUM(LBEDQqui)u + SUM(IOOMUPqui)u + SUM(IOOMDNqui)u]

Where:

c	Resource category

i	interval

u	individual generation unit (including units within an Aggregated Unit)

v	Aggregated Unit

z	zone

q	QSE

MCPEiz	Market Clearing Price of Energy for that interval of the zone in which unit resides

PEOOMUPiuq	Out of Merit Energy Up Payment ($) for that interval per unit per QSE

PEOOMUPivq	Out of Merit Energy Up Payment ($) for that interval per Aggregated Unit per QSE

PEOOMUPiq	Out of Merit Energy Up Payment ($) for that interval per QSE

PEOOMUPi	Summation of Out of Merit Energy Up Payment ($) per interval for all QSEs in the market

EOOMUPiuq	OOM Up quantity deployed for that unit per interval per QSE

EOOMUPivq	OOM Up quantity deployed for that Aggregated Unit per interval per QSE

IOOMUPiuq	OOM Energy Up Instructions for that interval for that unit converted to MWh for that interval by dividing by the interval count per hour (4 per hour for 15 minute Settlement Interval)

IOOMDNiuq	OOM Energy Down Instructions for that interval for that unit converted to MWh for that interval by dividing by the interval count per hour (4 per hour for 15 minute Settlement Interval)

MRiuq	Meter Reading for that unit for that interval per QSE

MRivq	Meter Reading for that Aggregated Unit for that interval per QSE.

OLiuq	Resource Plan output level submitted by the QSE for the specific unit given the OOM instruction for that interval.

OLivq	Resource Plan output level submitted by the QSE for the Aggregated Unit containing the specific unit given the OOM instruction for that interval.

RCGFCc	Resource Category Generic Fuel Cost for a specific category.

NETOOMUEQivq The net of all unit specific OOM instructions (Up minus Down) that result in a positive value.

NETLBEUQivq	The net of all unit specific Local Balancing Energy Instructions (Up minus Down) that result in a positive value.

NETOOMDEQivq The net of all unit specific OOM instructions (Down minus Up) that result in a positive value.

NETLBEDQivq The net of all unit specific Local Balancing Energy Instructions (Down minus Up) that result in a positive value.

LBEUQqui	Local Balancing Energy Up Instructions for that interval per specific unit

LBEDQqui	Local Balancing Energy Down Instructions for that interval per specific unit

NETUEQiv	The overall net direction per the instructions given to the Aggregated Unit results in an up direction

OOMAGRiv 	The percentage of the aggregated instructions to the aggregated unit that represents the OOM Energy market.

- If a Resource is called to provide OOME Up service and the payment for OOME service is insufficient to cover all costs of providing the Service, then that Resource will be paid, in addition to the energy payment, all verifiable, costs in excess of the OOME payment that are directly attributable to the OOME Service.  These costs may include the cost of exceeding swing gas contract limits, additional gas demand costs set by the fuel supply or transportation contracts, and any additional costs to purchase emissions credits or other costs incurred due to environmental regulations.  Verification of these costs must be submitted to ERCOT by the QSE or the Resource to allow resolution by the end of the dispute process for Settlement True-Up as defined in Section 9.2.6, True-Up Statement.  QSEs requesting such cost recovery shall perform the following:

(a)	Submit a settlement dispute in accordance with the dispute process outlined in Section 9.5.  In addition to the standard information required on the entry form on the ERCOT Portal, the dispute should clearly indicate:

(i)	The deployment instruction received from ERCOT to provide services.

(ii)	The payment received for providing the service.

(iii)	The actual cost of providing the service.

(iv)	A reference to the documentation to be provided in writing as indicated in section

(b)	Requests for additional compensation shall provide documentation in writing to allow for verification of the claimed amounts.  Documentation requirements for each cost type are as follows:

(i)	Fuel cost for providing the service shall be determined by taking the product of the deployed energy (MWH) the incremental fuel requirement (MMBtu/MWH) and the associated fuel cost ($/MMBtu) of the resource for deployed.

(ii)	Incremental fuel requirement shall be the incremental fuel consumption from the scheduled loading of the resource to the deployed loading based upon a heat rate curve for the resource from the most recently conducted heat rate tests.  Test data shall be provide in sufficient detail to allow for the validation of the heat rate curve provided.

(iii)	Fuel cost, including transportation cost, will require supporting documentation of sufficient detail to allow for the verification of the cost of fuel consumed at the deployed resource.  Documentation may include contracts, invoices or other documents that will be treated as confidential.  All copies of such documentation will be returned upon completion of the review.  For gas fired resources, such documentation will not be required if the requested incremental fuel cost is less than 115% of the Fuel Index Price.

(iv)	Compensation for additional types of cost, not outlined above, will must be reviewed and approved by the ERCOT Board of Directors during an Executive Session of their next regularly scheduled meeting.  Requests must be presented in person by a representative of the company submitting the request and must also include language suitable to be included in the Protocols to define the documentation requirements for future request of a similar nature.  Subsequent to the approval of such costs, the requesting company shall submit a Protocol Revision Request, in accordance with Protocol 21, incorporating the necessary documentation standards provided to the ERCOT Board.

- Whenever a Resource is called on for OOME Down, the QSE will be paid an energy payment equal to the total amount of energy to be reduced by the Resource, multiplied by the greater of: (a) the difference between (i) the MCPE of the Congestion Zone and (ii) the RCFGC for that Resource or (b) $0.00.  The calculation for OOME Down is as follows:

PEOOMDNiuqz	=	 -1 * EOOMDNiu  * Max (0,(MCPEiz-RCGFCc))

EOOMDNiu = 	Max (0,(Min(OLue - MRii ),IOOMDNiu))

PEOOMDNiq 	= SUM (PEOOMDNiu)q

PEOOMDNiz 	= SUM (PEOOMDNiu)z +

The equation below will be used to determine the Total OOM Down Energy Payments to be allocated to each QSE in Section 6.9.7.2, OOM Energy Charge.

PEOOMDNi = SUM (PEOOMDNiq)q

Where:

i	interval

u	unit.

z	zone

q	QSE

PEOOMDNiuqz	Out of Merit Energy Down Payment ($) for that interval per unit for the QSE.

PEOOMDNiq	Total Out of Merit Energy Down Payment in the interval for the QSE.

PEOOMDNiz	Out of Merit Energy Down Payment ($) per interval per zone.

EOOMDNiu 	OOM Energy quantity instructed down for that unit per interval.

MCPEiz	Market Clearing Price for Energy per interval for that zone.

IOOMDNiu	OOM Energy Down Dispatch Instructions per interval per unit converted to MWh for that interval by dividing by the interval count per hour (four (4) per hour for fifteen (15) minute Settlement Interval).

MRiu	Meter reading per unit per interval.

OLiuq	Resource plan submitted by the QSE for the specific Resource given the OOM instruction for that interval.

RCGFCc	Resource Category Generic Fuel Cost for a specific category as defined in Section 6.8.2.

PEOOMDNi	Total OOM Down Energy Payments.

[PRR369: Replace 6.8.2.3(4) as follows upon system implementation & notice to the Market.]

(4)	Whenever a Resource is called on for OOME Down, the QSE will be paid an energy payment equal to the total amount of energy to be reduced by the Resource, multiplied by the greater of: (a) the difference between (i) the MCPE of the Congestion Zone and (ii) the RCFGC for that Resource or (b) $0.00.  The calculation for OOME Down is as follows:

PEOOMDNiuqz	=	 -1 * EOOMDNiu  * Max (0,(MCPEiz-RCGFCc))

EOOMDNiu = 	Max (0, (Min(OLuiq - MRiu ),IOOMDNiuq))

PEOOMDNivqz	=	 -1 * EOOMDNiv  * Max (0,(MCPEiz-RCGFCc))

EOOMDNiv = 	Max (0,(Min(OLivq - MRivq ), NETDEQivq)) * OOMAGRivq

PEOOMDNiq 	= SUM (PEOOMDNiu)q + SUM (PEOOMDNiv)q

PEOOMDNiz 	= SUM (PEOOMDNiu)z + SUM (PEOOMDNiv)z

The equation below will be used to determine the Total OOM Down Energy Payments to be allocated to each QSE in Section 6.9.7.2, OOM Energy Charge.

PEOOMDNi = SUM (PEOOMDNiq)q

NETDEQivq 	=	Max [0, ((NETOOMDEQivq + NETLBEDQivq) – (NETOOMUEQivq + NETLBEUQivq))]

NETOOMUEQivq 	=	Max(0, (SUM(IOOMUPqui)u – SUM(IOOMDNqui)u))

NETLBEUQivq 	=	Max(0, (SUM(LBEUQqui)u – SUM(LBEDQqui)u))

NETOOMDEQivq 	=	Max(0, (SUM(IOOMDNqui)u – SUM(IOOMUPqui)u))

NETLBEDEQivq 	=	Max(0, (SUM(LBEDQqui)u – SUM(LBEUQqui)u))

OOMAGRivq 	=	[SUM(IOOMUPqui)u + SUM(IOOMDNqui)u] / [SUM(LBEUQqui)u + SUM(LBEDQqui)u + SUM(IOOMUPqui)u + SUM(IOOMDNqui)u]

Where:

c		  resource category

i	interval

u	individual generation unit (including units within an Aggregated Unit)

v	Aggregated Unit

z	zone

q	QSE

PEOOMDNiuqz	Out of Merit Energy Down Payment ($) for that interval per unit for the QSE

PEOOMDNivqz	Out of Merit Energy Down Payment ($) for that interval per Aggregated Unit per QSE

PEOOMDNiq	Total Out of Merit Energy Down Payment in the interval for the QSE

PEOOMDNiz	Out of Merit Energy Down Payment ($) per interval per zone

PEOOMDNiv	Out of Merit Energy Down Payment ($) per interval per zone to Aggregated Units

EOOMDNiu 	OOM Energy quantity instructed down for that unit per interval

EOOMDNiv 	OOM Energy quantity instructed down for that Aggregated Unit per interval

MCPEiz	Market Clearing Price for Energy per interval for that zone

IOOMUPiuq	OOM Energy Up Instructions for that interval for that unit converted to MWh for that interval by dividing by the interval count per hour (4 per hour for 15 minute Settlement Interval)

I

IOOMDNiuq	OOM Energy Down Instructions for that interval for that unit converted to MWh for that interval by dividing by the interval count per hour (4 per hour for 15 minute Settlement Interval)

MRiu	Meter reading per unit per interval

MRiv	Meter reading per Aggregated Unit per interval

OLiuq	Resource Plan submitted by the QSE for the specific unit given the OOM instruction for that interval.

OLivq	Resource Plan submitted by the QSE for the Aggregated Unit containing the individual unit(s) given the OOM instruction for that interval.

RCGFCc	Resource Category Generic Fuel Cost for a specific category as defined in Section 6.8.2

PEOOMDNi	Total OOM Down Energy Payments

LBEUQqui	Local Balancing Energy Up Instructions for that interval per specific unit

LBEDQqui	Local Balancing Energy Down Instructions for that interval per specific unit

NETOOMUEQivq The net of all unit specific OOM instructions (Up minus Down) that result in a positive value.

NETLBEUQivq	The net of all unit specific Local Balancing Energy Instructions (Up minus Down) that result in a positive value.

NETOOMDEQivq The net of all unit specific OOM instructions (Down minus Up) that result in a positive value.

NETLBEDQivq The net of all unit specific Local Balancing Energy Instructions (Down minus Up) that result in a positive value.

NETDEQiv	The overall net direction per the instructions given to the Aggregated Unit results in a down direction

OOMAGRiv 	The percentage of the aggregated instructions to the aggregated unit that represents the OOM Energy market.

*******************

7.4.3.1 Balancing Energy Up from a Specific Resource

In accordance with section 6.7.1.2, Deployment of Balancing Energy when Congestion Occurs, if a Market Solution exists, whenever an incremental specific Resource is deployed to solve Local Congestion, the QSE will be paid the incremental premium specified.  Aggregated Unit rules as described in Section 6.8.2, Capacity and Energy Payments for Out of Merit Service, apply to Aggregated Units approved by ERCOT.

LPCRSUqiz 	= 	-1 * Σ ((PMqiuz – MCPEiz )* Max (0, Min (MRqiuz -OLquiz, IOLqiuz - OLqiuz)))

PMqiuz	=	Max (BPMqiuz, BPMqiuz + MCPEiz )

Where:

LPCRSUqiz:	Payment to QSE for providing Balancing Energy Up from Specific Resource to solve local congestion per QSE per interval per zone.

PMqiuz:	Incremental Premium Specified for that specific Resource per interval.

BPMqiuz:	Incremental bid premium for that specific Resource per interval.

OLquiz:	Resource Plan output level submitted by the QSE for the specific Resource given the resource-specific instruction for that interval.

IOLquiz:	The Instructed output level for the Resource given the resource-specific instruction.

MRqiuz:	Meter reading per unit per interval.

MCPEiz:	Market clearing price of Energy per interval per zone.

[PRR369:  The following text will be taken out of the box upon system implementation and notice to the market.  Add the following sentence at the end of Section 7.4.3.1. Balancing Energy Up from a Specific Resource.]

Aggregated Unit rules as described in Section 6.8.2, Capacity and Energy Payments for Out of Merit Service, apply to Aggregated Units approved by ERCOT.

LPCRSUquiz 	= 	-1 * Σ ((PMqiuz – MCPEiz )* Max (0, Min (MRqiuz -OLquiz, IOLqiuz - OLqiuz)))

LPCRSUqviz	=	 -1 * Σ ((PMqivz – MCPEiz )* Max (0, Min (MRqivz -OLqviz, NETUEQivq)) * LBEAGRivq)

PMqivz	=	Max (BPMqivz, BPMqivz + MCPEiz )

BPMqivz 	= 	Min (BPMqiuz)

NETUEQivq	=	Max [0, ((NETOOMUEQivq + NETLBEUQivq) – (NETOOMDEQivq + NETLBEDQivq))]

NETOOMUEQivq	=	Max(0, (SUM(IOOMUPqui)u – SUM(IOOMDNqui)u))

NETLBEUQivq 	=	Max (0, (SUM(LBEUQqui)u – SUM(LBEDQqui)u))

NETOOMDEQivq 	=	Max(0, (SUM(IOOMDNqui)u – SUM(IOOMUPqui)u))

NETLBEDQivq 	=	Max(0, (SUM(LBEDQqui)u – SUM(LBEUQqui)u))

LBEAGRivq 	=	[(SUM(LBEUQqui)u + SUM(LBEDQqui)u)] / [SUM(LBEUQqui)u + SUM(LBEDQqui)u + SUM(IOOMUPqui)u + SUM(IOOMDNqui)u]

Where:

i	interval

q	QSE

u	individual generation unit (including units within an Aggregated Unit)

v	Aggregated Unit

z	zone

LPCRSUquiz:	Payment to QSE for providing Balancing Energy Up from Specific unit to solve local congestion per QSE per interval per zone.

LPCRSUqviz:	Payment to QSE for providing Balancing Energy Up from Aggregated units to solve local congestion per QSE per interval per zone.

PMqiuz:	Incremental Premium Specified for that specific unit per interval.

PMqivz:	Incremental Premium Specified for that specific Aggregated Unit per interval.

BPMqiuz:	Incremental bid premium for that specific unit per interval.

BPMqivz:	Incremental bid premium for that specific Aggregated Unit per interval.

OLquiz:	Resource Plan output level submitted by the QSE for the specific unit given the unit-specific instruction for that interval.

OLqviz:	Resource Plan output level submitted by the QSE for the Aggregated Unit containing the unit given the unit-specific instruction for that interval.

IOLquiz:	The Instructed output level for the unit given the unit-specific instruction.

IOLqviz:	The Instructed output level for the Aggregated Unit containing the unit given a unit-specific instruction.

IOOMUPiuq	OOM Energy Up Instructions for that interval for that unit converted to MWh for that interval by dividing by the interval count per hour (4 per hour for 15 minute Settlement Interval)

IOOMDNiuq	OOM Energy Down Instructions for that interval for that unit converted to MWh for that interval by dividing by the interval count per hour (4 per hour for 15 minute Settlement Interval)

MRqiuz:	Meter reading per unit per interval.

MRqivz:	Meter reading per Aggregated Unit per interval.

MCPEiz:	Market clearing price of Energy per interval per zone.

LBEUQqui	Local Balancing Energy Up Instructions for that interval per specific unit

LBEDQqui	Local Balancing Energy Down Instructions for that interval per specific unit

NETOOMUEQivq 	The net of all unit specific OOM instructions (Up minus Down) that result in a positive value.

NETLBEUQivq	The net of all unit specific Local Balancing Energy Instructions (Up minus Down) that result in a positive value.

NETOOMDEQivq 	The net of all unit specific OOM instructions (Down minus Up) that result in a positive value.

NETLBEDQivq 	The net of all unit specific Local Balancing Energy Instructions (Down minus Up) that result in a positive value.

NETUEQivq	The overall net direction per the instructions given to the Aggregated Unit results in an up direction

LBEAGRiv 	The percentage of the aggregated instructions to the aggregated unit that represents the OOM Energy market.

[7.4.3.2 – Revise formulae and variable definitions as follows:]

LPCRSDquiz 	= 	-1 * Σ((MCPEiz - BPMqiuz ) * Max(0, Min(OLqiuz -MRquiz , OLqiuz - IOLqiuz)))u

LPCRSDqviz 	=	-1 * Σ((MCPEiz - BPMqivz ) * Max(0, Min(OLqivz -MRqviz , NETDEQiv)))v * LBEAGRiv

BPMqivz = Max (BPMqiuz)

NETDEQivq 	=	Max [0, ((NETOOMDEQivq + NETLBEDQivq) – (NETOOMUEQivq + NETLBEUQivq))]

NETOOMUEQivq 	=	Max [0, (SUM (IOOMUPqui)u – SUM(IOOMDNqui)u)]

NETLBEUQivq 	=	Max [0, (SUM(LBEUQqui)u – SUM(LBEDQqui)u)]

NETOOMDEQivq 	=	Max [0, (SUM(IOOMDNqui)u – SUM(IOOMUPqui)u)]

NETLBEDEQivq 	=	Max[0, (SUM(LBEDQqui)u – SUM(LBEUQqui)u)]

LBEAGRivq 	=	[SUM(LBEUQqui)u + SUM(LBEDQqui)u] / [SUM(LBEUQqui)u + SUM(LBEDQqui)u + SUM(IOOMUPqui)u + SUM(IOOMDNqui)u]

Where:

i	interval

q	QSE

u	individual generation unit (including units within an Aggregated Unit)

v	Aggregated Unit

z	zone

LPCRSDqiz:	Payment to QSE for providing Balancing Energy Down from specific unit to solve local congestion per QSE per interval per zone

BPMqiuz:	Decremental bid Premium Specified for that specific unit per interval

BPMqivz:	Decremental bid Premium Specified for that specific Aggregated Unit per interval

OLquiz:	Resource Plan output level submitted by the QSE for the unit given the unit-specific instruction for that interval

OLqviz:	Resource Plan output level submitted by the QSE for the Aggregated Unit containing the unit given the unit-specific instruction for that interval

IOLquiz:	The Instructed output level for the unit given a unit-specific instruction

IOOMDNiuq	OOM Energy Down Instructions for that interval for that unit converted to MWh for that interval by dividing by the interval count per hour (4 per hour for 15 minute Settlement Interval)

IOOMUPiuq	OOM Energy Up Instructions for that interval for that unit converted to MWh for that interval by dividing by the interval count per hour (4 per hour for 15 minute Settlement Interval)

MRqiuz:	Meter reading per unit per interval.

MRqivz:	Meter reading per Aggregated Unit per interval.

MCPEiz:	Market clearing price of Energy per interval per zone

LBEUQqui	Local Balancing Energy Up Instructions for that interval per specific unit

LBEDQqui	Local Balancing Energy Down Instructions for that interval per specific unit

NETOOMUEQivq The net of all unit specific OOM instructions (Up minus Down) that result in a positive value.

NETLBEUQivqThe net of all unit specific Local Balancing Energy Instructions (Up minus Down) that result in a positive value.

NETOOMDEQivq The net of all unit specific OOM instructions (Down minus Up) that result in a positive value.

NETLBEDQivq The net of all unit specific Local Balancing Energy Instructions (Down minus Up) that result in a positive value.

NETDEQivq	The overall net direction per the instructions given to the Aggregated Unit results in a down direction

LBEAGRiv 	The percentage of the aggregated instructions to the aggregated unit that represents the OOM Energy market.

7.4.3.2 Balancing Energy Down from a Specific Resource

In accordance with section 6.7.1.2, if a Market Solution exists, whenever a decremental specific Resource is deployed to resolve the Local Congestion, the QSE will be paid the difference of MCPE in the commercial zone, which the specific Resource is in, and the decremental bid premium specified.

LPCRSDqiz 	= 	-1 * Σ( (MCPEiz - BPMqiuz ) * Max(0, Min(OLqiuz -MRquiz  , OLqiuz - IOLqiuz)))u-

Where:

LPCRSDqiz:	Payment to QSE for providing Balancing Energy Down from specific Resource to solve local congestion per QSE per interval per zone

BPMqiuz:	Decremental bid Premium Specified for that specific Resource per interval

OLquiz:	Resource Plan output level submitted by the QSE for the specific Resource given the resource-specific instruction for that interval

IOLquiz:	The Instructed output level for the Resource given a resource-specific instruction

MRqiuz:	Meter reading per unit per interval.

MCPEiz:	Market clearing price of Energy per interval per zone

| ## Sponsor |  |
| --- | --- |
| Name | Paula Feuerbacher |
| E-mail Address | pfeuerbacher@ercot.com |
| Company | ERCOT |
| Company Address | 2705 West Lake Drive, Taylor, TX 76574 |
| Phone Number | 512/248-3947 |
| Fax Number |  |
| ## ERCOT Impact Analysis |  |
| Date |  |
| Financial Impact |  |
| Analysis |  |
| ## TAC Action |  |
| TAC Vote Date |  |
| TAC Proposed Implementation Date |  |
| TAC Vote Results |  |
| ## ERCOT Board Action |  |
| ERCOT Board Vote Date |  |
| ERCOT Board Implementation Date |  |
| ERCOT Board Results |  |
| ## Appeal |  |
| Appeal Date |  |
| Entity Initiating Appeal |  |
| Nature of Appeal |  |