| PRR Number | 399PRR | PRR Title |  | Requirements for Replacing an IDR with a Non- IDR Meter |
| --- | --- | --- | --- | --- |
| ###### Status |  | Posted. |  |  |
| Protocol Section Requiring Revision |  | - Interval Data Recorder (IDR) Installation and Use in Settlement 18.6.6 Peak Demand Determination for Non-IDR Premises |  |  |
| Requested Resolution |  | ## Urgent |  |  |
| Revision Description |  | This revision will allow for: 1. IDR installations where they are needed or required and provide the most effective utilization of this type of reporting to ERCOT and for Settlement - Removal of IDR installations that are no longer valid for a premise or ESI ID because of reduced load or consumption based upon twelve (12) consecutive months of usage. - A means of correcting the provision that once a premise (ESI ID) has been identified and IDR meter installed it will always and forever be maintained as an IDR premise without considering significant changes in customer’s load and peak demand. - Also with future changes and request for IDR meters by customer’s that are generated from vendors through the Competitive Metering process and meter ownership this Protocol Revision would help to handle these types of issues and offer customers more choices via meter ownership. |  |  |
| ###### Reason for Revision |  | See Revision Description section of this document |  |  |
| Timeline |  |  |  |  |
| Date Received |  |  | 04/14/2003 |  |
| Date Posted |  |  | 04/15/2003 |  |
| Comments Due |  |  |  |  |
| PRS Review Date |  |  |  |  |
| PRS Recommendation |  |  |  |  |
| TAC Consideration |  |  |  |  |
| Board Consideration |  |  |  |  |

Proposed Protocol Language Revision

### 18.6.1 Interval Data Recorder (IDR) Installation and Use in Settlement

- IDR Requirement:  IDRs shall be installed and utilized for settlement of Premises having either:

  - A peak demand greater than 1000 kW (or 1000 kVa), or

  - Service provided at transmission voltage (above 60 kV).

- A Competitive Retailer may have an IDR installed and used for settlement purposes at any associated Premise outside the IDR Requirement.  Except as stated in item (4) or (7) of this subsection, IDRs in place or installed after September 1, 1999 shall be used for settlement.  Once an IDR is installed on a Premise and used for settlement purposes, the given premise shall continue to be settled with its interval data, except as stated in item (7) of this subsection.

- All Non-Metered Loads such as street lighting, regardless of the aggregation level, shall not be required to install IDRs under the IDR Requirement.  These loads shall be settled using Load Profiles.

- For premises not subject to the IDR Requirement in item (1) of this subsection, IDRs installed at the request of ERCOT, a TDSP, a municipal, or a cooperative for load research, rate/tariff design calculation, coincident demand calculation, or Load Profiling purposes shall be exempt from the requirement to use an IDR for settlement purposes.

- For IDR installation procedures reference Section 10.2.2, TDSP Metered Entities.

- TDSPs responsible for any Load transfer schemes between ERCOT and Non-ERCOT regions shall install IDR metering capable of measuring the load served during the period the Load transfer is implemented.

- Requirements for Replacing IDR with Non-IDR:

For a premise where an IDR meter already exist, however the peak demand identified in item (1a) of this section are no longer valid the following criteria must be satisfied to replace the IDR with a Non-IDR meter:

  - The Peak Demand is equal to or less than 200 kW (or 200 kVa) for a period of twelve (12) consecutive months.

### 18.6.6 Peak Demand Determination for Non-IDR Premises

For the purpose of determining the peak demand level for the IDR Requirement in Section 18.6.1, Interval Data Recorder (IDR) Installation and Use in Settlement, the demand will be determined in accordance with PUCT rulemaking or through a consensus process with ERCOT and Market Participants.  In the absence of a clear definition of peak demand in the “price to beat” rulemaking, the following application shall be used in determining the peak demand level for IDR Requirement in Section 18.6.1, Interval Data Recorder (IDR) Installation and Use in Settlement:

A Premise (ESI ID) has a peak demand greater than 1000 kW when a demand greater than 1000 kW has been measured in any two (2) billing months of the most recent twelve (12) month period. .  If an IDR meter has been installed for a premise (ESI ID) where the peak demand is found to be equal to or less than 200 kW (or 200 kVa) for a period of twelve (12) consecutive months, the IDR installation should be replaced with a Non-IDR meter.  Competitive Retailers may dispute an IDR assignment through the ERCOT settlement dispute process, described in Section 9.5 Settlement and Billing Dispute Process.

ERCOT shall be responsible for receiving and storing demand information necessary for determining mandatory IDR installations.

| ## Sponsor |  |
| --- | --- |
| Name | Kathy D. Scott |
| E-mail Address | Kathy.scott@CenterPointEnergy.com |
| Company | CenterPoint Energy |
| Company Address | 1111 Louisiana Street Houston, TX 77002-5200 |
| Phone Number | 713.207.7830 |
| Fax Number | 713.207.9921 |
| ## ERCOT Impact Analysis |  |
| Date |  |
| Financial Impact |  |
| Analysis |  |
| ## TAC Action |  |
| TAC Vote Date |  |
| TAC Proposed Implementation Date |  |
| TAC Vote Results |  |
| ## ERCOT Board Action |  |
| ERCOT Board Vote Date |  |
| ERCOT Board Implementation Date |  |
| ERCOT Board Results |  |
| ## Appeal |  |
| Appeal Date |  |
| Entity Initiating Appeal |  |
| Nature of Appeal |  |