##### 2.1 Definitions

##### Controllable Resource

##### Any resource that is not an Uncontrollable Renewable Resource

##### Renewable Production Potential

The maximum generation in MWH/interval from an Uncontrollable Renewable Resource that could be generated from all available units not on maintenance or forced outage given the available energy from wind or solar radiation for that interval.  The Renewable Production Potential is a function of the renewable energy available (wind or solar radiation), the energy conversion characteristics of each generator, units declared completely or partially unavailable, and units on forced outage.

##### Uncontrollable Renewable Resource

Generators that, when they are declared available, can produce only up to the Renewable Production Potential which is directly related to their supply of variable energy from renewable resources such as wind and solar.  Uncontrollable Renewable Resource output may be reduced from the Renewable Production Potential for orders given by ERCOT, as well as dispatch instructions of the controlling QSE.

- 6.8.1.15.1  Uninstructed Resource Charge Process

Once ERCOT has determined the Congestion Zones in which the Uninstructed Deviations of the QSE have occurred, an uninstructed pricing differential will be applied to a QSE representing Resources whenever one of the following two conditions exist:

- The QSE representing Resources has total metered Resources for any Settlement Interval that are greater than the larger of 101.5% of the QSE’s total of schedules plus instructions or five (5) MWh over the total of schedules plus instructions when the integrated amount of ERCOT-wide regulation is less than –25 MWhs in the interval. Upon one-day notice, ERCOT can reduce the percentage and MWh tolerance to 101% and 3 MWh, respectively, if significant price chasing exists.

- The QSE representing Resources has total metered Resources for any Settlement Interval that are less than the lesser of 98.5% of the total schedule plus instructions or five (5) MWh under the total of schedules plus instructions when the integrated amount of ERCOT-wide regulation is greater than + 25 MWhs in the interval. Upon one day notice, ERCOT can increase the percentage and MWh tolerance to 99% and 3 MWh, respectively, if significant price chasing exists.

- A QSE that schedules Uncontrollable Renewable Resources  will not be subject to Uninstructed Deviation Pricing for the Uncontrollable Renewable Resources to the extent that the total metered uncontrollable renewable resources does not exceed 150% or is not less than 50% of the Uncontrollable Renewable Resource’s  schedule in any schedule interval. The Uncontrollable Renewable Resource’s Renewable Production Potential may be used in lieu of the Uncontrollable Renewable Resource’ schedule for this purpose if the resource owner also makes an irrevocable commitment to use the Uncontrollable Renewable Resource’s Renewable Production Potential for all OOME Down payments in lieu of the resource plan submitted for the resource as specified in Section 6.8.2.3.