| PRR Number | 418PRR | PRR Title | Creation of New CSC |
| --- | --- | --- | --- |
| ###### Status |  | Posted |  |
| Protocol Section Requiring Revision |  | Section 7.2.1.3, Determining Closely Related Elements (CREs) |  |
| Requested Resolution |  | ## Urgent |  |
| Revision Description |  | Modify Section 7.2.1.3, create a new section 7.2.1.4, Approval Process and add language related to a new CSC effective July 1, 2003 through the remainder of 2003. |  |
| ###### Reason for Revision |  | Operational conditions require a new CSC to allow Zonal deployment of Balancing Energy to resolve a North to Houston constraint. |  |
| ## Timeline |  |  |  |
| Date Received |  | 05/23/03 |  |
| Date Posted |  | 05/23/03 |  |
| PRS Review Date |  | 05/30/03 |  |
| PRS Recommendation |  |  |  |

Proposed Protocol Language Revision

#### 7.2.1.3  Determining Closely Related Elements (CREs)

The appropriate ERCOT Technical Advisory Committee (TAC) Subcommittee will review the CRE identification process used to determine the CREs that should be recommended for approval to the TAC and the Board.  ERCOT may determine CREs beginning with the year 2003 as follows:

(1)	Determine the Zonal Average Shift Factor for a particular CSC (Xz) for each Zone z.

(2)	Determine the zonal average shift factor for the candidate CRE (Yz) for each Zone z using the same generation weighting as in (1) but ignoring Boundary Generation Resource busses that would cluster into a different Congestion Zone with respect to the CRE.

(3)	Determine positive “a” applying least-square curve fitting to the following equation:

Yz = a (Xz) + bz  	for all Zones z.

(4)	Using “a” from (3), determine the maximum absolute value of bz.

(5)	Also determine the total capacity (MW) of Boundary Generation Resources that would cluster into a different Congestion Zone.

(6)	If the maximum absolute value of bz is less than a threshold set by the appropriate TAC subcommittee, not to exceed 0.2, and the total capacity of Boundary Generation Resources that would cluster into a different Congestion Zone is less than 1,500 MW, then the element is a CRE for the particular CSC.

#### 7.2.1.4  Approval Process

By November 1 of each year, the appropriate ERCOT subcommittee will report to the TAC and ERCOT Board with recommended CSC designations, resulting Congestion Zone boundaries, CRE designations and associated Boundary Generation Resources for ERCOT Board review and approval.

Immediately following ERCOT Board approval changes in the CSCs and the resulting Congestion Zone boundaries and CRE designations will be posted on the MIS.  This posting will include a bus-by-bus identification of each Congestion Zone.

Effective July 1, 2003, for the remainder of the year 2003, a North to Houston CSC will be established with no changes to existing Congestion Zone boundaries, CRE designations, or associated Boundary Generation Resources.

| ## Sponsor |  |
| --- | --- |
| Name | Kent Saathoff |
| E-mail Address | ksaathoff@ercot.com |
| Company | ERCOT |
| Company Address | 2700 W. Lake Drive, Taylor TX |
| Phone Number | 512 248 3011 |
| Fax Number | 512 248 3082 |
| ## ERCOT Impact Analysis |  |
| Date |  |
| Financial Impact |  |
| Analysis |  |
| ## TAC Action |  |
| TAC Vote Date |  |
| TAC Proposed Implementation Date |  |
| TAC Vote Results |  |
| ## ERCOT Board Action |  |
| ERCOT Board Vote Date |  |
| ERCOT Board Implementation Date |  |
| ERCOT Board Results |  |
| ## Appeal |  |
| Appeal Date |  |
| Entity Initiating Appeal |  |
| Nature of Appeal |  |