| PRR Number | 418PRR | PRR Title | Creation of New CSC |
| --- | --- | --- | --- |
| ###### Recommended Action |  | Approve |  |
| Proposed Effective Date |  | July 1, 2003 |  |
| Priority & Rank Assigned |  |  |  |
| Summary of Impact Analysis |  | Impacts TCR auction process. |  |
| Benefit |  | Improved management of a transmission constraint. |  |
| PRR Background |  | Modify Section 7.2.1.3, create a new section 7.2.1.4, Approval Process and add language related to a new CSC effective July 1, 2003 through the remainder of 2003. Operational conditions require a new CSC to allow Zonal deployment of Balancing Energy to resolve a North to Houston constraint. |  |
| Protocol Revision Subcommittee Recommendation |  | PRS recommends approval of PRR418 as revised. |  |

Recommended Language

#### Approval Process

By November 1 of each year, the appropriate ERCOT subcommittee will report to the TAC and ERCOT Board with recommended CSC designations, resulting Congestion Zone boundaries, CRE designations and associated Boundary Generation Resources for ERCOT Board review and approval.

Immediately following ERCOT Board approval changes in the CSCs and the resulting Congestion Zone boundaries and CRE designations will be posted on the MIS.  This posting will include a bus-by-bus identification of each Congestion Zone.

- Special CSC and CREs for 2003 Only

Effective July 1, 2003, for the remainder of the year 2003 only, a North to Houston CSC will be established with no changes to existing Congestion Zone boundaries, CRE designations, or associated Boundary Generation Resources.  The new CSC for the remainder of 2003 is in addition to the CSCs approved by ERCOT market participants in November of 2002, and it is comprised of the double circuit Gibbons Creek – O’Brien 345 kV line and Jewett – T.H. Wharton 345  kV line. New CREs for this CSC are:

1.the Twin Oak – Gibbons Creek 345 kV double circuit line,

2. the double circuit comprised of Jewett – T.H. Wharton 345  kV line and the Jewett– Tomball 345 kV line,

3.  the double circuit comprised of the Roan’s Prairie – Kuykendahl 345 kV line, and the Jewett– Tomball 345 kV line.

4.  the double circuit comprised of Gibbons Creek to Roan’s Prairie 345kV line, and the Jewett to Tomball 345 kV line,

- the double circuit comprised of the 345 kV Gibbons Creek to O’Brien line and the Gibbons Creek to Roan’s Prairie 345 kV line,

- the 345-138 kV autotransformer 1 at T.H. Wharton.

In addition to the changes shown above, the following changes need to be made to other sections of the Protocols:

      - Special Monthly TCR Auction Procedure for the Special CSC for 2003

For the special CSC described in Section 7.2.1.5, two separate TCR auction processes will be used.  For the July 2003 TCR auction only, a non-combinatorial auction shall be used (similar to the process previously used by ERCOT prior to implementation of the single-round, simultaneous combinatorial auction process currently used) with an abbreviated notice period.  For the monthly TCR auctions for August through December 2003, a single-round, simultaneous combinatorial auction process shall be used.  There will be no change in the TCR Auction Notices contained in Section 7.5.3.2.1 for this special CSC except for the initial month.  No annual TCR auction process will be used for this special CSC.

7.5.6 (2) The cost of PCRs shall be equal to fifteen (15) percent of the applicable annual TCR auction clearing price for each CSC for which a PCR is allocated.  For special CSCs created in accordance with Section 7.2.1.5 above, the cost of the PCRs shall be equal to fifteen (15) percent of the applicable monthly TCR auction clearing price.

| PRR Evaluation |  |
| --- | --- |
| Non-ERCOT Market Comparison | No Comparison conducted. |
| Comments Author | Comments Summary |
| TXU | Revised language. |
| Constellation | Revised language. |
| ## Sponsor |  |
| Name | Kent Saathoff |
| E-mail Address | ksaathoff@ercot.com |
| Company | ERCOT |
| Company Address | 2700 W. Lake Drive, Taylor TX |
| Phone Number | 512 248 3011 |
| Fax Number | 512 248 3082 |