| PRR Number | 413 | PRR Title | Optimization for the whole operating day in RPRS procurement process |
| --- | --- | --- | --- |
| ###### Date |  | 6-13-2003 |  |
| ## Submitter’s Information |  |  |  |
| Name |  | Julie Gauldin |  |
| E-mail Address |  | Julie.Gauldin@puc.state.tx.us |  |
| Company |  | PUCT |  |
| Company Address |  |  |  |
| Phone Number |  | 512-936-7340 |  |
| Fax Number |  |  |  |

Comments

PUCT supports this PRR; optimization of RPRS market over the whole day is an improvement over the current methodology.

These comments are meant to clarify the language and not to change the proposal.

I’ve added one change to section 7.4.2 – this section which wasn’t originally included in the PRR.  The statement I removed from 7.4.2  is confusing and neither Jun nor I could figured out what it is intended to mean.

| PRR Number | 413PRR | PRR Title | Optimization for the whole operating day in RPRS procurement process |
| --- | --- | --- | --- |
| ###### Status |  |  |  |
| Protocol Section Requiring Revision |  | 4.4.16, ERCOT Receipt of Replacement Reserve Service Bids 6.6.3.2.1, ERCOT Ancillary Services Procurement during Adjustment Period (AP) 6.8.1.10, Zonal or System Wide Replacement Reserve Capacity Payment to QSE 6.8.1.11, Local Congestion Replacement Reserve Payment to QSE |  |
| Requested Resolution |  | ## Normal |  |
| Revision Description |  | Modify RPRS procurement process so that RPRS procurement process is an optimum solution for the whole operating day. |  |
| ###### Reason for Revision |  | To change ERCOT software system to produce an optimum solution for the whole operating day in RPRS procurement process. |  |
| ## Timeline |  |  |  |
| Date Received |  | 5/13/2003 |  |
| Date Posted |  | 5/13/2003 |  |
| Comments Due |  | 6/03/2003 |  |
| PRS Review Date |  | 6/19/2003 |  |
| PRS Recommendation |  | 7/11/2003 |  |
| TAC Consideration |  | 8/07/2003 |  |
| Board Consideration |  | 8/19/2003 |  |

### 4.4.16	ERCOT Receipt of Replacement Reserve Service Bids

QSEs may submit unit-specific Replacement Reserve Service (RPRS) bids to ERCOT by 1600.  RPRS bids will have the following components:

- A dollar per megawatt capacity price at which the supplier will provide the service;

- An hourly dollar per megawatt operational price;

- Designation of the specific Resource;

- Designation of the amount of capacity represented by the bid;

- The hours that the bid is effective;

- The minimum time in minutes until the Resource is available for Balancing Energy deployment; and

- An expiration time for the bid.

[PRR211: Current design does not allow for the submission of a unit specific UBES bid curve with RPRS capacity bids. Current design allows only one UBES curve per zone per QSE and one DBES curve per zone per QSE.]

(3)	A Balancing Energy bid  point in $/MWh for each hour that the RPRS capacity bid is effective;

##### Specific Procurement Process Requirements for Replacement Reserve Service in the Adjustment Period

ERCOT shall procure Replacement Reserve Service (RPRS) in the AP as follows:

- ERCOT will evaluate Zonal Congestion, Local Congestion, and capacity insufficiency using ERCOT’s Operational Model, balanced QSE schedules, Resource Plans and ERCOT forecast of next day Load.

- ERCOT will define the level of Resources available to meet next-day reliability needs of the ERCOT System based on QSE schedule submissions, Resource Plans and ERCOT Load forecast.  ERCOT will determine incremental Resource capacity available from Generation Resources that are off-line, or Generation Resources that are expected to be off-line in the requested hours or Loads acting as a Resource shown as available in the Resource Plans.

- RPRS procurement produces an optimum solution for the whole Operating Day.  The RPRS procurement resolves capacity inadequacy, Zonal Congestion and Local Congestion problems for each hour simultaneously based on the capacity price and the operational price submitted by the QSEs, as described in Section 4.4.16, ERCOT Receipt of Replacement Reserve Service Bids.  The solution of the RPRS is a result of ERCOT performing analysis of the current physical system operations for each hour to recognize potential transmission constraints that would require Resources not currently planned to be available. The purpose and use of the RPRS procurement is to provide capacity from which energy would be available to solve the following system security violations:

- ERCOT System capacity insufficiency using any RPRS bid;

- Zonal Congestion using the RPRS bids by Congestion Zone in bid price Merit Order and the current physical system operations in the ERCOT System; and

- Local Congestion using location specific Resource bids for RPRS and the current physical system operations in the ERCOT System.

- ERCOT will solve security violations using a transmission security-constrained mathematical optimization application.  The application will solve as if each bid can be proportioned into individual MW bids. The objective of the optimization is to minimize the total bid-based cost based on capacity price, operation price, resource shift factors, and minimum time until available for BES deployment, as described in Section 4.4.16, ERCOT Receipt of Replacement Reserve Service Bids, for the whole Operating Day while satisfying all the security constraints for each hour.

- In the event there is an insufficient amount of RPRS bids submitted to provide a feasible solution to the system security violations or to resolve Local Congestion, ERCOT will use OOMC to acquire the needed capacity.

- The costs associated with resolving system security violations will be identified separately into the following categories: capacity inadequacy, Zonal Congestion and Local Congestion.

- The Market Clearing Prices on the capacity insufficiency, CSC constraint and Operational Constraint will represent the marginal cost for the solution of each constraint and will be produced as an output of the mathematical optimization application.  The output of the application will be as follows:

- The marginal cost (Shadow Price of the power balance constraint) to solve system insufficiency defines MCPC for insufficiency.

- The marginal cost (Shadow Price of the CSC constraint) to solve a CSC constraint defines the congestion price of the CSC constraint.

- The bidder of RPRS is paid the higher of RPRS bid price and MCPC of the Congestion Zone unless the bid has been selected to solve Local Congestion. RPRS bidders whose bids are taken to solve Local Congestion are paid the OOMC price when there is no Market Solution and are paid bid price when there is a Market Solution.  RPRS bid price is equal to the operation price plus the capacity price, as described in Section 4.4.16, ERCOT Receipt of Replacement Reserve Service Bids, divided by the number of hours during which the Resource is continuously procured.

(8)	QSEs whose schedules have impacts on CSCs according to the Commercial Model (using zonal Shift Factors at the time of RPRS procurement for each Zone) shall be charged Congestion costs associated with the impact.

- (9)	The costs of resolving Local Congestion are based on the amount of capacity required to solve Local Congestion. This cost will be tracked by specific constraint to aid the determination of the potential addition to the constraint as a CSC.

(10)	If all of the cost of RPRS is not allocated by one of the above methods, then the allocation will be uplifted to all QSEs based on the Load Ratio Share for the relevant period.  If ERCOT collects more RPRS costs in this manner than is necessary, the excess funds collected by ERCOT will be credited to all QSEs based on the Load Ratio Share for the relevant period.

(11)	The RMR Units will be considered as unavailable in RPRS procurement.

(12)	In the case of tied bids for the selection of RPRS, ERCOT will select the bid that meets the requirement most closely (achieving the optimal solution).  When the price and capacity are identical from unaffiliated bidders, ERCOT may request re-bids.

(13)	For RPRS, for each hour, for each Congestion Zone, ERCOT will post the quantity of capacity procured and the MCPCs and Shadow Prices.

(14)	On award of RPRS capacity, the energy bid  point provided by the QSE selected to provide RPRS will be added to the Balancing Energy Up Service bid stack for the period of time the RPRS is awarded.

#### 6.8.1.10	Zonal or System Wide Replacement Reserve Service Capacity Payment to QSE

A QSE whose unit bid to provide RPRS to ERCOT is accepted in ERCOT’s Ancillary Service procurement process shall be paid for services in the amount (in megawatts) of RPRS capacity accepted by ERCOT, multiplied by the maximum of the bid price or the highest MCPC of that interval per zone of all the RPRS procurement processes for a single Operating Hour, excluding any RPRS procured to resolve Local Congestion and is paid as follows:

PCRPqi 	= 	 SUM (PCRPqiz)z

Given:

PCRPqiuz 	= 	-1 * Max(PABCRPiu, MCPCRPiz )* ZCRPqiuz

PABCRpiu	=	CBPRPiu/N + HOBPRPiu

The equation below will be used to determine the Total Replacement Reserve Payment to be allocated to each QSE as described in Section 6.9.2, QSE Obligations for Capacity Services Procured in the Day Ahead and Adjustment Periods.

PCRPi 		= 	SUM (PCRPqi)q

Where:

i 	interval being calculated

z	zone

u 	single Resource

N	Number of hours that this Resource is continuously procured

CBPRPiu	Capacity bid price submitted by the QSE for the single Resource

HOBPRPiu	Hourly operational bid price submitted by the QSE for the single Resource

MCPCRPiz	Highest Replacement Reserve Service Market Clearing Price of all procurement processes of Capacity ($/MW) per interval per zone, excluding any RPRS procured to resolve Local Congestion.

PABCRPiu	Bid Price of the single Resource awarded Replacement Reserve in that interval

PCRPqiuz	Replacement Reserve Payments ($) Service Payment by zone per interval for that QSE for that Resource

PCRPqiu	Replacement Reserve Payments ($) Service Payment per interval for that QSE for that Resource

PCRPi	Summation of Replacement Reserve Payments ($) per interval for all QSEs in the market

ZCRPqiuz	Accepted Unit Adjustment Replacement Reserve Service Capacity Quantity (MW) to solve insufficiency or Zonal Congestion per interval per zone per QSE per single Resource.

#### 6.8.1.11	Local Congestion Replacement Reserve Payment to QSE when a Market Solution exists

LPCRPiq 	= 	 -1 *	SUM (PABCRPiu * LCRPqiu)u

PABCRpiu	=	CBPRPiu/N + HOBPRPiu

The equation below will be used to determine the Total Replacement Reserve Payment to be allocated to each QSE as described in Section 6.9.2, QSE Obligations for Capacity Services Procured in the Day Ahead and Adjustment Periods.

LPCRPi	=	SUM (LPCRPiq)q

Where:

i 	interval being calculated

u 	single Resource

q	QSE

N	Number of hours that this Resource is continuously procured

CBPRPiu	Capacity bid price submitted by the QSE for the single Resource

HOBPRPiu	Hourly operational bid price submitted by the QSE for the single Resource

LPCRPiq	Replacement Reserve Payments ($) Service Payment per interval for that QSE

LPCRPi	Summations of Local Congestion Replacement Reserve Payments ($) per interval for all QSEs.

LCRPqiu	Accepted single Resource Replacement Reserve Service Capacity to solve Local Congestion (MW) per interval per QSE for that single Resource

PABCRPiu	Bid Price of the single Resource awarded Replacement Reserve of that interval

### 7.4.2 Resolution of Local Congestion

ERCOT will procure RPRS if necessary to provide sufficient capacity to resolve forecast Local Congestion. ERCOT will purchase RPRS from the Resource(s) having the lowest cost per megawatt based on each Resource’s bid price and its ability to resolve the Congestion given its Shift Factor on the Local Congestion.

| ## Sponsor |  |
| --- | --- |
| Name | Jun Yu |
| E-mail Address | jyu@ercot.com |
| Company | ERCOT |
| Company Address | 2705 West Lake Drive, Taylor, TX 76574 |
| Phone Number | 512-248-3969 |
| Fax Number | 512-248-5650 |
| ## ERCOT Impact Analysis |  |
| Date |  |
| Financial Impact |  |
| Analysis |  |
| ## TAC Action |  |
| TAC Vote Date |  |
| TAC Proposed Implementation Date |  |
| TAC Vote Results |  |
| ## ERCOT Board Action |  |
| ERCOT Board Vote Date |  |
| ERCOT Board Implementation Date |  |
| ERCOT Board Results |  |
| ## Appeal |  |
| Appeal Date |  |
| Entity Initiating Appeal |  |
| Nature of Appeal |  |