Texas

Standard

Electronic

Transaction

814_05:

##### Premise Information and Enrollment Response

Electronic Data Interchange

ANSI ASC X12 Ver/Rel 004010

Transaction Set 814

Texas 814_05:

PREMISE INFORMATION AND

ENROLLMENT RESPONSE

This transaction set, from ERCOT to the new CR is essentially a pass through of the TDSP’s 814_04 information.  This transaction will complete the new CR’s enrollment request.

Document Flow:

- ERCOT to New CR

|  |  | # Summary of Changes |
| --- | --- | --- |
| July 3, 2000 Version 1.0 |  | Initial Release |
| August 23, 2000 Version 1.1 |  | The following changes were made: |
|  |  | - Changed Historical Usage request codes and examples and removed the LIN10 through LIN13. |
|  |  | - Changed Life Support (REF~SU) to remove “Required” and add the following note, “We are not sure of the requirements of this information at this time. When the rules are finalized we will revisit whether the life support indicator will be sent.” |
|  |  | - Added rejection reason codes “A83” and “API” in REF~7G |
|  |  | - Corrected REF~1P~W11 Status code to say that the date will be provided in DTM~150, not DTM~MRR. |
|  |  | - Change transaction description page |
| December 12, 2000 Version 1.2 |  | The following changes were made: |
|  |  | - Change the term Utility to Transmission Distribution Service Provider (TDSP) to match ERCOT Protocols. |
|  |  | - Change the term Retail Electric Provider (REP) to Competitive Retailer (CR) to match ERCOT Protocols. |
|  |  | - Change description page transaction purpose to read the same as ERCOT Protocols. |
|  |  | - Insert the “How to use this Implementation Guide” page. |
|  |  | - Remove “.” between X and 12 in all references to ANSI ASC X12. |
|  |  | - Add BGN08 Element to indicate the TX SET Transaction Number per Change Control #2000-015. |
|  |  | - Add examples to end of transaction. |
|  |  | - Add Customer Name and Service Address—N1, N3 and N4 per Change Control #2000-034. |
|  |  | - Add the wording, “The following examples are all inclusive. Other combinations cannot be requested; for example, both HI and HU cannot be requested for the same ESI ID” to the LIN loop gray box. per Change Control #2000-026. |
|  |  | - Add REF~AQ Distribution Loss Factor to the LIN loop per Change Control #2000-035. |
|  |  | - Add REF~SPL Station ID to the LIN loop per Change Control #2000-004. |
|  |  | - Add Time of Use code 71-- Summer Super On-Peak to REF04 of REF~4P and REF~IX Segments in the NM1 loop per Change Control #2000-18. |
|  |  | - Add REF~PRT Unmetered Service class in the NM1 loop per Change Control #2000-002. |
|  |  | - Add NFI to REF02 of REF~7G segment in the LIN loop per Change Control #2000-010. |
| March 16, 2001 Version 1.3 |  | The following changes were made: |
|  |  | - Fixed X12 Structure Page |
|  |  | - Fixed How to Use this Implementation Guideline Page |
|  |  | - Removed MVO (Move Out) from LIN07 and LIN09 and fixed examples |
|  |  | - Removed "DTM~MRR is required when requesting an off cycle read." from LIN SW code because this is a carryover from the Request transaction and has been confusing market participants by making them think they needed to include the DTM~MRR in the Response, which is not valid. |
|  |  | - Removed second “Enrollment Request” in the LIN gray box |
| March 16, 2001 Version 1.3 (Continued) |  | - Removed Scenario Names from page two |
|  |  | - Changed RA/Clearinghouse to ERCOT |
|  |  | - Example 1 – changed the Rate Class to be the same for both meters (RS1) |
| August 3, 2001 Version 1.4 |  | The following changes were made: |
|  |  | - Modify gray box in REF~4P. Ref. 2001-118 |
|  |  | - Add new un-metered service types. Ref. 2001-047 |
|  |  | - Modify gray box in BGN06. Remove reference to 814_24. Ref. 2001-078 |
|  |  | - Modify gray box in LIN. Remove reference to 814_24. Ref. 2001-078 |
|  |  | - Modify gray box in REF~LO. Ref. 2001-116 |
|  |  | - Modify examples to show real load profiles and station id’s. Ref. 2001-116 |
|  |  | - Modify gray box on REF01 of REF~LO. Ref. 2001-116 |
|  |  | - Modify gray box in REF~SPL. Ref. 2001-116 |
|  |  | - Modify gray box in REF~7G. Ref. 2001-116 |
|  |  | - Modified example in REF~IX. Changed from K1MON to KHMON |
|  |  | - Fixed example in gray box of REF~MT from “REF~IX~6.0~TU^51” to “REF~IX~6.0~KHMON~TU^51” |
|  |  | Included REDLINES from CHANGE CONTROLS: 2001-193 10/24/01 - 12/05/01 2002-222 02/22/02 |
|  |  | 2001-168 – applied changes to REF~TZ and DTM~313 for consistency 2001-119 – 02/27/02 – Supports multiple reject reasons |
| June 17th, 2002 Version 1.5 |  | The following changes were made: |
|  |  | Change Control 2001-161 – Added REF~PTC (Premise Type) |
|  |  | Change Control 2001-179 – Removed “Quarterly” from gray box on REF02 in REF~MT |
|  |  | Change Control 2001-215 – Modified Gray box language in the REF~PRT (Un-metered Service Type) |
|  |  | Change Control 2002-236 – Added detail to Codes for the REF~PTC |
|  |  | Change Control 2002-246 – Added example of Reject transaction from ERCOT to New CR |
|  |  | Change Control 2002-259 – Changed Life Support Indicator to Special Needs and eliminated “I” value. |
|  |  | Change Control 2002-266 - This change control allows ERCOT to internally close the ‘In Review” switches when an 814_04 reject comes in from the TDSP as well as allows the CRs to receive the 814_05 reject to close the instance of the switch. All parties will be in synch with this change |
|  |  | Change Control 2002-267 - Removed “9” value from the N103 in the N1~AY loop. |
|  |  | Change Control 2002-271 – Added reject reason of “BIM” to REF~7G |
|  |  | Change Control 2002-272 – Added reject reason of “FRB” to REF~7G |
|  |  | Change Control 2002-285 – Added language to gray box in REF~SU and REF~LO and provided example of rejection transaction to new CR |
|  |  | Change Control 2002-304 – Added reject reasons of “ABN” and “NFI” to REF~7G |
| 8/5/02 |  | Change Control 2002-338 – Correct Typo in REF~SU |
| 8/5/02 |  | Change Control 2002-345 – Corrected SE count in Examples |
| 9/15/02 |  | Change Control 2002-359 – Removed Gray box in REF~7G on “ACI” code. |
|  |  | Change Control 2002-363 – CLEAN UP: Added missed language from CC 2002-246 to ASI gay box. |
| 9/15/02 |  | Change Control 2002-365 – Modified gray box of the “BIM” reject code in the REF~7G |
| 10/15/02 |  | Change Control 2002-417 – Added reject reason “ANK” to the REF~7G |
|  |  | Change Control 2002-420 – Replaced references to POLR with AREP |

How to Use this Implementation Guide

This section is used to show the X12 Rules for this segment.  You must look further into the grayboxes below for Texas Rules.

Segment:	REF Reference Identification (ESI ID)

Position:	030

Loop:	LIN        Optional

Level:	Detail

Usage:	Optional

Max Use:	>1

Purpose:	To specify identifying information

Syntax Notes:	1	At least one of REF02 or REF03 is required.

2	If either C04003 or C04004 is present, then the other is required.

3	If either C04005 or C04006 is present, then the other is required.

Semantic Notes:	1	REF04 contains data relating to the value cited in REF02.

Comments:

This section is used to show the Texas Rules for implementation of this segment.

| Notes: |  | Required |
| --- | --- | --- |
|  |  | REF~Q5~~10111111234567890ABCDEFGHIJKLMNOPQRS |

Data Element Summary

One or more examples.

Ref.	Data

Des.	Element	Name	Attributes

| # Must Use | REF01 | 128 | Reference Identification Qualifier |  |  |  | M |  | ID 2/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code qualifying the Reference Identification |  |  |  |  |  |  |  |  |
|  |  |  |  | Q5 |  | Property Control Number |  |  |  |  |  |
|  |  |  |  |  |  | Electric Service Identifier (ESI ID) |  |  |  |  |  |
| # Must Use | REF03 | 352 | Description |  |  |  | X |  | AN 1/80 |  |  |
|  |  |  | A free-form description to clarify the related data elements and their content |  |  |  |  |  |  |  |  |
|  |  |  | ESI ID |  |  |  |  |  |  |  |  |

These are X12 code descriptions, which often do not relate to the Texas descriptions.

X12 cannot keep up with Texas needs, thus, Texas often changes the meaning of existing codes.  See the corresponding graybox for the Texas definitions.

This column shows the X12 attributes for each data element.

M = Mandatory

O= Optional

X = Conditional

AN = Alphanumeric

N# = Implied Decimal at position #

ID = Identification

R = Real

1/30 = Minimum 1, Maximum 30

This column shows the Texas use of each data element.

814 General Request, Response or Confirmation

ANSI ASC X12 Structure

Functional Group ID=GE

Introduction:

This Draft Standard for Trial Use contains the format and establishes the data contents of the General Request, Response or Confirmation Transaction Set (814) for use within the context of an Electronic Data Interchange (EDI) environment. This standard can be used to request actions to be performed, to respond to a request for actions to be performed or to confirm information related to actions performed.

Heading:

Pos.	Seg.		Req.		Loop	Notes and

No.	ID	Name	Des.	Max.Use	Repeat	Comments

| M | 010 | ST | Transaction Set Header | M | 1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| M | 020 | BGN | Beginning Segment | M | 1 |  |  |  |  |  |  |  |  |
|  |  |  | LOOP ID - N1 |  |  | >1 |  |  |  |  |  |  |  |
|  | 040 | N1 | Name | O | 1 |  | n1 |  |  |  |  |  |  |
|  | 060 | N3 | Address Information | O | 2 |  | nl |  |  |  |  |  |  |
|  | 070 | N4 | Geographic Location | O | 1 |  | nl |  |  |  |  |  |  |

Detail:

Pos.	Seg.		Req.		Loop	Notes and

No.	ID	Name	Des.	Max.Use	Repeat	Comments

|  |  |  | LOOP ID - LIN |  |  | >1 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 010 | LIN | Item Identification | O | 1 |  |  |  |  |  |  |  |  |
|  | 020 | ASI | Action or Status Indicator | O | 1 |  |  |  |  |  |  |  |  |
|  | 030 | REF | Reference Identification | O | >1 |  |  |  |  |  |  |  |  |
|  | 040 | DTM | Date/Time Reference | O | >1 |  |  |  |  |  |  |  |  |
|  |  |  | LOOP ID - NM1 |  |  | >1 |  |  |  |  |  |  |  |
|  | 080 | NM1 | Individual or Organizational Name | O | 1 |  | n2 |  |  |  |  |  |  |
|  | 130 | REF | Reference Identification | O | >1 |  |  |  |  |  |  |  |  |
|  | 140 | DTM | Date/Time Reference | O | >1 |  |  |  |  |  |  |  |  |
| M | 150 | SE | Transaction Set Trailer | M | 1 |  |  |  |  |  |  |  |  |

Transaction Set Notes

1.	The N1 loop is used to identify the transaction sender and receiver.

2.	The NM1 loop is used to identify the parties associated with the individual line item (LIN), such as an individual consumer in a consolidated third party Consumer Service Provider transaction.

Segment:	ST Transaction Set Header

Position:	010

Loop:

Level:	Heading

Usage:	Mandatory

Max Use:	1

Purpose:	To indicate the start of a transaction set and to assign a control number

Syntax Notes:

Semantic Notes:	1	The transaction set identifier (ST01) is used by the translation routines of the interchange partners to select the appropriate transaction set definition (e.g., 810 selects the Invoice Transaction Set).

Comments:

| Notes: |  | Required |
| --- | --- | --- |
|  |  | ST~814~000000001 |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | ST01 | 143 | Transaction Set Identifier Code |  |  |  | M |  | ID 3/3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code uniquely identifying a Transaction Set |  |  |  |  |  |  |  |
|  |  |  |  | 814 |  | General Request, Response or Confirmation |  |  |  |  |
| Must Use | ST02 | 329 | Transaction Set Control Number |  |  |  | M |  | AN 4/9 |  |
|  |  |  | Identifying control number that must be unique within the transaction set functional group assigned by the originator for a transaction set |  |  |  |  |  |  |  |

Segment:	BGN Beginning Segment

Position:	020

Loop:

Level:	Heading

Usage:	Mandatory

Max Use:	1

Purpose:	To indicate the beginning of a transaction set

Syntax Notes:	1	If BGN05 is present, then BGN04 is required.

Semantic Notes:	1	BGN02 is the transaction set reference number.

2	BGN03 is the transaction set date.

3	BGN04 is the transaction set time.

4	BGN05 is the transaction set time qualifier.

5	BGN06 is the transaction set reference number of a previously sent transaction affected by the current transaction.

Comments:

| Notes: |  | Required |
| --- | --- | --- |
|  |  | BGN~11~200104021200719~20000402~~~200104011956531~~5 |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | BGN01 | 353 | Transaction Set Purpose Code |  |  |  | M |  | ID 2/2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code identifying purpose of transaction set |  |  |  |  |  |  |  |  |
|  |  |  |  | 11 |  | Response |  |  |  |  |  |
| Must Use | BGN02 | 127 | Reference Identification |  |  |  | M |  | AN 1/30 |  |  |
|  |  |  | Reference information as defined for a particular Transaction Set or as specified by the Reference Identification Qualifier |  |  |  |  |  |  |  |  |
|  |  |  | A unique transaction identification number assigned by the originator of this transaction. This number must be unique over time. Transaction Reference numbers will only contain uppercase letters (A to Z) and digits (0 to 9). Note that punctuation (spaces, dashes, etc.) must be excluded. |  |  |  |  |  |  |  |  |
| Must Use | BGN03 | 373 | Date |  |  |  | M |  | DT 8/8 |  |  |
|  |  |  | Date expressed as CCYYMMDD |  |  |  |  |  |  |  |  |
|  |  |  | The transaction creation date - the date that the data was processed by the sender's application system. |  |  |  |  |  |  |  |  |
| Must Use | BGN06 | 127 | Reference Identification |  |  |  | O |  | AN 1/30 |  |  |
|  |  |  | Reference information as defined for a particular Transaction Set or as specified by the Reference Identification Qualifier |  |  |  |  |  |  |  |  |
|  |  |  | Refers to the BGN02 of the... ... Enrollment Request (814_01), ... Drop to AREP Request (814_10), ... Move In Request (814_16), or ... Continuous Service Agreement CR Move In Request (814_22) This number will be tracked in the BGN06 through the lifecycle of the respective process. |  |  |  |  |  |  |  |  |
| Must Use | BGN08 | 306 | Action Code |  |  |  | O |  | ID 1/2 |  |  |
|  |  |  | Code indicating type of action |  |  |  |  |  |  |  |  |
|  |  |  | This segment is used to initially identify the type of 814 that is being sent or received. Ignore the ANSI X12 definition of the code. |  |  |  |  |  |  |  |  |
|  |  |  |  | 5 |  | Send |  |  |  |  |  |
|  |  |  |  |  |  | Indicates Texas SET Transaction 814_05 |  |  |  |  |  |

Segment:	N1 Name (Customer)

Position:	040

Loop:	N1        Optional

Level:	Heading

Usage:	Optional

Max Use:	1

Purpose:	To identify a party by type of organization, name, and code

Syntax Notes:	1	At least one of N102 or N103 is required.

2	If either N103 or N104 is present, then the other is required.

Semantic Notes:

Comments:	1	This segment, used alone, provides the most efficient method of providing organizational identification. To obtain this efficiency the "ID Code" (N104) must provide a key to the table maintained by the transaction processing party.

2	N105 and N106 further define the type of entity in N101.

| Notes: |  | Accept Response: Required Reject Response: Not Used |
| --- | --- | --- |
|  |  | N1~8R~PREMISE N1~8R~X |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | N101 | 98 | Entity Identifier Code |  |  |  | M |  | ID 2/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code identifying an organizational entity, a physical location, property or an individual |  |  |  |  |  |  |  |  |
|  |  |  |  | 8R |  | Consumer Service Provider (CSP) Customer |  |  |  |  |  |
|  |  |  |  |  |  | Customer |  |  |  |  |  |
| Must Use | N102 | 93 | Name |  |  |  | X |  | AN 1/60 |  |  |
|  |  |  | Free-form name |  |  |  |  |  |  |  |  |
|  |  |  | This name field must be used in order to populate the service address in N3 and N4. The sender can determine what they put in this field. |  |  |  |  |  |  |  |  |

Segment:	N3 Address Information (Customer Service Address)

Position:	060

Loop:	N1        Optional

Level:	Heading

Usage:	Optional

Max Use:	2

Purpose:	To specify the location of the named party

Syntax Notes:

Semantic Notes:

Comments:

| Notes: |  | Accept Response: Required Reject Response: Not Used |
| --- | --- | --- |
|  |  | N3~123 N MAIN ST~ANY ADDRESS OVERFLOW |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | N301 | 166 | Address Information | M |  | AN 1/55 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Address information |  |  |  |  |
|  |  |  | Customer Service Address |  |  |  |  |
| Dep | N302 | 166 | Address Information | O |  | AN 1/55 |  |
|  |  |  | Address information |  |  |  |  |
|  |  |  | Customer Service Address Overflow Condition: If there is an overflow, it must be sent. |  |  |  |  |

Segment:	N4 Geographic Location (Customer Service Address)

Position:	070

Loop:	N1        Optional

Level:	Heading

Usage:	Optional

Max Use:	1

Purpose:	To specify the geographic place of the named party

Syntax Notes:	1	If N406 is present, then N405 is required.

Semantic Notes:

Comments:	1	A combination of either N401 through N404, or N405 and N406 may be adequate to specify a location.

2	N402 is required only if city name (N401) is in the U.S. or Canada.

| Notes: |  | Accept Response: Required Reject Response: Not Used |
| --- | --- | --- |
|  |  | N4~ANYTOWN~TX~78111 |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | N401 | 19 | City Name | O |  | AN 2/30 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Free-form text for city name |  |  |  |  |
| Must Use | N402 | 156 | State or Province Code | O |  | ID 2/2 |  |
|  |  |  | Code (Standard State/Province) as defined by appropriate government agency |  |  |  |  |
| Must Use | N403 | 116 | Postal Code | O |  | ID 3/15 |  |
|  |  |  | Code defining international postal zone code excluding punctuation and blanks (zip code for United States) |  |  |  |  |
|  |  |  | Postal codes will only contain digits (0 to 9). Note that punctuation (spaces, dashes, etc.) must be excluded. |  |  |  |  |

Segment:	N1 Name (Transmission Distribution Service Provider)

Position:	040

Loop:	N1        Optional

Level:	Heading

Usage:	Optional

Max Use:	1

Purpose:	To identify a party by type of organization, name, and code

Syntax Notes:	1	At least one of N102 or N103 is required.

2	If either N103 or N104 is present, then the other is required.

Semantic Notes:

Comments:	1	This segment, used alone, provides the most efficient method of providing organizational identification. To obtain this efficiency the "ID Code" (N104) must provide a key to the table maintained by the transaction processing party.

2	N105 and N106 further define the type of entity in N101.

| Notes: |  | Required |
| --- | --- | --- |
|  |  | N1~8S~TDSP COMPANY~1~007909411 |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | N101 | 98 | Entity Identifier Code |  |  |  | M |  | ID 2/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code identifying an organizational entity, a physical location, property or an individual |  |  |  |  |  |  |  |  |
|  |  |  |  | 8S |  | Consumer Service Provider (CSP) |  |  |  |  |  |
|  |  |  |  |  |  | Transmission Distribution Service Provider (TDSP) |  |  |  |  |  |
| Must Use | N102 | 93 | Name |  |  |  | X |  | AN 1/60 |  |  |
|  |  |  | Free-form name |  |  |  |  |  |  |  |  |
|  |  |  | TDSP Name |  |  |  |  |  |  |  |  |
| Must Use | N103 | 66 | Identification Code Qualifier |  |  |  | X |  | ID 1/2 |  |  |
|  |  |  | Code designating the system/method of code structure used for Identification Code (67) |  |  |  |  |  |  |  |  |
|  |  |  |  | 1 |  | D-U-N-S Number, Dun & Bradstreet |  |  |  |  |  |
|  |  |  |  | 9 |  | D-U-N-S+4, D-U-N-S Number with Four Character Suffix |  |  |  |  |  |
| Must Use | N104 | 67 | Identification Code |  |  |  | X |  | AN 2/80 |  |  |
|  |  |  | Code identifying a party or other code |  |  |  |  |  |  |  |  |
|  |  |  | TDSP D-U-N-S Number or D-U-N-S + 4 Number |  |  |  |  |  |  |  |  |

Segment:	N1 Name (ERCOT)

Position:	040

Loop:	N1        Optional

Level:	Heading

Usage:	Optional

Max Use:	1

Purpose:	To identify a party by type of organization, name, and code

Syntax Notes:	1	At least one of N102 or N103 is required.

2	If either N103 or N104 is present, then the other is required.

Semantic Notes:

Comments:	1	This segment, used alone, provides the most efficient method of providing organizational identification. To obtain this efficiency the "ID Code" (N104) must provide a key to the table maintained by the transaction processing party.

2	N105 and N106 further define the type of entity in N101.

| Notes: |  | Required |
| --- | --- | --- |
|  |  | N1~AY~ERCOT~1~183529049~~41 |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | N101 | 98 | Entity Identifier Code |  |  |  | M |  | ID 2/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code identifying an organizational entity, a physical location, property or an individual |  |  |  |  |  |  |  |  |
|  |  |  |  | AY |  | Clearinghouse |  |  |  |  |  |
|  |  |  |  |  |  | ERCOT |  |  |  |  |  |
| Must Use | N102 | 93 | Name |  |  |  | X |  | AN 1/60 |  |  |
|  |  |  | Free-form name |  |  |  |  |  |  |  |  |
|  |  |  | ERCOT Name |  |  |  |  |  |  |  |  |
| Must Use | N103 | 66 | Identification Code Qualifier |  |  |  | X |  | ID 1/2 |  |  |
|  |  |  | Code designating the system/method of code structure used for Identification Code (67) |  |  |  |  |  |  |  |  |
|  |  |  |  | 1 |  | D-U-N-S Number, Dun & Bradstreet |  |  |  |  |  |
| Must Use | N104 | 67 | Identification Code |  |  |  | X |  | AN 2/80 |  |  |
|  |  |  | Code identifying a party or other code |  |  |  |  |  |  |  |  |
|  |  |  | ERCOT D-U-N-S Number or D-U-N-S + 4 Number |  |  |  |  |  |  |  |  |
| Must Use | N106 | 98 | Entity Identifier Code |  |  |  | O |  | ID 2/3 |  |  |
|  |  |  | Code identifying an organizational entity, a physical location, property or an individual |  |  |  |  |  |  |  |  |
|  |  |  |  | 41 |  | Submitter |  |  |  |  |  |

Segment:	N1 Name (Competitive Retailer)

Position:	040

Loop:	N1        Optional

Level:	Heading

Usage:	Optional

Max Use:	1

Purpose:	To identify a party by type of organization, name, and code

Syntax Notes:	1	At least one of N102 or N103 is required.

2	If either N103 or N104 is present, then the other is required.

Semantic Notes:

Comments:	1	This segment, used alone, provides the most efficient method of providing organizational identification. To obtain this efficiency the "ID Code" (N104) must provide a key to the table maintained by the transaction processing party.

2	N105 and N106 further define the type of entity in N101.

| Notes: |  | Required |
| --- | --- | --- |
|  |  | N1~SJ~CR NAME~1~007909422~~40 |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | N101 | 98 | Entity Identifier Code |  |  |  | M |  | ID 2/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code identifying an organizational entity, a physical location, property or an individual |  |  |  |  |  |  |  |  |
|  |  |  |  | SJ |  | Service Provider |  |  |  |  |  |
|  |  |  |  |  |  | Competitive Retailer (CR) |  |  |  |  |  |
| Must Use | N102 | 93 | Name |  |  |  | X |  | AN 1/60 |  |  |
|  |  |  | Free-form name |  |  |  |  |  |  |  |  |
|  |  |  | CR Name |  |  |  |  |  |  |  |  |
| Must Use | N103 | 66 | Identification Code Qualifier |  |  |  | X |  | ID 1/2 |  |  |
|  |  |  | Code designating the system/method of code structure used for Identification Code (67) |  |  |  |  |  |  |  |  |
|  |  |  |  | 1 |  | D-U-N-S Number, Dun & Bradstreet |  |  |  |  |  |
|  |  |  |  | 9 |  | D-U-N-S+4, D-U-N-S Number with Four Character Suffix |  |  |  |  |  |
| Must Use | N104 | 67 | Identification Code |  |  |  | X |  | AN 2/80 |  |  |
|  |  |  | Code identifying a party or other code |  |  |  |  |  |  |  |  |
|  |  |  | CR D-U-N-S Number or D-U-N-S + 4 Number |  |  |  |  |  |  |  |  |
| Must Use | N106 | 98 | Entity Identifier Code |  |  |  | O |  | ID 2/3 |  |  |
|  |  |  | Code identifying an organizational entity, a physical location, property or an individual |  |  |  |  |  |  |  |  |
|  |  |  |  | 40 |  | Receiver |  |  |  |  |  |

Segment:	LIN Item Identification

Position:	010

Loop:	LIN        Optional

Level:	Detail

Usage:	Optional

Max Use:	1

Purpose:	To specify basic item identification data

Syntax Notes:	1	If either LIN04 or LIN05 is present, then the other is required.

2	If either LIN06 or LIN07 is present, then the other is required.

3	If either LIN08 or LIN09 is present, then the other is required.

4	If either LIN10 or LIN11 is present, then the other is required.

5	If either LIN12 or LIN13 is present, then the other is required.

6	If either LIN14 or LIN15 is present, then the other is required.

7	If either LIN16 or LIN17 is present, then the other is required.

8	If either LIN18 or LIN19 is present, then the other is required.

9	If either LIN20 or LIN21 is present, then the other is required.

10	If either LIN22 or LIN23 is present, then the other is required.

11	If either LIN24 or LIN25 is present, then the other is required.

12	If either LIN26 or LIN27 is present, then the other is required.

13	If either LIN28 or LIN29 is present, then the other is required.

14	If either LIN30 or LIN31 is present, then the other is required.

Semantic Notes:	1	LIN01 is the line item identification

Comments:	1	See the Data Dictionary for a complete list of IDs.

2	LIN02 through LIN31 provide for fifteen different product/service IDs for each item. For example: Case, Color, Drawing No., U.P.C. No., ISBN No., Model No., or SKU.

| Notes: |  | This LIN will contain the same services (e.g., HU) listed in the LIN of the... ... Enrollment Request (814_01) ... Drop to AREP Request (814_10) ... Move In Request (814_16) ... Continuous Service Agreement CR Move In Request (814_22) Essentially, it is an "echo" except that the LIN01 may be different and the services do not have to be in the same order. |
| --- | --- | --- |
|  |  | Required |
|  |  | The following examples are all inclusive. Other combinations cannot be requested; for example, both HI and HU, or both SW and MVI, cannot be requested for the same ESI ID. LIN~1~SH~EL~SH~CE Normal Switch LIN~1~SH~EL~SH~CE~SH~SW Switch with Off Cycle Read LIN~1~SH~EL~SH~CE~SH~HU Normal Switch with Historical Usage Request LIN~1~SH~EL~SH~CE~SH~SW~SH~HU LIN~1~SH~EL~SH~CE~SH~HU~SH~SW Switch with Off Cycle Read and Historical Usage Request LIN~1~SH~EL~SH~CE~SH~HI Normal Switch with Detail Historical Interval Usage Request LIN~1~SH~EL~SH~CE~SH~SW~SH~HI LIN~1~SH~EL~SH~CE~SH~HI~SH~SW Switch with Off Cycle Read and Detail Historical Interval Usage Request LIN~1~SH~EL~SH~CE~SH~MVI Move In LIN~1~SH~EL~SH~CE~SH~MVI~SH~HU LIN~1~SH~EL~SH~CE~SH~HU~SH~MVI Move In with Historical Usage Request LIN~1~SH~EL~SH~CE~SH~MVI~SH~HI LIN~1~SH~EL~SH~CE~SH~HI~SH~MVI Move In with Detail Historical Interval Usage Request |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | LIN01 | 350 | Assigned Identification |  |  |  | O |  | AN 1/20 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Alphanumeric characters assigned for differentiation within a transaction set |  |  |  |  |  |  |  |  |
| Must Use | LIN02 | 235 | Product/Service ID Qualifier |  |  |  | M |  | ID 2/2 |  |  |
|  |  |  | Code identifying the type/source of the descriptive number used in Product/Service ID (234) |  |  |  |  |  |  |  |  |
|  |  |  |  | SH |  | Service Requested |  |  |  |  |  |
| Must Use | LIN03 | 234 | Product/Service ID |  |  |  | M |  | AN 1/48 |  |  |
|  |  |  | Identifying number for a product or service |  |  |  |  |  |  |  |  |
|  |  |  | Commodity |  |  |  |  |  |  |  |  |
|  |  |  |  | EL |  | Electric Service |  |  |  |  |  |
| Must Use | LIN04 | 235 | Product/Service ID Qualifier |  |  |  | X |  | ID 2/2 |  |  |
|  |  |  | Code identifying the type/source of the descriptive number used in Product/Service ID (234) |  |  |  |  |  |  |  |  |
|  |  |  |  | SH |  | Service Requested |  |  |  |  |  |
| Must Use | LIN05 | 234 | Product/Service ID |  |  |  | X |  | AN 1/48 |  |  |
|  |  |  | Identifying number for a product or service |  |  |  |  |  |  |  |  |
|  |  |  |  | CE |  | Energy Services |  |  |  |  |  |
|  | LIN06 | 235 | Product/Service ID Qualifier |  |  |  | X |  | ID 2/2 |  |  |
|  |  |  | Code identifying the type/source of the descriptive number used in Product/Service ID (234) |  |  |  |  |  |  |  |  |
|  |  |  |  | SH |  | Service Requested |  |  |  |  |  |
|  | LIN07 | 234 | Product/Service ID |  |  |  | X |  | AN 1/48 |  |  |
|  |  |  | Identifying number for a product or service |  |  |  |  |  |  |  |  |
|  |  |  |  | HI |  | Historical Interval Usage |  |  |  |  |  |
|  |  |  |  |  |  | Request to obtain Historical Interval Usage for this premise by interval. If the intervals are available, the TDSP will send the 867_02 Historical Usage, containing the following two PTD Loops: PTD~BO (Interval Summary) PTD~PM (Interval Detail) If this ESI ID does not have interval meters, or if the intervals are not available, the TDSP will send the 867_02 Historical Usage, containing any of the following PTD Loops, as appropriate, along with a status code REF~1P~HIU: PTD~BD (Unmetered Services) PTD~BO (Interval Summary) PTD~PL (Non-Interval Detail) |  |  |  |  |  |
|  |  |  |  | HU |  | Historical Summarized Usage |  |  |  |  |  |
|  |  |  |  |  |  | Request to obtain Historical Summarized Usage for this premise (interval, non-interval or unmetered). The TDSP will send the 867_02 Historical Usage, which may contain any of the following PTD Loops: PTD~BD (Unmetered Services) PTD~BO (Interval Summary) PTD~PL (Non-Interval Detail) |  |  |  |  |  |
|  |  |  |  | MVI |  | Move In |  |  |  |  |  |
|  |  |  |  |  |  | Request for a customer enrollment when a new customer moves into a premise. |  |  |  |  |  |
|  |  |  |  | SW |  | Special Read for Off-cycle Switch |  |  |  |  |  |
|  |  |  |  |  |  | Request for a special meter read for a customer to switch off cycle, i.e., not at the next, regularly scheduled meter read. |  |  |  |  |  |
|  | LIN08 | 235 | Product/Service ID Qualifier |  |  |  | X |  | ID 2/2 |  |  |
|  |  |  | Code identifying the type/source of the descriptive number used in Product/Service ID (234) |  |  |  |  |  |  |  |  |
|  |  |  |  | SH |  | Service Requested |  |  |  |  |  |
|  | LIN09 | 234 | Product/Service ID |  |  |  | X |  | AN 1/48 |  |  |
|  |  |  | Identifying number for a product or service |  |  |  |  |  |  |  |  |
|  |  |  |  | HI |  | Historical Interval Usage |  |  |  |  |  |
|  |  |  |  | HU |  | Historical Summarized Usage |  |  |  |  |  |
|  |  |  |  | MVI |  | Move In |  |  |  |  |  |
|  |  |  |  | SW |  | Special Read for Off-cycle Switch |  |  |  |  |  |

Segment:	ASI Action or Status Indicator

Position:	020

Loop:	LIN        Optional

Level:	Detail

Usage:	Optional

Max Use:	1

Purpose:	To indicate the action to be taken with the information provided or the status of the entity described

Syntax Notes:

Semantic Notes:

Comments:

| Notes: |  | Identifies the action to be taken or the status of a requested action for the service identified in the LIN segment. |
| --- | --- | --- |
|  |  | Required |
|  |  | ASI~WQ~021 |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | ASI01 | 306 | Action Code |  |  |  | M |  | ID 1/2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code indicating type of action |  |  |  |  |  |  |  |
|  |  |  |  | U |  | Reject |  |  |  |  |
|  |  |  |  |  |  | The CR will have to create a new original if still needed. |  |  |  |  |
|  |  |  |  | WQ |  | Accept |  |  |  |  |
| Must Use | ASI02 | 875 | Maintenance Type Code |  |  |  | M |  | ID 3/3 |  |
|  |  |  | Code identifying the specific type of item maintenance |  |  |  |  |  |  |  |
|  |  |  |  | 021 |  | Addition |  |  |  |  |

Segment:	REF Reference Identification (Status Reason)

Position:	030

Loop:	LIN        Optional

Level:	Detail

Usage:	Optional

Max Use:	>1

Purpose:	To specify identifying information

Syntax Notes:	1	At least one of REF02 or REF03 is required.

2	If either C04003 or C04004 is present, then the other is required.

3	If either C04005 or C04006 is present, then the other is required.

Semantic Notes:	1	REF04 contains data relating to the value cited in REF02.

Comments:

| Notes: |  | Accept Response: Required when status information must be conveyed. Reject Response: Not Used |
| --- | --- | --- |
|  |  | REF~1P~HUU~HISTORICAL USAGE UNAVAILABLE |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | REF01 | 128 | Reference Identification Qualifier |  |  |  | M |  | ID 2/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code qualifying the Reference Identification |  |  |  |  |  |  |  |  |
|  |  |  |  | 1P |  | Accessorial Status Code |  |  |  |  |  |
|  |  |  |  |  |  | Used on the response when the request is accepted, and additional status information must be provided. |  |  |  |  |  |
| Must Use | REF02 | 127 | Reference Identification |  |  |  | X |  | AN 1/30 |  |  |
|  |  |  | Reference information as defined for a particular Transaction Set or as specified by the Reference Identification Qualifier |  |  |  |  |  |  |  |  |
|  |  |  |  | A13 |  | Other |  |  |  |  |  |
|  |  |  |  |  |  | Explanation Required in REF03. |  |  |  |  |  |
|  |  |  |  | HIU |  | Historical Interval Usage Unavailable |  |  |  |  |  |
|  |  |  |  |  |  | Historical Usage summarized to a meter level will be provided instead (867_02). |  |  |  |  |  |
|  |  |  |  | HUU |  | Historical Usage Unavailable |  |  |  |  |  |
|  |  |  |  |  |  | No Historical Usage |  |  |  |  |  |
|  |  |  |  | MDI |  | Move Date Inconsistent |  |  |  |  |  |
|  |  |  |  |  |  | The dates of the Move Out and Move In overlap. The TDSP will use the Move In date as the official switch date. |  |  |  |  |  |
|  |  |  |  | MMO |  | Missing Move Out |  |  |  |  |  |
|  |  |  |  |  |  | No Move Out Pending - This will result in a forced move out (i.e., the current CR will receive a Drop Request - 814_06). |  |  |  |  |  |
|  |  |  |  | W08 |  | Switch Date or Switch Code Invalid or Missing. Meter will be read on the normal, on-cycle read date |  |  |  |  |  |
|  |  |  |  | W09 |  | Special, Off-cycle meter reading cannot be performed. Meter will be read on the normal, on-cycle read date. |  |  |  |  |  |
|  |  |  |  | W11 |  | Special, Off-cycle meter read will occur on a date other than the requested date, as identified in DTM~150. |  |  |  |  |  |
|  |  |  |  |  |  | Special meter read must be before the next scheduled meter read. |  |  |  |  |  |
| Dep | REF03 | 352 | Description |  |  |  | X |  | AN 1/80 |  |  |
|  |  |  | A free-form description to clarify the related data elements and their content |  |  |  |  |  |  |  |  |
|  |  |  | Used to further describe the reason code sent in REF02. Code "A13" requires a text explanation in this element. |  |  |  |  |  |  |  |  |

Segment:	REF Reference Identification (Rejection Reason)

Position:	030

Loop:	LIN        Optional

Level:	Detail

Usage:	Optional

Max Use:	>1

Purpose:	To specify identifying information

Syntax Notes:	1	At least one of REF02 or REF03 is required.

2	If either C04003 or C04004 is present, then the other is required.

3	If either C04005 or C04006 is present, then the other is required.

Semantic Notes:	1	REF04 contains data relating to the value cited in REF02.

Comments:

| Notes: |  | More than one rejection reason code may be sent by repeating the REF~7G segment. Accept Response: Not Used Reject Response: Required |
| --- | --- | --- |
|  |  | REF~7G~A13~ADDITIONAL REASON TEXT HERE |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | REF01 | 128 | Reference Identification Qualifier |  |  |  | M |  | ID 2/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code qualifying the Reference Identification |  |  |  |  |  |  |  |  |
|  |  |  |  | 7G |  | Data Quality Reject Reason |  |  |  |  |  |
|  |  |  |  |  |  | Reject reasons associated with a reject status notification. |  |  |  |  |  |
| Must Use | REF02 | 127 | Reference Identification |  |  |  | X |  | AN 1/30 |  |  |
|  |  |  | Reference information as defined for a particular Transaction Set or as specified by the Reference Identification Qualifier |  |  |  |  |  |  |  |  |
|  |  |  |  | A13 |  | Other |  |  |  |  |  |
|  |  |  |  |  |  | Explanation Required in REF03. |  |  |  |  |  |
|  |  |  |  | A76 |  | ESI ID invalid or not found |  |  |  |  |  |
|  |  |  |  | A83 |  | Invalid or Unauthorized Action |  |  |  |  |  |
|  |  |  |  |  |  | Information provided was not supported in the Texas SET Standards. |  |  |  |  |  |
|  |  |  |  | ACI |  | Action Code (ASI01) Invalid |  |  |  |  |  |
|  |  |  |  | ANK |  | Invalid Source Information |  |  |  |  |  |
|  |  |  |  |  |  | Unnecessary Billing Information Included |  |  |  |  |  |
|  |  |  |  | API |  | Required information missing |  |  |  |  |  |
|  |  |  |  |  |  | Explanation Required in REF03. May not be used in place of other, more specific error codes. |  |  |  |  |  |
|  |  |  |  | BIM |  | Billing Information Missing |  |  |  |  |  |
|  |  |  |  |  |  | Information required in the N1~BT (Customer Billing Loop) not received. Used by Muni/Coops only. |  |  |  |  |  |
|  |  |  |  | D76 |  | DUNs Number invalid or not found |  |  |  |  |  |
|  |  |  |  | FRB |  | Incorrect Billing Type (REF~BLT) Requested |  |  |  |  |  |
|  |  |  |  |  |  | Billing type indicator not supported by Muni/Coop market. |  |  |  |  |  |
|  |  |  |  | MTI |  | Maintenance Type Code (ASI02) Invalid |  |  |  |  |  |
|  |  |  |  | NFI |  | Not First In |  |  |  |  |  |
|  |  |  |  |  |  | Used if the switch date is greater than the Drop to AREP date. |  |  |  |  |  |
| Dep | REF03 | 352 | Description |  |  |  | X |  | AN 1/80 |  |  |
|  |  |  | A free-form description to clarify the related data elements and their content |  |  |  |  |  |  |  |  |
|  |  |  | Used to further describe the reason code sent in REF02. Codes "A13" and "API" require a text explanation in this element. |  |  |  |  |  |  |  |  |

Segment:	REF Reference Identification (Distribution Loss Factor Code)

Position:	030

Loop:	LIN        Optional

Level:	Detail

Usage:	Optional

Max Use:	>1

Purpose:	To specify identifying information

Syntax Notes:	1	At least one of REF02 or REF03 is required.

2	If either C04003 or C04004 is present, then the other is required.

3	If either C04005 or C04006 is present, then the other is required.

Semantic Notes:	1	REF04 contains data relating to the value cited in REF02.

Comments:

| Notes: |  | The Distribution Loss Factor Code is used to identify the related loss factor value that is used in settlement. The code must be the contained in a loss factor table that the TDSP has provided to ERCOT. It is usually related to the delivery voltage or rate class. It is not the same as the transformer loss factor used when the TDSP meters service at a different point than it delivers service. |
| --- | --- | --- |
|  |  | Accept Response: Required Reject Response: Not Used |
|  |  | REF~AQ~T |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | REF01 | 128 | Reference Identification Qualifier |  |  |  | M |  | ID 2/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code qualifying the Reference Identification |  |  |  |  |  |  |  |  |
|  |  |  |  | AQ |  | Access Code |  |  |  |  |  |
|  |  |  |  |  |  | Distribution Loss Factor Code |  |  |  |  |  |
| Must Use | REF02 | 127 | Reference Identification |  |  |  | X |  | AN 1/30 |  |  |
|  |  |  | Reference information as defined for a particular Transaction Set or as specified by the Reference Identification Qualifier |  |  |  |  |  |  |  |  |
|  |  |  | With the exception of code "T", each Distribution Company will post what each of the next 5 codes represent and how they are applied. |  |  |  |  |  |  |  |  |
|  |  |  |  | A |  |  |  |  |  |  |  |
|  |  |  |  | B |  |  |  |  |  |  |  |
|  |  |  |  | C |  |  |  |  |  |  |  |
|  |  |  |  | D |  |  |  |  |  |  |  |
|  |  |  |  | E |  |  |  |  |  |  |  |
|  |  |  |  | T |  | Transmission Only |  |  |  |  |  |

Segment:	REF Reference Identification (Premise Type)

Position:	030

Loop:	LIN        Optional

Level:	Detail

Usage:	Optional

Max Use:	>1

Purpose:	To specify identifying information

Syntax Notes:	1	At least one of REF02 or REF03 is required.

2	If either C04003 or C04004 is present, then the other is required.

3	If either C04005 or C04006 is present, then the other is required.

Semantic Notes:	1	REF04 contains data relating to the value cited in REF02.

Comments:

| Notes: |  | Accept Response: Required Reject Response: Not Used |
| --- | --- | --- |
|  |  | REF~PTC~01 |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | REF01 | 128 | Reference Identification Qualifier |  |  |  | M |  | ID 2/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code qualifying the Reference Identification |  |  |  |  |  |  |  |  |
|  |  |  |  | PTC |  | Patent Type |  |  |  |  |  |
|  |  |  |  |  |  | Premise Type |  |  |  |  |  |
| Must Use | REF02 | 127 | Reference Identification |  |  |  | X |  | AN 1/30 |  |  |
|  |  |  | Reference information as defined for a particular Transaction Set or as specified by the Reference Identification Qualifier |  |  |  |  |  |  |  |  |
|  |  |  |  | 01 |  | Residential |  |  |  |  |  |
|  |  |  |  |  |  | Designates a premise where a customer will reside or an umetered service located at such a premise (e.g. Outdoor Lights) provided it is not a master metered, multi-family facility, or a facility metered as a commercial facility. |  |  |  |  |  |
|  |  |  |  | 02 |  | Small Non-residential |  |  |  |  |  |
|  |  |  |  |  |  | Designates a nonresidential premise with peak demand of 1 Megawatt or below (i.e. Where the demand for the previous 12 months is less than or equal to 1 megawatt) or an unmetered service located at such a premise. |  |  |  |  |  |
|  |  |  |  | 03 |  | Large Non-residential |  |  |  |  |  |
|  |  |  |  |  |  | Designates a nonresidential premise with peak demand greater than 1 Megawatt (i.e. Where demand for at least 1 month of the previous 12 months is greater than 1 Megawatt) or an unmetered service located at such a premise. |  |  |  |  |  |

Segment:	REF Reference Identification (ESI ID)

Position:	030

Loop:	LIN        Optional

Level:	Detail

Usage:	Optional

Max Use:	>1

Purpose:	To specify identifying information

Syntax Notes:	1	At least one of REF02 or REF03 is required.

2	If either C04003 or C04004 is present, then the other is required.

3	If either C04005 or C04006 is present, then the other is required.

Semantic Notes:	1	REF04 contains data relating to the value cited in REF02.

Comments:

| Notes: |  | Required |
| --- | --- | --- |
|  |  | REF~Q5~~10111111234567890ABCDEFGHIJKLMNOPQRS |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | REF01 | 128 | Reference Identification Qualifier |  |  |  | M |  | ID 2/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code qualifying the Reference Identification |  |  |  |  |  |  |  |  |
|  |  |  |  | Q5 |  | Property Control Number |  |  |  |  |  |
|  |  |  |  |  |  | Electric Service Identifier (ESI ID) |  |  |  |  |  |
| Must Use | REF03 | 352 | Description |  |  |  | X |  | AN 1/80 |  |  |
|  |  |  | A free-form description to clarify the related data elements and their content |  |  |  |  |  |  |  |  |
|  |  |  | ESI ID |  |  |  |  |  |  |  |  |

Segment:	REF Reference Identification (Station ID)

Position:	030

Loop:	LIN        Optional

Level:	Detail

Usage:	Optional

Max Use:	>1

Purpose:	To specify identifying information

Syntax Notes:	1	At least one of REF02 or REF03 is required.

2	If either C04003 or C04004 is present, then the other is required.

3	If either C04005 or C04006 is present, then the other is required.

Semantic Notes:	1	REF04 contains data relating to the value cited in REF02.

Comments:

| Notes: |  | The value shall correspond to an ERCOT-defined Station ID that is published on the ERCOT website. |
| --- | --- | --- |
|  |  | Accept Response: Required Reject Response: Not Used |
|  |  | REF~SPL~~WEST |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | REF01 | 128 | Reference Identification Qualifier |  |  |  | M |  | ID 2/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code qualifying the Reference Identification |  |  |  |  |  |  |  |  |
|  |  |  |  | SPL |  | Standard Point Location Code (SPLC) |  |  |  |  |  |
|  |  |  |  |  |  | Substation at which the TDSP's distribution system is connected to the transmission grid for the Service Delivery Point (SDP). The value shall correspond to an ERCOT-defined Station ID that is published on the ERCOT website. |  |  |  |  |  |
| Must Use | REF03 | 352 | Description |  |  |  | X |  | AN 1/80 |  |  |
|  |  |  | A free-form description to clarify the related data elements and their content |  |  |  |  |  |  |  |  |
|  |  |  | Station ID |  |  |  |  |  |  |  |  |

Segment:	REF Reference Identification (Special Needs Indicator)

Position:	030

Loop:	LIN        Optional

Level:	Detail

Usage:	Optional

Max Use:	>1

Purpose:	To specify identifying information

Syntax Notes:	1	At least one of REF02 or REF03 is required.

2	If either C04003 or C04004 is present, then the other is required.

3	If either C04005 or C04006 is present, then the other is required.

Semantic Notes:	1	REF04 contains data relating to the value cited in REF02.

Comments:

| Notes: |  | Accept Response: Required Reject Response: Not Used A "Y" in this field means that the customer is either: A residential customer who qualifies through the Residential Critical Care Eligibility Determination Form, as issued by the PUCT. Or A customer for which electric service is considered crucial for the protection and maintenance of public safety pursuant to Subst. Rules 25.52 and 25.53. Or An industrial customer for whom an interruption or suspension of service will create a dangerous or life threatening condition at the customers premise. |
| --- | --- | --- |
|  |  | REF~SU~Y |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | REF01 | 128 | Reference Identification Qualifier |  |  |  | M |  | ID 2/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code qualifying the Reference Identification |  |  |  |  |  |  |  |  |
|  |  |  |  | SU |  | Special Processing Code |  |  |  |  |  |
|  |  |  |  |  |  | Special Needs Indicator |  |  |  |  |  |
| Must Use | REF02 | 127 | Reference Identification |  |  |  | X |  | AN 1/30 |  |  |
|  |  |  | Reference information as defined for a particular Transaction Set or as specified by the Reference Identification Qualifier |  |  |  |  |  |  |  |  |
|  |  |  |  | N |  | No |  |  |  |  |  |
|  |  |  |  |  |  | Special Needs are not required |  |  |  |  |  |
|  |  |  |  | Y |  | Yes |  |  |  |  |  |
|  |  |  |  |  |  | Special Needs are required |  |  |  |  |  |

Segment:	DTM Date/Time Reference (Service Period Start)

Position:	040

Loop:	LIN        Optional

Level:	Detail

Usage:	Optional

Max Use:	>1

Purpose:	To specify pertinent dates and times

Syntax Notes:	1	At least one of DTM02 DTM03 or DTM05 is required.

2	If DTM04 is present, then DTM03 is required.

3	If either DTM05 or DTM06 is present, then the other is required.

Semantic Notes:

Comments:

| Notes: |  | Accept Response: Required Reject Response: Not Used |
| --- | --- | --- |
|  |  | DTM~150~20010115 |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | DTM01 | 374 | Date/Time Qualifier |  |  |  | M |  | ID 3/3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code specifying type of date or time, or both date and time |  |  |  |  |  |  |  |
|  |  |  |  | 150 |  | Service Period Start |  |  |  |  |
| Must Use | DTM02 | 373 | Date |  |  |  | X |  | DT 8/8 |  |
|  |  |  | Date expressed as CCYYMMDD |  |  |  |  |  |  |  |

Segment:	NM1 Individual or Organizational Name (Meter Level Information)

Position:	080

Loop:	NM1        Optional

Level:	Detail

Usage:	Optional

Max Use:	1

Purpose:	To supply the full name of an individual or organizational entity

Syntax Notes:	1	If either NM108 or NM109 is present, then the other is required.

2	If NM111 is present, then NM110 is required.

Semantic Notes:	1	NM102 qualifies NM103.

Comments:	1	NM110 and NM111 further define the type of entity in NM101.

| Notes: |  | Accept Response: Required Reject Response: Not Used |
| --- | --- | --- |
|  |  | NM1~MQ~3~~~~~~32~GE10349811 NM1~MQ~3~~~~~~93~UNMETERED |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | NM101 | 98 | Entity Identifier Code |  |  |  | M |  | ID 2/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code identifying an organizational entity, a physical location, property or an individual |  |  |  |  |  |  |  |  |
|  |  |  |  | MQ |  | Metering Location |  |  |  |  |  |
| Must Use | NM102 | 1065 | Entity Type Qualifier |  |  |  | M |  | ID 1/1 |  |  |
|  |  |  | Code qualifying the type of entity |  |  |  |  |  |  |  |  |
|  |  |  |  | 3 |  | Unknown |  |  |  |  |  |
| Must Use | NM108 | 66 | Identification Code Qualifier |  |  |  | X |  | ID 1/2 |  |  |
|  |  |  | Code designating the system/method of code structure used for Identification Code (67) |  |  |  |  |  |  |  |  |
|  |  |  |  | 32 |  | Assigned by Property Operator |  |  |  |  |  |
|  |  |  |  |  |  | Meter Number |  |  |  |  |  |
|  |  |  |  | 93 |  | Code assigned by the organization originating the transaction set |  |  |  |  |  |
|  |  |  |  |  |  | This code used when a Meter Number is NOT provided in NM109 (i.e., when NM109 contains "UNMETERED"). |  |  |  |  |  |
| Must Use | NM109 | 67 | Identification Code |  |  |  | X |  | AN 2/80 |  |  |
|  |  |  | Code identifying a party or other code |  |  |  |  |  |  |  |  |
|  |  |  | Meter Number or, UNMETERED - for Unmetered Services Meter numbers will only contain uppercase letters (A to Z) and digits (0 to 9). Note that punctuation (spaces, dashes, etc.) must be excluded, and leading and trailing zeros that are part of the meter number must be present. Use NONE when meter or unmetered devices are yet to be installed. |  |  |  |  |  |  |  |  |

Segment:	REF Reference Identification (Meter Multiplier)

Position:	130

Loop:	NM1        Optional

Level:	Detail

Usage:	Optional

Max Use:	>1

Purpose:	To specify identifying information

Syntax Notes:	1	At least one of REF02 or REF03 is required.

2	If either C04003 or C04004 is present, then the other is required.

3	If either C04005 or C04006 is present, then the other is required.

Semantic Notes:	1	REF04 contains data relating to the value cited in REF02.

Comments:

| Notes: |  | Not Used when NM109 = NONE. There will be one REF~4P segment for each Meter Type, creating the potential for more than one REF~4P segment per NM1 Loop. For instance, if the meter has multiple meter types, and therefore the REF~MT segment is REF~MT~COMBO, there will be multiple REF~4P segments, one to indicate the meter multiplier for each type. For example, NM1~MQ~3~~~~~~32~1234568MG REF~MT~COMBO REF~4P~1200~K1015~TU^41 REF~4P~1~KH015~TU^41 Note: Format of the multiplier could be in the form of a fraction (e.g. 48.78432) Also note that since the Meter Multiplier is required for each Time of Use, this segment is also used to determine which Time of Use will be sent on the 867 transaction. |
| --- | --- | --- |
|  |  | Accept Response: Required for each meter type for each meter that is used for billing purposes. This segment will not be provided in the NM1 loop for UNMETERED services. Reject Response: Not Used |
|  |  | REF~4P~10~KHMON~TU^41 REF~4P~1~K1MON~TU^41 |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | REF01 | 128 | Reference Identification Qualifier |  |  |  | M |  | ID 2/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code qualifying the Reference Identification |  |  |  |  |  |  |  |  |
|  |  |  |  | 4P |  | Affiliation Number |  |  |  |  |  |
|  |  |  |  |  |  | Meter Multiplier (Ending Reading - Beginning Reading) * Meter Multiplier = Billed Usage |  |  |  |  |  |
| Must Use | REF02 | 127 | Reference Identification |  |  |  | X |  | AN 1/30 |  |  |
|  |  |  | Reference information as defined for a particular Transaction Set or as specified by the Reference Identification Qualifier |  |  |  |  |  |  |  |  |
|  |  |  | Meter Multiplier |  |  |  |  |  |  |  |  |
| Must Use | REF03 | 352 | Description |  |  |  | X |  | AN 1/80 |  |  |
|  |  |  | A free-form description to clarify the related data elements and their content |  |  |  |  |  |  |  |  |
|  |  |  | Meter Type (see REF~MT for valid codes). "COMBO" is not a valid code for this element. |  |  |  |  |  |  |  |  |
| Must Use | REF04 | C040 | Reference Identifier |  |  |  | O |  |  |  |  |
|  |  |  | To identify one or more reference numbers or identification numbers as specified by the Reference Qualifier |  |  |  |  |  |  |  |  |
|  |  |  | Note this is a composite data element. Populate C04001 & C04002. |  |  |  |  |  |  |  |  |
| Must Use | C04001 | 128 | Reference Identification Qualifier |  |  |  | M |  | ID 2/3 |  |  |
|  |  |  | Code qualifying the Reference Identification |  |  |  |  |  |  |  |  |
|  |  |  |  | TU |  | Trial Location Code |  |  |  |  |  |
|  |  |  |  |  |  | Time of Use |  |  |  |  |  |
| Must Use | C04002 | 127 | Reference Identification |  |  |  | M |  | AN 1/30 |  |  |
|  |  |  | Reference information as defined for a particular Transaction Set or as specified by the Reference Identification Qualifier |  |  |  |  |  |  |  |  |
|  |  |  |  | 41 |  | Off Peak |  |  |  |  |  |
|  |  |  |  | 42 |  | On Peak |  |  |  |  |  |
|  |  |  |  | 43 |  | Intermediate Peak |  |  |  |  |  |
|  |  |  |  | 51 |  | Totalizer |  |  |  |  |  |
|  |  |  |  |  |  | Totalizer/Total/Max (Demand) |  |  |  |  |  |
|  |  |  |  | 71 |  | Summer Super On Peak |  |  |  |  |  |

Segment:	REF Reference Identification (Number of Dials)

Position:	130

Loop:	NM1        Optional

Level:	Detail

Usage:	Optional

Max Use:	>1

Purpose:	To specify identifying information

Syntax Notes:	1	At least one of REF02 or REF03 is required.

2	If either C04003 or C04004 is present, then the other is required.

3	If either C04005 or C04006 is present, then the other is required.

Semantic Notes:	1	REF04 contains data relating to the value cited in REF02.

Comments:

| Notes: |  | Not Used when NM109 = NONE. There will be one REF~IX segment for each applicable Meter Type, creating the potential for more than one REF~IX segment per NM1 Loop. For instance, if the meter has multiple types, and therefore the REF~MT segment is REF~MT~COMBO, there may be multiple REF~IX segments, one to indicate the number of dials for each type that has dials. For example, NM1~MQ~3~~~~~~32~1234568MG REF~MT~COMBO REF~IX~6.0~KHMON~TU^51 REF~IX~5.1~KHMON~TU^51 |
| --- | --- | --- |
|  |  | Accept Response: Required for each meter type for each meter that is used for billing purposes that has dials. This segment will not be provided in the NM1 loop for UNMETERED services or for demand. Reject Response: Not Used |
|  |  | REF~IX~6.0~KHMON~TU^51 |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | REF01 | 128 | Reference Identification Qualifier |  |  |  | M |  | ID 2/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code qualifying the Reference Identification |  |  |  |  |  |  |  |  |
|  |  |  |  | IX |  | Item Number |  |  |  |  |  |
|  |  |  |  |  |  | Number of Dials on the Meter displayed as X.Y. The notation X.Y means that the meter has X dials to the left of the decimal point, and Y dials to the right. |  |  |  |  |  |
| Must Use | REF02 | 127 | Reference Identification |  |  |  | X |  | AN 1/30 |  |  |
|  |  |  | Reference information as defined for a particular Transaction Set or as specified by the Reference Identification Qualifier |  |  |  |  |  |  |  |  |
|  |  |  | Number of Dials |  |  |  |  |  |  |  |  |
| Must Use | REF03 | 352 | Description |  |  |  | X |  | AN 1/80 |  |  |
|  |  |  | A free-form description to clarify the related data elements and their content |  |  |  |  |  |  |  |  |
|  |  |  | Meter Type (see REF~MT for valid codes). "COMBO" is not a valid code for this element. |  |  |  |  |  |  |  |  |
| Must Use | REF04 | C040 | Reference Identifier |  |  |  | O |  |  |  |  |
|  |  |  | To identify one or more reference numbers or identification numbers as specified by the Reference Qualifier |  |  |  |  |  |  |  |  |
|  |  |  | Note that this is a composite data element. Populate C04001 and C04002. |  |  |  |  |  |  |  |  |
| Must Use | C04001 | 128 | Reference Identification Qualifier |  |  |  | M |  | ID 2/3 |  |  |
|  |  |  | Code qualifying the Reference Identification |  |  |  |  |  |  |  |  |
|  |  |  |  | TU |  | Trial Location Code |  |  |  |  |  |
|  |  |  |  |  |  | Time of Use |  |  |  |  |  |
| Must Use | C04002 | 127 | Reference Identification |  |  |  | M |  | AN 1/30 |  |  |
|  |  |  | Reference information as defined for a particular Transaction Set or as specified by the Reference Identification Qualifier |  |  |  |  |  |  |  |  |
|  |  |  |  | 41 |  | Off Peak |  |  |  |  |  |
|  |  |  |  | 42 |  | On Peak |  |  |  |  |  |
|  |  |  |  | 43 |  | Intermediate Peak |  |  |  |  |  |
|  |  |  |  | 51 |  | Totalizer |  |  |  |  |  |
|  |  |  |  |  |  | Totalizer/Total/Max (Demand) |  |  |  |  |  |
|  |  |  |  | 71 |  | Summer Super On Peak |  |  |  |  |  |

Segment:	REF Reference Identification (Load Profile)

Position:	130

Loop:	NM1        Optional

Level:	Detail

Usage:	Optional

Max Use:	>1

Purpose:	To specify identifying information

Syntax Notes:	1	At least one of REF02 or REF03 is required.

2	If either C04003 or C04004 is present, then the other is required.

3	If either C04005 or C04006 is present, then the other is required.

Semantic Notes:	1	REF04 contains data relating to the value cited in REF02.

Comments:

| Notes: |  | The load profile is made up of the following codes, with mandatory underscores between each: Profile Group and Segment Code, Weather Zone Code, Meter Data Type Code, Weather Sensitivity Code, TOU Schedule Code. Accept Response: Required Reject Response: Not Used |
| --- | --- | --- |
|  |  | REF~LO~RESLOWR_WEST_NIDR_NWS_NOTOU |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | REF01 | 128 | Reference Identification Qualifier |  |  |  | M |  | ID 2/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code qualifying the Reference Identification |  |  |  |  |  |  |  |  |
|  |  |  |  | LO |  | Load Planning Number |  |  |  |  |  |
|  |  |  |  |  |  | Load profile |  |  |  |  |  |
| Must Use | REF02 | 127 | Reference Identification |  |  |  | X |  | AN 1/30 |  |  |
|  |  |  | Reference information as defined for a particular Transaction Set or as specified by the Reference Identification Qualifier |  |  |  |  |  |  |  |  |
|  |  |  | Load Profile |  |  |  |  |  |  |  |  |

Segment:	REF Reference Identification (Meter Type)

Position:	130

Loop:	NM1        Optional

Level:	Detail

Usage:	Optional

Max Use:	>1

Purpose:	To specify identifying information

Syntax Notes:	1	At least one of REF02 or REF03 is required.

2	If either C04003 or C04004 is present, then the other is required.

3	If either C04005 or C04006 is present, then the other is required.

Semantic Notes:	1	REF04 contains data relating to the value cited in REF02.

Comments:

| Notes: |  | Not Used when NM109 = NONE. There will only be one REF~MT segment for each NM1 loop. If there are multiple meter types on this meter, only one REF~MT will be sent. It will contain the code COMBO. The specific meter types will be identified in REF03 for the attribute segments contained within this loop. For example, REF~IX~6.0~KHMON~TU^51. |
| --- | --- | --- |
|  |  | Accept Response: Required for each meter that is used for billing purposes. This segment will not be provided in the NM1 loop for UNMETERED services. Reject Response: Not Used |
|  |  | REF~MT~KHMON REF~MT~COMBO |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | REF01 | 128 | Reference Identification Qualifier |  |  |  | M |  | ID 2/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code qualifying the Reference Identification |  |  |  |  |  |  |  |  |
|  |  |  |  | MT |  | Meter Ticket Number |  |  |  |  |  |
|  |  |  |  |  |  | Meter Type |  |  |  |  |  |
| Must Use | REF02 | 127 | Reference Identification |  |  |  | X |  | AN 1/30 |  |  |
|  |  |  | Reference information as defined for a particular Transaction Set or as specified by the Reference Identification Qualifier |  |  |  |  |  |  |  |  |
|  |  |  | When REF01 is MT, the meter type is expressed as a five-character field. The first two characters are the type of consumption, the last three characters are the metering interval reported for billing purposes. "COMBO" is used for a meter that records more than one measurement. Valid values can be a combination of the following values: Type of Consumption K1 Kilowatt Demand (kW) K2 Kilovolt Amperes Reactive Demand (kVAR) K3 Kilovolt Amperes Reactive Hour (kVARH) K4 Kilovolt Amperes (kVA) KH Kilowatt Hour (kWh) Metering Interval Reported for Billing Purposes nnn Number of minutes from 001 to 999 DAY Daily MON Monthly For Example: KHMON Kilowatt Hours Per Month K1015 Kilowatt Demand per 15 minute interval Other Valid Codes COMBO This code is used to indicate that the meter has multiple measurements, e.g., one meter that measures both kWh and Demand. |  |  |  |  |  |  |  |  |

Segment:	REF Reference Identification (TDSP Rate Class)

Position:	130

Loop:	NM1        Optional

Level:	Detail

Usage:	Optional

Max Use:	>1

Purpose:	To specify identifying information

Syntax Notes:	1	At least one of REF02 or REF03 is required.

2	If either C04003 or C04004 is present, then the other is required.

3	If either C04005 or C04006 is present, then the other is required.

Semantic Notes:	1	REF04 contains data relating to the value cited in REF02.

Comments:

| Notes: |  | Not Used when NM109 = NONE. Accept Response: Required for each meter that is used for billing purposes. This segment must also be sent when account has UNMETERED services available. Reject Response: Not Used |
| --- | --- | --- |
|  |  | REF~NH~RS1 |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | REF01 | 128 | Reference Identification Qualifier |  |  |  | M |  | ID 2/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code qualifying the Reference Identification |  |  |  |  |  |  |  |  |
|  |  |  |  | NH |  | Rate Card Number |  |  |  |  |  |
|  |  |  |  |  |  | TDSP Rate Class or Tariff |  |  |  |  |  |
| Must Use | REF02 | 127 | Reference Identification |  |  |  | X |  | AN 1/30 |  |  |
|  |  |  | Reference information as defined for a particular Transaction Set or as specified by the Reference Identification Qualifier |  |  |  |  |  |  |  |  |
|  |  |  | TDSP Rate Class or Tariff |  |  |  |  |  |  |  |  |

Segment:	REF Reference Identification (TDSP Rate Subclass)

Position:	130

Loop:	NM1        Optional

Level:	Detail

Usage:	Optional

Max Use:	>1

Purpose:	To specify identifying information

Syntax Notes:	1	At least one of REF02 or REF03 is required.

2	If either C04003 or C04004 is present, then the other is required.

3	If either C04005 or C04006 is present, then the other is required.

Semantic Notes:	1	REF04 contains data relating to the value cited in REF02.

Comments:

| Notes: |  | Not Used when NM109 = NONE. Accept Response: Required if maintained by TDSP, must be sent for each meter that is used for billing purposes. This segment must also be sent when account has UNMETERED services available. Reject Response: Not Used |
| --- | --- | --- |
|  |  | REF~PR~123 |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | REF01 | 128 | Reference Identification Qualifier |  |  |  | M |  | ID 2/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code qualifying the Reference Identification |  |  |  |  |  |  |  |  |
|  |  |  |  | PR |  | Price Quote Number |  |  |  |  |  |
|  |  |  |  |  |  | TDSP Rate Subclass - Used to provide further classification of a rate. |  |  |  |  |  |
| Must Use | REF02 | 127 | Reference Identification |  |  |  | X |  | AN 1/30 |  |  |
|  |  |  | Reference information as defined for a particular Transaction Set or as specified by the Reference Identification Qualifier |  |  |  |  |  |  |  |  |
|  |  |  | TDSP Rate Subclass |  |  |  |  |  |  |  |  |

Segment:	REF Reference Identification (Unmetered Service Type)

Position:	130

Loop:	NM1        Optional

Level:	Detail

Usage:	Optional

Max Use:	>1

Purpose:	To specify identifying information

Syntax Notes:	1	At least one of REF02 or REF03 is required.

2	If either C04003 or C04004 is present, then the other is required.

3	If either C04005 or C04006 is present, then the other is required.

Semantic Notes:	1	REF04 contains data relating to the value cited in REF02.

Comments:

| Notes: |  | There will be one REF~PRT segment for each applicable Unmetered Service Meter Type, creating the potential for more than one REF~PRT segment per NM1 Loop. This segment is used to provide additional information to the CR for the specified Unmetered Service. It describes the type of device which this measurement loop references and can include additional text information which may be useful to the CR (i.e., a specific wattage of a light, additional text information for further clarification, etc.) |
| --- | --- | --- |
|  |  | Accept Response: Required for each unmetered service type used for billing purposes. Reject Response: Not Used |
|  |  | REF~PRT~MV~750~QQ^100 REF~PRT~SD~400 Company Owned~QQ^20 REF~PRT~MV~400 Customer Owned~QQ^15 REF~PRT~MV~Third party maintained facilities~QQ^100 |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | REF01 | 128 | Reference Identification Qualifier |  |  |  | M |  | ID 2/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code qualifying the Reference Identification |  |  |  |  |  |  |  |  |
|  |  |  |  | PRT |  | Product Type |  |  |  |  |  |
|  |  |  |  |  |  | Defined Unmetered Service Type |  |  |  |  |  |
| Must Use | REF02 | 127 | Reference Identification |  |  |  | X |  | AN 1/30 |  |  |
|  |  |  | Reference information as defined for a particular Transaction Set or as specified by the Reference Identification Qualifier |  |  |  |  |  |  |  |  |
|  |  |  | If a new code is needed, it must be requested via the Change Control process. |  |  |  |  |  |  |  |  |
|  |  |  |  | AN |  | Antenna |  |  |  |  |  |
|  |  |  |  | AR |  | Argon |  |  |  |  |  |
|  |  |  |  | BB |  | Bill Boards |  |  |  |  |  |
|  |  |  |  | BS |  | Bus Shelters |  |  |  |  |  |
|  |  |  |  | CU |  | Cat Unit |  |  |  |  |  |
|  |  |  |  | ED |  | Electronic Devise |  |  |  |  |  |
|  |  |  |  | FL |  | Fluorescent |  |  |  |  |  |
|  |  |  |  | HA |  | Historical/Antique |  |  |  |  |  |
|  |  |  |  | IN |  | Incandescent |  |  |  |  |  |
|  |  |  |  | LV |  | Levys |  |  |  |  |  |
|  |  |  |  | MH |  | Metal Halide |  |  |  |  |  |
|  |  |  |  | MV |  | Mercury Vapor |  |  |  |  |  |
|  |  |  |  | OT |  | Other Unmetered |  |  |  |  |  |
|  |  |  |  | PA |  | Power Analog node |  |  |  |  |  |
|  |  |  |  | PB |  | Phone Booth |  |  |  |  |  |
|  |  |  |  | PO |  | Phone Outlet |  |  |  |  |  |
|  |  |  |  | PS |  | Pump Station |  |  |  |  |  |
|  |  |  |  | RR |  | Rail Road Crossings |  |  |  |  |  |
|  |  |  |  | SD |  | Sodium |  |  |  |  |  |
|  |  |  |  | TL |  | Traffic Lights |  |  |  |  |  |
|  |  |  |  | TR |  | Transceiver |  |  |  |  |  |
|  |  |  |  | WM |  | Wallpacked Mercury Vapor |  |  |  |  |  |
|  |  |  |  | WS |  | Warning Sirens |  |  |  |  |  |
| Dep | REF03 | 352 | Description |  |  |  | X |  | AN 1/80 |  |  |
|  |  |  | A free-form description to clarify the related data elements and their content |  |  |  |  |  |  |  |  |
|  |  |  | Used to provide additional clarification information to the CR for the Unmetered Service when necessary. When applicable, used to provide the specific wattage/lumens for a light. This free-form text can not contain any characters that may be used as element delimiters, sub-element delimiters, segment terminators, or field separators(This includes: asterisk *, pipes ¦, tabs, linefeeds, carets ^, angle brackets < >, and tildes ~). |  |  |  |  |  |  |  |  |
| Must Use | REF04 | C040 | Reference Identifier |  |  |  | O |  |  |  |  |
|  |  |  | To identify one or more reference numbers or identification numbers as specified by the Reference Qualifier |  |  |  |  |  |  |  |  |
|  |  |  | Note that this is a composite data element. Populate C04001 and C04002. |  |  |  |  |  |  |  |  |
| Must Use | C04001 | 128 | Reference Identification Qualifier |  |  |  | M |  | ID 2/3 |  |  |
|  |  |  | Code qualifying the Reference Identification |  |  |  |  |  |  |  |  |
|  |  |  |  | QQ |  | Unit Number |  |  |  |  |  |
|  |  |  |  |  |  | Number of devices |  |  |  |  |  |
| Must Use | C04002 | 127 | Reference Identification |  |  |  | M |  | AN 1/30 |  |  |
|  |  |  | Reference information as defined for a particular Transaction Set or as specified by the Reference Identification Qualifier |  |  |  |  |  |  |  |  |
|  |  |  | Number of unmetered devices of this type. |  |  |  |  |  |  |  |  |

Segment:	REF Reference Identification (Meter Reading Cycle)

Position:	130

Loop:	NM1        Optional

Level:	Detail

Usage:	Optional

Max Use:	>1

Purpose:	To specify identifying information

Syntax Notes:	1	At least one of REF02 or REF03 is required.

2	If either C04003 or C04004 is present, then the other is required.

3	If either C04005 or C04006 is present, then the other is required.

Semantic Notes:	1	REF04 contains data relating to the value cited in REF02.

Comments:

| Notes: |  | Meter cycles will be provided on the TDSP's website. |
| --- | --- | --- |
|  |  | Accept Response: Required when the meter read is done by cycle. Required for unmetered services. Reject Response: Not Used One of either the REF~TZ or the DTM~313 is required for metered services but not both. |
|  |  | REF~TZ~15 REF~TZ~01 |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | REF01 | 128 | Reference Identification Qualifier |  |  |  | M |  | ID 2/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code qualifying the Reference Identification |  |  |  |  |  |  |  |  |
|  |  |  |  | TZ |  | Total Cycle Number |  |  |  |  |  |
|  |  |  |  |  |  | Cycle number when the meter will be read. |  |  |  |  |  |
| Must Use | REF02 | 127 | Reference Identification |  |  |  | X |  | AN 1/30 |  |  |
|  |  |  | Reference information as defined for a particular Transaction Set or as specified by the Reference Identification Qualifier |  |  |  |  |  |  |  |  |
|  |  |  | Metered: Meter Reading Cycle Unmetered: Billing Cycle Must be numeric and only two digits. |  |  |  |  |  |  |  |  |

Segment:	DTM Date/Time Reference (Meter Cycle by Day of the Month)

Position:	140

Loop:	NM1        Optional

Level:	Detail

Usage:	Optional

Max Use:	>1

Purpose:	To specify pertinent dates and times

Syntax Notes:	1	At least one of DTM02 DTM03 or DTM05 is required.

2	If DTM04 is present, then DTM03 is required.

3	If either DTM05 or DTM06 is present, then the other is required.

Semantic Notes:

Comments:

| Notes: |  | Accept Response: Required on metered services if meter read based on a day of the month. Not used for unmetered services. Reject Response: Not Used One of either the REF~TZ or DTM~313 is required for metered services but not both. |
| --- | --- | --- |
|  |  | DTM~313~~~~DD~01 |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | DTM01 | 374 | Date/Time Qualifier |  |  |  | M |  | ID 3/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code specifying type of date or time, or both date and time |  |  |  |  |  |  |  |  |
|  |  |  |  | 313 |  | Cycle |  |  |  |  |  |
|  |  |  |  |  |  | Meter Cycle by Day of the Month |  |  |  |  |  |
| Must Use | DTM05 | 1250 | Date Time Period Format Qualifier |  |  |  | X |  | ID 2/3 |  |  |
|  |  |  | Code indicating the date format, time format, or date and time format |  |  |  |  |  |  |  |  |
|  |  |  |  | DD |  | Day of Month in Numeric Format |  |  |  |  |  |
| Must Use | DTM06 | 1251 | Date Time Period |  |  |  | X |  | AN 1/35 |  |  |
|  |  |  | Expression of a date, a time, or range of dates, times or dates and times |  |  |  |  |  |  |  |  |
|  |  |  | Day of month that the meter will be read, in DD format (e.g., 01 - 28). |  |  |  |  |  |  |  |  |

Segment:	SE Transaction Set Trailer

Position:	150

Loop:

Level:	Detail

Usage:	Mandatory

Max Use:	1

Purpose:	To indicate the end of the transaction set and provide the count of the transmitted segments (including the beginning (ST) and ending (SE) segments)

Syntax Notes:

Semantic Notes:

Comments:	1	SE is the last segment of each transaction set.

| Notes: |  | Required |
| --- | --- | --- |
|  |  | SE~21~000000001 |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | SE01 | 96 | Number of Included Segments | M |  | N0 1/10 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Total number of segments included in a transaction set including ST and SE segments |  |  |  |  |
| Must Use | SE02 | 329 | Transaction Set Control Number | M |  | AN 4/9 |  |
|  |  |  | Identifying control number that must be unique within the transaction set functional group assigned by the originator for a transaction set |  |  |  |  |

814_05 Example #1 of 3

Premise Information and Enrollment Accept Response – ERCOT to New CR

| ERCOT forwards Premise Information and Start Date to New CR |  |  |  |
| --- | --- | --- | --- |
| ST~814~000000001 |  |  | Transaction Type, Control Number |
| BGN~11~200104030800001~20010403~~~200104011956531~~5 |  |  | Response, Unique Transaction Number, Transaction Date, SET Transaction Number |
| N1~8R~PREMISE |  |  | Customer Name |
|  | N3~123 N MAIN ST~ANY ADDRESS OVERFLOW |  | Customer Service Address |
|  | N4~ANYTOWN~TX~78111 |  | Customer City, State, Zip |
| N1~8S~TDSP COMPANY~1~007909411 |  |  | TDSP Name and DUNS Number |
| N1~AY~ERCOT~1~183529049~~41 |  |  | ERCOT and DUNS Number, Sender |
| N1~SJ~CR NAME~9~007909422CRN1~~40 |  |  | New CR Name and DUNS Number, Receiver |
| LIN~1~SH~EL~SH~CE~SH~SW~SH~HI |  |  | Off –Cycle Switch and Historical Interval Usage Request an Echo LIN from Request 814_04 |
|  | ASI~WQ~021 |  | Accept Addition |
|  | REF~AQ~A |  | Distribution Loss Factor |
|  | REF~1P~HUU~HISTORICAL USAGE UNAVAILABLE |  | Status Reason Code and Text |
|  | REF~PTC~02 |  | Premise Type |
|  | REF~Q5~~10111111234567890ABCDEFGHIJKLMNOPQRS |  | ESI ID |
|  | REF~SPL~~WEST |  | Station ID |
|  | REF~SU~Y |  | Special Needs Indicator |
|  | DTM~150~20010415 |  | Service Period Start Date |
|  | NM1~MQ~3~~~~~~32~1234568MG |  | Start of Meter Loop, Meter Number |
|  |  | REF~4P~1200~KHMON~TU^51 | Meter Multiplier |
|  |  | REF~4P~1~KHMON~TU^41 | Meter Multiplier |
|  |  | REF~4P~1~KHMON~TU^42 | Meter Multiplier |
|  |  | REF~IX~6.0~KHMON~TU^51 | Number of Dials |
|  |  | REF~IX~5.1~KHMON~TU^41 | Number of Dials |
|  |  | REF~IX~5.1~KHMON~TU^42 | Number of Dials |
|  |  | REF~LO~ RESLOWR_WEST_NIDR_NWS_TOUØ1 | Load Profile |
|  |  | REF~MT~COMBO | Meter Type |
|  |  | REF~NH~RS1 | TDSP Rate Class |
|  |  | REF~PR~123 | TDSP Rate Subclass |
|  |  | REF~TZ~15 | Meter Cycle |
|  | NM1~MQ~3~~~~~~32~2796584MG |  | Start of Meter Loop, Meter Number |
|  |  | REF~4P~1~KHMON~TU^41 | Meter Multiplier |
|  |  | REF~4P~1~KHMON~TU^43 | Meter Multiplier |
|  |  | REF~4P~1~KHMON~TU^51 | Meter Multiplier |
|  |  | REF~IX~5.1~KHMON~TU^41 | Number of Dials |
|  |  | REF~IX~5.1~KHMON~TU^43 | Number of Dials |
|  |  | REF~IX~6.0~KHMON~TU^51 | Number of Dials |
|  |  | REF~LO~ RESLOWR_WEST_NIDR_NWS_TOUØ1 | Load Profile |
|  |  | REF~MT~COMBO | Meter Type |
|  |  | REF~NH~RS2 | TDSP Rate Class |
|  |  | REF~PR~123 | TDSP Rate Subclass |
|  |  | REF~TZ~15 | Meter Cycle |
| SE~42~000000001 |  |  | Number of Segments, Control Number |

814_05 Example #2 of 3

Premise Information and Enrollment Accept Response – ERCOT to New CR

| ERCOT forwards Premise Information and Start Date to New CR This example shows a response for Unmetered Services |  |  |  |
| --- | --- | --- | --- |
| ST~814~000000001 |  |  | Transaction Type, Control Number |
| BGN~11~200104030800001~20010403~~~200104011956531~~5 |  |  | Response, Unique Transaction Number, Transaction Date, SET Transaction Number |
| N1~8R~PREMISE |  |  | Customer Name |
|  | N3~123 N MAIN ST~ANY ADDRESS OVERFLOW |  | Customer Service Address |
|  | N4~ANYTOWN~TX~78111 |  | Customer City, State, Zip |
| N1~8S~TDSP COMPANY~1~007909411 |  |  | TDSP Name and DUNS Number |
| N1~AY~ERCOT~1~183529049~~41 |  |  | ERCOT and DUNS Number, Sender |
| N1~SJ~CR NAME~9~007909422CRN1~~40 |  |  | New CR Name and DUNS Number, Receiver |
| LIN~1~SH~EL~SH~CE~SH~HU |  |  | Cycle Switch and Historical Summarized Usage Request an Echo LIN from Request 814_04 |
|  | ASI~WQ~021 |  | Accept CR Change |
|  | REF~AQ~T |  | Distribution Loss Factor |
|  | REF~PTC~02 |  | Premise Type |
|  | REF~Q5~~10111111234567890ABCDEFGHIJKLMNOPQRS |  | ESI ID |
|  | REF~SPL~~WEST |  | Station ID |
|  | REF~SU~Y |  | Special Needs Indicator |
|  | DTM~150~20010415 |  | Service Period Start Date |
|  | NM1~MQ~3~~~~~~93~UNMETERED |  | Beginning of Meter Loop, Unmetered Services Indicator |
|  |  | REF~LO~ NMFLAT_WEST_NIDR_NWS_NOTOU | Load Profile |
|  |  | REF~PRT~MV~750~QQ^100 | Unmetered Service Type, Wattage/Lumens, Number of Devices of this type. |
|  |  | REF~NH~SL1 | TDSP Rate Class |
|  |  | REF~TZ~15 | Meter Cycle |
| SE~22~000000001 |  |  | Number of Segments, Control Number |

814_05 Example #3 of 3

Switch CR Notification Reject Response –ERCOT to New CR

| TDSP rejects switch notification, ERCOT forwards rejection to new CR who submitted 814_01 or 814_16 |  |  |
| --- | --- | --- |
| ST~814~000000001 |  | Transaction Type, Control Number |
| BGN~11~200104021201001~20010402~~~200104011956531~~5 |  | Response, Unique Transaction Number, Transaction Date, SET Transaction Number |
| N1~8S~TDSP COMPANY~1~007909411 |  | TDSP Name and DUNS Number |
| N1~AY~ERCOT~1~183529049~~41 |  | ERCOT and DUNS Number, Sender |
| N1~SJ~CR NAME~9~007909422CRN1~~40 |  | New CR Name and DUNS Number, Receiver |
| LIN~1~SH~EL~SH~CE~SH~SW~SH~HI |  | Off –Cycle Switch and Historical Interval Usage Request an Echo LIN from Request 814_03 |
|  | ASI~U~101 | Reject CR Change |
|  | REF~7G~A13~REASON TEXT | Reject Reason Code and Text |
|  | REF~Q5~~10111111234567890ABCDEFGHIJKLMNOPQRS | ESI ID |
| SE~10~000000001 |  | Number of Segments, Control Number |