# Texas SET Change Control Form

Texas SET Change Request

Change Control #2002-

This TX SET Change Request can be found on the ERCOT website at www.ercot.com  .

| Requester’s Name: Ed Skiba | Company Name: Entergy | Phone #: 770-513-7935 |
| --- | --- | --- |
| Date of Request: 05/31/2002 | Affected EDI Transaction Set #(s): 867_03 | E-Mail Address: edskiba@us.ibm.com |
| Emergency Priority* (Y/N): N | Requested Implementation Date: Version 1.5 | Production Implementation Date: |
| Testing Required for this CC (Y/N): | Testing Flight Number: (ready to test for this flight) | Status:Approved |

Brief Explanation (This will be copied into the description in the Change Control Summary Spreadsheet): Correct errors in the MEA05 of the Non-Interval Detail (PL) Loop.

Reason for Request (Explain why this change is needed. For business or technical purposes?):

The descriptions and codes in the MEA05 Gray box do not match the MEA04

Detail Explanation  (Exactly what change is required? To which TX SET Standards? Why?):   See Redline document.

For Change Control Manager Use Only:

| Date of TX SET Discussion: | Expected Implementation Date: |  |
| --- | --- | --- |

TX SET Discussion and Resolution:

| *Emergency Priority | Used for Change Controls that require immediate implementation. |
| --- | --- |

Please submit this form via e-mail to txsetchangecontrol@ercot.com .

Your request will be evaluated and prioritized at an upcoming TX SET meeting or conference call.

Non-Interval Detail

Segment:	MEA Measurements (Meter Reads)

Position:	160

Loop:	QTY        Optional

Level:	Detail

Usage:	Optional

Max Use:	40

Purpose:	To specify physical measurements or counts, including dimensions, tolerances, variances, and weights  (See Figures Appendix for example of use of C001)

Syntax Notes:	1	At least one of MEA03 MEA05 MEA06 or MEA08 is required.

2	If MEA05 is present, then MEA04 is required.

3	If MEA06 is present, then MEA04 is required.

4	If MEA07 is present, then at least one of MEA03 MEA05 or MEA06 is required.

5	Only one of MEA08 or MEA03 may be present.

Semantic Notes:	1	MEA04 defines the unit of measure for MEA03, MEA05, and MEA06.

Comments:	1	When citing dimensional tolerances, any measurement requiring a sign (+ or -), or any measurement where a positive (+) value cannot be assumed, use MEA05 as the negative (-) value and MEA06 as the positive (+) value.

| Notes: |  | PTD = PL (Non-Interval Detail) |
| --- | --- | --- |
|  |  | Conditional - Required unless reporting Additive/Subtractive Metering Usage or Adjusted (missing or abundance) Consumption. (if the PTD06 equals "AI" , "AO","CD","DC","DM", or "MD") MEA with an MEA07 = "51" (total) is required unless reporting Additive/Subtractive Metering Usage (the PTD06 equals "AI" or "AO","CD", "DC","DM", or "MD"). MEA03 must equal QTY02. |
|  |  | MEA~AA~PRQ~1000~KH~1000~1100~51 |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | MEA01 | 737 | Measurement Reference ID Code |  |  |  |  |  | O |  | ID 2/2 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Code identifying the broad category to which a measurement applies |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | AA |  | Meter reading-beginning actual/ending actual |  |  |  |  |  |  |  |
|  |  |  |  |  | AE |  | Meter reading-beginning actual/ending estimated |  |  |  |  |  |  |  |
|  |  |  |  |  | EA |  | Meter reading-beginning estimated/ending actual |  |  |  |  |  |  |  |
|  |  |  |  |  | EE |  | Meter reading-beginning estimated/ending estimated |  |  |  |  |  |  |  |
| Must Use | MEA02 | 738 | Measurement Qualifier |  |  |  |  |  | O |  | ID 1/3 |  |  |  |
|  |  |  |  | Code identifying a specific product or process characteristic to which a measurement applies |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | PRQ |  | Product Reportable Quantity |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | Consumption |  |  |  |  |  |  |
| Must Use | MEA03 | 739 | Measurement Value |  |  |  |  |  | X |  | R 1/20 |  |  |  |
|  |  |  |  | The value of the measurement |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Total consumption after meter multiplier and transformer loss factor are applied. |  |  |  |  |  |  |  |  |  |  |
| Must Use | MEA04 | C001 | Composite Unit of Measure |  |  |  |  |  | X |  |  |  |  |  |
|  |  |  |  | To identify a composite unit of measure (See Figures Appendix for examples of use) |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Please note that this is a composite data element, populate C00101. |  |  |  |  |  |  |  |  |  |  |
| Must Use | C00101 | 355 | Unit or Basis for Measurement Code |  |  |  |  |  | M |  | ID 2/2 |  |  |  |
|  |  |  |  | Code specifying the units in which a value is being expressed, or manner in which a measurement has been taken |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | K1 |  | Kilowatt Demand |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | kW |  |  |  |  |  |  |
|  |  |  |  |  | K2 |  | Kilovolt Amperes Reactive Demand |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | kVAR |  |  |  |  |  |  |
|  |  |  |  |  | K3 |  | Kilovolt Amperes Reactive Hour |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | kVARH |  |  |  |  |  |  |
|  |  |  |  |  | K4 |  | Kilovolt Amperes |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | kVA |  |  |  |  |  |  |
|  |  |  |  |  | KH |  | Kilowatt Hour |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | kWh |  |  |  |  |  |  |
| Dep | MEA05 | 740 | Range Minimum |  |  |  |  |  | X |  | R 1/20 |  |  |  |
|  |  |  |  | The value specifying the minimum of the measurement range |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | The Beginning reading is required when C00101, element 355 Unit of Measure is equal to: "K3" - kVARH (Kilovolt Amperes Reactive Hour) "K4" - kVA (Kilovolt Amperes) "KH" - kWh (Kilowatt Hour). The Beginning read is not used for: "K1" - kW (Kilowatt Demand) "K2" - kVAR (Kilovolt Amperes Reactive Demand). |  |  |  |  |  |  |  |  |  |  |
| Must Use | MEA06 | 741 | Range Maximum |  |  |  |  |  | X |  | R 1/20 |  |  |  |
|  |  |  |  | The value specifying the maximum of the measurement range |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Ending reading when C00101, element 355 Unit of Measure is equal to: "K3" - kVARH (Kilovolt Amperes Reactive Hour) "K4" - kVA (Kilovolt Amperes) "KH" - kWh (Kilowatt Hour). Single reading when: "K1" - kW (Kilowatt Demand) "K2" - kVAR (Kilovolt Amperes Reactive Demand). |  |  |  |  |  |  |  |  |  |  |
| Must Use | MEA07 | 935 | Measurement Significance Code |  |  |  |  |  | O |  | ID 2/2 |  |  |  |
|  |  |  |  | Code used to benchmark, qualify or further define a measurement value |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 41 |  | Off Peak |  |  |  |  |  |  |  |
|  |  |  |  |  | 42 |  | On Peak |  |  |  |  |  |  |  |
|  |  |  |  |  | 43 |  | Intermediate |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | Mid-Peak |  |  |  |  |  |  |
|  |  |  |  |  | 51 |  | Total |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | Totalizer/Total/Max (Demand) |  |  |  |  |  |  |
|  |  |  |  |  | 71 |  | Low |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | Summer Super On-Peak |  |  |  |  |  |  |