| PRR Number | 356PRR | PRR Title | SCE Performance Requirement |
| --- | --- | --- | --- |
| ###### Status |  | Posted |  |
| Protocol Section Requiring Revision |  | 6.10.5 |  |
| Requested Resolution |  | ## Normal |  |
| Revision Description |  | To provide additional incentive to QSEs to closely follow SCE, and to reduce the burden of Ancillary Service capacity costs on QSEs that do closely follow SCE. |  |
| ###### Reason for Revision |  | To cause QSEs to make better efforts to follow their Resource schedule so that ERCOT can accurately procure Balancing Energy, and to change the allocation of Ancillary Service capacity costs from pure load ratio share to a method that assigns costs to causation. Both reasons are currently of increased importance to assure the operationally smooth implementation of Relaxed Balanced Schedules. |  |
| ###### Known Benefits for Revision |  | - Provides incentive to improve SCE performance without the need to precisely define and enforce an acceptable SCE - Compensates QSEs that make efforts to improve unit response and otherwise minimize SCE - Allows each QSE to perform its own analysis of the benefits and costs of approaching zero SCE, and/or committing to deliver energy and services at ramp rates it cannot perform - Provides the benefits outlined above with enough uncertainty to minimize gaming. - Reduces the demand on ERCOT to police SCE. |  |
| ## Timeline |  |  |  |
| Date Received |  | 08/22/2002 |  |
| Date Posted |  | 08/22/2002 |  |
| Comments Due |  | 09/12/2002 |  |
| PRS Review Date |  | 09/24/2002 |  |
| PRS Recommendation |  | 09/26/2002 |  |
| TAC Consideration |  | 11/07/2002 |  |
| Board Consideration |  | 11/19/2002 |  |

Proposed Protocol Language Revision

### 6.10.5	QSE Real Power Performance Criteria

#### 6.10.5.1	Ancillary Service Demand Factor

For each Settlement Interval ERCOT shall calculate an Ancillary Service Demand Factor for each QSE in accordance with the following:

ASDFqi = (ABSCEqi / ABSCEti )

Where:	  ABSCEqi = the 15 minute integration of the absolute value of the SCE of the QSE    during that interval, multiplied by ( 1 – ( 0.5 * ( UNCRESqi / TOTRESqi ) ) )

ABSCEti = Sum of ABSCEqi for all QSEs during the interval

UNCRESqi = Net capability of online generation of the QSE in that interval that is deemed by ERCOT to be uncontrollable

TOTRESqi = Net Capability of all online generation of the QSE in that interval.

6.10.5.2	Ancillary Service Cost Reallocation

For each Settlement Interval ERCOT shall calculate an Ancillary Service Cost Reallocation for each QSE in accordance with the following:

ASCRqi = ( 1 – (ELFPFi / ( ELFPFi + ABSCEti ) ) ) * IECASi * ( ASDFqi – LRSqi )

Where:	  LRSqi = Load Ratio Share for Ancillary Service allocation for the QSE, adjusted for bilateral transactions and bid awards, as used to deploy Ancillary Services

ELFPFi = the absolute value of the difference between i) ERCOT’s load forecast made to determine Balancing Energy deployments for the interval, and ii) ERCOT integrated actual load for the interval.

IECASi = The Interval Equivalent Cost of Ancillary Services, determined by ERCOT to be 25% of the cost of Regulation Service plus Non-Spin Reserve Service plus Responsive Reserve Service for the hour containing the interval, based on the capacity clearing prices for each Service for the hour, and based on the total amount of these Services allocated to all QSEs: provided; that if ABSCEti < 100 MW in any interval, then IECASi is set to zero for that interval.

In the event ASCRqi for any QSE for any Settlement Interval is positive, the QSE will be charged that amount.  In the eventASCRqi for any QSE for any Settlement Interval is negative, the QSE will be paid that amount.

PIP112: Add the following two paragraphs when BUL is implemented:

On deployment of BULs, the QSE shall control its Loads to the difference between its Scheduled power and the BUL instructed amount.  ERCOT, at a time established at their discretion, shall determine the Base Load of the sum of all Loads qualified by the QSE as BUL Resources by obtaining actual meter data from IDR meters or directly from the TDSP who has agreed to supply the data.  While IDRs must normally be present on all Loads providing BUL Resources, IDRs may be installed and read on a sample of the Loads qualified by a QSE as a BUL Resource, provided the Loads are sampled in accordance with Section 18.7.2.  The Base Load is determined by the finding the average actual metered energy during the same interval as the instruction to deploy BUL from the previous 10 days (in the case of weekday deployment) or six days (in the case of weekend or holiday deployment).  Days in which BUL instructions occurred during the same interval are ignored.  If the instruction to deploy occurs on a weekday, only weekdays are included in the average.  If the instruction to deploy occurs on a weekend, only weekend days and ERCOT-defined holidays are included in the average. For small customer load management programs, Base Load for each interval is determined from the aggregated appropriate baseline load profile for participating customers assuming the load management program is not in operation.

ERCOT shall also integrate for each Settlement Interval the signal provided by the QSE representing the amount of BUL power deployed. The expected total reduction of Load shall be determined by subtracting the integrated signal for each interval from the Base Load as determined above.  For small customer load management programs during each interval, the expected total reduction of Load shall be determined by subtracting the aggregated participating customer load during operation of the load management program, which is calculated from the appropriate load profile, from the Base Load for these customers as calculated above.  Control performance of the QSE providing BUL shall be deemed satisfactory when during the first hour in which BUL is deployed in an Operating Day, the actual metered Load for any Settlement Interval during the hour is equal to or less than the amount of energy expected.  Failure of the QSE to provide satisfactory control performance during the first hour in which BUL is deployed in an Operating Day will cause ERCOT to withhold payments for any capacity provided as BUL for the Operating Day.  Failure to provide at least 90% of the expected total Load during the first hour of deployment will also disqualify those specific Loads and will reduce the amount the QSE is qualified to bid accordingly.  QSEs may apply for re-qualification of a specific load according to Section 6.10.2. Repeated failures of the same Load to perform in three Operating Days in a Calendar Year will result in a 90 day waiting period after which the QSE may apply for re-qualification of the specific Load according to Section 6.10.2.

#### 6.10.5.3	Effect on SCE of ERCOT Instructions

Operational instructions by ERCOT should be included in SCE to the greatest extent possible.  In the event of operational instructions by ERCOT that directly cause any QSE to increase or decrease its SCE, appropriate adjustments to ABSCEqi shall be made in settlement when performing the calculations in 6.10.5.1.  The extent of the need to make such adjustments shall be reported periodically to the appropriate subcommittee.

6.10.5.4	ERCOT Load Forecast Error

ERCOT shall post on its website the interval values of ELFPFi.  ERCOT shall make periodic reports to the appropriate subcommittee showing the trend of ELFPFi and its impact on the amount of Ancillary Services that ERCOT determines is necessary

6.10.5.5	SCE Obligation

Ancillary Service Cost Reallocation under 6.10.5.2 is not intended to substitute for a requirement that all QSEs make a continuous good faith effort to operate at or near a zero SCE.  ERCOT shall post on its website daily averages of the absolute value of every QSE SCE as a percentage of the daily average respective QSE load.  When such data shows average absolute SCE to be more than five percent of average load for any QSE on any day, ERCOT will report that QSE to the appropriate regulatory authority.

| ## Sponsor |  |
| --- | --- |
| Name | Paul S. Breitzman |
| E-mail Address | paulb@ci.garland.tx.us |
| Company | City of Garland |
| Company Address | 525 East Avenue B, Garland, TX 75040 |
| Phone Number | 972-205-3011 |
| Fax Number | 972-205-3106 |
| ## ERCOT Impact Analysis |  |
| Date |  |
| Financial Impact |  |
| Analysis |  |
| ## TAC Action |  |
| TAC Vote Date |  |
| TAC Proposed Implementation Date |  |
| TAC Vote Results |  |
| ## ERCOT Board Action |  |
| ERCOT Board Vote Date |  |
| ERCOT Board Implementation Date |  |
| ERCOT Board Results |  |
| ## Appeal |  |
| Appeal Date |  |
| Entity Initiating Appeal |  |
| Nature of Appeal |  |