|  | Integrated Model | Auction Model |
| --- | --- | --- |
| PROs | - Allows price certainty for load and resources cleared for congestion in the day-ahead - Assures resources of receiving no less than their bid price for energy - Results in greater day-ahead market liquidity since TCRs could only clear in Day-ahead market (strong financial incentive) - Provides somewhat more operational data to ERCOT to use in system planning than today’s market design - It is more transparent for decision-making - Allows virtual bidding. - Creates greater efficiency for the market by developing an ERCOT wide integrated resource plan - Energy and A/S can be co-optimized on a locational basis - Allows the proper incentives for mid-load plants to commit and allows for their ramping through train reconfigurations | - Simple and least cost to implement overall. - Requires no mitigation or market monitoring due to lack of interaction between bids, congestion, CRRs and SCED in day-ahead and real-time. - Is a centralized auction clearing exchange similar to Nord Pool DAM and UKPX, by far the most liquid electricity markets in the world. - Allows for flexibility in congestion cost hedging until real-time since CRRs don’t expire day-ahead. Any changes in individual operating plans or schedules due to outages, changes in market conditions, fuel constraints or weather conditions from the day-ahead plan can still be hedged until real-time. - Allows for Dynamic Scheduling which is consistent with current protocols – NOIEs can avoid the problem of buying and selling imbalance energy. - Keeps the DAEM voluntary as required by the PUCT Rule – the activities of others in the DAEM does not impact non-participants. Non-participants are not economically “forced” to participate in the DAEM to counter the actions of others. - Allows price certainty for load cleared day-ahead at load zone or generation cleared at generation node - Model is simple and straightforward to implement for both ERCOT and market participants. - Settles CRRs on actual congestion and sets LMPs on the true real-time SCED. The financial model better matches physics of the system - Allows virtual bidding. |
| CONs | - Can be easily gamed since the day-ahead SCED resolves congestion based on bilateral schedules and bids and offers that may be either physical or virtual. - Requires greater sophistication on the part of market participants to effectively counter the actions of others in day-ahead market – participants are forced to virtual bid to counter the adverse impacts of the virtual bidding of others. - Requires extensive market mitigation and daily market monitoring at ERCOT due to manipulative activities surrounding CRRs and virtual bidding resulting in market inefficiency. - Is more complex to implement and much more expensive to monitor. - Resource costs cannot be accurately represented in SCUC leading to market inefficiency– e.g. tiered Natural Gas contracts cannot be accurately represented by multi-part bids used in any centralized unit commitment and ERCOT is heavily dependent on Natural Gas fired Resources (over 60% compared to 6% for PJM). - Dynamic Scheduling by NOIEs are not readily accommodated under IDAM. To this point no acceptable solution has been suggested. - May require a bid sufficiency mechanism to assure that adequate resources are offered into the market. This would not meet the PUCT Rule requirement for a voluntary market. | - May not have much liquidity due to supply-side risk - A formalized exchange market creating limited value above today’s broker market at a potentially higher cost - Will likely take longer than the integrated model to develop a liquid market. - Less likely to have full market transparency, therefore, it will not enhance forward contracting - No RTO/ISO uses as day-ahead market - Cannot lock in day-ahead the cost of transmission congestion without CRRs - Creates a ‘lumpy” unit commitment that may lead to inefficiencies for dispatched units leading to improper bidding incentives - Main issue little to no value added over today’s model |