| PRR Number | 404PRR | PRR Title | ## Energy Procured from ERCOT |
| --- | --- | --- | --- |
| Recommended Action |  | Approve |  |
| Protocol Section Requiring Revision |  | 4.3.2, Schedule Components |  |
| Proposed Effective Date |  | Contingent on results of Impact Analysis. |  |
| Priority & Rank Assigned |  | Pending. |  |
| Summary of Impact Analysis |  | Impact Analysis pending. |  |
| Revision Description |  | This PRR provides for the procurement of balancing energy from ERCOT. |  |
| ###### Benefit |  | Expands the functionality of Relaxed Balanced Schedules by allowing Market Participants to schedule generation via ERCOT. |  |
| PRS Recommendation |  | PRS recommends approval of PRR404 as revised by ERCOT comments and PRS and requires a credit analysis from ERCOT. |  |

Proposed Protocol Language Revision

### 4.3.2	Schedule Components

Included in each Balanced Schedule is:

- Energy to be produced by Resources that the QSE represents, or Balancing Energy scheduled via ERCOT, by Congestion Zone;

- Energy to be consumed by Loads that the QSE represents (including T&D Losses) by Congestion Zone;

- ERCOT allocated Ancillary Services Obligations for the QSE;

- Any energy and Ancillary Services scheduled to or from other QSEs(e.g. Inter-QSE Trades); and

- Any Balancing Energy scheduled via ERCOT processes.

- Any Self-Arranged Ancillary Services.

When relaxed Balanced Schedules are allowed by ERCOT, pursuant to Section 4.3.5, Requirement for Balanced Schedules, the requirement for subpart (2) above shall be relaxed and QSEs shall be allowed to schedule energy for Load that is equivalent to the amount of scheduled energy for Resources as specified in subpart (1) and (5) above.

Use of Balancing Energy scheduled via ERCOT to meet Obligations shall be limited to an amount supported by the QSE’s credit collateral with ERCOT.

| PRR Evaluation |  |
| --- | --- |
| Non-ERCOT Market Comparison | No Comparison conducted. |
| Comments Author | Comments Summary |
| ERCOT 050803 | Clarifications. |
| ERCOT 052103 | Recommends remand. |
| Fulcrum 100203 | Revised language. |

| ## Sponsor |  |
| --- | --- |
| Name | Cesar Seymour |
| E-mail Address | cseymour@tractebelusa.com |
| Company | Tractebel Energy Marketing |
| Company Address | Houston, TX 78744 |
| Phone Number | 713/350-1734 |
| Fax Number |  |

| Sponsor's Revision Description | This PRR provides for the procurement of balancing energy from ERCOT. |
| --- | --- |
| ###### Sponsor's Reason for Revision | Expands the functionality of Relaxed Balanced Schedules by allowing Market Participants to schedule up to 500MW of generation from ERCOT. |