On January 9, ERCOT is hosting a meeting to review Dynamics generation data with generation owners to support development of Dynamics working group data.  It is recommended that a technical representative from every PGC attend this meeting.  Please contact Juan Santos (jsantos@ercot.com, 512-248-3139) for details about this meeting.

On January ?, ERCOT is hosting a meeting to review the Oncor proposed Northeast Short Term Protection Scheme.

TAC Approved RMR Exit Strategy for B.M. Davis #1 goes to the Board on Dec. 16.  Represents a two phased approach totaling to about 33 millions dollars in transmission additions.  This cost will be more than offset by the

Hosting North Regional Planning Meeting (Dec. 11, tomorrow in Taylor):  Agenda focuses on DFW/Northeast Congestion including Short-term DFW, Long-term DFW and Long-term Northeast planning issues.

Completed Phase II of the ERCOT Voltage Security Study:  Determined reactive margins at several transmission nodes based on contingencies (B, C and D).  Summer ‘03, Fall ’03 and Winter ‘03/’04 cases were studied.  Overall, the ERCOT system has sufficient margin for voltage stability for a large majority of outages.  Low margin areas (or negative margin) for B, C and D contingencies include Houston and Brian/College.  For type D contingencies, low margins were also found in the Valley (Lon Hill contingency), Kendall (Zorn), DFW (Forney), and West Texas (Morgan Creek).  Hosted briefing in Taylor on 12/3

Completed ERCOT Regional Transmission Project Planning Group Peer Review and Acceptance letter for two projects:

CenterPoint’s Oasis project:  Construction of the Oasis substation at the point where the 345 kV circuits coming north out of Dow separate, one heading east to PHR and one heading west to WAP.  Letter sent 11/19.

LCRA’s Medina Project:  Construct a new 138-kV 795-ACSR transmission line (approximately 14 miles in length) between the LCRA Medina Lake and CPS Anderson substations.  Convert and upgrade the existing 69-kV 1/0-ACSR transmission line T-279 (approximately 7.82 miles in length) between the LCRA Pipe Creek and LCRA Medina Lake substations to 138-kV with 795-ACSR.  Letter sent 11/19.

Several transmission projects are going through the regional review process including Jacksboro-West Denton 345 kV line and Watermill-Norwood 345 kV Line.  More are being received everyday.

Hosted McCamey Area Technical Conference to help optimize operations relating to output from Wind Farms.

Transmission Project Information Tacking (TPIT) for November – inclusion of 50% of 138 kV projects along with 345 kV projects, list has more than tripled than what was first submitted.

Quarterly updates of ROS Steady State WG 2204 Data Set A Base Cases – over eighty updates to the eight 2004 seasonal base cases were received.  Required a time consuming generation redispatch of the cases.

Completion of ROS Steady State WG 2005-2009 Data Set B Base Cases.  This was a major task and included development of two different generation dispatches for each case.  One dispatch was based upon CSC constraints and the other dispatch was based upon pure economics.

Completed American Superconductor's ERCOT Voltage Scan Study: results were presented to ERCOT stake holders on Nov. 14th.  May be applied for development of a Transient Voltage Dip Criteria.

Completed System Planning review of Texas Genco RMR Request.  Results were provided to System Operations and legal staff.

Continued Review of Northeast ERCOT Oncor Proposed Scheme by ERCOT Staff.

Posted updated contingency files Powerworld, PSSE/ACCC, and VSAT formats along with MUST format.

Developed new Wind Generation Data forms for processing generation interconnection requests and maintaining generation planning data.

System Protection Working group cases completed.

Complete security studies for three generation interconnects, also received several new requests for review.