Texas

Standard

Electronic

Transaction

650_01:

##### Service Order Request

Electronic Data Interchange

ANSI ASC X12 Ver/Rel 004010

Transaction Set 650

Texas 650_01:

Service Order Request

This transaction set, from the Competitive Retailer (CR) to the Transmission Distribution Service Provider (TDSP), is used to initiate the original Service Order, Cancel or Change (Update) Request.

Document Flow:

- CR to TDSP

|  |  | # Summary of Changes |
| --- | --- | --- |
| January 17, 2001 Version 1.0 |  | Initial Release |
| March 27, 2001 Version 1.3 |  | The following changes were made: |
|  |  | - Removed Scenario Names from Transaction Description page |
|  |  | - Corrected the How to Use this Implementation Guide page |
|  |  | - Changed the gray box in the REF (Purpose Code) to BGN07 = KH and BGN07 = IN from BGN07 = RH and BGN07 = TE |
|  |  | - Changed Name in PER to be similar to 814_PC, noting that the Outage Work Group has suggested Last Name, First Name as the proposed format. |
|  |  | - Corrected gray box for N403 Service Address Zip Code to be numeric only |
|  |  | - Changed Customer City and State to Must Use |
|  |  | - Corrected REF~TD Reason for change code “REPPH” to “REFPH” |
|  |  | - Changed Life Support Indicator to be consistent with other Texas SET transactions. |
|  |  | - Removed the language "When direction and comments are both used each must be limited to 80 characters. If only one is used that one can be no more then 160 characters." from the MTX segments on the 650_01. This language was in conflict with the gray box below that says 80 characters only (and the Service Order report, which also said 80 characters only). |
|  |  | - Corrected Example #3: changed CR to POLR |
|  |  | - Corrected the SE segment count in examples |
| July 27, 2001 Version 1.4 |  | The following changes were made: Change Control 2001-152: - Added language to the HL Parent Loop: “For an ESI ID with multiple meters the CR will generate a service order request for each meter affected.” Multiple meters at an ESI ID requires multiple service orders.” - Removed the second example in the HL Parent Loop (Service Order Level Information):HL~1~EV~1 - Modified the following text in the HL01 gray box to “This unique number will be used to identify the HL loop. One parent loop (Service Order Level Information) per transaction”. - Modified HL04. Changed gray box “Indicates one hierarchical loop will be provided”. Changed gray box language for code “0” to “Service order applies to one meter affected at the ESI ID.” Removed code “1” and the gray box. - Removed the “HL Hierarchical Level (Meter Level Information) segment. - Modified REF (Meter Number). - Changed the gray box from: “HL Child Loop (Meter Level Information)” to “HL Parent Loop (Service Order Level Information)” - Removed from the gray box “This segment may not be repeated, there must be one HL child loop for each meter. - Changed the gray box sentence: “ Meter Number is required on the following service orders if the work is to be performed for a specific meter on a multi-meter installation:” to “Required: “ Meter Number is required for the following service order types.” |
|  |  | - Example #1 of 5: Modified “HL~1~EV~1” to “HL~1~~EV~0”; Removed the “HL~2~1~-EV” line; Modified “SE~19~000000001” to “SE~18~000000001”; Moved Meter Number under Purpose Code - Example #2 of 5: Changed title from “Meter Test Multiple Meters Request” to “Meter Test”; Remove ‘Indicates Multiple Meters in gray box title; Removed the “HL~2~1~EV” Line; Removed the “HL~3~1~EV” Line; Moved Meter Number under Purpose Code; Removed the REF~MG~495951R” Line - Example #4 of 5: Modified “HL~1~EV~1” to “HL~1~EV~0”; Removed the “HL~2~1~EV” Line; Moved Meter Number under Purpose Code; Modified “SE~20~000000001” to “SE~19~000000001” Change Control 2001-101: - Added Purpose Codes (SL010 Street Light – Remove a Specific Lamp; GL009 Guard Light – Remove a Specific Lamp). Change Control 2001-132: - Added gray box language to the PER segment stating that the Customer Contact Name should be formatted as Last Name, First Name and stated that “Only one comma will be used”. |
| June 17th, 2002 Version 1.5 |  | The following changes were made: |
|  |  | - Change Control 2001-198 - Provided clarification in gray box for YNQ PDL (Premium Disconnect Location) |
|  |  | - Change Control 2002-254 - In the REF~8X (Purpose Codes) added DC003. - Change Control 2002-259 – Changed Life Support Indicator to Special Needs and eliminated the “I” value. |
|  |  | - Change Control 2002-305 – Clean-up of gray box for RD code in the BGN07. |
|  |  | - Applied Change Control 2002-311 |
| 10/15/02 |  | Change Control 2002-419 – Changed references to POLR in example 3 to AREP |
|  |  | Change Control 2002-435 – Added RCOO3 code to REF~8X |
| 6/12/03 |  | Change Control 2003-528 Add Business Process Overviews to the appropriate implementation guides |

| May 29th, 2003 Version 1.6 |  | The following changes were made: |
| --- | --- | --- |
|  |  | Change Control 2003-474 Provide additional detail on when the REF~TD (Reason for Change) should be populated on the 650_01 transaction |
| 5/30/03 |  | Change Control 2003-511 This change control is being requested due to the fact some TDSP's have been receiving TX SET transactions with telephone (TE) or communication numbers with punctuation and symbols in the PER04 and PER06 data elements of the PER Segment, which is incorrect. |

Business Process Overview					June 10, 2003

| Overview |  | The 650_01 (CR to TDSP) business process overview addresses the following areas: - Service Order Request Overview - Clarifying Request - Multi-Metered/Un-Metered ESI Id’s |
| --- | --- | --- |
| Service Order Request Overview |  | - Transaction used to send Discretionary Service Request. CR’s send this transaction via point to point Protocol to the TDSP for service type work such as those listed under the Purpose Code REF~8X. |
| Clarifying Request |  | - For every 650_01 Request there will be a 650_02 Response. - For Reconnects BGN06 must contain the BGN02 of the original disconnect request except when the party requesting the reconnect is not the party that initiated the disconnect, in which case the BGN06 is left blank. - For Cancels/Updates BGN06 must contain the BGN02 of the original service order request. |
| Multi-Metered/Un –Metered ESI Id’s |  | When requesting action on a specific meter on a multi-metered/un-metered ESI ID you must specify in REF~8X the appropriate purpose for the transaction and provide specific information in the REF~MG for meter service request or REF~ADE for un-metered service request. |

How to Use this Implementation Guide

This section is used to show the X12 Rules for this segment.  You must look further into the grayboxes below for Texas Rules.

Segment:	REF Reference Identification (ESI ID)

Position:	030

Loop:	LIN        Optional

Level:	Detail

Usage:	Optional

Max Use:	>1

Purpose:	To specify identifying information

Syntax Notes:	1	At least one of REF02 or REF03 is required.

2	If either C04003 or C04004 is present, then the other is required.

3	If either C04005 or C04006 is present, then the other is required.

Semantic Notes:	1	REF04 contains data relating to the value cited in REF02.

Comments:

This section is used to show the Texas Rules for implementation of this segment.

| Notes: |  | Required |
| --- | --- | --- |
|  |  | REF~Q5~~10111111234567890ABCDEFGHIJKLMNOPQRS |

Data Element Summary

One or more examples.

Ref.	Data

Des.	Element	Name	Attributes

| # Must Use | REF01 | 128 | Reference Identification Qualifier |  |  |  | M |  | ID 2/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code qualifying the Reference Identification |  |  |  |  |  |  |  |  |
|  |  |  |  | Q5 |  | Property Control Number |  |  |  |  |  |
|  |  |  |  |  |  | Electric Service Identifier (ESI ID) |  |  |  |  |  |
| # Must Use | REF03 | 352 | Description |  |  |  | X |  | AN 1/80 |  |  |
|  |  |  | A free-form description to clarify the related data elements and their content |  |  |  |  |  |  |  |  |
|  |  |  | ESI ID |  |  |  |  |  |  |  |  |

These are X12 code descriptions, which often do not relate to the Texas descriptions.

X12 cannot keep up with Texas needs, thus, Texas often changes the meaning of existing codes.  See the corresponding graybox for the Texas definitions.

This column shows the X12 attributes for each data element.

M = Mandatory

O = Optional

X = Conditional

AN = Alphanumeric

N# = Implied Decimal at position #

ID = Identification

R = Real

1/30 = Minimum 1, Maximum 30

This column shows the Texas use of each data element.

650 Maintenance Service Order

ANSI ASC X12 Structure

Functional Group ID=MO

Introduction:

This Draft Standard for Trial Use contains the format and establishes then data contents of the Maintenance Service Order Transaction Set (650) for use within the context of an Electronic Data Interchange (EDI) environment. This transaction set provides a uniform, singular medium for the exchange of maintenance related information among organizations involved in the reporting, requesting, scheduling, planning, estimating, coordinating and performing of maintenance actions. It provides the structure to convey maintenance-related information, including maintenance action directives, maintenance actions, cost estimates, maintenance action assignments, maintenance action status, and completion reports. This transaction set can be used in a bi-directional environment alone or in conjunction with the Project Schedule Reporting Transaction Set (806) to link schedule and maintenance action information as well as with the Specifications/Technical Information Transaction Set (841) to link maintenance-related, media independent, technical data.

Heading:

Pos.	Seg.		Req.		Loop	Notes and

No.	ID	Name	Des.	Max.Use	Repeat	Comments

| M | 010 | ST | Transaction Set Header | M | 1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| M | 020 | BGN | Beginning Segment | M | 1 |  |  |  |  |  |  |  |  |
|  |  |  | LOOP ID - N1 |  |  | >1 |  |  |  |  |  |  |  |
|  | 050 | N1 | Name | O | 1 |  | n1 |  |  |  |  |  |  |
|  | 060 | N2 | Additional Name Information | O | 2 |  |  |  |  |  |  |  |  |
|  | 070 | N3 | Address Information | O | 2 |  |  |  |  |  |  |  |  |
|  | 080 | N4 | Geographic Location | O | 1 |  |  |  |  |  |  |  |  |
|  | 090 | PER | Administrative Communications Contact | O | >1 |  |  |  |  |  |  |  |  |

Detail:

Pos.	Seg.		Req.		Loop	Notes and

No.	ID	Name	Des.	Max.Use	Repeat	Comments

|  |  |  | LOOP ID - HL |  |  | >1 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| M | 010 | HL | Hierarchical Level | M | 1 |  | n2 |  |  |  |  |  |  |
|  | 030 | REF | Reference Identification | O | >1 |  |  |  |  |  |  |  |  |
|  | 050 | DTM | Date/Time Reference | O | >1 |  |  |  |  |  |  |  |  |
|  | 070 | YNQ | Yes/No Question | O | >1 |  | n3 |  |  |  |  |  |  |
|  |  |  | LOOP ID - MTX |  |  | >1 |  |  |  |  |  |  |  |
|  | 250 | MTX | Text | O | 1 |  |  |  |  |  |  |  |  |
| M | 290 | SE | Transaction Set Trailer | M | 1 |  |  |  |  |  |  |  |  |

Transaction Set Notes

1.	The N1 segment identifies the organization originating and receiving the transaction set.

2.	The HL levels are group work candidate and work candidate.  Valid HL parent-child relationships are 1) group work candidate-group work candidate and 2) group work candidate-work candidate.

3.	The YNQ segment identifies conditions related to a maintenance or repair requirement.

Segment:	ST Transaction Set Header

Position:	010

Loop:

Level:	Heading

Usage:	Mandatory

Max Use:	1

Purpose:	To indicate the start of a transaction set and to assign a control number

Syntax Notes:

Semantic Notes:	1	The transaction set identifier (ST01) is used by the translation routines of the interchange partners to select the appropriate transaction set definition (e.g., 810 selects the Invoice Transaction Set).

Comments:

| Notes: |  | Required |
| --- | --- | --- |
|  |  | ST~650~000000001 |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | ST01 | 143 | Transaction Set Identifier Code |  |  |  | M |  | ID 3/3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code uniquely identifying a Transaction Set |  |  |  |  |  |  |  |
|  |  |  |  | 650 |  | Maintenance Service Order |  |  |  |  |
| Must Use | ST02 | 329 | Transaction Set Control Number |  |  |  | M |  | AN 4/9 |  |
|  |  |  | Identifying control number that must be unique within the transaction set functional group assigned by the originator for a transaction set |  |  |  |  |  |  |  |

Segment:	BGN Beginning Segment

Position:	020

Loop:

Level:	Heading

Usage:	Mandatory

Max Use:	1

Purpose:	To indicate the beginning of a transaction set

Syntax Notes:	1	If BGN05 is present, then BGN04 is required.

Semantic Notes:	1	BGN02 is the transaction set reference number.

2	BGN03 is the transaction set date.

3	BGN04 is the transaction set time.

4	BGN05 is the transaction set time qualifier.

5	BGN06 is the transaction set reference number of a previously sent transaction affected by the current transaction.

Comments:

| Notes: |  | Required |
| --- | --- | --- |
|  |  | BGN~13~200105031956531~20010531~~~~79~IT BGN~13~200106030958742~20010603~~~200105031956531~79~C |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | BGN01 | 353 | Transaction Set Purpose Code |  |  |  | M |  | ID 2/2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code identifying purpose of transaction set |  |  |  |  |  |  |  |  |
|  |  |  |  | 13 |  | Request |  |  |  |  |  |
| Must Use | BGN02 | 127 | Reference Identification |  |  |  | M |  | AN 1/30 |  |  |
|  |  |  | Reference information as defined for a particular Transaction Set or as specified by the Reference Identification Qualifier |  |  |  |  |  |  |  |  |
|  |  |  | A unique transaction identification number assigned by the originator of this transaction. This number must be unique over time. Transaction Reference numbers will only contain uppercase letters (A to Z) and digits (0 to 9). Note that punctuation (spaces, dashes, etc.) must be excluded. |  |  |  |  |  |  |  |  |
| Must Use | BGN03 | 373 | Date |  |  |  | M |  | DT 8/8 |  |  |
|  |  |  | Date expressed as CCYYMMDD |  |  |  |  |  |  |  |  |
|  |  |  | The transaction creation date - the date that the data was processed by the sender's application system. |  |  |  |  |  |  |  |  |
| Dep | BGN06 | 127 | Reference Identification |  |  |  | O |  | AN 1/30 |  |  |
|  |  |  | Reference information as defined for a particular Transaction Set or as specified by the Reference Identification Qualifier |  |  |  |  |  |  |  |  |
|  |  |  | Reconnect: Original Request: Required when Purpose Code (REF.02 of the REF~8X) = “RC001” or “RC002”; Refers to the BGN.02 of the original disconnect request, otherwise not used. Cancel: Required - Refers to the BGN02 of the original Service Order Request. Change (Update): Required - Refers to the BGN02 of the original Service Order Request. This number will be tracked in the BGN06 through the lifecycle of the Service Order Request Process. |  |  |  |  |  |  |  |  |
| Must Use | BGN07 | 640 | Transaction Type Code |  |  |  | O |  | ID 2/2 |  |  |
|  |  |  | Code specifying the type of transaction |  |  |  |  |  |  |  |  |
|  |  |  |  | 13 |  | Maintenance Request |  |  |  |  |  |
|  |  |  |  |  |  | Meter Maintenance |  |  |  |  |  |
|  |  |  |  | 38 |  | Test |  |  |  |  |  |
|  |  |  |  |  |  | Meter Test |  |  |  |  |  |
|  |  |  |  | 72 |  | Termination |  |  |  |  |  |
|  |  |  |  |  |  | Disconnect |  |  |  |  |  |
|  |  |  |  | 79 |  | Continuation |  |  |  |  |  |
|  |  |  |  |  |  | Reconnect |  |  |  |  |  |
|  |  |  |  | AN |  | Material Obligation Inquiry |  |  |  |  |  |
|  |  |  |  |  |  | Lighting |  |  |  |  |  |
|  |  |  |  | IN |  | Inquiry |  |  |  |  |  |
|  |  |  |  |  |  | Technical/Environmental |  |  |  |  |  |
|  |  |  |  | KH |  | Change Order |  |  |  |  |  |
|  |  |  |  |  |  | Meter Exchange |  |  |  |  |  |
|  |  |  |  | RD |  | Returns Detail |  |  |  |  |  |
|  |  |  |  |  |  | Read (Out of Cycle) |  |  |  |  |  |
|  |  |  |  | XZ |  | Facility Confirmation |  |  |  |  |  |
|  |  |  |  |  |  | Facilities Investigation The TDSP will not complete the order until it has done everything within it's business processes to meet the requirements of the original service order. |  |  |  |  |  |
| Must Use | BGN08 | 306 | Action Code |  |  |  | O |  | ID 1/2 |  |  |
|  |  |  | Code indicating type of action |  |  |  |  |  |  |  |  |
|  |  |  |  | 2 |  | Change (Update) |  |  |  |  |  |
|  |  |  |  |  |  | Disconnect for Non-Pay: Not Used Reconnect for Non-Pay: Not Used All Others: Used when updating any other Service Order |  |  |  |  |  |
|  |  |  |  | C |  | Cancelled |  |  |  |  |  |
|  |  |  |  |  |  | Disconnect for Non-Pay: Not Used (Send an Original Reconnect Non-Pay instead) All Others: Used when canceling any other Service Order (including Reconnect for Non-Pay) |  |  |  |  |  |
|  |  |  |  | IT |  | Initiate |  |  |  |  |  |
|  |  |  |  |  |  | Original Request |  |  |  |  |  |

Segment:	N1 Name (Customer)

Position:	050

Loop:	N1        Optional

Level:	Heading

Usage:	Optional

Max Use:	1

Purpose:	To identify a party by type of organization, name, and code

Syntax Notes:	1	At least one of N102 or N103 is required.

2	If either N103 or N104 is present, then the other is required.

Semantic Notes:

Comments:	1	This segment, used alone, provides the most efficient method of providing organizational identification. To obtain this efficiency the "ID Code" (N104) must provide a key to the table maintained by the transaction processing party.

2	N105 and N106 further define the type of entity in N101.

| Notes: |  | Required |
| --- | --- | --- |
|  |  | N1~8R~JANE DOE |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | N101 | 98 | Entity Identifier Code |  |  |  | M |  | ID 2/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code identifying an organizational entity, a physical location, property or an individual |  |  |  |  |  |  |  |  |
|  |  |  |  | 8R |  | Consumer Service Provider (CSP) Customer |  |  |  |  |  |
|  |  |  |  |  |  | Customer |  |  |  |  |  |
| Must Use | N102 | 93 | Name |  |  |  | X |  | AN 1/60 |  |  |
|  |  |  | Free-form name |  |  |  |  |  |  |  |  |
|  |  |  | Customer Name as documented in the sender's application system. |  |  |  |  |  |  |  |  |

Segment:	N2 Additional Name Information

Position:	060

Loop:	N1        Optional

Level:	Heading

Usage:	Optional

Max Use:	2

Purpose:	To specify additional names or those longer than 35 characters in length

Syntax Notes:

Semantic Notes:

Comments:

| Notes: |  | Optional |
| --- | --- | --- |
|  |  | N2~D/B/A CUSTOMER CORP |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | N201 | 93 | Name | M |  | AN 1/60 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Free-form name |  |  |  |  |
|  | N202 | 93 | Name | O |  | AN 1/60 |  |
|  |  |  | Free-form name |  |  |  |  |

Segment:	N3 Address Information (Customer Service Address)

Position:	070

Loop:	N1        Optional

Level:	Heading

Usage:	Optional

Max Use:	2

Purpose:	To specify the location of the named party

Syntax Notes:

Semantic Notes:

Comments:

| Notes: |  | Required |
| --- | --- | --- |
|  |  | N3~123 N MAIN ST MS FLR 13~ANY ADDRESS OVERFLOW |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | N301 | 166 | Address Information | M |  | AN 1/55 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Address information |  |  |  |  |
|  |  |  | Customer Service Address |  |  |  |  |
| Dep | N302 | 166 | Address Information | O |  | AN 1/55 |  |
|  |  |  | Address information |  |  |  |  |
|  |  |  | Customer Service Address Overflow Condition: If there is an overflow, it must be sent. |  |  |  |  |

Segment:	N4 Geographic Location (Customer Service Address)

Position:	080

Loop:	N1        Optional

Level:	Heading

Usage:	Optional

Max Use:	1

Purpose:	To specify the geographic place of the named party

Syntax Notes:	1	If N406 is present, then N405 is required.

Semantic Notes:

Comments:	1	A combination of either N401 through N404, or N405 and N406 may be adequate to specify a location.

2	N402 is required only if city name (N401) is in the U.S. or Canada.

| Notes: |  | Required - The first 5 characters of the Zip Code (N403) will be used for validation. |
| --- | --- | --- |
|  |  | N4~ANYTOWN~TX~78111 |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | N401 | 19 | City Name | O |  | AN 2/30 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Free-form text for city name |  |  |  |  |
| Must Use | N402 | 156 | State or Province Code | O |  | ID 2/2 |  |
|  |  |  | Code (Standard State/Province) as defined by appropriate government agency |  |  |  |  |
| Must Use | N403 | 116 | Postal Code | O |  | ID 3/15 |  |
|  |  |  | Code defining international postal zone code excluding punctuation and blanks (zip code for United States) |  |  |  |  |
|  |  |  | Postal codes will only contain digits (0 to 9). Note that punctuation (spaces, dashes, etc.) must be excluded. |  |  |  |  |

Segment:	PER Administrative Communications Contact (Customer Contact)

Position:	090

Loop:	N1        Optional

Level:	Heading

Usage:	Optional

Max Use:	>1

Purpose:	To identify a person or office to whom administrative communications should be directed

Syntax Notes:	1	If either PER03 or PER04 is present, then the other is required.

2	If either PER05 or PER06 is present, then the other is required.

3	If either PER07 or PER08 is present, then the other is required.

Semantic Notes:

Comments:

| Notes: |  | Required if telephone number is available and/or customer contact name is different than the customer name provided in N1~8R. Required when Segment YNQ Call Ahead YNQ02 = Y (Yes). The name and phone number are both required The Customer Contact Name should be formatted as follows: Last Name, First Name Only one comma will be used per the following examples: |
| --- | --- | --- |
|  |  | PER~IC~SNOW, JOE RAY JR PER~IC~SNOW, JOE RAY JR~TE~8005551212 PER~IC~SNOW, JOE RAY JR~TE~8005551212~TE~8005552121 |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | PER01 | 366 | Contact Function Code |  |  |  | M |  | ID 2/2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code identifying the major duty or responsibility of the person or group named |  |  |  |  |  |  |  |
|  |  |  |  | IC |  | Information Contact |  |  |  |  |
| Dep | PER02 | 93 | Name |  |  |  | O |  | AN 1/60 |  |
|  |  |  | Free-form name |  |  |  |  |  |  |  |
|  |  |  | Customer Contact Name - Required if this name is different than the customer name provided in N1~8R. The Outage Workshop Group will investigate and determine the solution concerning the customer name format for this transaction. The recommendation from this group is: Last Name, First Name The comma ( , ) follows the customer's last name as a means of easily segmenting this data element into a system format. The Last Name, First Name format would be used when applicable. |  |  |  |  |  |  |  |
| Dep | PER03 | 365 | Communication Number Qualifier |  |  |  | X |  | ID 2/2 |  |
|  |  |  | Code identifying the type of communication number |  |  |  |  |  |  |  |
|  |  |  | Required if providing a telephone number. |  |  |  |  |  |  |  |
|  |  |  |  | TE |  | Telephone |  |  |  |  |
| Dep | PER04 | 364 | Communication Number |  |  |  | X |  | AN 1/80 |  |
|  |  |  | Complete communications number including country or area code when applicable |  |  |  |  |  |  |  |
|  |  |  | Required if providing a telephone number. Punctuation (dashes, symbols etc.) must be excluded. |  |  |  |  |  |  |  |
| Dep | PER05 | 365 | Communication Number Qualifier |  |  |  | X |  | ID 2/2 |  |
|  |  |  | Code identifying the type of communication number |  |  |  |  |  |  |  |
|  |  |  | Condition: Required if providing a second telephone number. |  |  |  |  |  |  |  |
|  |  |  |  | TE |  | Telephone |  |  |  |  |
| Dep | PER06 | 364 | Communication Number |  |  |  | X |  | AN 1/80 |  |
|  |  |  | Complete communications number including country or area code when applicable |  |  |  |  |  |  |  |
|  |  |  | Condition: Required if providing a second telephone number. Punctuation (dashes, symbols etc.) must be excluded. |  |  |  |  |  |  |  |

Segment:	N1 Name (Transmission Distribution Service Provider)

Position:	050

Loop:	N1        Optional

Level:	Heading

Usage:	Optional

Max Use:	1

Purpose:	To identify a party by type of organization, name, and code

Syntax Notes:	1	At least one of N102 or N103 is required.

2	If either N103 or N104 is present, then the other is required.

Semantic Notes:

Comments:	1	This segment, used alone, provides the most efficient method of providing organizational identification. To obtain this efficiency the "ID Code" (N104) must provide a key to the table maintained by the transaction processing party.

2	N105 and N106 further define the type of entity in N101.

| Notes: |  | Required |
| --- | --- | --- |
|  |  | N1~8S~TDSP NAME~1~007909411~~40 |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | N101 | 98 | Entity Identifier Code |  |  |  | M |  | ID 2/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code identifying an organizational entity, a physical location, property or an individual |  |  |  |  |  |  |  |  |
|  |  |  |  | 8S |  | Consumer Service Provider (CSP) |  |  |  |  |  |
|  |  |  |  |  |  | Transmission Distribution Service Provider (TDSP) |  |  |  |  |  |
| Must Use | N102 | 93 | Name |  |  |  | X |  | AN 1/60 |  |  |
|  |  |  | Free-form name |  |  |  |  |  |  |  |  |
|  |  |  | TDSP Name |  |  |  |  |  |  |  |  |
| Must Use | N103 | 66 | Identification Code Qualifier |  |  |  | X |  | ID 1/2 |  |  |
|  |  |  | Code designating the system/method of code structure used for Identification Code (67) |  |  |  |  |  |  |  |  |
|  |  |  |  | 1 |  | D-U-N-S Number, Dun & Bradstreet |  |  |  |  |  |
|  |  |  |  | 9 |  | D-U-N-S+4, D-U-N-S Number with Four Character Suffix |  |  |  |  |  |
| Must Use | N104 | 67 | Identification Code |  |  |  | X |  | AN 2/80 |  |  |
|  |  |  | Code identifying a party or other code |  |  |  |  |  |  |  |  |
|  |  |  | TDSP D-U-N-S Number or D-U-N-S + 4 Number |  |  |  |  |  |  |  |  |
| Must Use | N106 | 98 | Entity Identifier Code |  |  |  | O |  | ID 2/3 |  |  |
|  |  |  | Code identifying an organizational entity, a physical location, property or an individual |  |  |  |  |  |  |  |  |
|  |  |  |  | 40 |  | Receiver |  |  |  |  |  |

Segment:	N1 Name (Competitive Retailer)

Position:	050

Loop:	N1        Optional

Level:	Heading

Usage:	Optional

Max Use:	1

Purpose:	To identify a party by type of organization, name, and code

Syntax Notes:	1	At least one of N102 or N103 is required.

2	If either N103 or N104 is present, then the other is required.

Semantic Notes:

Comments:	1	This segment, used alone, provides the most efficient method of providing organizational identification. To obtain this efficiency the "ID Code" (N104) must provide a key to the table maintained by the transaction processing party.

2	N105 and N106 further define the type of entity in N101.

| Notes: |  | Required |
| --- | --- | --- |
|  |  | N1~SJ~CR NAME~9~007909411ESP1~~41 |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | N101 | 98 | Entity Identifier Code |  |  |  | M |  | ID 2/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code identifying an organizational entity, a physical location, property or an individual |  |  |  |  |  |  |  |  |
|  |  |  |  | SJ |  | Service Provider |  |  |  |  |  |
|  |  |  |  |  |  | Competitive Retailer (CR) |  |  |  |  |  |
| Must Use | N102 | 93 | Name |  |  |  | X |  | AN 1/60 |  |  |
|  |  |  | Free-form name |  |  |  |  |  |  |  |  |
|  |  |  | CR Name |  |  |  |  |  |  |  |  |
| Must Use | N103 | 66 | Identification Code Qualifier |  |  |  | X |  | ID 1/2 |  |  |
|  |  |  | Code designating the system/method of code structure used for Identification Code (67) |  |  |  |  |  |  |  |  |
|  |  |  |  | 1 |  | D-U-N-S Number, Dun & Bradstreet |  |  |  |  |  |
|  |  |  |  | 9 |  | D-U-N-S+4, D-U-N-S Number with Four Character Suffix |  |  |  |  |  |
| Must Use | N104 | 67 | Identification Code |  |  |  | X |  | AN 2/80 |  |  |
|  |  |  | Code identifying a party or other code |  |  |  |  |  |  |  |  |
|  |  |  | CR D-U-N-S Number or D-U-N-S + 4 Number |  |  |  |  |  |  |  |  |
| Must Use | N106 | 98 | Entity Identifier Code |  |  |  | O |  | ID 2/3 |  |  |
|  |  |  | Code identifying an organizational entity, a physical location, property or an individual |  |  |  |  |  |  |  |  |
|  |  |  |  | 41 |  | Submitter |  |  |  |  |  |

Segment:	HL Hierarchical Level (Service Order Level Information)

Position:	010

Loop:	HL        Mandatory

Level:	Detail

Usage:	Mandatory

Max Use:	1

Purpose:	To identify dependencies among and the content of hierarchically related groups of data segments

Syntax Notes:

Semantic Notes:

Comments:	1	The HL segment is used to identify levels of detail information using a hierarchical structure, such as relating line-item data to shipment data, and packaging data to line-item data.

The HL segment defines a top-down/left-right ordered structure.

2	HL01 shall contain a unique alphanumeric number for each occurrence of the HL segment in the transaction set. For example, HL01 could be used to indicate the number of occurrences of the HL segment, in which case the value of HL01 would be "1" for the initial HL segment and would be incremented by one in each subsequent HL segment within the transaction.

3	HL02 identifies the hierarchical ID number of the HL segment to which the current HL segment is subordinate.

4	HL03 indicates the context of the series of segments following the current HL segment up to the next occurrence of an HL segment in the transaction. For example, HL03 is used to indicate that subsequent segments in the HL loop form a logical grouping of data referring to shipment, order, or item-level information.

5	HL04 indicates whether or not there are subordinate (or child) HL segments related to the current HL segment.

| Notes: |  | HL Parent Loop (Service Order Level Information) |
| --- | --- | --- |
|  |  | Required For an ESI ID with multiple meters the CR will generate a service order request for each meter affected. "Multiple meters at an ESI ID requires multiple service orders." |
|  |  | HL~1~~EV~0 |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | HL01 | 628 | Hierarchical ID Number |  |  |  | M |  | AN 1/12 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | A unique number assigned by the sender to identify a particular data segment in a hierarchical structure |  |  |  |  |  |  |  |  |
|  |  |  | This unique number will be used to identify the HL Loop. One parent loop (Service Order Level Information) per transaction. |  |  |  |  |  |  |  |  |
|  |  |  |  | 1 |  | Parent HL Loop - Service Order Level Information |  |  |  |  |  |
| Must Use | HL03 | 735 | Hierarchical Level Code |  |  |  | M |  | ID 1/2 |  |  |
|  |  |  | Code defining the characteristic of a level in a hierarchical structure |  |  |  |  |  |  |  |  |
|  |  |  |  | EV |  | Event |  |  |  |  |  |
|  |  |  |  |  |  | Other type of action or report |  |  |  |  |  |
| Must Use | HL04 | 736 | Hierarchical Child Code |  |  |  | O |  | ID 1/1 |  |  |
|  |  |  | Code indicating if there are hierarchical child data segments subordinate to the level being described |  |  |  |  |  |  |  |  |
|  |  |  | Indicates one hierarchical loop will be provided. |  |  |  |  |  |  |  |  |
|  |  |  |  | 0 |  | No Subordinate HL Segment in This Hierarchical Structure. |  |  |  |  |  |
|  |  |  |  |  |  | Service order applies to one meter affected at the ESI ID. |  |  |  |  |  |

Segment:	REF Reference Identification (Purpose Code)

Position:	030

Loop:	HL        Mandatory

Level:	Detail

Usage:	Optional

Max Use:	>1

Purpose:	To specify identifying information

Syntax Notes:	1	At least one of REF02 or REF03 is required.

2	If either C04003 or C04004 is present, then the other is required.

3	If either C04005 or C04006 is present, then the other is required.

Semantic Notes:	1	REF04 contains data relating to the value cited in REF02.

Comments:

| Notes: |  | HL Parent Loop (Service Order Level Information) All service orders must be for known ESI IDs, including service orders for Street Lights and Guard Lights. The following Purpose Codes can only be used when requesting the associated Transaction Type Code as noted below: Purpose Codes Beginning With: DC use only when BGN07 = 72 Disconnect FI use only when BGN07 = XZ Facilities Investigation GL use only when BGN07 = AN Lighting SL use only when BGN07 = AN Lighting RC use only when BGN07 = 79 Reconnect ME use only when BGN07 = KH Meter Exchange MT use only when BGN07 = 38 Meter Test MM use only when BGN07 = 13 Meter Maintenance RD use only when BGN07 = RD Read (Out of Cycle) TE use only when BGN07 = IN Technical Environmental |
| --- | --- | --- |
|  |  | Required |
|  |  | REF~8X~DC001 |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | REF01 | 128 | Reference Identification Qualifier |  |  |  | M |  | ID 2/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code qualifying the Reference Identification |  |  |  |  |  |  |  |  |
|  |  |  |  | 8X |  | Transaction Category or Type |  |  |  |  |  |
|  |  |  |  |  |  | Purpose Code |  |  |  |  |  |
| Must Use | REF02 | 127 | Reference Identification |  |  |  | X |  | AN 1/30 |  |  |
|  |  |  | Reference information as defined for a particular Transaction Set or as specified by the Reference Identification Qualifier |  |  |  |  |  |  |  |  |
|  |  |  |  | DC001 |  | Disconnect for Non-Pay |  |  |  |  |  |
|  |  |  |  |  |  | When this purpose code is sent, the Segment YNQ Yes/No Disconnect Premium Location is Required to be sent with either a Yes or a No in the YNQ02 |  |  |  |  |  |
|  |  |  |  | DC002 |  | Disconnect for Customer Requested Clearance |  |  |  |  |  |
|  |  |  |  |  |  | The TDSP must be able to accept the Disconnect for Customer Requested Clearance and Reconnect for Customer Requested Clearance at the same time. |  |  |  |  |  |
|  |  |  |  | DC003 |  | Remove One Specific Meter on a Multi-Meter Premise/ESI-ID. |  |  |  |  |  |
|  |  |  |  |  |  | This can only be used to Remove One Specific Meter from a Multi-Meter Premise/ESI-ID. Not to be used for Non-Multi-Metered Premises/ESI-ID. This does not close out the Premise/ESI-ID. |  |  |  |  |  |
|  |  |  |  | FI001 |  | Relocation of Service |  |  |  |  |  |
|  |  |  |  | FI002 |  | Relocation of Facilities |  |  |  |  |  |
|  |  |  |  | FI003 |  | Plan Review |  |  |  |  |  |
|  |  |  |  | FI004 |  | Dead Animal on Facilities |  |  |  |  |  |
|  |  |  |  | FI005 |  | Transformer Open |  |  |  |  |  |
|  |  |  |  | FI006 |  | Fire Ants |  |  |  |  |  |
|  |  |  |  | FI007 |  | Wire |  |  |  |  |  |
|  |  |  |  | FI008 |  | Pole |  |  |  |  |  |
|  |  |  |  | FI009 |  | Transformer |  |  |  |  |  |
|  |  |  |  | FI010 |  | Crossed Meters |  |  |  |  |  |
|  |  |  |  | FI011 |  | Possible Meter Damage |  |  |  |  |  |
|  |  |  |  | GL001 |  | Guard Light - Lights Out |  |  |  |  |  |
|  |  |  |  | GL002 |  | Guard Light - Dim Light |  |  |  |  |  |
|  |  |  |  | GL003 |  | Guard Light - Light Hanging |  |  |  |  |  |
|  |  |  |  | GL004 |  | Guard Light - Pole Leaning |  |  |  |  |  |
|  |  |  |  | GL005 |  | Guard Light - Light On All Day |  |  |  |  |  |
|  |  |  |  | GL006 |  | Guard Light - Light Goes On and Off |  |  |  |  |  |
|  |  |  |  | GL007 |  | Guard Light - Install Shield |  |  |  |  |  |
|  |  |  |  | GL008 |  | Guard Light - Re-aim |  |  |  |  |  |
|  |  |  |  | GL009 |  | Guard Light - Remove a Specific Lamp |  |  |  |  |  |
|  |  |  |  | ME001 |  | Exchange with AMR (Automated Meter Read) Meter |  |  |  |  |  |
|  |  |  |  | ME002 |  | Exchange with OMR (Off-site Meter Read) Meter |  |  |  |  |  |
|  |  |  |  | ME003 |  | Exchange with IDR |  |  |  |  |  |
|  |  |  |  | ME004 |  | Exchange with TOU |  |  |  |  |  |
|  |  |  |  | ME005 |  | Exchange with Demand Meter |  |  |  |  |  |
|  |  |  |  | ME006 |  | Exchange with kVAR Meter |  |  |  |  |  |
|  |  |  |  | ME007 |  | Exchange with kWh Meter |  |  |  |  |  |
|  |  |  |  | ME008 |  | Exchange with Electrical Pulse Metering |  |  |  |  |  |
|  |  |  |  | ME009 |  | Exchange (like for like) |  |  |  |  |  |
|  |  |  |  | ME010 |  | Upgrade (Demand Capacity, Scale # of Dials) |  |  |  |  |  |
|  |  |  |  |  |  | Upgrade is an exchange of a like meter for like meter where there is a need for a change due to increased functionality or customer request. On upgrades, comments will be used for details. |  |  |  |  |  |
|  |  |  |  | ME011 |  | Damaged |  |  |  |  |  |
|  |  |  |  |  |  | A damaged meter purpose code indicates that the customer knows the meter has been damaged. |  |  |  |  |  |
|  |  |  |  | ME012 |  | Dead Meter |  |  |  |  |  |
|  |  |  |  | MM001 |  | Install Lock Band |  |  |  |  |  |
|  |  |  |  | MM002 |  | Remove Lock Band |  |  |  |  |  |
|  |  |  |  | MM003 |  | Meter Based Angle Adapter "elbow" |  |  |  |  |  |
|  |  |  |  | MM004 |  | Dual Socket Adapter |  |  |  |  |  |
|  |  |  |  | MM005 |  | Verify Meter Data |  |  |  |  |  |
|  |  |  |  | MM006 |  | Tampering Suspected |  |  |  |  |  |
|  |  |  |  | MT001 |  | Meter Test |  |  |  |  |  |
|  |  |  |  | RC001 |  | Reconnect after Disconnect for Non-Pay |  |  |  |  |  |
|  |  |  |  | RC002 |  | Reconnect for Customer Requested Clearance |  |  |  |  |  |
|  |  |  |  |  |  | The TDSP must be able to accept the Disconnect for Customer Requested Clearance and Reconnect for Customer Requested Clearance at the same time. |  |  |  |  |  |
|  |  |  |  | RC003 |  | Reconnect of Requested Suspension |  |  |  |  |  |
|  |  |  |  | RD001 |  | Special Out of Cycle Read |  |  |  |  |  |
|  |  |  |  | RD002 |  | Re-Read/Potential Error |  |  |  |  |  |
|  |  |  |  | SL001 |  | Street Light - Lights Out |  |  |  |  |  |
|  |  |  |  | SL002 |  | Street Light - Dim Light |  |  |  |  |  |
|  |  |  |  | SL003 |  | Street Light - Light Hanging |  |  |  |  |  |
|  |  |  |  | SL004 |  | Street Light - Pole Leaning |  |  |  |  |  |
|  |  |  |  | SL005 |  | Street Light - Light On All Day |  |  |  |  |  |
|  |  |  |  | SL006 |  | Street Light - Light Goes On and Off |  |  |  |  |  |
|  |  |  |  | SL007 |  | Street Light - Install Shield |  |  |  |  |  |
|  |  |  |  | SL008 |  | Street Light - Needs Tag |  |  |  |  |  |
|  |  |  |  | SL009 |  | Street Light - Pole Paint |  |  |  |  |  |
|  |  |  |  | SL010 |  | Street Light - Remove a Specific Lamp |  |  |  |  |  |
|  |  |  |  | TE001 |  | Radio/TV Interference |  |  |  |  |  |
|  |  |  |  | TE002 |  | EMF (Electro-Magnetic Field) |  |  |  |  |  |
|  |  |  |  | TE003 |  | PCB |  |  |  |  |  |
|  |  |  |  | TE004 |  | Audible Interference |  |  |  |  |  |
|  |  |  |  | TE005 |  | Power Quality |  |  |  |  |  |
|  |  |  |  | TE006 |  | Tree Trimming |  |  |  |  |  |
|  |  |  |  | TE007 |  | Right of Way Cut |  |  |  |  |  |

Segment:	REF Reference Identification (Pole Number)

Position:	030

Loop:	HL        Mandatory

Level:	Detail

Usage:	Optional

Max Use:	>1

Purpose:	To specify identifying information

Syntax Notes:	1	At least one of REF02 or REF03 is required.

2	If either C04003 or C04004 is present, then the other is required.

3	If either C04005 or C04006 is present, then the other is required.

Semantic Notes:	1	REF04 contains data relating to the value cited in REF02.

Comments:

| Notes: |  | HL Parent Loop (Service Order Level Information) Pole numbers will only contain uppercase letters (A to Z) and digits (0 to 9). Note that punctuation (spaces, dashes, etc.) must be excluded, and leading and trailing zeros that are part of the meter number must be present. |
| --- | --- | --- |
|  |  | Required if available for Street Lights. This segment may be repeated as many times as necessary to provide all pole numbers. |
|  |  | REF~ADE~1Z295820394820 |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | REF01 | 128 | Reference Identification Qualifier |  |  |  | M |  | ID 2/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code qualifying the Reference Identification |  |  |  |  |  |  |  |  |
|  |  |  |  | ADE |  | Associated Property Number |  |  |  |  |  |
|  |  |  |  |  |  | Pole Number |  |  |  |  |  |
| Must Use | REF02 | 127 | Reference Identification |  |  |  | X |  | AN 1/30 |  |  |
|  |  |  | Reference information as defined for a particular Transaction Set or as specified by the Reference Identification Qualifier |  |  |  |  |  |  |  |  |
|  |  |  | Pole Number |  |  |  |  |  |  |  |  |

Segment:	REF Reference Identification (Meter Number)

Position:	030

Loop:	HL        Mandatory

Level:	Detail

Usage:	Optional

Max Use:	>1

Purpose:	To specify identifying information

Syntax Notes:	1	At least one of REF02 or REF03 is required.

2	If either C04003 or C04004 is present, then the other is required.

3	If either C04005 or C04006 is present, then the other is required.

Semantic Notes:	1	REF04 contains data relating to the value cited in REF02.

Comments:

| Notes: |  | HL Parent Loop (Service Order Level Information) Meter numbers will only contain uppercase letters (A to Z) and digits (0 to 9). Note that punctuation (spaces, dashes, etc.) must be excluded, and leading and trailing zeros that are part of the meter number must be present. |
| --- | --- | --- |
|  |  | Meter Number is required on the following service order types: Disconnect for Customer Requested Clearance (DC002) Remove One Specific Meter on a Multi-Meter Premise/ESI-ID (DC003). Meter Exchanges (ME0xx) All Meter Maintenance (MM0xx) All Reconnect for Customer Requested Clearance (RC002) Special Out of Cycle Read (RD001) Re-Read/Potential Error (RD002) Meter Test (MT001) Meter Number is Not Used on all other Service Orders. |
|  |  | REF~MG~394820R |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | REF01 | 128 | Reference Identification Qualifier |  |  |  | M |  | ID 2/3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code qualifying the Reference Identification |  |  |  |  |  |  |  |
|  |  |  |  | MG |  | Meter Number |  |  |  |  |
| Must Use | REF02 | 127 | Reference Identification |  |  |  | X |  | AN 1/30 |  |
|  |  |  | Reference information as defined for a particular Transaction Set or as specified by the Reference Identification Qualifier |  |  |  |  |  |  |  |
|  |  |  | Meter Number |  |  |  |  |  |  |  |

Segment:	REF Reference Identification (Priority Code)

Position:	030

Loop:	HL        Mandatory

Level:	Detail

Usage:	Optional

Max Use:	>1

Purpose:	To specify identifying information

Syntax Notes:	1	At least one of REF02 or REF03 is required.

2	If either C04003 or C04004 is present, then the other is required.

3	If either C04005 or C04006 is present, then the other is required.

Semantic Notes:	1	REF04 contains data relating to the value cited in REF02.

Comments:

| Notes: |  | HL Parent Loop (Service Order Level Information) On their respective websites, each TDSP will provide a table showing: TDSP Priority Codes Used (02 - 99; Note that 01 will always be "standard") Meaning of each Priority Code Service Orders to which each Priority Code applies Number of days each service order will take to execute |
| --- | --- | --- |
|  |  | Required |
|  |  | REF~PH~01 |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | REF01 | 128 | Reference Identification Qualifier |  |  |  | M |  | ID 2/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code qualifying the Reference Identification |  |  |  |  |  |  |  |  |
|  |  |  |  | PH |  | Priority Rating |  |  |  |  |  |
|  |  |  |  |  |  | Priority Code |  |  |  |  |  |
| Must Use | REF02 | 127 | Reference Identification |  |  |  | X |  | AN 1/30 |  |  |
|  |  |  | Reference information as defined for a particular Transaction Set or as specified by the Reference Identification Qualifier |  |  |  |  |  |  |  |  |
|  |  |  |  | 01 |  | Priority 01 - This must be "standard" |  |  |  |  |  |
|  |  |  |  | 02 |  | Priority 02 |  |  |  |  |  |
|  |  |  |  | 03 |  | Priority 03 |  |  |  |  |  |
|  |  |  |  | 04 |  | Priority 04 |  |  |  |  |  |
|  |  |  |  | 05 |  | Priority 05 |  |  |  |  |  |
|  |  |  |  | 06 |  | Priority 06 |  |  |  |  |  |
|  |  |  |  | 07 |  | Priority 07 |  |  |  |  |  |
|  |  |  |  | 08 |  | Priority 08 |  |  |  |  |  |
|  |  |  |  | 09 |  | Priority 09 |  |  |  |  |  |
|  |  |  |  | 10 |  | Priority 10 |  |  |  |  |  |
|  |  |  |  |  |  | 11 - 99 Priority Codes may go as high as 99 |  |  |  |  |  |

Segment:	REF Reference Identification (ESI ID)

Position:	030

Loop:	HL        Mandatory

Level:	Detail

Usage:	Optional

Max Use:	>1

Purpose:	To specify identifying information

Syntax Notes:	1	At least one of REF02 or REF03 is required.

2	If either C04003 or C04004 is present, then the other is required.

3	If either C04005 or C04006 is present, then the other is required.

Semantic Notes:	1	REF04 contains data relating to the value cited in REF02.

Comments:

| Notes: |  | HL Parent Loop (Service Order Level Information) Required |
| --- | --- | --- |
|  |  | REF~Q5~~10111111234567890ABCDEFGHIJKLMNOPQRS |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | REF01 | 128 | Reference Identification Qualifier |  |  |  | M |  | ID 2/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code qualifying the Reference Identification |  |  |  |  |  |  |  |  |
|  |  |  |  | Q5 |  | Property Control Number |  |  |  |  |  |
|  |  |  |  |  |  | Electric Service Identifier (ESI ID) |  |  |  |  |  |
| Must Use | REF03 | 352 | Description |  |  |  | X |  | AN 1/80 |  |  |
|  |  |  | A free-form description to clarify the related data elements and their content |  |  |  |  |  |  |  |  |
|  |  |  | ESI ID |  |  |  |  |  |  |  |  |

Segment:	REF Reference Identification (Special Needs Indicator)

Position:	030

Loop:	HL        Mandatory

Level:	Detail

Usage:	Optional

Max Use:	>1

Purpose:	To specify identifying information

Syntax Notes:	1	At least one of REF02 or REF03 is required.

2	If either C04003 or C04004 is present, then the other is required.

3	If either C04005 or C04006 is present, then the other is required.

Semantic Notes:	1	REF04 contains data relating to the value cited in REF02.

Comments:

| Notes: |  | HL Parent Loop (Service Order Level Information) Required A "Y" in this field means that the customer is either: · A residential customer who qualifies through the Residential Critical Care eligibility Determination Form, as issued by the PUCT. Or · A customer for which electric service is considered crucial for the protection and maintenance of public safety pursuant to Subst. Rules §§25.52 and 25.53. Or · An industrial customer for whom an interruption or suspension of service will create a dangerous or life threatening condition at the customer's premise. |
| --- | --- | --- |
|  |  | REF~SU~Y |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | REF01 | 128 | Reference Identification Qualifier |  |  |  | M |  | ID 2/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code qualifying the Reference Identification |  |  |  |  |  |  |  |  |
|  |  |  |  | SU |  | Special Processing Code |  |  |  |  |  |
|  |  |  |  |  |  | Special Needs Indicator |  |  |  |  |  |
| Must Use | REF02 | 127 | Reference Identification |  |  |  | X |  | AN 1/30 |  |  |
|  |  |  | Reference information as defined for a particular Transaction Set or as specified by the Reference Identification Qualifier |  |  |  |  |  |  |  |  |
|  |  |  |  | N |  | No |  |  |  |  |  |
|  |  |  |  |  |  | Special Needs Not Required |  |  |  |  |  |
|  |  |  |  | Y |  | Yes |  |  |  |  |  |
|  |  |  |  |  |  | Special Needs Required |  |  |  |  |  |

Segment:	REF Reference Identification (Reason for Change)

Position:	030

Loop:	HL        Mandatory

Level:	Detail

Usage:	Optional

Max Use:	>1

Purpose:	To specify identifying information

Syntax Notes:	1	At least one of REF02 or REF03 is required.

2	If either C04003 or C04004 is present, then the other is required.

3	If either C04005 or C04006 is present, then the other is required.

Semantic Notes:	1	REF04 contains data relating to the value cited in REF02.

Comments:

| Notes: |  | HL Parent Loop (Service Order Level Information) The REF~TD segment may repeat as many times as necessary to provide all change reasons. The REF~TD segment should only occur for those segments that are changing from what was previously submitted. If a segment is present on the change transaction but is not changing from what was submitted previously, a REF~TD should not be included for the segment that is not changing. For Example, if a CR is changing from one non-standard Priority Code to another non-standard Priority Code but is not changing the Service Requested Date, the DTM~211 will be contained within the transaction. In this example, there will be REF~TD~REFPH segment, but there should not be a REF~TD~DTM211 segment. |
| --- | --- | --- |
|  |  | Required on Change (Update) (BGN08 = "2") |
|  |  | REF~TD~REFMG |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | REF01 | 128 | Reference Identification Qualifier |  |  |  | M |  | ID 2/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code qualifying the Reference Identification |  |  |  |  |  |  |  |  |
|  |  |  |  | TD |  | Reason for Change |  |  |  |  |  |
| Must Use | REF02 | 127 | Reference Identification |  |  |  | X |  | AN 1/30 |  |  |
|  |  |  | Reference information as defined for a particular Transaction Set or as specified by the Reference Identification Qualifier |  |  |  |  |  |  |  |  |
|  |  |  |  | DTM211 |  | Change Service Requested Date |  |  |  |  |  |
|  |  |  |  | DTM843 |  | Change Not Before Date/Time |  |  |  |  |  |
|  |  |  |  | MTXACC |  | Change Access Instructions |  |  |  |  |  |
|  |  |  |  | MTXRPT |  | Change Report Remarks |  |  |  |  |  |
|  |  |  |  | N18R |  | Change Customer Information |  |  |  |  |  |
|  |  |  |  |  |  | Includes customer name, customer contact name, telephone number. |  |  |  |  |  |
|  |  |  |  | REFADE |  | Change Pole Number |  |  |  |  |  |
|  |  |  |  | REFMG |  | Change Meter Number |  |  |  |  |  |
|  |  |  |  | REFPH |  | Change Priority Code |  |  |  |  |  |
|  |  |  |  | YNQCAL |  | Change Call Ahead |  |  |  |  |  |
|  |  |  |  | YNQPDL |  | Change Premium Disconnect Location |  |  |  |  |  |

Segment:	DTM Date/Time Reference (Service Requested Date)

Position:	050

Loop:	HL        Mandatory

Level:	Detail

Usage:	Optional

Max Use:	>1

Purpose:	To specify pertinent dates and times

Syntax Notes:	1	At least one of DTM02 DTM03 or DTM05 is required.

2	If DTM04 is present, then DTM03 is required.

3	If either DTM05 or DTM06 is present, then the other is required.

Semantic Notes:

Comments:

| Notes: |  | HL Parent Loop (Service Order Level Information) If the date requested is within the standard time frame (Priority Code 01), the TDSP will treat it as a wish date, and will work on that date if possible. If the TDSP cannot accomplish the task on that date, they will complete it within the standard time window. For customer requested clearance, the date requested by the CR must be adhered to by the TDSP if it is within the time specified by the tariff. |
| --- | --- | --- |
|  |  | Required when Priority Code is other than Standard Not Used when BGN08 = C (Cancel) Optional when requesting standard service with a "wish to be worked date" which is within the standard time window. |
|  |  | DTM~211~20010601 |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | DTM01 | 374 | Date/Time Qualifier |  |  |  | M |  | ID 3/3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code specifying type of date or time, or both date and time |  |  |  |  |  |  |  |
|  |  |  |  | 211 |  | Service Requested |  |  |  |  |
| Must Use | DTM02 | 373 | Date |  |  |  | X |  | DT 8/8 |  |
|  |  |  | Date expressed as CCYYMMDD |  |  |  |  |  |  |  |

Segment:	DTM Date/Time Reference (Not Before Date/Time)

Position:	050

Loop:	HL        Mandatory

Level:	Detail

Usage:	Optional

Max Use:	>1

Purpose:	To specify pertinent dates and times

Syntax Notes:	1	At least one of DTM02 DTM03 or DTM05 is required.

2	If DTM04 is present, then DTM03 is required.

3	If either DTM05 or DTM06 is present, then the other is required.

Semantic Notes:

Comments:

| Notes: |  | HL Parent Loop (Service Order Level Information) This segment is used to indicate that action should not be performed before the date and time provided. |
| --- | --- | --- |
|  |  | Disconnect for Clearance: Required if applicable All others: Not Used |
|  |  | DTM~843~20010602 DTM~843~20010602~0800 |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | DTM01 | 374 | Date/Time Qualifier |  |  |  | M |  | ID 3/3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code specifying type of date or time, or both date and time |  |  |  |  |  |  |  |
|  |  |  |  | 843 |  | Not Before |  |  |  |  |
| Must Use | DTM02 | 373 | Date |  |  |  | X |  | DT 8/8 |  |
|  |  |  | Date expressed as CCYYMMDD |  |  |  |  |  |  |  |
| Dep | DTM03 | 337 | Time |  |  |  | X |  | TM 4/8 |  |
|  |  |  | Time expressed in 24-hour clock time as follows: HHMM, or HHMMSS, or HHMMSSD, or HHMMSSDD, where H = hours (00-23), M = minutes (00-59), S = integer seconds (00-59) and DD = decimal seconds; decimal seconds are expressed as follows: D = tenths (0-9) and DD = hundredths (00-99) |  |  |  |  |  |  |  |
|  |  |  | Used when requesting service to be performed at a specific time. HHMM, where H = Hours (00 to 23) and M = Minutes (00 to 59) in Central Prevailing Time (CT). For this transaction, since X12 does not allow 2400 for time, 2359 will be used to indicate midnight. For example, midnight between October 15th and October 16th will be reflected as 2359 of October 15th. |  |  |  |  |  |  |  |

Segment:	YNQ Yes/No Question (Call Ahead)

Position:	070

Loop:	HL        Mandatory

Level:	Detail

Usage:	Optional

Max Use:	>1

Purpose:	To identify and answer yes and no questions, including the date, time, and comments further qualifying the condition

Syntax Notes:	1	Only one of YNQ01 YNQ09 or YNQ10 may be present.

2	If either YNQ03 or YNQ04 is present, then the other is required.

3	If YNQ09 is present, then YNQ08 is required.

Semantic Notes:	1	YNQ02 confirms or denies the statement made in YNQ01, YNQ09 or YNQ10. A "Y" indicates the statement is confirmed; an "N" indicates the statement is denied.

2	YNQ10 contains a free-form question when codified questions are not available.

Comments:

| Notes: |  | HL Parent Loop (Service Order Level Information) Required if customer requests a call before the work is to be performed. This requires a phone number to be populated in the PER Segment. |
| --- | --- | --- |
|  |  | YNQ~~Y~~~~~~9~CAL |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | YNQ02 | 1073 | Yes/No Condition or Response Code |  |  |  | M |  | ID 1/1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code indicating a Yes or No condition or response |  |  |  |  |  |  |  |  |
|  |  |  |  | N |  | No |  |  |  |  |  |
|  |  |  |  |  |  | No need to call the customer |  |  |  |  |  |
|  |  |  |  | Y |  | Yes |  |  |  |  |  |
|  |  |  |  |  |  | Call the customer before performing the work |  |  |  |  |  |
| Must Use | YNQ08 | 1270 | Code List Qualifier Code |  |  |  | X |  | ID 1/3 |  |  |
|  |  |  | Code identifying a specific industry code list |  |  |  |  |  |  |  |  |
|  |  |  |  | 9 |  | Indicator Code |  |  |  |  |  |
| Must Use | YNQ09 | 1271 | Industry Code |  |  |  | X |  | AN 1/30 |  |  |
|  |  |  | Code indicating a code from a specific industry code list |  |  |  |  |  |  |  |  |
|  |  |  |  | CAL |  | Call Ahead |  |  |  |  |  |
|  |  |  |  |  |  | Does this customer require a call before the work is to be performed? |  |  |  |  |  |

Segment:	YNQ Yes/No Question (Premium Disconnect Location)

Position:	070

Loop:	HL        Mandatory

Level:	Detail

Usage:	Optional

Max Use:	>1

Purpose:	To identify and answer yes and no questions, including the date, time, and comments further qualifying the condition

Syntax Notes:	1	Only one of YNQ01 YNQ09 or YNQ10 may be present.

2	If either YNQ03 or YNQ04 is present, then the other is required.

3	If YNQ09 is present, then YNQ08 is required.

Semantic Notes:	1	YNQ02 confirms or denies the statement made in YNQ01, YNQ09 or YNQ10. A "Y" indicates the statement is confirmed; an "N" indicates the statement is denied.

2	YNQ10 contains a free-form question when codified questions are not available.

Comments:

| Notes: |  | HL Parent Loop (Service Order Level Information) |
| --- | --- | --- |
|  |  | Required only when the Segment REF Purpose Code REF02 equals DC001 (Disconnent Non-Pay.) Not used with any other Segment REF Purpose Codes. |
|  |  | YNQ~~N~~~~~~9~PDL |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | YNQ02 | 1073 | Yes/No Condition or Response Code |  |  |  | M |  | ID 1/1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code indicating a Yes or No condition or response |  |  |  |  |  |  |  |  |
|  |  |  |  | N |  | No |  |  |  |  |  |
|  |  |  |  |  |  | Standard for that meter (normal business practice - usually most economical.) |  |  |  |  |  |
|  |  |  |  | Y |  | Yes |  |  |  |  |  |
|  |  |  |  |  |  | Premium - Do what you have to do to get it done - complying with the tariff. |  |  |  |  |  |
| Must Use | YNQ08 | 1270 | Code List Qualifier Code |  |  |  | X |  | ID 1/3 |  |  |
|  |  |  | Code identifying a specific industry code list |  |  |  |  |  |  |  |  |
|  |  |  |  | 9 |  | Indicator Code |  |  |  |  |  |
| Must Use | YNQ09 | 1271 | Industry Code |  |  |  | X |  | AN 1/30 |  |  |
|  |  |  | Code indicating a code from a specific industry code list |  |  |  |  |  |  |  |  |
|  |  |  |  | PDL |  | Premium Disconnect Location |  |  |  |  |  |
|  |  |  |  |  |  | Indicates whether the CR will pay for a premium disconnect location. |  |  |  |  |  |

Segment:	MTX Text (Directions)

Position:	250

Loop:	MTX        Optional

Level:	Detail

Usage:	Optional

Max Use:	1

Purpose:	To specify textual data

Syntax Notes:	1	If MTX01 is present, then MTX02 is required.

2	If MTX03 is present, then MTX02 is required.

Semantic Notes:

Comments:

| Notes: |  | HL Parent Loop (Service Order Level Information) Required if directions add value to the Service Order. |
| --- | --- | --- |
|  |  | MTX~ACC~9 MILES FROM SQUARE ON HWY 243 ON RIGHT |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | MTX01 | 363 | Note Reference Code |  |  |  | O |  | ID 3/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code identifying the functional area or purpose for which the note applies |  |  |  |  |  |  |  |  |
|  |  |  |  | ACC |  | Access Instructions |  |  |  |  |  |
|  |  |  |  |  |  | Directions |  |  |  |  |  |
| Must Use | MTX02 | 1551 | Message Text |  |  |  | X |  | AN 1/4096 |  |  |
|  |  |  | To transmit large volumes of message text |  |  |  |  |  |  |  |  |
|  |  |  | Message Text not to exceed 80 characters total. |  |  |  |  |  |  |  |  |

Segment:	MTX Text (Comments)

Position:	250

Loop:	MTX        Optional

Level:	Detail

Usage:	Optional

Max Use:	1

Purpose:	To specify textual data

Syntax Notes:	1	If MTX01 is present, then MTX02 is required.

2	If MTX03 is present, then MTX02 is required.

Semantic Notes:

Comments:

| Notes: |  | HL Parent Loop (Service Order Level Information) Required if comments add value to the Service Order. |
| --- | --- | --- |
|  |  | MTX~RPT~PLS WORK NO SOONER THAN AUG 10 AND NO LATER THAN AUG 15. WARNING - IRATE CUST. |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | MTX01 | 363 | Note Reference Code |  |  |  | O |  | ID 3/3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code identifying the functional area or purpose for which the note applies |  |  |  |  |  |  |  |
|  |  |  |  | RPT |  | Report Remarks |  |  |  |  |
| Must Use | MTX02 | 1551 | Message Text |  |  |  | X |  | AN 1/4096 |  |
|  |  |  | To transmit large volumes of message text |  |  |  |  |  |  |  |
|  |  |  | Message Text not to exceed 80 characters total. |  |  |  |  |  |  |  |

Segment:	SE Transaction Set Trailer

Position:	290

Loop:

Level:	Detail

Usage:	Mandatory

Max Use:	1

Purpose:	To indicate the end of the transaction set and provide the count of the transmitted segments (including the beginning (ST) and ending (SE) segments)

Syntax Notes:

Semantic Notes:

Comments:	1	SE is the last segment of each transaction set.

| Notes: |  | Required |
| --- | --- | --- |
|  |  | SE~13~000000001 |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | SE01 | 96 | Number of Included Segments | M |  | N0 1/10 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Total number of segments included in a transaction set including ST and SE segments |  |  |  |  |
| Must Use | SE02 | 329 | Transaction Set Control Number | M |  | AN 4/9 |  |
|  |  |  | Identifying control number that must be unique within the transaction set functional group assigned by the originator for a transaction set |  |  |  |  |

650_01 Example#1 of 5

## Reread with Call Ahead Request  - CR to TDSP

| CR submits Reread Request Includes Call Ahead Request |  |  |
| --- | --- | --- |
| ST~650~000000001 |  | Transaction Type, Control Number |
| BGN~13~200105031956531~20010531~~~~RD~IT |  | Request, Reference Number, Date, Transaction Type, Initial Request |
| N1~8R~CUSTOMER NAME |  | Customer Name |
|  | N2~D/B/A CUSTOMER CORP | Additional Name Information |
|  | N3~123 NORTH MAIN | Customer Service Address |
|  | N4~ANYTOWN~TX~78111 | Customer Service Address City, State, Zip Code |
|  | - PER~IC~DOE, JOHN~TE~8005551212 | - Contact Name, Phone Number |
| N1~8S~TDSP COMPANY~1~007909411~~40 |  | TDSP Name, DUNS Number, Receiver |
| N1~SJ~CR~9~007909422CRN1~~41 |  | CR Name, DUNS Number, Sender |
| HL~1~~EV~0 |  | Service Order Level Information |
|  | REF~8X~RD002 | Purpose Code |
|  | REF~MG~394820R | Meter Number |
|  | REF~PH~01 | Priority Code |
|  | REF~Q5~~10111111234567890ABCDEFGHIJKLMNOPQRS | ESI ID |
|  | REF~SU~N | Special Needs Indicator |
|  | DTM~211~20010601 | Service Request Date |
|  | YNQ~~Y~~~~~~9~CAL | Call Ahead |
| SE~18~000000001 |  | Number of Segments, Control Number |

650_01 Example#2 of 5

## Meter Test Request  - CR to TDSP

| CR submits Meter Test Request |  |  |
| --- | --- | --- |
| ST~650~000000001 |  | Transaction Type, Control Number |
| BGN~13~200105031956531~20010531~~~~38~IT |  | Request, Reference Number, Date, Transaction Type, Initial Request |
| N1~8R~CUSTOMER NAME |  | Customer Name |
|  | N3~123 NORTH MAIN | Customer Service Address |
|  | N4~ANYTOWN~TX~78111 | Customer Service Address City, State, Zip Code |
|  | - PER~IC~DOE, JOHN~TE~8005551212 | - Contact Name, Phone Number |
| N1~8S~TDSP COMPANY~1~007909411~~40 |  | TDSP Name, DUNS Number, Receiver |
| N1~SJ~CR~9~007909422CRN1~~41 |  | CR Name, DUNS Number, Sender |
| HL~1~~EV~0 |  | Service Order Level Information |
|  | REF~8X~MT001 | Purpose Code |
|  | REF~MG~394820R | Meter Number |
|  | REF~PH~02 | Priority Code |
|  | REF~Q5~~10111111234567890ABCDEFGHIJKLMNOPQRS | ESI ID |
|  | REF~SU~N | Special Needs Indicator |
|  | DTM~211~20010601 | Service Request Date |
| SE~16~000000001 |  | Number of Segments, Control Number |

650_01 Example#3 of 5

## Disconnect for Non-Pay Request  - CR to TDSP

| AREP submits Disconnect for Non-Pay request Includes Premium Disconnect Location |  |  |
| --- | --- | --- |
| ST~650~000000001 |  | Transaction Type, Control Number |
| BGN~13~200105031956531~20010531~~~~72~IT |  | Request, Reference Number, Date, Transaction Type, Initial Request |
| N1~8R~CUSTOMER NAME |  | Customer Name |
|  | N2~D/B/A CUSTOMER CORP | Additional Name Information |
|  | N3~123 NORTH MAIN | Customer Service Address |
|  | N4~ANYTOWN~TX~78111 | Customer Service Address City, State, Zip Code |
|  | - PER~IC~DOE, JOHN~TE~8005551212~TE~ - 8005551212 | - Contact Name, Phone Number, Additional Phone Number |
| N1~8S~TDSP COMPANY~1~007909411~~40 |  | TDSP Name, DUNS Number, Receiver |
| N1~SJ~AREP~9~007909422AREP~~41 |  | AREP Name, DUNS Number, Sender |
| HL~1~~EV~0 |  | Service Order Level Information |
|  | REF~8X~DC001 | Purpose Code |
|  | REF~PH~02 | Priority Code |
|  | REF~Q5~~10111111234567890ABCDEFGHIJKLMNOPQRS | ESI ID |
|  | REF~SU~N | Special Needs Indicator |
|  | DTM~211~20010601 | Service Request Date |
|  | YNQ~~Y~~~~~~9~PDL | Premium Disconnect Location |
| SE~17~000000001 |  | Number of Segments, Control Number |

650_01 Example#4 of 5

## Change/Update Reread Request  - CR to TDSP

| CR submits Change/Update Reread Request |  |  |
| --- | --- | --- |
| ST~650~000000001 |  | Transaction Type, Control Number |
| BGN~13~200106030958742~20010603~~~200105031956531~RD~2 |  | Request, Reference Number, Date, Transaction Type, Change Update Request |
| N1~8R~CUSTOMER NAME |  | Customer Name |
|  | N2~D/B/A CUSTOMER CORP | Additional Name Information |
|  | N3~123 NORTH MAIN | Customer Service Address |
|  | N4~ANYTOWN~TX~78111 | Customer Service Address City, State, Zip Code |
|  | - PER~IC~DOE, JOHN~TE~8005551212 | - Contact Name, Phone Number |
| N1~8S~TDSP COMPANY~1~007909411~~40 |  | TDSP Name, DUNS Number, Receiver |
| N1~SJ~CR~9~007909422CRN1~~41 |  | CR Name, DUNS Number, Sender |
| HL~1~~EV~0 |  | Service Order Level Information |
|  | REF~8X~RD002 | Purpose Code |
|  | REF~MG~394820R | Meter Number |
|  | REF~PH~01 | Priority Code |
|  | REF~Q5~~10111111234567890ABCDEFGHIJKLMNOPQRS | ESI ID |
|  | REF~SU~N | Special Needs Indicator |
|  | REF~TD~YNQCAL | Reason for Change |
|  | DTM~211~20010601 | Service Request Date |
|  | YNQ~~N~~~~~~9~CAL | Call Ahead |
| SE~19~000000001 |  | Number of Segments, Control Number |

650_01 Example#5 of 5

## Cancel Facilities Investigation Request  - CR to TDSP

| CR submits Cancel Facilities Investigation Request |  |  |
| --- | --- | --- |
| ST~650~000000001 |  | Transaction Type, Control Number |
| BGN~13~200106030958742~20010603~~~200105031956531~XZ~C |  | Request, Reference Number, Date, Transaction Type, Cancel Request |
| N1~8R~CUSTOMER NAME |  | Customer Name |
|  | N2~D/B/A CUSTOMER CORP | Additional Name Information |
|  | N3~123 NORTH MAIN | Customer Service Address |
|  | N4~ANYTOWN~TX~78111 | Customer Service Address City, State, Zip Code |
|  | - PER~IC~DOE, JOHN~TE~8005551212 | - Contact Name, Phone Number |
| N1~8S~TDSP COMPANY~1~007909411~~40 |  | TDSP Name, DUNS Number, Receiver |
| N1~SJ~CR~9~007909422CRN1~~41 |  | CR Name, DUNS Number, Sender |
| HL~1~~EV~0 |  | Service Order Level Information |
|  | REF~8X~FI003 | Purpose Code |
|  | REF~PH~01 | Priority Code |
|  | REF~Q5~~10111111234567890ABCDEFGHIJKLMNOPQRS | ESI ID |
|  | REF~SU~N | Special Needs Indicator |
| SE~15~000000001 |  | Number of Segments, Control Number |