# Major Market Component

ERCOT LMP Load Zones

# Purpose

The purpose of this paper is

- to describe the initial designation of Texas Nodal Market (TNM) Load Zones and the rules associated with the assignment of load busses to these zones;

- to describe the rules associated with the assignment of new transmission facilities to an existing zone; and,

- to describe the rules associated with the creation of new or deletion/realignment of existing load zones.

# Time Frame

The concepts described in this paper shall apply following the approval of an initial set of load zones for the Texas Nodal Market by the ERCOT Board of Directors and shall continue until revised in accordance with the Texas Nodal Market Protocols.

# Definitions

Load Buss – a buss at which real power is delivered to load connect to the buss.

Load Zone – a collection of designated load busses.  Every load buss in ERCOT must be assigned to a load zone for purposes of settling the Texas Nodal Market.  Each load zone is assigned a load zone LMP that is computed in each settlement interval as the load weighted average (based on State Estimator values) of the LMP at every buss assigned to the load zone.

# Description

## Initial Load Zone Assignments

The initial Load Zone designation for the Texas Nodal Market (TNM) recommended by the Texas Nodal Team to the ERCOT Board of Directors proposes to utilize the four (4) Competitively Significant Constraint (CSC) Zones that were in effect in the ERCOT Market during 2003.  Load busses within each 2003 CSC Zone, other than those busses that are owned by a Non Opt-In Entity (NOIE), will be assigned to the equivalent TNM designated Load Zone.

## Special Consideration For NOIEs

TBD

## Addition of New Load Busses

New load busses shall be assigned to a TNM Load Zone in accordance with the following rules:

- If the new load busses connect to busses all located in the same TNM Load Zone then assign the new load buses to that TNM Load Zone.

- If the new load busses connect to busses located in different TNM Load Zones then ERCOT shall simulate recent on-peak LMP markets with the new facility incorporated into the network model.  Each new buss shall be assigned to the TNM Load Zone with the closest matching Zonal LMP to the new buss's LMP.  In the event that this test produces no definitive answer, ERCOT shall expand its simulation to include a recent low load, off-peak, market and consider both simulations in making an assignment.  Expansion of simulated recent markets that are considered shall continue until an assignment is made.

## Load Zone Modifications

TNM Load Zones (including NOIE Load Zones) may be added/deleted, or otherwise reformed, only upon the approval of the ERCOT Board of Directors.  Such changes shall not become effective until after 36 months following the month in which the ERCOT Board renders its approval.  To the extent practicable, assignment of load busses in a newly approved TNM Load Zone structure shall comport with existing assignments.  For example, if an existing TNM Load Zone is unaffected by a change and continues to exist in the new Load Zone structure, buss assignments to such a Load Zone should also remain unchanged.  On the other hand, busses assignments within TNM Load Zones that change as a result of a newly approved structure shall be made consistent with the rules that apply in the case of new load buss installations.

# Inputs

NA

# Outputs

NA

# Settlement

NA

#### Example

NA