| PRR Number | 485PRR | PRR Title | Revision to Unit-Specific Deployment Based on Generic Cost |
| --- | --- | --- | --- |
| Recommended Action |  | Approve as revised |  |
| Protocol Section(s) Requiring Revision (include Section No. and Title) |  | 4.4.20, Posting of Resource Category Bid Price 6.7.1.2, Deployment of Balancing Energy when Congestion Occurs 7.4.3.1, Balancing Energy Up from a Specific Resource 7.4.3.2, Balancing Energy Down from a Specific Resource |  |
| Proposed Effective Date |  | Upon system implementation |  |
| Priority & Rank Assigned |  | To be Included in Release 4 requirements |  |
| Summary of Impact Analysis |  | ERCOT computer system impact with code changes in the Market Database, Lodestar, and the MOS to Lodestar interface; no staffing or business function impacts upon system implementation; however, a manual workaround (after a minor Lodestar project is complete) would require 0.75 FTE and have minor business function impacts; no grid operations impacts. |  |
| Revision Description |  | This revision includes RMR Units in the list of exceptions and would avoid the bids for certain types of units exceeding the $1,000/MW bid cap. |  |
| Benefit |  | WMS passed a motion to set unit-specific bid limits based on a modified generic cost structure. Prior to the implementation of this motion, in the interim, WMS voted to develop and process on an urgent timeline a PRR to set unit-specific bid prices based on generic cost plus zero percent (0%). This revision would avoid the bids for certain types of units exceeding the $1,000/MW bid cap and includes RMR Units in the list of exceptions and also address the methods for handling non-bid Resources. |  |
| PRS Recommendation |  | PRS recommends approval as revised by the WMS & ERCOT comments dated 03/25/04. |  |

| Sponsor |  |
| --- | --- |
| Name | Lori Johnson |
| E-mail Address | Ljohnson@ercot.com |
| Company | ERCOT |
| Company Address | 2705 West Lake Drive Taylor, TX |
| Phone Number | (512) 248-3956 |
| Fax Number | (512) 248-6560 |

ERCOT/Market Segment Impacts and Benefits

Instructions:  To allow for comprehensive PRR consideration, please fill out each block below completely, even if your response is “none,” “not known,” or “not applicable.”  Wherever possible, please include reasons, explanations, and cost/benefit analyses pertaining to the PRR.

|  | Impact |  | Benefit |
| --- | --- | --- | --- |
|  | Business | Computer Systems |  |
| ERCOT |  | Modifications to automate bid limit posting, bid limit validation, and calculation of payment based on bid or generic cost for non-bid units | Consistency between Resource deployment and Resource payment |
| MARKET SEGMENT |  |  |  |
| Consumer |  |  | Potential reduction in Uplift cost as compared to Market Solution approach |
| LSE: General, Including NOIE |  |  | Potential reduction in Uplift cost as compared to Market Solution approach |
| LSE: CR & REP |  |  | Potential reduction in Uplift cost as compared to Market Solution approach |
| QSE |  | Modify Resource Plan submission process to use published bid limits | Consistency between Resource deployment and Resource payment Bid limits provide market mechanism |
| Resource |  |  | Bid limits provide market mechanism |
| TDSP | None | None | None |

Proposed Protocol Language Revision

{Protocol language below includes WMS and ERCOT Comments dated 032504, ANP Comments dated 021304, and Comments from CEDI Task Force and ERCOT dated 031704.}

# 4 Scheduling

## 4.4 Day Ahead Scheduling Process

    - Posting of Resource Category Bid Price

Two Business Days prior to the end of the month, ERCOT will calculate and post the Resource Category Generic Price for Balancing Energy Up deployments (RCGUP) and the Resource Category Generic Price for Balancing Energy Down deployments (RCGDP) by Resource Category for QSEs to use as the Resource-specific bid premiums submitted in their Resource Plans for the following month.  ERCOT will calculate the RCGUP and RCGDP based on the Resource Category Generic Fuel Cost specified in Section 6.8.2.1, Resource Category Generic Costs, item (3), using the average of the FIP from the previous month.

- In order to eliminate overlap between the RCGUP and RCGDP, except as provided in subsections (2) and (3), below, the RCGUP will be adjusted  as follows by adding an UPSHIFT value to the RCGUP calculation:

RCGUPc	=	RCGUPBASE,c + UPSHIFT

UPSHIFT	=	max(RCGDP)c – min(RCGUPBASE)c + 0.01

Where:

c			Resource Category

RCGDP		Resource Category Generic Down Price following Resource Category Generic Fuel Cost calculation

RCGUP		Resource Category Generic Up Price

RCGUPBASE		Base Resource Category Generic Up Price following Resource Category Generic Fuel Cost calculation

UPSHIFT		The additional amount necessary to shift RCGUP above RCGDP

- The following Resource types are not eligible for an UPSHIFT adjustment.  The BES Up and BES Down prices for these Resource types will be set as follows:

| Resource Type | RCGUP | RCGDN |
| --- | --- | --- |
| Nuclear | $996 | - $996 |
| Hydro | $997 | - $997 |
| Qualifying Facilities | $998 | - $998 |
| Renewable | $999 | $0 |
| Load Acting as a Resource | $1,000 | - $1,000 |

- RMR Resources and Generation Resources scheduled for testing are not eligible for an UPSHIFT adjustment.  QSEs scheduling for RMR Resources and Generation Resources scheduled for testing must use the highest calculated RCGUP and the lowest calculated RCGDP for use in the submitted Resource Plan for such Resources.

- In order to avoid oscillation between Resources with the same cost category at the same plant, a QSE may alternatively add and subtract from the posted RCGUP and RCGDP in increments of $0.01 for use in the submitted Resource Plan for each Resource, such that the following holds true:

SUM (IBPu – RCGUPu)p = 0

SUM (DBPu – RCGDPu)p = 0

Where:

p		Plant

u		Unit at Plant p

DBPu	Submitted Resource-specific decremental bid premium in Resource Plan for unit, u

IBPu	Submitted Resource specific incremental bid premium in Resource Plan for unit, u

RCGDPu	Resource Category Generic Down Price for unit, u

RCGUPu	Resource Category Generic Up Price for unit, u

[Replace Section 4.4.20 with the following upon system implementation:] 4.4.20 Publication of Resource Category Bid Limits ERCOT will calculate and publish the Resource Category Generic bid limit for Balancing Energy Up and the Resource Category Generic bid limit for Balancing Energy Down for each Resource Category on each day for which the FIP is published, pursuant to Section 6.8.2.1, Resource Category Generic Cost, item (3). - The Resource Category Generic bid limits for Balancing Energy Up for all gas-fired units will be calculated by multiplying the most recent FIP by the sum of a constant heat rate adder and the Resource Category heat rate, as used in the Resource Category Generic Fuel Cost calculation for upward instructions specified in Section 6.8.2.1, Resource Category Generic Cost. - The Resource Category Generic bid limits for Balancing Energy Down for all gas-fired units will be calculated by multiplying the most recent FIP by the difference of a constant heat rate adder and the Resource Category heat rate, as used in the Resource Category Generic Fuel Cost calculation for downward instructions specified in Section 6.8.2.1, Resource Category Generic Cost. - For all other Resource Categories, the Resource Category Generic bid limit for Balancing Energy Up and the Resource Category Generic bid limit for Balancing Energy Down will be calculated using the appropriate Resource Category Generic Fuel Cost calculation using the most recent FIP, if necessary, as specified in Section 6.8.2.1, Resource Category Generic Cost. - The proposed value of the heat rate adder will be recommended by the appropriate TAC subcommittee and will be re-evaluated on a quarterly basis. The value of the heat rate adder must be approved by the ERCOT Board.

#### Deployment of Balancing Energy when Congestion Occurs

- If the Operational Model indicates there is Zonal Congestion, ERCOT will separate the Balancing Energy Service bids into a Bid Stack for each Congestion Zone.

- ERCOT will use the Operational Model to determine the amount and location of Balancing Energy deployment for clearing Zonal Congestion as well as balancing the system.

- Except as stated in item (4) below, ERCOT will deploy Balancing Energy bids within a zone in bid price Merit Order.

- ERCOT may form specific Resource prices for both incrementing and decrementing a specific Resource to resolve Local Congestion.

- As part of the submittal of the Resource Plan for each Resource at a plant, QSEs may specify bid premiums by Resource.  For each Resource, a submitted bid premium for Balancing Energy Up must be less than or equal to the Resource Category Generic bid limit for Balancing Energy Up deployments; the submitted bid premium for Balancing Energy Down must be greater than or equal to the Resource Category Generic bid limit for Balancing Energy Down deployments calculated pursuant to Section 4.4.20, Publication of Resource Category Bid Limits.  With the exception of eligible non-bid Resources [as defined in item (6) below], if a bid premium is not specified in the Resource Plan, ERCOT will set the incremental bid premium equal to the Resource Category Generic bid limit for Balancing Energy Up and the decremental bid premium equal to the Resource Category Generic bid limit for Balancing Energy Down.  Resource-specific incremental prices will be formed by ERCOT by adding the incremental bid premium as specified in Section 4.4.20, Posting of Resource Category Bid Limits, to the MCPE of the Congestion Zone.  The Resource-specific premium price will not be less than the bid premium.  Resource-specific decremental prices will be the decremental premium specified in Section 4.4.20, Posting of Resource Category Bid Limits.

- A QSE may specify within the Resource Plan that a Resource is a “non-bid” Resource for the following Resource Categories:  Nuclear, Hydro, and Renewable.  In addition, a QSE may also specify Qualifying Facilities, and Loads Acting as a Resource as non-bid Resources within the Resource Plan.  A QSE must specify an RMR unit as a non-bid Resource.  A non-bid Resource would only be deployed by the system as necessary to maintain reliability.  To accomplish this, the system will set an incremental price for the non-bid Resources such that the pricing order from lowest to highest for deployment of Balancing Energy Up among the non-bid Resources will be as follows:  RMR, Qualifying Facility, Hydro, Renewable, Nuclear, Load Acting as a Resource.  The system will set a decremental price for the non-bid Resources such that the pricing order from lowest to highest for deployment of Balancing Energy Down among the non-bid Resources will be as follows:  Nuclear, Renewable, Qualifying Facility, RMR, Hydro.  If a Resource is specified as a non-bid Resource, then ERCOT will use OOME to determine the amount paid to the QSE in connection with such Resources dispatched to resolve the constraint.

- The actual Shift Factors with respect to the Local Congestion of Resources’ individual incremental and decremental prices from above are used to determine the most economical deployment of individual Resources to solve Local Congestion.

- ERCOT will instruct QSEs to deploy Balancing Energy Service from a specific Resource through the issuance of a Dispatch Instruction to each Resource according to the most economical solution to resolve the Local Congestion.  The deployment of Resource specific Balancing Energy Service must be in equal incremental and decremental amounts.  The decremental amount will be selected such that it does not adversely affect any other constraint.

- The Dispatch Instruction will specify the current output level, the amount of Balancing Energy Service, and the range of acceptable operation of the specific Resource.

- For an incremental Resource-specific instruction, the QSE will be paid the incremental premium specified, times the difference in megawatts between the Resource’s current output level and the minimum of the allowed range specified in the Dispatch Instruction at the time of receipt of the Dispatch Instruction for all hours or portions of an hour the Resource is individually dispatched.

- For a decremental unit-specific Dispatch Instruction, the QSE will be paid the product of:  1) the difference in MCPE minus the decremental premium specified, times 2) the Instruction for all hours or portions of an hour the Resource is individually dispatched.

- QSEs shall first meet the specific Resource deployment performance requirements of Section 6.10.7, Individual Resource Dispatch Performance and then provide the Balancing Energy Service deployment instructed pursuant to Section 6.7.1, Deployment of Balancing Energy Service.  In the event that a QSE is unable to provide the Balancing Energy Service due to a specific Resource deployment then the QSE will follow the notification procedures established in Section 5, Dispatch.

[PRR359 & PRR424: Replace Section 6.7.1.2 (5)-(12) with Section 6.7.1.2 (5)-(13) as follows when the system change is implemented.]

- As part of the submittal of the Resource Plan for each Resource at a plant, QSEs may specify bid premiums by Resource.  For each Resource, a submitted bid premium for Balancing Energy Up must be less than or equal to the Resource Category Generic Bid Limit for Balancing Energy Up deployments; the submitted bid premium for Balancing Energy Down must be greater than or equal to the Resource Category Generic Bid Limit for Balancing Energy Down deployments calculated pursuant to Section 4.4.20, Publication of Resource Category Bid Limits.  With the exception of eligible non-bid Resources [as defined in item (6) below], if a bid premium is not specified in the Resource Plan, ERCOT will set the incremental bid premium equal to the Resource Category Generic bid limit for Balancing Energy Up and the decremental bid premium equal to the Resource Category Generic bid limit for Balancing Energy Down.  Resource-specific incremental prices will be the incremental bid premium specified by the QSE based on the Resource Category Generic Price for Balancing Energy Up and the Resource-specific decremental prices will be the decremental premium specified by the QSE based on the Resource Category Generic Price for Balancing Energy Down.

- A QSE may specify within the Resource Plan that a Resource is a “non-bid Resource” for the following Resource Categories:  Nuclear, Hydro, and Renewable.  In addition, a QSE may also specify Qualifying Facilities, and Loads Acting as a Resource as non-bid Resources within the Resource Plan.  A QSE must specify an RMR unit as a non-bid Resource. A non-bid Resource would only be deployed by the system as necessary to maintain reliability.  To accomplish this, the system will set an incremental price for the non-bid Resources such that the pricing order from lowest to highest for deployment of Balancing Energy Up among the non-bid Resources will be as follows:  RMR, Qualifying Facility, Hydro, Renewable, Nuclear, Load Acting as a Resource.  The system will set a decremental price for the non-bid Resources such that the pricing order from lowest to highest for deployment of Balancing Energy Down among the non-bid Resources will be as follows:  Nuclear, Renewable, Qualifying Facility, RMR, Hydro.

- The actual Shift Factors with respect to the Local Congestion of Resources’ individual incremental and decremental prices from above are used to determine the most economical deployment of individual Resources to solve Local Congestion.

- ERCOT will instruct QSEs to deploy Balancing Energy Service from a specific Resource through the issuance of a Dispatch Instruction for each Resource using the most economical solution to resolve the Local Congestion.

- The Dispatch Instruction will specify the instructed output level, the amount of Balancing Energy Service, and the range of acceptable operation of the specific Resource.

- For an incremental Resource-specific instruction, the QSE will be paid in accordance with Section 7.4.3.1, Balancing Energy Up from a Specific Resource.

- For a decremental unit-specific Dispatch Instruction, the QSE will be paid in accordance with Section 7.4.3.2, Balancing Energy Down from a Specific Resource.

- QSEs shall first meet the specific Resource deployment performance requirements of Section 6.10.7, Individual Resource Dispatch Performance and then provide the Balancing Energy Service deployment instructed pursuant to Section 6.7.1, Deployment of Balancing Energy Service.  In the event that a QSE is unable to provide the Balancing Energy Service due to a specific Resource deployment then the QSE will follow the notification procedures established in Section 5, Dispatch.

- If a Resource is specified as a non-bid Resource, then ERCOT will use OOME to determine the amount paid to the QSE in connection with the Resources dispatched to resolve the constraint

7.4.3.1 Balancing Energy Up from a Specific Resource

In accordance with Section 6.7.1.2, Deployment of Balancing Energy when Congestion Occurs, whenever an incremental specific Resource with a submitted bid premium, in accordance with the Resource Category Generic bid limit for Balancing Energy Up, is deployed to solve Local Congestion, the QSE will be paid the incremental premium specified.  Aggregated Unit rules as described in Section 6.8.2, Capacity and Energy Payments for Out of Merit Service, apply to Aggregated Units approved by ERCOT.

LPCRSUquiz 	= 	-1 * Σ ((PMqiuz – MCPEiz )* Max (0, Min (MRqiuz -OLquiz, IOLqiuz - OLqiuz)))

LPCRSUqviz	=	 -1 * Σ ((PMqivz – MCPEiz )* Max (0, Min (MRqivz -OLqviz, NETUEQivq)) * LBEAGRivq)

PMqiuz	=	Max (BPMqiuz, BPMqiuz + MCPEiz )

PMqivz	=	Max (BPMqivz, BPMqivz + MCPEiz )

[PRR424:  Replace the formulas above for PMqiuz and PMqivz with the following upon system implementation:]

PMqiuz	=	Max (BPMqiuz, MCPEiz )

PMqivz	=	Max (BPMqivz, MCPEiz )

BPMqivz 	= 	Min (BPMqiuz)

NETUEQivq			=	Max [0, ((NETOOMUEQivq + NETLBEUQivq) –(NETOOMDEQivq + NETLBEDQivq))]

NETOOMUEQivq	=	Max(0, (SUM(IOOMUPqui)u – SUM(IOOMDNqui)u))

NETLBEUQivq 	=	Max (0, (SUM(LBEUQqui)u – SUM(LBEDQqui)u))

NETOOMDEQivq 	=	Max(0, (SUM(IOOMDNqui)u – SUM(IOOMUPqui)u))

NETLBEDQivq 	=	Max(0, (SUM(LBEDQqui)u – SUM(LBEUQqui)u))

LBEAGRivq 		=	[(SUM(LBEUQqui)u + SUM(LBEDQqui)u)] /

[SUM(LBEUQqui)u + SUM(LBEDQqui)u +

SUM(IOOMUPqui)u + SUM(IOOMDNqui)u]

Where:

i	interval

q	QSE

u	individual generation unit (including units within an Aggregated Unit)

v	Aggregated Unit

z	zone

LPCRSUquiz	Payment to QSE for providing Balancing Energy Up from Specific unit to solve Local Congestion per QSE per interval per zone.

LPCRSUqviz	Payment to QSE for providing Balancing Energy Up from Aggregated Units to solve Local Congestion per QSE per interval per zone.

PMqiuz	Incremental Premium Specified for that specific unit per interval.

PMqivz	Incremental Premium Specified for that specific Aggregated Unit per interval.

BPMqiuz	Incremental bid premium for that specific unit per interval.

BPMqivz	Incremental bid premium for that specific Aggregated Unit per interval.

OLquiz	Resource Plan output level submitted by the QSE for the specific unit given the unit-specific instruction for that interval.

OLqviz	Resource Plan output level submitted by the QSE for the Aggregated Unit containing the unit given the unit-specific instruction for that interval.

IOLquiz	The Instructed output level for the unit given the unit-specific instruction.

IOLqviz	The Instructed output level for the Aggregated Unit containing the unit given a unit-specific instruction.

IOOMUPiuq	OOM Energy Up Instructions for that interval for that unit converted to MWh for that interval by dividing by the interval count per hour (4 per hour for 15 minute Settlement Interval)

IOOMDNiuq	OOM Energy Down Instructions for that interval for that unit converted to MWh for that interval by dividing by the interval count per hour (4 per hour for 15 minute Settlement Interval)

MRqiuz	Meter reading per unit per interval.

MRqivz	Meter reading per Aggregated Unit per interval.

MCPEiz	Market Clearing Price for Energy per interval per zone.

LBEUQqui	Local Balancing Energy Up Instructions for that interval per specific unit

LBEDQqui	Local Balancing Energy Down Instructions for that interval per specific unit

NETOOMUEQivq	The net of all unit specific OOM instructions (Up minus Down) that result in a positive value.

NETLBEUQivq	The net of all unit specific Local Balancing Energy Instructions (Up minus Down) that result in a positive value.

NETOOMDEQivq	The net of all unit specific OOM instructions (Down minus Up) that result in a positive value.

NETLBEDQivq	The net of all unit specific Local Balancing Energy Instructions (Down minus Up) that result in a positive value.

NETUEQivq	The overall net direction per the instructions given to the Aggregated Unit results in an up direction

LBEAGRiv	The percentage of the aggregated instructions to the Aggregated Unit that represents the OOM Energy market.

7.4.3.2 Balancing Energy Down from a Specific Resource

In accordance with Section 6.7.1.2, whenever a decremental specific Resource with a submitted bid premium, in accordance with the Resource Category Generic bid limit for Balancing Energy Down , is deployed to resolve the Local Congestion, the QSE will be paid the difference of MCPE in the commercial zone, which the specific Resource is in, and the decremental bid premium specified.

LPCRSDquiz 	= 	-1 * Σ(MAX(0,(MCPEiz - BPMqiuz )) * Max(0, Min(OLqiuz -MRquiz OLqiuz - IOLqiuz)))u

LPCRSDqviz 	=	-1 * Σ(MAX(0,(MCPEiz - BPMqivz )) * Max(0, Min(OLqivz -MRqviz  NETDEQiv)))v * LBEAGiv

BPMqivz = Max (BPMqiuz)

NETDEQivq   =	Max [0, ((NETOOMDEQivq + NETLBEDQivq) – (NETOOMUEQivq + NETLBEUQivq))]

NETOOMUEQivq 	=		Max [0, (SUM (IOOMUPqui)u – SUM(IOOMDNqui)u)]

NETLBEUQivq 	=		Max [0, (SUM(LBEUQqui)u – SUM(LBEDQqui)u)]

NETOOMDEQivq	=		Max [0, (SUM(IOOMDNqui)u – SUM(IOOMUPqui)u)]

NETLBEDEQivq 	=	Max[0, (SUM(LBEDQqui)u – SUM(LBEUQqui)u)]

LBEAGRivq	=	[SUM(LBEUQqui)u + SUM(LBEDQqui)u] /

[SUM(LBEUQqui)u + SUM(LBEDQqui)u +

SUM(IOOMUPqui)u + SUM(IOOMDNqui)u]

Where:

i	interval

q	QSE

u	individual generation unit (including units within an Aggregated Unit)

v	Aggregated Unit

z	zone

LPCRSDqiz	Payment to QSE for providing Balancing Energy Down from specific unit to solve Local Congestion per QSE per interval per zone

BPMqiuz	Decremental bid premium specified for that specific unit per interval

BPMqivz	Decremental bid premium specified for that specific Aggregated Unit per interval

OLquiz	Resource Plan output level submitted by the QSE for the unit given the unit-specific instruction for that interval

OLqviz	Resource Plan output level submitted by the QSE for the Aggregated Unit containing the unit given the unit-specific instruction for that interval

IOLqui	The instructed output level for the unit given a unit-specific instruction

IOOMDNiuq	OOM Energy Down instructions for that interval for that unit converted to MWh for that interval by dividing by the interval count per hour (4 per hour for 15 minute Settlement Interval)

IOOMUPiuq	OOM Energy Up instructions for that interval for that unit converted to MWh for that interval by dividing by the interval count per hour (4 per hour for 15 minute Settlement Interval)

MRqiuz	Meter reading per unit per interval.

MRqivz	Meter reading per Aggregated Unit per interval.

MCPEiz	Market Clearing Price of Energy per interval per zone

LBEUQqui	Local Balancing Energy Up instructions for that interval per specific unit

LBEDQqui	Local Balancing Energy Down instructions for that interval per specific unit

NETOOMUEQivq The net of all unit specific OOM instructions (Up minus Down) that result in a positive value.

NETLBEUQivqThe net of all unit specific Local Balancing Energy instructions (Up minus Down) that result in a positive value.

NETOOMDEQivq The net of all unit specific OOM instructions (Down minus Up) that result in a positive value.

NETLBEDQivq The net of all unit specific Local Balancing Energy instructions (Down minus Up) that result in a positive value.

NETDEQivq	The overall net direction per the instructions given to the Aggregated Unit results in a down direction

LBEAGRiv	The percentage of the aggregated instructions to the Aggregate Unit that represents the OOM Energy market.

| PRR Evaluation |  |
| --- | --- |
| Non-ERCOT Market Comparison | No Comparison conducted. |
| Comments Author | Comments Summary |
| ERCOT | Revised language. |
| ANP, Inc. | Author’s intent is to make a better alternative to the current Market Solution process per the PUCT’s and the ERCOT Board’s requests on the comments from PRR 440, Interim Removal of Market Solution. Makes modifications to the following Sections: 4.4.20 Publication of Resource Category Generic Bid Limits; 6.7.1.2 Deployment of Balancing Energy when Congestion Occurs; 7.4.3.1 Balancing Energy Up from a Specific Resource, and 7.4.3.2 Balancing Energy Down from a Specific Resource. |
| ERCOT & CEDI Task Force | Revised Language. |
| WMS and ERCOT | Revised Language. |

| Sponsor's Revision Description | This revision includes RMR Units in the list of exceptions and would avoid the bids for certain types of units exceeding the $1,000/MW bid cap. |
| --- | --- |
| Sponsor's Reason for Revision | WMS passed a motion to set unit-specific bid limits based on a modified generic cost structure. Prior to the implementation of this motion, in the interim, WMS voted to develop and process on an urgent timeline a PRR to set unit-specific bid prices based on generic cost plus zero percent (0%). This revision would avoid the bids for certain types of units exceeding the $1,000/MW bid cap and includes RMR Units in the list of exceptions. |