# Major Market Component

Day-Ahead Reliability Unit Commitment (DaRUC)

# Purpose

A Day-Ahead Reliability Unit Commitment (DaRUC) will be used by ERCOT to maintain system reliability and security by committing generation as necessary beyond that committed by the bilateral, Ancillary Services and Day-Ahead energy markets.  DaRUC will use the Security Constrained Unit Commitment (SCUC) algorithm with a power flow model to enforce transmission constraints.  DaRUC is intended to solve day-ahead unit reliability issues and not to minimize prices of balancing energy in real time.

The fundamental characteristics of the DaRUC process in the ERCOT market are:

- RUC will be run by the Independent System Operator (i.e., ERCOT),

- Self Committed Resources for Bilateral Schedules, Day-Ahead Schedules, and Ancillary Services will be supported,

- ERCOT forecasted load will be considered in the optimized solution,

- The Security-Constrained Unit Commitment technique will be adopted,

- It will be put in place to ensure enough online generation capacity to meet the reliability need and to allow secure real-time operation.

- The process will use two bid components and minimize expected cost subject to security constraints.  Cost components considered will include: Startup Cost and Minimum-energy operating cost.  Energy cost above minimum energy for all units will not be considered.

# Time Frame

RUC will be run after the close of the Day-Ahead Period, which includes the clearing of the Ancillary Services market.  Other reliability studies may be executed for other time periods and are addressed in specific in order to capture the differences in requirements.  It will consider hourly loads and unit commitments to optimize the entire 24 hour operating day.

# Applications/Process Description

The DaRUC model will use a power flow model that includes physical topology of the transmission system, buses and transmission constraints within the transmission grid, and an electrically equivalent representation of the DC ties. The RUC optimization routine will be used to minimize the cost of committing additional resources to meet forecasted demand subject to all transmission constraints and generator performance characteristics. DaRUC will resolve system energy balance and transmission congestion simultaneously. DaRUC process will ensure that generation patterns and load profiles are operationally feasible with respect to all transmission constraints, as well as with generator ramping and other performance constraints. This modeling is consistent with that described in the Fidelity Requirements for Transmission Element Modeling Document.

DaRUC will use a Security Constrained Unit Commitment (SCUC) technique. Market participants may bid into the RUC market since it is primarily used to commit additional Resources to resolve predicted operational security issue that will be faced by Real Time Operations. Market Participants submit offers to sell in the DaRUC Market. RUC will allow ERCOT to commit additional resources beyond those scheduled as needed to meet ERCOT’s system load forecast (plus ancillary services) and decommit resources, qualifying facilities (QFs) may be considered last, if no other method of maintaining transmission security is available.  Commit and decommit decisions will be sent automatically in the day-ahead with all decommit decisions confirmed in real time.  Transmission constraints and system load obligations are satisfied by the SCUC.   If ERCOT decommits units it will provide notification to the market monitor.  If ERCOT decommits a unit earlier than its scheduled shutdown within the DaRUC 24-hour period, then no compensation is due to the impacted QSE from ERCOT.  However, if ERCOT decommits a unit that it not scheduled to shutdown within the DaRUC 24-hour period, then ERCOT shall pay the impacted QSE the following sum, if positive: (a) the unit’s Start-up Cost and (b) if negative the difference between the product of LSL times the LMP at the unit’s node minus the units Minimum-energy Cost for the hours starting with the decommitment interval until the interval ERCOT determines that the unit may again be at LSL.    Any decommitment cost is allocated to all QSEs on a load ratio share basis.

# Inputs

ERCOT System-wide hourly Load Forecast allocated appropriately over load buses

Planned unit outages

Expected unit status at midnight and number of hours in that status

Market Participants’ Self Committed Resources

- Transmission constraints – Transfer limits on energy flows through the electricity network; e.g. thermal or stability limits. Ratings must be enforced for both a predicted network and single contingency outages. Binding transmission constraints indicate the existence of transmission congestion and it will cause nodal price separation. These constraints can represent:

  - Thermal constraints – protect transmission facilities against thermal overload.

  - Generic constraints – protect the transmission system against transient instability, dynamic stability or voltage collapse.

- Planned transmission topology

- Energy balancing constraints – the energy balance at each node in the electricity system must be served.

- Resource Constraints – the physical and security limits on generation or capacity requirements.

  - Resource output constraints – the minimum and maximum sustained output levels of each unit.

  - Resource operational constraints – includes minimum run time, minimum down time and ramp rate constraints.

  - Startup time – ERCOT will use a generic formula, with market participant provided constants, which is a function of hours offline and used by all market participants to model minimum startup time.  Market Participants may manually override the value; this may trigger market monitor action.

  - Mutually available units—limitations on simultaneous unit starts at a station (an alternative to software is to allow ERCOT operations to make manual decisions).

- Ancillary Service constraints

Factors included in the SCUC are:

- Generating unit operating status

- Combined Cycle Operating Configuration -

- Constraints on the minimum up and down time of the generators

- Offer prices for the Generation minimum energy and start up

- Plant-related startup and shutdown constraints

- Minimum and maximum generation constraints

- Ancillary services obligations

- Maintenance and derating schedules

- Availability including Environmental and Fuel Constraints

# Outputs

DaRUC produces the generating unit commitment profile for next day’s operation.  The DaRUC must have capability to modify the commitments to provide for ERCOT operator review of the feasibility of DaRUC outputs.  ERCOT operators may choose to use a DaRUC commitment recommendation or choose to use their judgment to make commitment decisions to maintain reliability or limit unreasonable commitment cost without merit.

ERCOT may also use the look ahead “8-Day” planning tool output and potentially keep a unit online (or start a unit for the next day) so that a unit “minimum duration between starts” does not limit the availability of the unit (for security reasons).

Any manual commitment input to DaRUC will automatically be posted to the MIS giving operator reason.

If ERCOT commits a unit earlier than its self-scheduled commitment start hour or keeps a unit committed beyond its self-scheduled shutdown hour, then the impacted QSE has the option of claiming a make whole payment only for the extra hours committed by ERCOT.  The make whole payment calculation in such instances shall be calculated based on the difference between the unit’s Minimum-energy Cost minus the energy revenues calculated as below.  Such make whole payment will be allocated in a similar manner for those hours as any unit committed through the DaRUC process.

# Definitions

DaRUC: Day-Ahead Reliability Unit Commitment

Committed Capacity Nomination – that amount of self-committed capacity attributed to a QSE in the Scheduling Process that represents the QSEs contribution to serving its capacity obligation.

Start-up Cost (in $/Start) component is provided to be able to reflect all the costs incurred by the resource in starting up and reaching breaker close.

The Minimum-energy Cost (in $/MWh) component is provided to be able to reflect costs incurred by the resource in producing Minimum-energy output after the Start-up Cost is incurred. Minimum-energy Cost is the product of the fuel consumption at Low Sustainable Limit multiplied by the Fuel Index Price divided by the Low Sustainable Limit.

Low Sustainable Limit is the minimum net capability of a Resource that may be delivered for an indefinite period.

# Detailed Description of DaRUC Process

Prior to the DaRUC, QSEs must submit any financially binding energy schedules from the Day-Ahead Market and any AS schedules for AS awarded/purchased in the AS market, self-provided AS, and AS purchased/sold bilaterally.  QSEs may also submit additional bilateral energy schedules and committed capacity nominations (self-provided, bought or sold).  Committed capacity nominations can only be made from unloaded resources capacity (having no energy or AS schedules) that is capable of providing BES.  These inputs are not explicitly used in the SCUC (except possibly resource-specific AS provision) but are used to settle DaRUC charges.  Upon implementation of a day-ahead energy market which has a unit commitment for resources with three part bids the units committed in the day-ahead market will be considered as self-committed in the DaRUC.

QSEs may submit two-part bids for use in the DaRUC process.  The SCUC of the DaRUC will first be run with only those resources that submit offers.  If additional resources are required, the SCUC is run again with those resources selected in the first run treated the same as self-committed resources and available off-line resources that did not submit bids having their bids set at their respective Start-up Cost and Minimum-energy Cost offer caps specified below. SCUC will use two bid components (Start-up Cost and Minimum-energy Cost) and minimize expected cost subject to security constraints.  If a resource is self-committed to be on-line at the start of the DaRUC 24-hour period and scheduled to go off-line at some time during the 24-hour period, ERCOT may decide to keep the resource on-line and not incur the Start-up Cost but rather incur the Minimum-energy Cost from the hour that the resource was scheduled to go off-line till the hour the resource is no longer required by ERCOT to be on-line. The Start-up Cost and Minimum-energy Cost offers are capped at the following levels:

Startup Cost offer caps by resource category are for O&M expenses per start in addition to verifiable fuel times FIP.  O&M expenses by resource category are as follows:

Combined Cycle greater than 90 MW** = $6,810

Combined Cycle less than or equal to 90 MW** = $5,310

Gas Steam Supercritical Boiler = $4,800

Gas Steam Reheat Boiler = $3,000

Gas Steam Non-reheat or boiler w/o air-preheater = $2,310

Simple Cycle greater than 90 MW = $5,000

Simple Cycle less than or equal to 90 MW = $2,300

Renewable = 0

Unit specific verifiable fuel consumption rate per start will be established up front, and O&M expenses may be established in place of these proxies by providing verifiable data.  Regardless of the method selected QSEs retain the right to submit verifiable costs on any start.  The Market Monitor may require that the QSE provide justification of the verifiable costs.  Any resource called upon and paid using these proxy costs will submit “actual” costs for cold , warm and Hot starts,  by season if necessary,  within 30 days after the 1st such start.  This set of costs will be used for all future payments for this unit subject to approval by the market monitor.  In such cases, this new set of proxy costs will be defined for future use in settlements. .

**Determined by capacity of largest simple-cycle combustion turbine in the train.

Where:

FIP = Fuel Index Price defined as the Midpoint price, expressed in $/MMBTU, published in Gas Daily, in the Daily Price Survey, under the heading “East-Houston-Katy, Houston Ship Channel” for the day

RMC = Resource Maximum Capacity (in MW)

u = unit

Resource Minimum-energy Cost is the heat rate of a unit, in one of these categories, at its Low Sustainable Limit multiplied by the Fuel Index Price, and is capped at the following based on the resource category:

Combined Cycle greater than 90 MW** = 10 MMBtu/MWh * FIP

Combined Cycle less than or equal to 90 MW** = 10 MMBtu/MWh * FIP

Gas Steam Supercritical Boiler = 16.5 MMBtu/MWh * FIP

Gas Steam Reheat Boiler = 17.0 MMBtu/MWh * FIP

Gas Steam Non-reheat or boiler without air-preheater = 19.0 MMBtu/MWh * FIP

Simple Cycle greater than 90 MW = 15.0 MMBtu/MWh * FIP

Simple Cycle less than or equal to 90 MW = 15.0 MMBtu/MWh * FIP

Where:

FIP = 	Fuel Index Price defined as the Midpoint price, expressed in $/MMBTU, published in Gas Daily, in the Daily Price Survey, under the heading “East-Houston-Katy, Houston Ship Channel” for the day

The SCUC will treat all resource capacity providing AS as unavailable, unless such treatment leads to infeasibility (i.e. that capacity is needed to resolve some local transmission problem that cannot be resolved by any other means).  In such cases, ERCOT shall inform the impacted QSE(s) of the amount of its (their) capacity that does not qualify to provide AS.  In such event and within two hours of notification, the QSE(s) may either substitute capacity to meet its (their) AS obligations or ask ERCOT to replace the capacity.  For substituted capacity, ERCOT shall determine the feasibility of the capacity for providing AS.  ERCOT will not pay for ancillary services, which cannot be provided if ERCOT provided such notification in the process. ERCOT may then procure additional AS for the amount of substituted capacity that is infeasible and the amount of capacity that QSE(s) want ERCOT to replace and appropriately charge all impacted QSEs. The appropriate charge is the weighted average of the market clearing prices of the particular ancillary services procured by ERCOT in the multiple procurement processes.  [All buyers will pay the weighted average price of the two markets]

Conceptually

- Unit offering/providing AS does not work due to congestion

- ERCOT tells provider it cannot work

- Provider has option to provide elsewhere or buy it from ERCOT

  - If original AS sold to ERCOT – NO PAYMENT

  - If self-provided – NO CREDIT

- New Auction – New Clearing price

- All who buy from ERCOT this AS pay the weighted average of the two auctions.

Solely for purposes of make whole provisions, the BES bid curves of resources are capped as follows:

Nuclear = $15.00/MWh

Hydro = $10.00/MWh

Coal and Lignite = $18.00/MWh

Combined Cycle greater than 90 MW** = FIP * 9 MMBtu/MWh

Combined Cycle less than or equal to 90 MW** = FIP * 10 MMBtu/MWh

Gas-Steam Supercritical Boiler = FIP * 10.5 MMBtu/MWh

Gas Steam Reheat Boiler = FIP * 11.5 MMBtu/MWh

Gas Steam Non-reheat or boiler without air-preheater = FIP * 14.5	MMBtu/MWh

Simple Cycle greater than 90 MW = FIP * 14 MMBtu/MWh

Simple Cycle less than or equal to 90 MW = FIP * 15 MMBtu/MWh

Diesel = FIP * 16 MMBtu/MWh

Other Renewable = $0/MWh

Each provider made whole under these generic caps has the option to provide to ERCOT verifiable costs to be used as the basis of all future payments for this unit.  The Market Monitor may require that the QSE provide justification of the verifiable costs.  Any resource called upon and paid using these proxy costs may submit “typical” energy costs, by season if necessary,  within 30 days after such calculation of make whole payments which it considers invalid.  This review may be requested by a resource only one time per year, but data submitted may cover multiple seasonal periods. This set of costs will be used for all future make whole payments for this unit subject to approval by the market monitor.

ERCOT shall inform resources selected for commitment through the DaRUC process of the commitment decision.  ERCOT’s commitment decision shall specify the hours for which the resource is required to be at least at Minimum-energy – such hours shall respect any minimum run time for the resource.

# QSE De-commitment of Resources after DaRUC

In the event of a forced, or impending forced, outage of a resource, the QSE must inform ERCOT about the status of the resource, the QSE-desired and the earliest and latest times that the outage can start, the expected, minimum and maximum duration of the outage.  ERCOT shall coordinate the outage schedule and grant the QSE’s desired schedule unless system reliability necessitates changes within the parameters specified by the QSE.

If a QSE would like to de-commit a self-committed resource for purely economic reasons, the QSE must submit a request to ERCOT specifying the QSE-desired and the earliest and latest times that the resource would be off-line, the expected, minimum and maximum duration of the de-commitment.  ERCOT shall coordinate the de-commitment schedule and grant the QSE’s desired schedule unless system reliability necessitates changes within the parameters specified by the QSE.  A need to commit additional units is a reliability issue and would negate approval of a decommitment request.

Whatever the reason behind a QSE’s decision to de-commit a self-committed resource, that resource does not qualify as Committed Capacity for the hours of de-commitment in the settlement of DaRUC charges.

Resources committed by ERCOT through the DaRUC process that experience a forced outage after unit synchronization, shall be eligible for pro-rated “make-whole” payments based on costs incurred prior to the forced outage.  Resource gets the hourly minimum energy rate for the hours run.

If the resource experiences start-up failure which creates a forced outage prior to breaker close it may be compensated for its verifiable actual costs limited to costs which qualify as normal startup, (includes maintenance, labor and fuel) up to the forced outage event.

# Settlement Rules

DaRUC Resource commitment decisions are financially and physically binding on the participants and “make-whole” costs are assigned to market participants who have insufficient self-provided or purchased committed capacity and bilateral energy schedules to satisfy their after-the-fact metered load and committed capacity and bilateral energy schedule obligations.

The calculation for the “make-whole” payment the generator will receive in any of the allocation methods below:

a.  Calculate the “make-whole” payment for a particular resource for a particular day as follows:

- Calculate the total of Start-up Cost bid (if the resource is started in this day) plus the product of Minimum-energy Cost bid multiplied by the number of hours for which the resource is committed in that day.

- Calculate the energy revenue credit by multiplying the energy output of the resource for each interval of the period for which the resource was committed times the corresponding LMPs at the resource node and subtracting from this product the additional production cost (area under energy bid curve) for output that exceeds the Minimum-energy level and (ii) adding the following quantity, if positive and only up to the amount required to fully eliminate any make whole payment(s):

(A) the energy output of the resource for each interval outside of the commitment period during which the resource was producing energy times the corresponding LMPs at the resource node and subtracting from this product (B) the additional production cost (area under energy bid curve) for output that exceeds the Minimum-energy level plus the Minimium-energy Cost.  If the output of the resource is below its Minimum-energy level, then the amount (B) is equal to the Minimum-energy Cost ($/MWh) multiplied by the output of the resource.

- The “make-whole” payment to the resource is the difference between the amount calculated in (a1) minus the amount calculated in (a3)   If the resource  did not submit an offer for the RUC, 100% of all margin over “make-whole” payments are refunded to QSEs on a load ratio share basis.  If the resource submitted an offer for the RUC, 50% of all margin over “make-whole” payments are refunded to QSEs on a load ratio share basis.

- Any positive “make-whole” payment of this resource is then allocated evenly over all intervals for which the resource is committed in that day.

  - Calculate the DaRUC charge for each interval as follows:

1.	Calculate the hourly payment for each Resource by dividing the total make-whole payment by the number of hours committed.

2.	Calculate the total hourly payments by summing the payments for each Resource for the hour.

3.	Calculate the Hourly DaRUC Charge for an hour by dividing the total hourly payment by the total MWs capacity committed for the hour.

4.	For each QSE calculate any QSE capacity shortage equal to the difference between the adjusted load and the lesser of the amount of capacity in the Day-ahead RUC snapshot (adjusted by ERCOT de-commits) or the amount of available QSE capacity in the real time hour.

5.	Calculate the total shortfall for each hour for all QSEs.

6.	QSEs that are short will be charged the lesser of their “short ratio share” of the total DaRUC hourly payments or six (6) times the Hourly DaRUC Charge where  “short ratio share” is the amount a QSE is short in that hour divided by the total that all QSEs are short in that hour.

- Any DaRUC Charges not paid for in 5 above will be charged to all QSEs in Load Ratio Share.

- ERCOT will calculate and monitor the cost of local congestion for unit commitment based on unconstrained vs. constrained unit commitment of the unit involved.

- An incentive will be designed into the mitigation for non-competitive units that will be at least 25% above the energy offer caps used to calculate the make whole payments to incentivize self-commitment.

  - This is how decommitment is handled:

1.	Calculate the amount by which each QSE is short in each interval by taking the positive difference between (1) the sum of QSE self-committed capacity (capacity de-committed or forced-out during this interval would therefore not be counted, except capacity decommitted by ERCOT), Committed Capacity purchase nominations, and bilateral energy purchase schedules (all submitted prior to the DaRUC) and (2) the sum of the QSE after-the-fact adjusted metered load, Committed Capacity sale nominations, and bilateral energy sale schedules.

# Questions

RUC-1.	Self-committed resources after RUC – can they de-commit?  How? (RUC)

- De-committed by ERCOT   -- How much time notice to Resource? Should ERCOT be required to inform Resource of reason for de-committing?

- Approval by ERCOT of QSE request to de-commit capacity.  ERCOT will not approve the request if it affects a reliability or security constraint. (When this is done, capacity does not count for settlement.)  OUTSTANDING ISSUE:  if you want to de-commit because you don’t need capacity for your obligations, but ERCOT does not approve, should you be compensated for the capacity you wanted to shut down (but couldn’t for reliability)?  Should there be a “make whole” provision in this case?

- Forced outage and deration (notification obligation to ERCOT) timeline as defined in outage scheduler

RUC-4.	What is optimization objective?  Start-up versus energy? (RUC)

- Minimize the cost of (Start up, No Load, Expected energy).  Assumes self committed units have zero three part bids for RUC.

- Once a unit is committed in RUC, the energy bids cannot be changed until the RUC of the following day.

- OUTSTANDING ISSUE:  units that are constrained for fuel, environmental, or other reasons—how can they convey this characteristic in RUC? i.e. “non-bid” resources.  These are not deployed unless absolutely needed.

- Unit Classification:  Self committed, unavailable, “special,” and available units.  (All available units have an obligation to bid into RUC.)

- RC (Resource Committed) Committed for next day use by resource representative

- RA (Resource Available) Available but not committed by resource representative. (RUC bid Required)

- RU (Resource Unavailable) Not available for RUC due to Outage Scheduler status.

- RE (Resource Emergency) Available to RUC under reliability emergency only. “RE” status conveyed from Outage Scheduler and includes any “special” unit considerations as defined and approved by Outage Scheduler.

- Units procured out of RUC are not substitutable

RUC-9	Assume schedules are financially binding in RUC process.  Virtual schedules are not considered in RUC. (RUC) (if a unit is selected in RUC for Reliability wouldn’t it need to be physically biding?)

RUC-10	Agreements between unit commitment database and outage schedules (OUTAGE/DAEM)

- UC database should be initialized by the outage scheduler DB.  (The outage scheduler is definitive and high/low limits should come from the OS.)

RUC-12	Ramp rates must be known and honored (up rate, down rate) (RUC/RT)

RUC-13	Resource sustainable limits must be accurate and updated timely (RUC/RT/OUTAGE)

- UC database should be initialized by the outage scheduler DB.  (The outage scheduler is definitive and high/low limits should come from the OS—as well as changes to limits from outages and derates.)

RUC-14	High/Low operating limits must be accurate and updated timely (RT)

RUC-20	Ability to handle uncontrollable Resources (RUC/RT)

- Wind schedules?

- ERCOT forecast for wind?

- OUTSTANDING ISSUE:  White paper from Walter Reid

RUC-21	Model synchronous condensers, phase shifters, dynamic rating devices, DC ties (Network Model)

RUC-22	Model private networks in RUC

- LaaRs behind private networks (need info about load(s) and the generator(s))

RUC- 	Model QFs in RUC

RUC-	Model RMR Resources in RUC (RUC/RT)

RUC-23	Model combined cycle plants (RUC/RT/OUTAGE/Resource Bidding)

- CC to inform ERCOT of operating modes

- CC to inform ERCOT of which modes are available

- Modify body of the document “Input” section to incorporate CC operating modes

RUC-	Which document/source governs unit parameters (Resource Bidding White Paper)

RUC-24	Model “switchable” units (boundary units with ability to sell into other reliability areas) (RUC/RT/OUTAGE)

- Obligation for switchable units to inform ERCOT of capacity available for ERCOT (via outage scheduler at a minimum)

RUC-25	Model and evaluate SPS/RAP (Security Analysis)

RUC-26	Model Resource characteristics that qualify for A/S (RUC/RT/AS)

- Input from DA A/S market

- Resources which are providing A/S must self commit in RUC Capacity obligation transfers will be allowed prior to the RUC

  - Should A/S capacity obligation transfers be allowed to a RUC unit after the RUC is completed? (The revenue would count against make whole payment to the RUC unit.)

RUC-27	Need for locational A/S (AS)

RUC-28	Resource turnaround time (unit cycle time, minimum up/down time) (RUC/RT)

- What happens to resources with min op time > 24 hours?

- What happens to units that take > 24 hours to start?

RUC-30	Need for voltage control (RUC/RT/OUTAGE)

Commitment will honor voltage constraints through SA as part of the day-ahead ERCOT planning process.

RUC-31	Resource availability affected by fuel, emissions, staffing, number of starts (RUC/RT)

Added new status to denote a new type of “unavailability” [don’t run unless last resort]

RUC-32	Standard naming convention (GLOBAL)

Move to other document to be developed [in Fidelity paper].

RUC-33	Interface with state estimator (RUC/RT/OUTAGE)

SE will be used to provide parameters for bus load forecast that Security Analysis will pass to DaRUC

RUC-34	Accurate information from Market Participants and usage of AC power flow (GLOBAL) Security Analysis will be full AC power flow and will provide linearized constraints to DaRUC

RUC-35	Automatic shift factor calculation (RUC/RT/DAEM)

Shift factors are calculated out of the security analysis for each hour of the day-ahead.

RUC-36	When no solution, RUC must provide clear, easy to understand help information (RUC/GLOBAL)

DaRUC will commit all generation and if a shortage of supply remains, the program will relax constraints and list the constraints that cannot be resolved and remove the constraint.

RUC-37	Ability to input known voltage/stability limits (RUC/GLOBAL)

All transmission limits either thermal or stability will be modeled as constraints on flow of lines

RUC-38	Extended limitations/deratings must be interfaced with outage scheduler (RUC/RT/OUTAGE)

The Outage Scheduler (transmission and generation) will provide Security Analysis and DaRUC with Resource limits by hour for any time period within the next 10 days.  Outage scheduler will be used to update Resource limits due to forced de-ratings.

RUC-39	Adjustable shift factor threshold (RUC/RT/OUTAGE)

No threshold in the LMP model for RUC is recommended at this time; need to look at further.

RUC-40	Flexible study mode (RUC/RT)

Study mode is provided by the 7-8 day RUC that may be used to study day after tomorrow and the next seven days [The issue of mothball units can be addressed in the Protocols or other means.]

RUC-41	Ability to adjust/relax constraints with solution not found (RUC/RT/OUTAGE)

Only ERCOT operator action will be allowed to change constraints once they are established

RUC-42	Accommodation of dynamic ratings (RUC/RT/OC/DAEM)

All thermal limits on all transmission devices will be adjusted to expected ambient air conditions.

RUC-43	Access to calculated shift factors (RUC/RT/DAEM)

The output of the DaRUC will only consist of unit commitments by hour for the next day.    ERCOT requires availability of hourly shift factors as well as other information specified in the reliability calculation.

RUC-44	Generic Cost/Bidding Information (incremental, start-up, minimum load) (RUC/RT/OUTAGE)

Three part bids accommodated

RUC-45	Ability to consider all resources regardless of whether they are bid into the market (RUC/RT) All available resources will be considered.  Those that have bids will be dispatched according to their bid price and those without bids, the model will assign generic start-up, and no load costs at the value it uses for OOMC/OOME for real-time.

RUC-46	Direct assignment of RUC charges to entities using cost causation principle (RUC/SETTLEMENT)

Unresolved at this time.

RUC-47	Resource cannot be made unavailable for economic purposes (RUC/RT/OUTAGE)

See RUC – 1 above.

RUC-49	How are minimum run times exceeding 24 hours addressed? (RUC)

DaRUC does not.  The longer term WaRUC (Week Ahead RUC) will be used by ERCOT to study extended term events that may pose a security problem of provide significant cost improvement to current DaRUC commitments by bring on line units not available with shorter notice than 24 hours.

RUC-50	How does RUC determine if it is time to start large mothballed units? (RUC)

The WaRUC is used or other long term methods are used.  [This issue of mothball units can be addressed in the Protocols or other means]

RUC-51	Does tendency to have long-term period shutdowns improperly affect SCUC? (RUC)

No.

RUC-52	What is the Day-Ahead Market interface with RUC? (RUC/DAEM)

In the DA, QSEs will commit capacity through nominations or energy schedules (process to be developed) that establish a baseline of capacity available prior to DaRUC.  DaRUC adds new capacity to this mix and charges QSEs who when considering actual metered load and self-committed capacity shorted the DA

RUC-53	What, if any, seams issues exist between DaRUC and other similar reliability studies? (RUC) All Security Analysis systems including RUC will obtain all transmission and generation parameters and schedules from the same source data.  Generation parameters can be combined for market model.

RUC-54	What are the consequences of not supporting a DA generation commitment post-RUC? (RUC)

QSEs who do not support DA generation commitment are subject to HaRUC capacity charges for the amount they are short.  See above settlement formula

RUC-55	What effect do bilateral schedules have on RUC? (RUC)

Bilateral schedules and capacity nominations are used to establish the QSEs self-commitment of Resources(process to be developed)

RUC-56	What are the financial consequences of changing bilateral schedules post-RUC? (RUC) Schedules and capacity nominations are saved just prior to DaRUC and are used to settle DaRUC.  If commitments change and a new DoRUC (Day of RUC) is needed, the short QSE will pay for the new capacity

RUC-57	How are RUC charges calculated? (RUC)

Sum of all costs. See above

RUC-58	How does RUC unit status affect the ability to change status in the Adjustment Period? (RUC)

Changing of a units status may cause the QSE to become subject to new capacity charges if a DoRUC is required to replace the capacity

Questions 56, 57, and 58 will be addressed in examples.

RUC- 59	On Friday, do we want to do a 3-day commitment with binding bids?  Is this more risk than taking the gas risk directly?

No.  Increased risk of load forecast Error for 3 days out not worth the added value to purchasing fuel on Friday for Monday.  [Garland: Could cause units to file for fuel unavailability status would that be acceptable?]

RUC-60  How SPS and remedial action plans will be modeled in RUC?

They must be modeled in security analysis and flowed into Day-ahead RUC and in LMP calculator as linerized transmission constraints.