| OGRR Number | 142OGRR | OGRR Title | DOE and NERC Reporting |
| --- | --- | --- | --- |
| Operating Guide Section Requiring Revision |  | 3.1.3.4, QSE Responsibility for Department of Energy (DOE) and NERC Reportable Events [new] 3.1.5.1, Disturbance Reporting 2.1, Operational Duties |  |
| Protocol Section Revised |  | ## N/A |  |
| Proposed Effective Date |  | April 1, 2004 |  |
| Revision Description |  | Changes the reporting processes by having TDSPs and QSEs provide required disturbance data for DOE/NERC reportable events to the ERCOT Control Area Authority, who will distribute the report to the DOE and NERC. |  |
| ###### Benefit |  | Clarification of the Operating Guide’s DOE / NERC reporting requirements. Adds QSE reporting requirement. ERCOT, as the Control Area Authority, should be the reporting Entity for DOE/NERC events. |  |
| OWG Recommendation |  | OWG recommends approval of OGRR142 as revised by OGRTF comments. |  |
| ROS Recommendation |  | ROS recommends approval of OGRR142 as recommended by OWG. |  |
| TAC Action |  | TAC approved OGRR142 as recommended by ROS. |  |

| ## Sponsor |  |
| --- | --- |
| Name | Rick Keetch, on behalf of the OWG |
| E-mail Address | rkeetch@reliant.com |
| Company | Reliant Energy |
| Company Address | P.O. Box 1700, Houston, TX 77251 |
| Phone Number | (713) 497-2526 |
| Fax Number | (713) 497-9720 |

Proposed Operating Guides Revision

      - QSE Responsibility for Department of Energy (DOE) and NERC Reportable Events

QSEs shall provide ERCOT with the DOE Form EIA-417 report for each DOE reportable event that happens in areas they represent.  ERCOT, as the Control Area Authority, will forward these reports to the DOE and NERC as required.

#### 3.1.5.1	 Transmission Operator Responsibility for Department of Energy (DOE) and NERC Reportable Events

TOs shall provide ERCOT with the DOE Form EIA-417 report for each DOE reportable event that happens in areas they represent.  ERCOT, as the Control Area Authority, will forward these reports to the DOE and NERC as required.

## 2.1	Operational Duties

| CONTROL AREA AUTHORITY RESPONSIBILITY | CONTROL AREA AUTHORITY ACTION |
| --- | --- |
| Ensure the reliability of the ERCOT Transmission Grid in accordance with North American Electric Reliability Council/ERCOT reliability guidelines. | - Evaluate ERCOT Transmission Grid conditions on a continuous basis. - Oversee operation of the ERCOT Transmission Grid and act as the ERCOT Control Area Authority. - Serve as the point of contact for initiation of generation interconnection to the transmission system. - Implement Congestion Management of the ERCOT Transmission Grid on a nondiscriminatory basis. - Coordinate all planned outages of transmission system facilities. - Schedule ERCOT Interchange transactions across the DC Ties. - Deploy Ancillary Services - Compile a detailed operations plan to ensure that sufficient resources and required Ancillary Services have been provided for all expected load and transactions within the ERCOT Region. - Forecast the actual load, generation dispatch and transmission system operations for the next seven days and update available transmission capacity (TTC), as necessary. - Adjust resources in accordance with the ERCOT Protocols as needed to maintain ERCOT in a secure operating condition and meet NERC control performance requirements. |
| Supervise the ERCOT System operational transmission planning | - Perform operational transmission system security studies, including those related to generation and load interconnection responsibilities. - Review outages of generating units and major transmission lines to identify constraints that may affect transfers. - Perform load flows and security analyses of planned outages submitted by TDSPs as a basis for approval or rejection. - Withdraw approval of a scheduled Transmission Facility Maintenance Outage if unable to meet the applicable reliability standards even if all other reasonable options are exercised. |
| Collect and disseminate information | - Provide appropriate operational information to individual QSEs and TDSPs. - Provide relevant operational information to Market Participants over the ERCOT Market Information System. - Collect and maintain Control Area Authority operational data required by the PUCT. - Receive reports from TOs and QSEs and forward them to DOE and/or NERC as required. - Submit Reliability Coordinator reports to DOE and/or NERC as required. - Record and report accumulated time error. |

| OGRR Evaluation |  |
| --- | --- |
| Non-ERCOT Market Comparison | No Comparison conducted. |
| Comments Author | Comments Summary |
| OGRTF | Revised language. |

| Sponsor's Revision Description | Changes the reporting processes by having TDSPs and QSEs provide required disturbance data for DOE/NERC reportable events to the ERCOT Control Area Authority, who will distribute the report to the DOE and NERC. |
| --- | --- |
| ###### Sponsor's Reason for Revision | Clarification of the Operating Guide’s DOE / NERC reporting requirements. Adds QSE reporting requirement. ERCOT, as the Control Area Authority, should be the reporting Entity for DOE/NERC events. |