| PRR Number | 516PRR | PRR Title | Curtailment Prioritization for Wind Generators |
| --- | --- | --- | --- |
| Protocol Section(s) Requiring Revision (include Section No. and Title) |  | 7.4.2, Resolution of Local Congestion 7.8.8, Accommodation of New or Recommissioned WPGRs |  |
| Requested Resolution (Normal or Urgent) |  | Normal |  |
| Revision Description |  | Introduces the concept of using in-service date to determine curtailment priority in situations where only wind generators are needed to resolve local congestion. Requires new WPGRs locating in the McCamey area to install Special Protection Schemes to limit their output. |  |
| Reason for Revision |  | To address congestion situations that may arise in the McCamey area prior to the completion of the ERCOT approved 345kV transmission plan (Twin Buttes – McCamey – Odessa). Also addresses congestion situations that may arise in other locations since wind generation can be installed much more quickly than typically required transmission improvements can be constructed. |  |
| Timeline |  |  |  |
| Date Posted |  | 04/30/04 |  |

See Master List for current updates.

| Sponsor |  |
| --- | --- |
| Name | Beth Garza |
| E-mail Address | beth_garza@fpl.com |
| Company | FPL Energy |
| Company Address | 701 Brazos, Suite 970, Austin, TX 78701 |
| Phone Number | 512-320-5915 |
| Fax Number | 512-320-5988 |

ERCOT/Market Segment Impacts and Benefits

Instructions:  To allow for comprehensive PRR consideration, please fill out each block below completely, even if your response is “none,” “not known,” or “not applicable.”  Wherever possible, please include reasons, explanations, and cost/benefit analyses pertaining to the PRR.

|  | Impact |  | Benefit |
| --- | --- | --- | --- |
|  | Business | Computer Systems |  |
| ERCOT | Approve SPSs and implement limitations. | None | Implementation of limitations will be done in a more orderly manner since WPGRs affected would have agreed to the limitations in advance |
| MARKET SEGMENT |  |  |  |
| Consumer | None | None | Eliminate unnecessary transmission costs. |
| LSE: General, Including NOIE | None | None | Eliminate unnecessary transmission costs. |
| LSE: CR & REP | None | None | Eliminate unnecessary transmission costs. |
| QSE | None | None | Eliminate unnecessary transmission costs. |
| Resource | Potential increase in equipment costs. (Fast Response) | None | Clarifies transmission requirements. |
| TDSP | Design and implementation of SPSs | None | Clarifies transmission requirements |

Comments

Proposed Protocol Language Revision

### 7.4.2	Resolution of Local Congestion

ERCOT will procure RPRS if necessary to provide sufficient capacity to resolve forecasted Local Congestion.  ERCOT will purchase RPRS from the Resource(s) having the lowest cost per megawatt based on each Resource’s Resource Category Generic Cost and its ability to resolve the Congestion given its Shift Factor on the Local Congestion.

ERCOT will instruct and Dispatch Resources in Real Time in a manner that does not exceed the units’ operational limits known to ERCOT.  Other than in the McCamey area, which is addressed in Section 7.8, Congestion Management in McCamey Area, for situations where WPGRs provide the only options for resolving Congestion, the WPGRs shall be curtailed by reverse order of online date (last on, first off).  The Resources that have been instructed to operate within a range to resolve Local Congestion are paid or pay a price, in accordance with Section 6.8, Compensation for Services Provided, that depends on whether a Market Solution exists for resolving the Local Congestion.

The Local Congestion Costs are embedded in each service deployed to resolve Local Congestion such as OOMC, OOME, and unit specific premiums.

The formula to calculate Local Congestion Replacement Reserve payment can be found in Section 6.8.1.11, Local Congestion Replacement Reserve Payment to QSE, of these Protocols.

### Accommodation of New or Recommissioned WPGRs

Wind Powered Generating Resources in the McCamey Area which have completed the required registration activities as defined in Section 16.5.2, Registration Process for a Resource, after January 1, 2003 shall be required to install a Special Protection Scheme to allow them to operate in such a way as to not increase curtailments at existing WPGRs, until such time as additional Transmission Facilities are placed in service that would increase the transmission transfer capability out of the McCamey Area.  When only WPGRs are available to resolve Congestion in the McCamey Area, any WPGRs which have completed the required registration activities as defined in Section 16.5.2, Registration Process for a Resource, after January 1, 2003, including any WPGRs added beyond the quantity at which a SPS would be effective, shall be the first curtailed by reverse order of online date (last on, first off) when the capability of the installed Transmission Facilities is exceeded.

In the case of a new or recommissioned WPGR located in the McCamey Area, the Resource must demonstrate to ERCOT’s satisfaction that the anticipated capacity factor for the first month of commercial operation is greater than or equal to ten percent (10%) to be included in the determination of total generating capacity in the McCamey Area.

#### 7.8.8.1	New or Recommissioned Unit Startup and Testing

For new or recommissioned Generation Resources in the McCamey Area that rely on phased-in Facility interconnection, the WPGR owner shall Supply ERCOT with a test plan.  This plan must indicate how the Resource will increase capacity, along with the expected dates of such capacity becoming available.  During the testing period before commercial operation, ERCOT shall allocate to the new WPGR, TGRs equal to the test plan capability if it is less than five (5) MW.  ERCOT shall allocate to the new WPGR, TGRs proportional to the test plan’s proportion of available capacity if it is more than five (5) MW.

#### 7.8.8.2	New or Recommissioned Unit Commercial Operation

The owner of a WPGR coming online in the McCamey Area shall notify ERCOT three (3) Business Days before expected commercial operation.  The Notice must include the MW of generation capacity expected to become commercial based on the PUCT certification of the Generation Resource as a REC generator, the date of expected commercial operation, and the scheduling QSE(s) with the associated capacity that the QSE will be scheduling from the WPGR.  ERCOT shall allocate TGRs to the QSE(s) scheduling the WPGR as of the first day of expected commercial operation in accordance with the methods described in this Section 7.8.9, ERCOT Day Ahead and Real Time Operations.  The owner of the WPGR is responsible for notifying ERCOT of any delays in expected commercial operation, as the calculation of the TGR allocation is partially contingent upon this information.