Settlements with Enhanced Hybrid Day-Ahead Market

(Draft)

TNT COCG 5-4-04

Table Contents

Section 1.	Day-ahead Energy Settlements	2

Section 1.1	Inputs for Settlements	2

Section 1.2	Hourly Settlement Formulas	4

Section 2.	Real-time Energy Settlements	5

Section 2.1	Inputs for Settlements	5

Section 2.2	15-min Settlement Formulas	7

Section 3.	CRR Settlements	9

Section 3.1	CRRs to be settled with Day-ahead LMP and Shadow Prices	9

Section 3.1.1	Inputs for Settlements	9

Section 3.1.2	Hourly Settlement Formulas	11

Section 3.2	CRRs to be settled with Real-time LMP and Shadow Prices	12

Section 3.2.1	Inputs for Settlements	12

Section 3.2.2	Hourly Settlement Formulas	14

Section 4.	Day-ahead Reliability Unit Commitment Settlements	15

Section 5.	Hour-ahead Reliability Unit Commitment Settlements	15

Section 6.	Ancillary Service Settlements	15

Section 7.	Appendices	15

# Day-ahead Energy Settlements

In this section, settlements are presented for the offers and bids cleared in the E-HDAM, energy scheduled through inter-QSE trades (to be settled in E-HDAM), and make-whole for unit-commitment in E-HDAM.

## Inputs for Settlements

| ID | Input | Unit | Party | Settlement Point | Resource | Frequency | Description | Source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | DALMP | $/MWh | -- | Resource node, load zone, or hub. | -- | Hourly | DA Locational Marginal Price. | E-HDAM (for each load zone, calculated by using LMP and distribution factors; for each hub, calculated by using LMP and the number of buses with the hub) |
| 2 | DA Energy Sold | MW | QSE | Delivery location as specified in offers. | Name of the unit if committed in E_HDAM; otherwise, null. | Hourly | Energy sold in DA market at the specified settlement point. | E-HDAM |
| 3 | DA Energy Purchased | MW | QSE | Delivery location as specified in bids. | -- | Hourly | Energy purchased in DA market at the specified settlement point. | E-HDAM |
| 4 | DA Inter-QSE Energy Resource | MW | QSE | Delivery location as specified. | -- | Hourly | Energy purchased from the counter QSE at the specified settlement point. | E-HDAM |
| 5 | DA Inter-QSE Energy Obligation | MW | QSE | Delivery location as specified. | -- | Hourly | Energy sold to the counter QSE at the specified settlement point. | E-HDAM |

Section 1.1 Inputs for Settlements (Continued)

| ID | Input | Unit | Party | Settlement Point | Resource | Frequency | Description | Source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 6 | DA Unit Commitment Flag | -- | QSE | Resource node where the unit physically exists. | Name of the resource with 3-part offers. | Hourly | Flag to indicate whether the unit is committed for the hour in the E-HDAM (“Y” or null). | E-HDAM |
| 7 | LSL | MW | QSE | Resource node where the unit physically exists. | Name of the resource with 3-part offers. | Hourly | Low Sustained Limit. | Submitted by the QSE representing the resource to E-HDAM |
| 8 | Startup Price | $/Startup | QSE | Resource node where the unit physically exists. | Name of the resource with 3-part offers. | Hourly | Startup price as shown in the 3-part offers. | Submitted by the QSE representing the resource to E-HDAM |
| 9 | Minimum Energy Price | $/MWh | QSE | Resource node where the unit physically exists. | Name of the resource with 3-part offers. | Hourly | Minimum energy price as shown in the 3-part offers. | Submitted by the QSE representing the resource to E-HDAM |
| 10 | Energy Offer Curve | ($/MWh, MW) | QSE | Resource node where the unit physically exists. | Name of the resource with 3-part offers. | Hourly | Linear curve that represents energy offer prices as a function of the output level. | Submitted by the QSE representing the resource to E-HDAM |

## Hourly Settlement Formulas

| ID | Payment/Charge | Unit | Formula |
| --- | --- | --- | --- |
| 1 | DA Energy Sold | $ | (-1)  DALMP _Settlement Point  DA Energy Sold _QSE _Settlement Point [_Resource Name] |
| 2 | DA Energy Purchased | $ | DALMP _Settlement Point  DA Energy Purchased _QSE _Settlement Point |
| 3 | DA Inter-QSE Energy Resource | $ | (-1)  DALMP _Settlement Point  DA Inter-QSE Energy Resource _QSE _Settlement Point |
| 4 | DA Inter-QSE Energy Obligation | $ | DALMP _Settlement Point  DA Inter-QSE Energy Obligation _QSE _Settlement Point |
| 5 | DA Make-whole | $ | If DA Unit Commitment Flag _QSE _Settlement Point _Resource Name = “Y” (-1)  (Startup Price _QSE _Settlement Point _Resource Name / Number of Commitment Hours + Minimum Energy Price _QSE _Settlement Point _Resource Name  LSL _QSE _Settlement Point _Resource Name +Pr (q) dq - DALMP _Settlement Point  DA Energy Sold _QSE _Settlement Point _Resource Name) Where Number of Commitment Hours is determined based on DA Unit Commitment Flag _QSE _Settlement Point _Resource Name Pr (q) is the function that represents the Energy Offer Curve, the energy offer price ($/MWh) vs. the MW quantity Otherwise None |
| 6 | DA Make-whole Allocation | $ | ( All QSE ( All settlement points ( All resources DA Make-whole Payment _QSE _Settlement Point _Resource Name)  ( All settlement points DA Energy Purchased _QSE _Settlement Point) / (( All QSE  All settlement points DA Energy Purchased _QSE _Settlement Point) |

# Real-time Energy Settlements

In this section, settlements are presented for (1) energy carried forward from the E-HDAM, (2) energy scheduled through inter-QSE energy trades (to be settled with real-time LMP), (3) energy produced / consumed in real-time, and make-whole for unit commitment in E-HDAM.

## Inputs for Settlements

| ID | Input | Unit | Party | Settlement Point | Resource | Frequency | Description | Source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | LMP | $/MWh | -- | All settlement points (resource nodes, load zones, or hubs). | -- | Every 15 min | Real-time Locational Marginal Price. | Calculated from 5-min data from SCED and State Estimator |
| 2 | Base Point | MW | QSE | Resource node where resource is physically located. | Name of the resource. | Every 5 min | Resource set point. | SCED |
| 3 | Metered Resource | MWh | QSE | Resource node where resource is physically located. | Name of the resource. | Every 15 min | QSE’s metered generation (or curtailed load) at the settlement point. | DataAgg |
| 4 | (Adjusted) Metered Load | MWh | QSE | Load zone where load is physically located. | -- | Every 15 min | QSE’s adjusted meter load at the settlement point. | DataAgg |
| 5 | DA Energy Resource | MW | QSE | Delivery location as specified in bids. | -- | Hourly | QSE’s energy purchased in DA market at the specified settlement point. | Day-ahead energy model |
| 6 | DA Energy Obligation | MW | QSE | Delivery location as specified in offers. | -- | Hourly | QSE’s energy sold in DA market at the specified settlement point. | Day-ahead energy model |
| 7 | Inter-QSE Energy Resource | MW | QSE | Delivery location as specified in energy schedules. | -- | Every 15 min | QSE’s energy purchased through inter-QSE trades at the specified settlement point. | Energy schedules (Inter-QSE trades) |
| 8 | Inter-QSE Energy Obligation | MW | QSE | Delivery location as specified in energy schedules. | -- | Every 15 min | QSE’s energy sold through inter-QSE trades at the specified settlement point. | Energy schedules (Inter-QSE trades) |

Section 2.1 Inputs for Settlements (Continued)

| ID | Input | Unit | Party | Settlement Point | Resource | Frequency | Description | Source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 9 | RUS Deployment | MWh | QSE | -- | -- | Every 15 min | 15-min integration of regulation up deployment | LFC |
| 10 | RUS Participation Factor | -- | QSE | -- | Name of the resource. | Hourly | Each unit’s percentage of capacity available for regulation down service. | DA Ancillary Service Market |
| 11 | RDS Deployment | MWh | QSE | -- | -- | Every 15 min | 15-min integration of regulation up deployment | LFC |
| 12 | RDS Participation Factor | -- | QSE | -- | Name of the resource. | Hourly | Each unit’s percentage of capacity available for regulation down service. | DA Ancillary Service Market |
| 13 | RRS Deployment | MWh | QSE | -- | -- | Every 15 min | Responsive reserve deployment. ??? | LFC |
| 14 | RRS Participation Factor | -- | QSE | -- | Name of the resource. | Hourly | Each unit’s percentage of capacity available for responsive reserve service. | DA Ancillary Service Market |
| 15 | NSRS Deployment | MWh | QSE | -- | Name of resource | Every 15 min | Non-Spin reserve deployment. | Manual Instruction |
| 16 | DA Unit Commitment Flag | -- | QSE | Resource node where the unit physically exists. | Name of the resource with 3-part offers in E-HDAM. | Hourly | Flag to indicate whether the unit is committed for the hour in the E-HDAM (“Y” or null). | E-HDAM |
| 17 | Startup Price | $/Startup | QSE | Resource node where the unit physically exists. | Name of the resource with 3-part offers in E-HDAM. | Hourly | Startup price as shown in the 3-part offers. | Submitted by the QSE representing the resource to E-HDAM |

## 15-min Settlement Formulas

| ID | Payment/Charge | Unit | Formula |
| --- | --- | --- | --- |
| 1 | Metered Resource | $ | (-1)  LMP _Settlement Point  Metered Resource _QSE _Settlement Point _Resource Name |
| 2 | Metered Load | $ | LMP _Settlement Point  Metered Load _QSE _Settlement Point |
| 3 | DA Energy Resource | $ | (-1)  LMP _Settlement Point  (DA Energy Resource _QSE _Settlement Point / 4) |
| 4 | DA Energy Obligation | $ | LMP _Settlement Point  (DA Energy Obligation _QSE _Settlement Point / 4) |
| 5 | Inter-QSE Energy Resource | $ | (-1)  LMP _Settlement Point  (Inter-QSE Energy Resource _QSE _Settlement Point / 4) |
| 6 | Inter-QSE Energy Obligation | $ | LMP _Settlement Point  (Inter-QSE Energy Obligation _QSE _Settlement Point / 4) |
| 7 | Penalty to Resource | $ | Define A =  Three 5-min intervals (Base Point _QSE _Settlement Point _Resource Name / 12) + Ancillary Service Deployment _QSE _Settlement Point _Resource Name B = ABS (Metered Resource _QSE _Settlement Point _Resource Name – A) Where Ancillary Service Deployment _QSE _Settlement Point _Resource Name = RUS Deployment _QSE  RUS Participation Factor _QSE _Resource Name + RDS Deployment _QSE  RDS Participation Factor _QSE _Resource Name + RRS Deployment _QSE  RRS Participation Factor _QSE _Resource Name + NSRS Deployment _QSE _Settlement Point _Resource Name If Dynamic Scheduling Resources OR Qualifying Facilities not bidding OR Uncontrollable resources None Otherwise Max (0, LMP _Settlement Point)  Min (1, 2  Max (0, (B / A – Max (0.10, 1.25 / A))))  Metered Resource _QSE _Settlement Point _Resource Name |

Section 2.2 15-min Settlement Formulas (Continued)

| ID | Payment/Charge | Unit | Formula |
| --- | --- | --- | --- |
| 8 | Make-whole | $ | If DA Unit Commitment Flag _QSE _Settlement Point _Resource Name = “Y” Startup Price _QSE _Settlement Point _Resource Name / (Number of Commitment Hours  4)  Flag of Non-performance Where Flag of Non-performance is determined by using Metered Resource and DA Unit Commitment Flag _QSE _Settlement Point _Resource Name Otherwise None |
| 9 | Penalty Charge Allocation | $ | ( All QSE  All settlement points  All resources Penalty to Resource _QSE _Settlement Point _Resource Name)  ( All settlement points Metered Load _QSE _Settlement Point) / ( All QSE  All settlement points Metered Load _QSE _Settlement Point) |
| 10 | Make-whole Allocation | $ | ( All QSE  All settlement points  All resources Make-whole Payment _QSE _Settlement Point _Resource Name)  ( All settlement points DA Energy Resource _QSE _Settlement Point) / ( All QSE  All settlement points DA Energy Resource _QSE _Settlement Point) |

# CRR Settlements

Under the E-HDAM, CRRs are settled with day-ahead LMP and Shadow Prices by default. CRRs may be settled with real-time LMP and Shadow Prices according to the holder’s specification.

## CRRs to be settled with Day-ahead LMP and Shadow Prices

### Inputs for Settlements

| ID | Input | Unit | Party | Element | Bus | Settlement Point | Point of Injection | Point of Withdrawal | Frequency | Description | Source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | DA LMP | $/MWh | -- | -- | -- | Resource node, load zone, or hub. | -- | -- | Hourly | DA Locational Marginal Price. | E-HDAM model (Load-weighted average for each load zone and straight average for each hub) |
| 2 | DA SP | $/MWh | -- | Directional physical network element. | -- | -- | -- | -- | Hourly | DA Shadow Price. | E-HDAM model |
| 3 | DA PTP Obligation | MW | Holder | -- | -- | -- | Settlement point where power is injected. | Settlement point where power is withdrawn. | Hourly | Point-to-Point Obligation to be settled with DA LMP. | CRR auction and holder’s specification |

Section 3.1.1 Inputs for Settlements (Continued)

| ID | Input | Unit | Party | Element | Bus | Settlement Point | Point of Injection | Point of Withdrawal | Frequency | Description | Source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4 | DA PTP Option | MW | Holder | -- | -- | -- | Settlement point where power is injected. | Settlement point where power is withdrawn. | Hourly | Point-to-Point Option to be settled with DA shadow prices. | CRR auction and holder’s specification |
| 5 | DA Flowgate | MW | Holder | Directional network element that (partly) makes up a flowgate. | -- | -- | -- | -- | Hourly | Flowgate to be settled with DA shadow prices. | CRR auction and holder’s specification |
| 6 | Shift Factor | -- | -- | Directional physical network element. | Resource/Load bus where the directional physical network element starts or ends. | -- | -- | -- | Hourly | Shift factor represents the ratio of flow on the element to the injection at the bus under the associated topology. | Same as used in E-HDAM model. |
| 7 | Distribution Factor | -- | -- | -- | Resource/Load bus that (partly) makes up the settlement point. | Resource node, load zone, or hub. | -- | -- | Hourly | Distribution factor.  All buses Distribution Factor _Bus _Settlement Point = 1.0, for each settlement point. | Same as used in E-HDAM model. |

### Hourly Settlement Formulas

| ID | Payment/Charge | Unit | Formula |
| --- | --- | --- | --- |
| 1 | DA PTP Obligation | $ | (-1)  (DA LMP _Point of Withdrawal – DA LMP _Point of Injection)  DA PTP Obligation _Holder _Point of Injection _Point of Withdrawal |
| 2 | DA PTP Option | $ | (-1)  DA PTP Option Price _Point of Injection _Point of Withdrawal  DA PTP Option _Holder _Point of Injection _Point of Withdrawal Where DA PTP Option Price _Point of Injection _Point of Withdrawal =  All elements ( All topologies (DA SP _Element _Topology  Max (0, (Distribution Factor _Bus _ Point of Injection _Topology  Shift Factor _Element _Bus _Topology - Distribution Factor _Bus_ Point of Withdrawal _Topology  Shift Factor _Element _Bus _Topology)))) |
| 3 | DA Flowgate | $ | (-1)  ( All topologies DA SP _Element _Topology)  DA Flowgate _Holder _Element |

## CRRs to be settled with Real-time LMP and Shadow Prices

### Inputs for Settlements

| ID | Input | Unit | Party | Element | Bus | Settlement Point | Point of Injection | Point of Withdrawal | Frequency | Description | Source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | LMP | $/MWh | -- | -- | -- | Resource node, load zone, or hub. | -- | -- | Every 5 minutes | RT Locational Marginal Price. | Calculated by using data from SCED and SE |
| 2 | SP | $/MWh | -- | Directional network element. | -- | -- | -- | -- | Every 5 minutes | Shadow Price. | SCED |
| 3 | PTP Obligation | MW | Holder | -- | -- | -- | Settlement point where power is injected. | Settlement point where power is withdrawn. | Hourly | Point-to-Point Obligation to be settled with RT LMP. | CRR auction and holder’s specification |
| 4 | PTP Option | MW | Holder | -- | -- | -- | Settlement point where power is injected. | Settlement point where power is withdrawn. | Hourly | Point-to-Point Option to be settled with RT shadow prices. | CRR auction and holder’s specification |
| 5 | Flowgate | MW | Holder | Directional network element that (partly) makes up a flowgate. | -- | -- | -- | -- | Hourly | Flowgate to be settled with RT shadow prices. | CRR auction and holder’s specification |

Section 3.2.1 Inputs for Settlements (Continued)

| ID | Input | Unit | Party | Element | Bus | Settlement Point | Point of Injection | Point of Withdrawal | Frequency | Description | Source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 6 | Shift Factor | -- | -- | Settlement point where the directional element starts or ends. | Resource/Load bus where the directional physical network element starts or ends. | -- | -- | -- | Every 5 minutes | Shift factor represents the ratio of flow on the element to the injection at the bus under the associated topology. | Same as used in SCED model. |
| 7 | Distribution Factor | -- | -- | -- | Resource/Load bus that (partly) makes up the settlement point. | Resource node, load zone, or hub. | -- | -- | Every 5 minutes | Distribution factor.  All buses Distribution Factor _Bus _Settlement Point = 1.0, for each settlement point. | For each bus within a load zone, calculated by using SE load; for each bus within a hub, calculated by using the number of buses within the hub. |

### Hourly Settlement Formulas

| ID | Payment/Charge | Unit | Formula |
| --- | --- | --- | --- |
| 1 | PTP Obligation | $ | (-1)  ( Twelve 5-min intervals (LMP _Point of Withdrawal - LMP _Point of Injection)) / 12  PTP Obligation _Point of Injection _Point of Withdrawal _Holder |
| 2 | PTP Option | $ | (-1)  PTP Option Price _Point of Injection _Point of Withdrawal  PTP Option _Point of Injection _Point of Withdrawal _Holder Where PTP Option Price _Point of Injection _Point of Withdrawal =  Twelve 5-min intervals ( All elements ( All topologies (SP _Element _Topology  Max (0, (Distribution Factor _Bus _Point of Injection _Topology  Shift Factor _Element _Bus _Topology - Distribution Factor _Bus _Point of Withdrawal _Topology  Shift Factor _Element _Bus _Topology))))) / 12 |
| 3 | Flowgate | $ | (-1)  ( Twelve 5-min intervals ( All topologies SP _Element _Topology)) / 12  Flowgate _Element _Holder |

# Day-ahead Reliability Unit Commitment Settlements

Same as the settlements with ADAM.

# Hour-ahead Reliability Unit Commitment Settlements

Same as the settlements with ADAM.

# Ancillary Service Settlements

Same as the settlements with ADAM.

# Appendices

- Energy Settlement Examples with E-HDAM

- DaRUC (HaRUC) Make-Whole Payment Examples

- DaRUC (HaRUC) Charge Example