Settlements with Auction Day-Ahead Market

(Draft)

TNT COCG 5-4-04

Table Contents

Section 1.	Day-ahead Energy Settlements	2

Section 1.1	Inputs for Settlements	2

Section 1.2	Hourly Settlement Formulas	2

Section 2.	Real-time Energy Settlements	3

Section 2.1	Inputs for Settlements	3

Section 2.2	15-min Settlement Formulas	5

Section 3.	CRR Settlements	6

Section 3.1	Inputs for Settlements	6

Section 3.2	Hourly Settlement Formulas	8

Section 4.	Day-ahead Reliability Unit Commitment Settlements	9

Section 4.1	Day-ahead Reliability Unit Commitment Payments	9

Section 4.1.1	Inputs for Settlements	9

Section 4.1.2	Settlement Formulas	9

Section 4.2	Day-ahead Reliability Unit Commitment Charges	12

Section 4.2.1	Descriptions	12

Section 4.2.2	Inputs for Settlements	16

Section 4.2.3	Settlement Formulas	17

Section 5.	Hour-ahead Reliability Unit Commitment Settlements	18

Section 5.1	Hour-ahead Reliability Unit Commitment Payments	18

Section 5.2	Hour-ahead Reliability Unit Commitment Charges	18

Section 6.	Ancillary Service Settlements	19

Section 7.	Appendices	20

# Day-ahead Energy Settlements

In this section, settlements are presented for the offers and bids cleared in the Auction Day-Ahead energy Market (ADAM).

## Inputs for Settlements

| ID | Input | Unit | Party | Settlement Point | Frequency | Description | Source |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | DAMCPE | $/MWh | -- | Resource node, load zone, or hub. | Hourly | DAM Clearing Price for Energy. “NC” for non-convergence. | Auction day-ahead energy model |
| 2 | DA Energy Sold | MW | QSE | Delivery location as specified in offers. | Hourly | Energy sold in DA market at the specified settlement point. | Auction day-ahead energy model |
| 3 | DA Energy Purchased | MW | QSE | Delivery location as specified in bids. | Hourly | Energy purchased in DA market at the specified settlement point. | Auction day-ahead energy model |

## Hourly Settlement Formulas

| ID | Payment/Charge | Unit | Formula |
| --- | --- | --- | --- |
| 1 | DA Energy Sold | $ | (-1)  DAMCPE _Settlement Point  DA Energy Sold _QSE _Settlement Point |
| 2 | DA Energy Purchased | $ | DAMCPE _Settlement Point  DA Energy Purchased _QSE _Settlement Point |

# Real-time Energy Settlements

In this section, settlements are presented for energy (1) carried forward from the ADAM, (2) scheduled through inter-QSE energy trades, and (3) produced / consumed in real-time.

## Inputs for Settlements

| ID | Input | Unit | Party | Settlement Point | Resource | Frequency | Description | Source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | LMP | $/MWh | -- | All settlement points (resource nodes, load zones, or hubs). | -- | Every 15 min | Real-time Locational Marginal Price. | Calculated from 5 minute data from SCED and State Estimator |
| 2 | Base Point | MW | QSE | Resource node where resource is physically located. | Name of the resource. | Every 5 min | Resource set point. | SCED |
| 3 | Metered Resource | MWh | QSE | Resource node where resource is physically located. | Name of the resource. | Every 15 min | QSE’s metered generation (or curtailed load) at the settlement point. | DataAgg |
| 4 | (Adjusted) Metered Load | MWh | QSE | Load zone where load is physically located. | -- | Every 15 min | QSE’s adjusted meter load at the settlement point. | DataAgg |
| 5 | DA Energy Resource | MW | QSE | Delivery location as specified in bids. | -- | Hourly | QSE’s energy purchased in DA market at the specified settlement point. | Day-ahead energy model |
| 6 | DA Energy Obligation | MW | QSE | Delivery location as specified in offers. | -- | Hourly | QSE’s energy sold in DA market at the specified settlement point. | Day-ahead energy model |
| 7 | Inter-QSE Energy Resource | MW | QSE | Delivery location as specified in energy schedules. | -- | Every 15 min | QSE’s energy purchased through inter-QSE trades at the specified settlement point. | Energy schedules (Inter-QSE trades) |
| 8 | Inter-QSE Energy Obligation | MW | QSE | Delivery location as specified in energy schedules. | -- | Every 15 min | QSE’s energy sold through inter-QSE trades at the specified settlement point. | Energy schedules (Inter-QSE trades) |
| 9 | RUS Deployment | MWh | QSE | -- | -- | Every 15 min | 15-min integration of regulation up deployment | LFC |

Section 2.1 Inputs for Settlements (Continued)

| ID | Input | Unit | Party | Settlement Point | Resource | Frequency | Description | Source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | RUS Participation Factor | -- | QSE | -- | Name of the resource. | Hourly | Each unit’s percentage of capacity available for regulation down service. | DA Ancillary Service Market |
| 11 | RDS Deployment | MWh | QSE | -- | -- | Every 15 min | 15-min integration of regulation up deployment | LFC |
| 12 | RDS Participation Factor | -- | QSE | -- | Name of the resource. | Hourly | Each unit’s percentage of capacity available for regulation down service. | DA Ancillary Service Market |
| 13 | RRS Deployment | MWh | QSE | -- | -- | Every 15 min | Responsive reserve deployment. | LFC |
| 14 | RRS Participation Factor | -- | QSE | -- | Name of the resource. | Hourly | Each unit’s percentage of capacity available for responsive reserve service. | DA Ancillary Service Market |
| 15 | NSRS Deployment | MWh | QSE | -- | Name of resource. | Every 15 min | Non-spin reserve deployment. | Manual instruction. |

## 15-min Settlement Formulas

| ID | Payment/Charge | Unit | Formula |
| --- | --- | --- | --- |
| 1 | Metered Resource | $ | (-1)  LMP _Settlement Point  Metered Resource _QSE _Settlement Point _Resource Name |
| 2 | Metered Load | $ | LMP _Settlement Point  Metered Load _QSE _Settlement Point |
| 3 | DA Energy Resource | $ | (-1)  LMP _Settlement Point  (DA Energy Resource _QSE _Settlement Point / 4) |
| 4 | DA Energy Obligation | $ | LMP _Settlement Point  (DA Energy Obligation _QSE _Settlement Point / 4) |
| 5 | Inter-QSE Energy Resource | $ | (-1)  LMP _Settlement Point  (Inter-QSE Energy Resource _QSE _Settlement Point / 4) |
| 6 | Inter-QSE Energy Obligation | $ | LMP _Settlement Point  (Inter-QSE Energy Obligation _QSE _Settlement Point / 4) |
| 7 | Penalty to Resource | $ | Define A =  Three 5-min intervals (Base Point _QSE _Settlement Point _Resource Name / 12) + Ancillary Service Deployment _QSE _Settlement Point _Resource Name B = ABS (Metered Resource _QSE _Settlement Point _Resource Name – A) Where Ancillary Service Deployment _QSE _Settlement Point _Resource Name = RUS Deployment _QSE  RUS Participation Factor _QSE _Resource Name + RDS Deployment _QSE  RDS Participation Factor _QSE _Resource Name + RRS Deployment _QSE  RRS Participation Factor _QSE _Resource Name + NSRS Deployment _QSE _Resource Name If Dynamic Scheduling Resources OR Qualifying Facilities not bidding OR Uncontrollable resources None Otherwise Max (0, LMP _Settlement Point)  Min (1, 2  Max (0, (B / A – Max (0.10, 1.25 / A))))  Metered Resource _QSE _Settlement Point _Resource Name |

# CRR Settlements

Under the ADAM, all types of CRRs are settled with real-time LMP and Shadow Prices.

## Inputs for Settlements

| ID | Input | Unit | Party | Element | Bus | Settlement Point | Point of Injection | Point of Withdrawal | Frequency | Description | Source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | LMP | $/MWh | -- | -- | -- | Resource node, load zone, or hub. | -- | -- | Every 5 minutes | RT Locational Marginal Price. | Calculated by using data from SCED and SE |
| 2 | SP | $/MWh | -- | Directional network element. | -- | -- | -- | -- | Every 5 minutes | Shadow Price. | SCED |
| 3 | PTP Obligation | MW | Holder | -- | -- | -- | Settlement point where power is injected. | Settlement point where power is withdrawn. | Hourly | Point-to-Point Obligation to be settled with RT LMP. | CRR auction |
| 4 | PTP Option | MW | Holder | -- | -- | -- | Settlement point where power is injected. | Settlement point where power is withdrawn. | Hourly | Point-to-Point Option to be settled with RT shadow prices. | CRR auction |
| 5 | Flowgate | MW | Holder | Directional network element that (partly) makes up a flowgate. | -- | -- | -- | -- | Hourly | Flowgate to be settled with RT shadow prices. | CRR auction |

Section 3.1 Inputs for Settlements (Continued)

| ID | Input | Unit | Party | Element | Bus | Settlement Point | Point of Injection | Point of Withdrawal | Frequency | Description | Source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 6 | Shift Factor | -- | -- | Settlement point where the directional element starts or ends. | Resource/Load bus where the directional physical network element starts or ends. | -- | -- | -- | Every 5 minutes | Shift factor represents the ratio of flow on the element to the injection at the bus under the associated topology. | Same as used in the SCED model. |
| 7 | Distribution Factor | -- | -- | -- | Resource/Load bus that (partly) makes up the settlement point. | Resource node, load zone, or hub. | -- | -- | Every 5 minutes | Distribution factor.  All buses Distribution Factor _Bus _Settlement Point = 1.0, for each settlement point. | For each bus within a load zone, calculated by using SE load; for each bus within a hub, calculated by using the number of buses within the hub. |

## Hourly Settlement Formulas

| ID | Payment/Charge | Unit | Formula |
| --- | --- | --- | --- |
| 1 | PTP Obligation | $ | (-1)  ( Twelve 5-min intervals (LMP _Point of Withdrawal - LMP _Point of Injection)) / 12  PTP Obligation _Point of Injection _Point of Withdrawal _Holder |
| 2 | PTP Option | $ | (-1)  PTP Option Price _Point of Injection _Point of Withdrawal  PTP Option _Point of Injection _Point of Withdrawal _Holder Where PTP Option Price _Point of Injection _Point of Withdrawal =  Twelve 5-min intervals ( All elements ( All topologies (SP _Element _Topology  Max (0, (Distribution Factor _Bus _Point of Injection _Topology  Shift Factor _Element _Bus _Topology - Distribution Factor _Bus _Point of Withdrawal _Topology  Shift Factor _Element _Bus _Topology))))) / 12 |
| 3 | Flowgate | $ | (-1)  ( Twelve 5-min intervals ( All topologies SP _Element _Topology)) / 12  Flowgate _Element _Holder |

# Day-ahead Reliability Unit Commitment Settlements

## Day-ahead Reliability Unit Commitment Payments

### Inputs for Settlements

| ID | Inputs | Variable | Unit of Measure | Data Source | Freq. | Description |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Deployment Hour Count |  | # | Settlements |  | # of continuous hours per deployment |
| 2 | Energy Bid curve |  | $/MWh | QSE |  |  |
| 3 | Energy Bid Curve Cap |  | $/MWh | Settlements |  |  |
| 4 | Fuel Index Price | LTOTERCOT | $/MMBtu | Gas Daily | Daily |  |
| 5 | Generation Output of Unit |  | MWh | Data Ag | 96 |  |
| 6 | LMP | SFRS | $/MWh | MOS | 96 |  |
| 7 | LSL |  | MW | QSE | 24 |  |
| 8 | Minimum Energy Cost Cap |  | $/MWh | Settlements |  |  |
| 9 | Minimum Energy Cost Bid |  | $/MWh | QSE |  |  |
| 10 | MMBtu per Startup |  | MMBtu/Start | QSE | Start |  |
| 11 | No Load (set to zero) |  | $/h | QSE |  |  |
| 12 | Startup Cost Cap |  | $/Start | Settlements | Start | Calculated Total dollars per startup |
| 13 | Startup Costs Bid |  | $/Start | QSE | Start | Total dollars per startup |
| 14 | Type of Unit | LTOTQSE |  | QSE |  | Resource Category |

### Settlement Formulas

#### Make-whole Payment Formulas

| Make Whole Payment = | -1*(Max (0,RUC Cost - Energy Credit in RUC hours - Energy Credit in non-RUC hours)) |
| --- | --- |
| Margin over Make Whole = | -1*(Min (0,RUC Cost - Energy Credit in RUC hours - Energy Credit in non-RUC hours)) |

Note: Energy Credit in non-RUC hours can only be used to bring Make Whole to zero, not create a margin.

Where:

| RUC Cost = | Min (Startup Cost Bid, Startup Cost Cap) + No Load Cost + Min (((Min Energy Cost Bid, Min Energy Cost Cap) * LSL) * # Committed Hours) |
| --- | --- |
| Energy Credit in RUC hours = | Sum (Energy Payment - Energy Cost RUC) |
| Energy Credit in Non-RUC hours = | Max (0, Sum (Energy Payment - Energy Cost Non-RUC)) |
| Energy Payment = | Actual Generation * LMP |
| Energy Cost (RUC Hour) = | Max (0, Actual Generation - LSL/4) * Min (Real Time BES Bid Curve, Real Time BES Bid Curve Cap) |
| Energy Cost (Non-RUC Hour) = | IF (Actual Generation > LSL, Max (0, Actual Generation - LSL/4) * Min (Real Time BES Bid Curve, Real Time BES Bid Curve Cap) + Min ((Min Energy Cost Cap, Min Energy Cost Bid) * LSL/4), Actual Generation * Min (Min Energy Cost Cap, Min Energy Cost Bid)) |
| Minimum Energy Cost Cap = | (MMBTU (at LSL) * FIP)/LSL (Capped by resource category per protocols) |
| Minimum Energy Cost Bid = | Submitted in $/MWh |
| Startup Costs Bid = | Submitted in $/Start |
| Startup Cost Cap = | O&M Expenses + (verifiable MMBTU * FIP) (Capped by resource category per protocols) |
| Real Time BES Bid Curve = | Submitted in $/MWh |
| Real Time BES Bid Curve Cap = | Resource Category Heat Rate * FIP |

#### Other Formulas

The following calculations are applied to ERCOT De-Commit, ERCOT Extending a committed unit, and a Forced Outage.

A. Under the Applications/Process Description section:

“If ERCOT de-commits a unit earlier than its scheduled shutdown within the DaRUC 24-hour period, then no compensation is due to the impacted QSE from ERCOT.  However, if ERCOT de-commits a unit that it not scheduled to shutdown within the DaRUC 24-hour period, then ERCOT shall pay the impacted QSE the following sum, if positive: (a) the unit’s Start-up Cost and (b) if negative the difference between the product of LSL times the LMP at the unit’s node minus the units Minimum-energy Cost for the hours starting with the de-commitment interval until the interval ERCOT determines that the unit may again be at LSL.”

If (ERCOTDECOMITFLAG =1) AND (De-Commit Startup = 1), THEN

RUCDECOMIT COST = Max (0, Min (Startup Cost Bid, Startup Cost Cap) + Min (0, (LSL/4*LMP)-Min ((Min Energy Cost Cap, Min Energy Cost Bid) * LSL/4)))

New data cut is needed to indicate the intervals in which ERCOT has de-committed a unit to compare to the Resource Plan view, at time of de-commitment (another new data cut), to determine if the unit planned to shut down within the DaRUC 24 hour period. The example demonstrates how the payment will be calculated per the above scenario. The actual data sets received to develop the settlement calculation will drive how the final calculation will look.

B. Under the Outputs section:

“If ERCOT commits a unit earlier than its self-scheduled commitment start hour or keeps a unit committed beyond its self-scheduled shutdown hour, then the impacted QSE has the option of claiming a make whole payment only for the extra hours committed by ERCOT.  The make whole payment calculation in such instances shall be calculated based on the difference between the unit’s Minimum-energy Cost minus the energy revenues calculated as below.”

Since above section states the” QSE has the option of claiming a make whole payment”, it is assumed to mean the QSE will file a dispute for payment.

C. Under the Detailed Description of DaRUC Process section:

“If a resource is self-committed to be on-line at the start of the DaRUC 24-hour period and scheduled to go off-line at some time during the 24-hour period, ERCOT may decide to keep the resource on-line and not incur the Start-up Cost but rather incur the Minimum-energy Cost from the hour that the resource was scheduled to go off-line till the hour the resource is no longer required by ERCOT to be on-line.”

If (ERCOTEXTRACOMITFLAG = 1) AND (INTERVAL 1 = Committed), THEN

RUCEXTRACOMIT COST = Min ((Min Energy Cost Cap, Min Energy Cost Bid) * LSL/4)

A data cut is needed to indicate committed hours, including the extended hours. The example demonstrates how the payment will be calculated per the above scenario. The actual data sets received to develop the settlement calculation will drive how the final calculation will look.

D. Under the QSE De-commitment of Resources after DaRUC section:

“Resources committed by ERCOT through the DaRUC process that experience a forced outage after unit synchronization, shall be eligible for pro-rated “make-whole” payments based on costs incurred prior to the forced outage.  Resource gets the hourly minimum energy rate for the hours run.”

If RUCFORCEDOUTAGEFLAG = 1, THEN

RUCFOUT COST = Min ((Min Energy Cost Cap, Min Energy Cost Bid) * LSL/4)+Min (Startup Cost Bid, Startup Cost Cap)

Note: This is a pro-rated Make whole payment.

A data cut is needed to indicate the period of a forced outage.  The example demonstrates how the payment will be calculated per the above scenario. The actual data sets received to develop the settlement calculation will drive how the final calculation will look.

“If the resource experiences start-up failure which creates a forced outage prior to breaker close it may be compensated for its verifiable actual costs limited to costs which qualify as normal startup, (includes maintenance, labor and fuel) up to the forced outage event.”

Since the above section states the QSE ”may be compensated for its verifiable costs”, it is assumed to mean the QSE will file a dispute, by submitting verifiable costs, for a make-whole payment equal to the above limitations.

#### Open Issues

The white paper did not address the question of how to allocate Non-RUC hours when there is DaRUC and HaRUC activity on the same unit, in the same operating day.

A proposal for the Make-whole payment calculations for a unit that is committed in both DaRUC and HaRUC processes is as follows:

- Identify the Non-RUC hours for the DaRUC payment as the hours in which the unit is not committed during either the DaRUC process or any HaRUC process.

- Calculate DaRUC Make-whole payment as described in Section 4.1.2.1.

- Calculate HaRUC Make-whole payment assuming there were no Non-RUC hours.

## Day-ahead Reliability Unit Commitment Charges

### Descriptions

The DaRUC charge is described in the DaRUC white paper starting with Settlement Rules Section b, which states “Calculate the DaRUC charge for each interval as follows”:

1.	Calculate the hourly payment for each Resource by dividing the total make-whole payment by the number of hours committed.

This is the result of Step a.4 “Calculating DaRUC Make Whole Payments”.  (A DaRUC Make Whole payment for each applicable QSE by hourly interval.

2.	Calculate the total hourly DaRUC payments by summing the payments for each Resource for the hour.

Sum all QSE DaRUC Make Whole payments by hourly interval to get an hourly ERCOTTOT for DaRUC Make Whole payments.

Interval 1	Interval 2	Interval 3

Resource QSE A:  	$3000.00	$3000.00	$3000.00

Resource QSE B:  	      $0.00	 $ 900.00          $ 900.00

Resource QSE C:  	$2500.00	$2500.00	$2500.00

ERCOTTOT $:	$5500.00	$6400.00	$6400.00

- Calculate the Hourly DaRUC Charge for an hour by dividing the total hourly

payment by the total MWs capacity committed for the hour.

Calculate the hourly dollar per MW charge that will be used to calculate the charge for a shortfall.

Interval 1	Interval 2	Interval 3

Resource QSE A:  	400 MW	400 MW	400 MW

Resource QSE B:  	    0 MW	  90 MW	  90 MW

Resource QSE C:  	360 MW	360 MW	360 MW

ERCOTTOT MW:	760 MW	850 MW	850 MW

ERCOTTOT $:        $5500.00         $6400.00        $6400.00

ERCOTTOT MW:    760 MW         850 MW        850 MW

Hourly DaRUC Charge: $7.24/MW    $7.53/MW    $7.53/MW

4.	For each QSE calculate any QSE capacity shortage equal to the difference between the adjusted load and the lesser of the amount of capacity in the Day-ahead RUC snapshot (adjusted by ERCOT de-commits) or the amount of available QSE capacity in the real time hour.

Calculate the capacity shortfall quantity (MW) for each QSE/hour.

SFQTY = [Load Obligation–MIN((DaRUC Snapshot-DeCommits),Real Time QSE Cap.)]

5.	Calculate the total shortfall for each hour for all QSEs.

Sum the QSE specific results from b.4 to find the ERCOTTOT Shortfall MW quantity by hour.

Interval 1	Interval 2	Interval 3

Shortfall QSE A: 	 	100 MW	100 MW	100 MW

Shortfall QSE B: 	 	  50 MW	  50 MW	  50 MW

Shortfall QSE C: 	 	    0 MW	    0 MW	  20 MW

SFERCOTTOT MWqty:	150 MW	150 MW	170 MW

- QSEs that are short will be charged the lesser of their “short ratio share” of the total DaRUC hourly payments or six (6) times the Hourly DaRUC Charge where  “short ratio share” is the amount a QSE is short in that hour divided by the total that all QSEs are short in that hour.

This has two steps:

First, calculate an hourly Shortfall Ratio Share (SFRS).

For each Shortfall QSE: 	Shortfall QSE MWqty/ERCOTTOT Shortfall MWqty

Interval 1	Interval 2	Interval 3

Shortfall QSE A: 	 	100 MW	100 MW	100 MW

SFERCOTTOT MW:		150 MW	150 MW	170 MW

SFRS QSE A:			.666667	.666667	.588236

Interval 1	Interval 2	Interval 3

Shortfall QSE B: 	 	  50 MW	  50 MW	  50 MW

SFERCOTTOT MW:		150 MW	150 MW	170 MW

SFRS QSE B:			.333334	.333334	.294118

Interval 1	Interval 2	Interval 3

Shortfall QSE C: 	 	    0 MW	    0 MW	  20 MW

SFERCOTTOT MW:		150 MW	150 MW	170 MW

SFRS QSE C:			.000000	.000000	.117647

Second, calculate the actual hourly charge for a Shortfall QSE.  The hourly DaRUC Charge that we developed in b.3 enters the picture again.

QSE Shortfall DaRUC Charge = MIN [(SFRS * ERCOTTOT DaRUC Make Whole Payments), (6 * Hourly DaRUC Charge*QSE Shortfall MW Quantity)]

- Any DaRUC Charges not paid for in 5 above will be charged to all QSEs in Load Ratio Share.

This has 4 steps,

Step 1,

SUM all QSE Shortfall DaRUC Charges by interval, from Step b.6, to create an ERCOTOT for Shortfall DaRUC Charges.

Step 2,

Calculate the amount, not paid by the Shortfall QSEs, to be uplifted to LOAD by hour.

Uplift to Load = ERCOTTOT DaRUC Make Whole Payments – ERCOTOT Shortfall DaRUC Charges

Step 3,

Calculate the hourly Load Ratio Share for each QSE:

SUM4 (LTOTQSE) / SUM4 (LTOTERCOT)

Step 4,

Calculate the Load Charge by hour:

Load Charge = LRS * Uplift to Load

Therefore, in the end:

ERCOTTOT DaRUC Make Whole Payments = ERCOTOT Shortfall DaRUC Charges

+ ERCOTTOT Load Uplift Charges

- ERCOT will calculate and monitor the cost of local congestion for unit commitment based on unconstrained vs. constrained unit commitment of the unit involved.

- An incentive will be designed into the mitigation for non-competitive units that will be at least 25% above the energy offer caps used to calculate the make whole payments to incentivize self-commitment.

### Inputs for Settlements

| ID | Inputs | Variable | Unit of Measure | Data Source | Freq. | Description |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | int. ERCOTTOT DaRUC Make Whole Payment | DARUCMWAMT_ERCOTTOT | $ | Settlements | 24 | Sum all QSE/Unit DaRUC Make Whole payments by hour. |
| 2 | int. ERCOTTOT DaRUC Make Whole MW Quantity | DARUCMWQTY_ERCOTTOT | MW | Settlements | 24 | Sum all QSE/Unit DaRUC Make Whole MW Quantities by hour. |
| 3 | int. Hourly DaRUC Charge | DARUCHRCHG | $/MW | Settlements | 24 | Hourly DaRUC $/MW Charge used in Shortfall DaRUC Charge. |
| 4 | QSE Load Obligation | LDOBL | MW | MOS | 24 | QSE after-the-fact adjusted metered load, committed capacity sale nominations and bilateral energy sale schedules. |
| 5 | QSE DaRUC Snapshot | DARUCSNAP | MW | MOS | 24 | Sum of QSE self-committed capacity, capacity de-committed by ERCOT, committed capacity purchase nominations and bilateral energy purchase schedules (all submitted prior to the DaRUC). |
| 6 | QSE De-Commits | DECOMMIT | MW | MOS | 24 | Capacity taken off-line by either QSE or due to forced outage. |
| 7 | QSE Real Time Capacity | RTCAP | MW | MOS | 24 | Actual capacity provided in real time. |
| 8 | int. QSE Shortfall MW Quantity | DARUCSFQTY | MW | Settlements | 24 | Load Obligation–MIN((DaRUC Snapshot-De-Commits),Real Time QSE Cap.) |
| 9 | int. ERCOTTOT Shortfall MW Quantity | DARUCSFQTY_ERCOTTOT | MW | Settlements | 24 | Sum all QSE Capacity Shortfall quantities in MW. |
| 10 | int. QSE Short Ratio Share | SFRS | % | Settlements | 24 | Each QSE’s percentage of total ERCOT DaRUC Shortfall. |
| 11 | int. QSE Shortfall DaRUC Charge | DARUCSFAMT | $ | Settlements | 24 | QSE charge for Capacity shortfall. Shortfall Charge = MIN [(SFRS * ERCOTTOT DaRUC Make Whole Payments), (6 * Hourly DaRUC Charge*QSE Shortfall MW Quantity)] |
| 12 | int. ERCOTTOT Shortfall DaRUC Charge | DARUCSFAMT_ERCOTTOT | $ | Settlements | 24 | Sum all QSE Shortfall DaRUC Charges by hour. |
| 13 | int. Uplift to Load | UPTL | $ | Settlements | 24 | Total dollar amount uplifted to Load. This is the total dollar amount above the ERCOTTOT Shortfall DaRUC Charge. |
| 14 | ERCOT Total Load Actual | LTOTERCOT | MWh | Data Ag | 96 | Load Total for ERCOT by interval. |
| 15 | QSE Load Actual | LTOTQSE | MWh | Data Ag | 96 | Load Total for each QSE by interval. |
| 16 | int. QSE Load Ratio Share | LRS | % | Settlements | 24 | QSE percentage of Total ERCOT Load |
| 17 | int. QSE Uplift Charge | DARUCLAAMT | $ | Settlements | 24 | QSE charge for DaRUC above Shortfall DaRUC Charge |

### Settlement Formulas

The DaRUC charge equations below are based on the 3-29-04 TNT approved DaRUC white paper.

DaRUC ShortFall Charge (DARUCSF)

| DARUCSFAMT = | MIN (SFRS * DARUCMWAMT_ERCOTTOT), (6 * DARUCHRCHG * DARUCSFQTY)) |
| --- | --- |

Where:

| SFRS = | DARUCSFQTY / DARUCSFQTY_ERCOTTOT |
| --- | --- |
| DARUCHRCHG = | DARUCMWAMT_ERCOTTOT / DARUCMWQTY_ERCOTTOT |
| DARUCSFQTY = | LDOBL – MIN ((DARUCSNAP – SUM (DECOMMIT)), RTCAP) |

DaRUC Load Allocation Charge (DARUCLA)

DARUCLAAMT = LRS * DARUCUPTL

Where:

| LRS = | SUM4(LTOTQSE) / SUM4(LTOTERCOT) |
| --- | --- |
| DARUCUPTL = | DARUCMWAMT_ERCOTTOT - DARUCSFAMT_ERCOTTOT |

DaRUC Charge (DARUCCHG)

DARUCCHGAMT = DARUCSFAMT + DARUCLAAMT

The above variables are defined as follows:

| DARUCCHGAMT = | Total DARUC Charge to QSE ($) by hour for that QSE |
| --- | --- |
| DARUCHRCHG = | Hourly DARUC Charge used to calculate DARUC Shortfall Charge to QSE |
| DARUCLAAMT = | Total DARUC Load Allocation Charge to QSE ($) by hour for that QSE |
| DARUCMWAMT_ERCOTTOT = | Sum of DARUC Make Whole Payments to QSE ($) by hour for all QSEs in the market |
| DARUCMWQTY_ERCOTTOT = | Sum of DARUC Make Whole Quantities (MW) by hour for all QSEs in the market |
| DARUCSFAMT = | Total DARUC ShortFall Charge to QSE ($) by hour for that QSE |
| DARUCSFAMT_ERCOTTOT = | Sum of DARUC ShortFall Payments to QSE ($) by hour for all QSEs in the market |
| DARUCSFQTY = | Total DARUC ShortFall Quantity (MW) by hour for that QSE |
| DARUCSFQTY_ERCOTTOT = | Sum of DARUC ShortFall Quantities (MW) by hour for all QSEs in the market |

| DARUCSNAP = | Sum of QSE self-committed capacity, capacity de-committed by ERCOT, committed capacity purchase nominations and bilateral energy purchase schedules (all submitted prior to the DaRUC) by hour for that QSE |
| --- | --- |
| DARUCUPTL = | Total amount ($) uplifted to Load. |
| DECOMMIT = | Capacity taken off-line by either QSE or due to forced outage by hour for that QSE |
| LDOBL = | QSE after-the-fact adjusted metered load, committed capacity sale nominiations and bilateral energy sale schedules by hour for that QSE |
| LRS = | QSE percentage % of Total ERCOT Load by hour for that QSE |
| LTOTERCOT = | Load Total for ERCOT by 15-minute interval |
| LTOTQSE = | Load Total for each QSE by 15-minute interval |
| RTCAP = | QSE actual capacity provided in real time by hour for that QSE |
| SFRS = | QSE percentage % of Total ERCOT ShortFall by hour for that QSE |

# Hour-ahead Reliability Unit Commitment Settlements

## Hour-ahead Reliability Unit Commitment Payments

See Section 4.1.

## Hour-ahead Reliability Unit Commitment Charges

See Section 4.2.

# Ancillary Service Settlements

Day-Ahead Ancillary Services (A/S) Market (using Responsive Reserve as an example).  Could have used Regulation Up – RU, Regulation Down – RD or Non-Spin – NS, as well).

| ID | Inputs | Description | Unit of Measure | Frequency |
| --- | --- | --- | --- | --- |
| 1 | MCPCRS | Market Clearing Price for Responsive Reserve | $/MW | 24 |
| 2 | MRSQ | Market Responsive Reserve Quantity | MW | 24 |
| 3 | RSQ | Responsive Reserve Quantity | MW | 24 |
| 4 | MRRO | Market Obligated Responsive Reserve Quantity | MW | 24 |
| 5 | RRO | Obligated Responsive Reserve Quantity | MW | 24 |
| 6 | MRRSAQ | Market Responsive Reserve Self Arranged Quantity | MW | 24 |
| 7 | RRSAQ | Responsive Reserve Self Arranged Quantity | MW | 24 |
| 8 | RRTOT | Total Obligated Responsive Reserve Quantity | MW | 24 |
| 9 | RRSAQTOT | Total Responsive Reserve Self Arranged Quantity | MW | 24 |
| 10 | PCRRBILLAMT | Responsive Reserve Service Payment Billable Amount | $ | 24 |
| 11 | PCOOMRRBILLAMT | OOM Responsive Capacity Billable Amount | $ | 24 |
| 12 | MOOMRSQ | Market OOM Responsive Reserve Quantity | MW | 24 |
| 13 | OOMRSQ | OOM Responsive Reserve Quantity | MW | 24 |
| 14 | MCPOOMRS | Market Clearing Price for OOM Responsive Reserve | $/MW | 24 |

Payment to the QSEs providing Responsive Reserve = -1 * RSQ_QSE No. * MCPCRS

Note that the formula in white paper does not show “-1*”.

Collection from the QSEs who did not fully self arrange their Responsive Reserve Obligations = -1 * ((PCRRBILLAMT + PCOOMRRBILLAMT) / (RRTOT – RRSAQTOT)) * (RSO_QSE No.  – RRSAQ _ QSE  No.)

Collection from QSEs who did not fully self arrange their Responsive Reserve Obligations = (RRO_QSE No. –  RRSAQ _QSE No.) * MCPCRS [This formula works if ERCOT did not purchase any Responsive Reserves in excess of the total Responsive Reserve Obligations or under purchased Responsive Reserves, as compared to the total Responsive Reserve Obligations.

Scenario Two – Purchased Responsive Reserve for a Second Market (QSE defaulted on its quantity awarded).

Scenario Three – Purchased Responsive Reserve for a Second Market (due to change in forecasted weather conditions)

Scenario Four – ERCOT receives insufficient Responsive Reserve bids to procure required Responsive Reserve.

Scenario Five –  1) Unit offering/providing Responsive Reserve does not work due to congestion

2) ERCOT tells provider it cannot work

3) Provider has option to provide elsewhere or buy Responsive Reserve from ERCOT

a) If original Responsive Reserve sold to ERCOT – NO PAYMENT

b) If self provided – NO CREDIT

4) New Auction – New Clearing price

5) All who buy from ERCOT this Responsive Reserve pay the weighted average of the two auctions

using the Protocols and then using the proposed Texas Nodal Market:

The methodology to pay QSEs providing Responsive Reserve under this Scenario does not exist under the current Protocols.

Payment to the QSEs providing Responsive Reserve under this Scenario = RSQ_QSE No. * MCPCRS (weighted average of the two auctions)

Collection from QSEs who did not fully self arrange their Responsive Reserve Obligations under this Scenario  = (RRO_QSE No. – RRSAQ _QSE No.) * MCPCRS (weighted average of the two auctions)

# Appendices

- Energy Settlement Examples with ADAM

- DaRUC (HaRUC) Make-Whole Payment Examples

- DaRUC (HaRUC) Charge Example