| PRR Number | 513PRR | PRR Title | Verification of Operational Model after Recurring Local Congestion Event |
| --- | --- | --- | --- |
| Recommended Action |  | Approve |  |
| Protocol Section(s) Requiring Revision (include Section No. and Title) |  | 7.4.1, Determination of Local Congestion |  |
| Proposed Effective Date |  | Upon filling ERCOT staffing requirements |  |
| Priority & Rank Assigned |  | N/A |  |
| Revision Description |  | After an event has caused Local Congestion for two times within a three-day period or three times within a five-day period, this PRR would make it mandatory for ERCOT to contact the appropriate TO to (1) verify transmission line ratings, (2) verify modeling of the transmission system, and (3) determine with the TO if any transmission operational intervention could relieve congestion. |  |
| Benefit |  | Reduce risks of Local Congestion costs to the market due to recurring events |  |
| PRS Recommendation |  | PRS recommends approval of PRR513 as revised by ERCOT Comments and PRS. |  |
| TAC Recommendation |  | After voting to declare PRR 513 urgent, TAC recommends approval as recommended by PRS. |  |

| Sponsor |  |
| --- | --- |
| Name | Robert Kelly |
| E-mail Address | rkelly@brazoselectric.com |
| Company | Brazos Electric |
| Company Address | Box 2585, Waco, TX 76702-2585 |
| Phone Number | 254-750-6353 |
| Fax Number | 254-750-6340 |

ERCOT/Market Segment Impacts and Benefits

Instructions:  To allow for comprehensive PRR consideration, please fill out each block below completely, even if your response is “none,” “not known,” or “not applicable.”  Wherever possible, please include reasons, explanations, and cost/benefit analyses pertaining to the PRR.

|  | Impact |  | Benefit |
| --- | --- | --- | --- |
|  | Business | Computer Systems |  |
| ERCOT | Additional operator and engineering time | Potential data and configuration updates to Operational Model | Comprehensive Operational Model |
| MARKET SEGMENT | None | None | Reduce risks of local congestion costs |
| Consumer | None | None | Reduce risks of local congestion costs |
| LSE: General, Including NOIE | None | None | Reduce risks of local congestion costs |
| LSE: CR & REP | None | None | Reduce risks of local congestion costs |
| QSE | None | None | Reduce risks of local congestion costs |
| Resource | None | None | Potential of less OOM |
| TDSP | Additional operator and engineering time | None | Confirmation that ERCOT Operational Model coincides with physical transmission conditions. |

Proposed Protocol Language Revision

### 7.4.1	Determination of Local Congestion

ERCOT will use the Operational Model of the ERCOT System in combination with QSE energy schedules and ERCOT forecasted Load to determine the extent of Local Congestion.

When ERCOT identifies Local Congestion on a repeated basis ERCOT shall have procedures established to contact the appropriate TDSP to:

(1)	Verify that ratings of Transmission Facilities in the ERCOT Operational Model causing the event are current and correctly represented;

(2)	Verify, when the TDSP’s analysis results differ from those of ERCOT, that the configuration of the transmission system in the ERCOT Operational Model matches that in use by the TDSP.  To recognize operational time constraints, such verification will focus on transmission elements believed to have impacted the event; and

(3)	Mutually identify with the TDSP any additional operational intervention or system monitoring that could be implemented to manage recurring Congestion due to a recurring cause.

If it is determined that rating(s) in the ERCOT Operational Model or configuration of the Transmission Facilities are not correct, then the TDSP will provide, via submittal of a Service Request on the ERCOT MIS, written confirmation of the correction.  Once written confirmation is completed, ratings in the ERCOT Operational Model will be updated in the Real Time systems by the close of the next Business Day, and such rating changes and any needed configuration changes will be  included in the next scheduled Operational Model database update.   Once additional operational intervention to the transmission system has been identified (if any) to relieve Congestion and mutually accepted by both ERCOT and TDSP operators, the necessary operational instructions shall be prepared and implemented as soon as practicable.

ERCOT will use an interim practice between the identification of a problem and the final authorized submittal of a Service Request in order to implement corrections as soon as practicable.  The interim practice will allow an authorized or alternate contact to provide corrected ratings to ERCOT by email.  ERCOT will enter that rating into the Real Time systems to be used in current operating activities on a temporary basis.  After the Service Request documentation is received by ERCOT, permanent changes will be made in the Operational Model and loaded into the operating systems.

| PRR Evaluation |  |
| --- | --- |
| Non-ERCOT Market Comparison | No Comparison conducted. |
| Comments Author | Comments Summary |
| TXU | Revised Language. |
| ERCOT | Revised Language. |

| Sponsor's Revision Description | After an event has caused Local Congestion for two times within a three-day period or three times within a five-day period, this PRR would make it mandatory for ERCOT to contact the appropriate TO to (1) verify transmission line ratings, (2) verify modeling of the transmission system, and (3) determine with the TO if any transmission operational intervention could relieve congestion. |
| --- | --- |
| Sponsor's Reason for Revision | Reduce risks of Local Congestion costs to the market due to recurring events |