| PRR Number | 479PRR | PRR Title | IDR Optional Removal Threshold |
| --- | --- | --- | --- |
| Date |  | June 22, 2004 |  |
| Submitter’s Information |  |  |  |
| Name |  | Carl Raish |  |
| E-mail Address |  | craish@ercot.com |  |
| Company |  | ERCOT |  |
| Company Address |  | 2705 West Lake Drive, Taylor, Texas |  |
| Phone Number |  | 512-248-3876 |  |
| Fax Number |  |  |  |

ERCOT/Market Segment Impacts and Benefits

Instructions:  To allow for comprehensive PRR consideration, please fill out each block below completely, even if your response is “none,” “not known,” or “not applicable.”  Wherever possible, please include reasons, explanations, and cost/benefit analyses pertaining to the PRR.

|  | Impact |  | Benefit |
| --- | --- | --- | --- |
|  | Business | Computer Systems |  |
| ERCOT | Additional reporting and verification resources, which shall increase the administration cost | TBD system changes. Cost unknown. | None. |
| MARKET SEGMENT |  |  |  |
| Consumer | Possible reduced cost. | N/A | Possible reduction in TDSP wires charges. Customer Choice of a meter type. |
| LSE: General, Including NOIE | Loss of settlement accuracy. Changes in settlement charges | N/A | None. |
| LSE: CR & REP | Loss of settlement accuracy. Changes the relationship with the Customer. Changes in settlement charges. Possible changes in revenue. | Added software to manage. | None. |
| QSE | Changes in settlement charges. | N/A | None. |
| Resource | N/A | N/A | N/A |
| TDSP | Increased number of meter changes. | Software changes possible such as validation of meter type. | Less processing of IDR data. |

Comments

ERCOT revises language in the new Section 18.6.7 item (a), to clarify that a new Customer may request removal of an IDR meter only if the following criteria are met:

  - Their request is submitted within 120 days of their Move-In; and

  - They have at least 25 consecutive days worth of usage loaded in ERCOT’s system; and

  - Their demand has not exceeded the IDR Optional Removal Threshold at anytime since their Move-In.

ERCOT is also revising all of Section 18 and Section 10.9 to reflect the change from IDR Requirement to IDR Mandatory Installation Threshold.

Revised Proposed Protocol Language

Add IDR Optional Removal Threshold as a defined term and change IDR Requirement to IDR Mandatory Installation Threshold in Section 2.

IDR Optional Removal Threshold

The kW (or kVA) level at which an Interval Data Recorder may be removed as set forth in Section 18.6.7, IDR Optional Removal Threshold.

IDRMandatory Installation Threshold

The kW (kVA) level at which the installation of Interval Data Recorders are required for settlement purposes as set forth in Protocols Section 18.6.1, Interval Data Recorder Installation and Use in Settlement.

## 10.9	Standards for Metering Facilities

For settlement purposes, IDRs are required on any of the following locations/sites:

- Generation Resources (with the exception of those excluded in Section 10.2.2(2))

- Resources bidding into the Ancillary Services Market

- Non-opt-in-Entity metering points used to determine NOIE total Load.

- Service Delivery Points connected to the transmission system (>60KV)

- Locations meeting IDR requirements defined in Section 18, Load Profiling of these Protocols.

18.6	Installation and Use of Interval Data Recorders

### 18.6.1	Interval Data Recorder (IDR) Installation and Use in Settlement

- IDR Mandatory Installation Threshold:  IDRs shall be installed and utilized for settlement of Premises having either:

  - A peak demand greater than 1000 kW (or 1000 kVa), or

  - Service provided at transmission voltage (above 60 kV).

- A Competitive Retailer may have an IDR installed and used for settlement purposes at any associated Premise outside the IDR Mandatory Installation Threshold.  Except as stated in item (4) of this Section, IDRs in place or installed after September 1, 1999 shall be used for settlement.  Once an IDR is installed on a Premise and used for settlement purposes, the given Premise shall continue to be settled with its interval data, except as stated in Sections 18.6.7, IDR Optional Removal Threshold.  If a Customer or CR requests installation of an IDR meter, the same Customer or CR may not request removal of the IDR meter for a period of twelve (12) months following such installation.

- All Non-Metered Loads such as street lighting, regardless of the aggregation level, shall not be required to install IDRs under the IDR Mandatory Installation Threshold.  These Loads shall be settled using Load Profiles.

- For premises not subject to the IDR Mandatory Installation Threshold in item (1) of this Section:

  - IDRs installed at the request of ERCOT, a TDSP, a municipal, or a cooperative for Load research, rate/tariff design calculation, coincident demand calculation, or Load Profiling purposes shall be exempt from the requirement to use an IDR for settlement purposes; or

  - IDRs previously used specifically for separating NOIE load from competitive load shall be exempt from the requirement to use an IDR for settlement purposes, provided that the IDR meter has been removed within one hundred and twenty (120) days after the NOIE has fully implemented Customer Choice.

- For IDR installation procedures reference Section 10.2.2, TDSP Metered Entities.

- TDSPs responsible for any Load transfer schemes between ERCOT and Non-ERCOT Regions shall install IDR metering capable of measuring the Load served during the period the Load transfer is implemented.

### IDR Administration Issues

ERCOT shall produce a report informing the appropriate Market Participants of profiled Premises that have become subject to the IDR Mandatory Installation Threshold.  ERCOT shall put in place a system to track Market Participants’ timely adherence to this requirement.  This report shall be posted to MIS.

### 18.6.3	Adherence to Interval Data Recorder Requirements

Municipal Entities and Electric Cooperative Entities that opt-in to Customer Choice must install IDR meters at all Premises subject to the IDR Mandatory Installation Threshold for metering prior to the effective date of their participation in the testing and integration requirements of ERCOT systems for Customer Choice.

### 18.6.4	Technical Requirements

Regardless of data retrieval method, interval data shall be provided on a schedule that supports the requirements of final settlement (typical monthly billing cycle).

Interval data that is provided for settlement shall be consistent with the ERCOT defined Settlement Interval.

IDRs used for settlement shall meet technical metering requirements defined in the Load Profiling Guides.

### 18.6.5	Future Requirements for IDRs

ERCOT and the appropriate ERCOT TAC subcommittee shall evaluate the impact of the IDR Mandatory Installation Threshold as defined in this Section for possible revision prior to the introduction of competitive metering services to the market on January 1, 2004.

### 18.6.6	Peak Demand Determination for Non-IDR Premises

For the purpose of determining the peak demand level for the IDR Mandatory Installation Threshold in Section 18.6.1, Interval Data Recorder (IDR) Installation and Use in Settlement, the demand will be determined in accordance with PUCT rulemaking or through a consensus process with ERCOT and Market Participants.  In the absence of a clear definition of peak demand in the “price to beat” rulemaking, the following application shall be used in determining the peak demand level for IDR  Mandatory Installation Threshold in Section 18.6.1, Interval Data Recorder (IDR) Installation and Use in Settlement:

A Premise (ESI ID) has a peak demand greater than 1000 kW when a demand greater than 1000 kW has been measured in any two (2) billing months of the most recent twelve (12) month period.  Competitive Retailers may dispute an IDR assignment through the ERCOT settlement dispute process, described in Section 9.5 Settlement and Billing Dispute Process.

ERCOT shall be responsible for receiving and storing demand information necessary for determining mandatory IDR installations.

### 18.6.7 IDR Optional Removal Threshold

The IDR Optional Removal Threshold is established at one hundred and fifty (150) kW (or kVA).  A Customer, or the CR upon a Customer’s request, may request, in accordance with PUCT rules and regulations, removal of an IDR:

a) for a new Customer move-in, where the request is submitted within a one hundred and twenty (120) days of the Move-In provided the new Customer’s demand has remained below the Optional Removal Threshold and that ERCOT has at least twenty five (25) consecutive days of usage in its system for the new Customer; or

b) if an existing Customer’s load has never exceeded the IDR Optional Removal Threshold during the most recent twelve (12) consecutive months.

If a Customer or CR requests removal of an IDR meter, the same Customer or CR may not request reinstallation of an IDR meter for a period of twelve (12) months following such removal. Removal or re-installation of IDR meters may be subject to applicable tariff charges.