| PRR Number | 502PRR | PRR Title | Aggregation of Combined Cycle Units Providing RRS |
| --- | --- | --- | --- |
| Protocol Section(s) Requiring Revision (include Section No. and Title) |  | 6.10.5.4, Responsive Reserve Services Performance Monitoring Criteria |  |
| Effective Date |  | Upon System Implementation |  |
| Priority & Rank Assigned |  | 1.1 |  |
| Revision Description |  | Allows RRS obligations awarded to combined cycle plants to be aggregated for all units in a block for consideration of compliance. |  |
| Benefit |  | Combined cycle plants operate in an interdependent manner. RRS awarded to a single unit in a block will be met by the capacity of all units in the block in reality. The current A/S auction process does not accommodate awards to the units to follow actual operations. Submission is result of CEDI Task Force meeting on RRS compliance measurement. |  |
| PRS Recommendation |  | PRS recommended approval of PRR502 as revised by ERCOT Comments & PRS |  |
| TAC Recommendation |  | After voting to declare PRR502 urgent, TAC recommended approval as recommended by PRS. |  |
| Board Action |  | Board approved PRR502 as recommended by TAC. |  |

| Sponsor |  |
| --- | --- |
| Name | Clayton Greer |
| E-mail Address | Clayton.greer@constellation.com |
| Company | Constellation |
| Company Address | 8768 Polo Drive |
| Phone Number | (817) 938-3692 |
| Fax Number |  |

ERCOT/Market Segment Impacts and Benefits

Instructions:  To allow for comprehensive PRR consideration, please fill out each block below completely, even if your response is “none,” “not known,” or “not applicable.”  Wherever possible, please include reasons, explanations, and cost/benefit analyses pertaining to the PRR.

|  | Impact |  | Benefit |
| --- | --- | --- | --- |
|  | Business | Computer Systems |  |
| ERCOT |  |  |  |
| MARKET SEGMENT |  |  |  |
| Consumer |  |  |  |
| LSE: General, Including NOIE |  |  |  |
| LSE: CR & REP |  |  |  |
| QSE |  |  |  |
| Resource |  |  |  |
| TDSP |  |  |  |

Proposed Protocol Language Revision

#### 6.10.5.4	Responsive Reserve Services Performance Monitoring Criteria

QSEs providing Responsive Reserve Services must so indicate in the Scheduling Process.  QSEs shall have Resources available to meet the schedule while adhering to the Responsive Reserve Service requirements detailed in the Operating Guides.  On deployment of any Responsive Reserve Service, the QSE shall control its Resources to operate to the final Resource bilateral schedules as converted to a base power function plus the instructed Responsive Reserve Service power requirement.  ERCOT’s calculation of SCE ten (10) minutes after the deployment instruction will determine the level of Responsive Reserve Service provided.  Satisfactory control performance of the QSE providing only Responsive Reserve Services shall be deemed satisfactory when:

- Not less than ninety-five percent (95%) nor more than one hundred fifty percent (150%) of the RRS requested, subject to the declared capabilities of the QSE, is provided within ten (10) minutes of ERCOT’s deployment Dispatch Instruction and maintained until recalled or the QSEs service Obligation expires;

- Generation Resources providing the RRS requested shall return to within ninety percent (90%) to one hundred ten percent (110%) of their pre-deployment scheduled output, subject to the declared capabilities of the QSE, within ten (10) minutes following a recall instruction from ERCOT; and

[PRR436:  Add the following to Section 6.10.5.4, item (2) upon fulfilling of staffing requirements:]

If LaaRs are utilized to satisfy the Responsive Reserve Service, the RRS requested shall return as soon as practical considering process constraints, and within three (3) hours.

(3) 	RRS bids from QSEs may include contributions from combined cycle Resources in Aggregated Units meeting the criteria in Section 6.8.2.4, Aggregating Units.    Thus, to determine if a combined cycle Aggregated Unit is capable of performing its RRS oObligation, all Resources On-line in the Aggregated Unit will be measured as on an aggregate capacity basis and will be calculated from the lower of the High Sustainable Limits specified in the Resource Plan or through telemetry, or the seasonal tested Net Dependable Capability.

For all frequency deviations exceeding 0.175 Hz, ERCOT shall measure and record each two (2) second scan rate values of real power output for each QSE Resource providing Responsive Reserve Service.  ERCOT shall measure and record the MW data beginning one (1) minute prior to the start of the frequency excursion event or Manual / Dispatch Instruction until ten (10) minutes after the start of the frequency excursion event or Manual / Dispatch Instruction.  Satisfactory performance is measured by comparing the actual response to the frequency response capability required in the Operating Guides.

Where multiple observations of operating response are available, the QSE must deliver the required frequency response capability seventy-five percent (75%) of the time during any single calendar quarter.

| PRR Evaluation |  |
| --- | --- |
| Non-ERCOT Market Comparison | No Comparison conducted. |
| Comments Author | Comments Summary |
| Constellation | Suggests PRR be declared urgent. |
| ERCOT | Suggests that this PRR be remanded to ROS. |

| Sponsor's Revision Description | Allows RRS obligations awarded to combined cycle plants to be aggregated for all units in a block for consideration of compliance. |
| --- | --- |
| Sponsor's Reason for Revision | Combined cycle plants operate in an interdependent manner. RRS awarded to a single unit in a block will be met by the capacity of all units in the block in reality. The current A/S auction process does not accommodate awards to the units to follow actual operations. Submission is result of CEDI Task Force meeting on RRS compliance measurement. |