| PRR Number | 479PRR | PRR Title | IDR Optional Removal Threshold |
| --- | --- | --- | --- |
| Protocol Section(s) Requiring Revision |  | 2, Definitions and Acronyms 18.6.1, Interval Data Recorder (IDR) Installation and Use in Settlement 18.6.2, IDR Administration Issues 18.6.3, Adherence to Interval Data Recorder Requirements 18.6.5, Future Requirements for IDRs 18.6.6, Peak Demand Determination for Non-IDR Premises 18.6.7, IDR Optional Removal Threshold [new] |  |
| Effective Date |  | Changes to Sections 18.6.2 and new 18.6.7 can be implemented upon system implementation. All other revisions will be effective August 1, 2004. |  |
| Priority & Rank Assigned |  | N/A |  |
| Revision Description |  | Allow the removal of an IDR meter for settlement under certain conditions when load drops below a minimum threshold. Allow IDRs installed by NOIEs for load aggregation purposes only to be exempt from being used for retail Customer settlement purposes and allow the NOIE to remove IDRs used for NOIE load aggregation within one hundred and twenty (120) days after the NOIE fully implements Customer Choice. |  |
| Benefit |  | Allowing the removal of an IDR meter for settlement when a new Customer moves in to a premise and that Customer’s Load is much lower that the prior Customer who may have warranted the use of an IDR meter. The new Customer should not have to pay for an IDR meter needed by the prior Customer. In addition, some Customer’s load may have dropped for some time, which means the need for a mandatory IDR meter has also diminished. Exempt NOIEs from the cost of monitoring, compiling, and transmitting IDR data for small accounts that do not meet the IDR Mandatory Installation Threshold. |  |
| PRS Recommendation |  | PRS recommended approval of PRR479 as recommended by PWG and amended by Nueces Electric Cooperative comments and PRS. |  |
| TAC Recommendation |  | TAC approved urgent status for PRR 479 and recommended approval as modified by TAC. |  |
| Board Action |  | Board approved PRR479 as recommended by TAC. |  |
| Sponsor |  |  |  |
| Name |  | Ernie Podraza, PWG Chair |  |
| E-mail Address |  | epodraza@reliant.com |  |
| Company |  | Reliant Energy |  |
| Company Address |  | 1000 Main, Houston, Texas 77002 |  |
| Phone Number |  | 713-497-5295 |  |
| Fax Number |  |  |  |

Proposed Protocol Language Revision

Add IDR Optional Removal Threshold as a defined term and change IDR Requirement to IDR Mandatory Installation Threshold in Section 2.

IDR Optional Removal Threshold

The kW (kVA) level at which an interval data recorder may be removed as set forth in Section 18.6.7, IDR Optional Removal Threshold.

IDRMandatory Installation Threshold

The kW (kVA) level at which the installation of interval data recorders are required for settlement purposes as set forth in Section 18.6.1, Interval Data Recorder Installation and Use in Settlement

18.6	Installation and Use of Interval Data Recorders

### Interval Data Recorder (IDR) Installation and Use in Settlement

- IDR Mandatory Installation Threshold:  IDRs shall be installed and utilized for settlement of Premises having either:

  - A peak demand greater than 1000 kW (or 1000 kVA), or

  - Service provided at transmission voltage (above 60 kV).

- A Competitive Retailer may have an IDR installed and used for settlement purposes at any associated Premise outside the IDR Mandatory Installation Threshold.  Except as stated in item (4) of this Section, IDRs in place or installed after September 1, 1999 shall be used for settlement.  Once an IDR is installed on a Premise and used for settlement purposes, the given Premise shall continue to be settled with its interval data, except as stated in Sections 18.6.7, IDR Optional Removal Threshold.  If a Customer or CR requests installation of an IDR meter, the same Customer may not request removal of the IDR meter for a period of twelve (12) months following such installation.

- All Non-Metered Loads such as street lighting, regardless of the aggregation level, shall not be required to install IDRs under the IDR Mandatory Installation Threshold.  These Loads shall be settled using Load Profiles.

- For Premises not subject to the IDR Mandatory Installation Threshold in item (1) of this Section:

  - IDRs installed at the request of ERCOT, a TDSP, a municipal, or a cooperative for Load research, rate/tariff design calculation, coincident Demand calculation, or Load Profiling purposes shall be exempt from the requirement to use an IDR for settlement purposes; or

  - IDRs previously used specifically for separating NOIE Load from competitive Load shall be exempt from the requirement to use an IDR for retail Customer settlement purposes, provided that the IDR meter has been removed within one hundred and twenty (120) days after the NOIE has fully implemented Customer Choice.   IDR meters used for NOIE separation that do not meet the IDR Mandatory Installation Threshold shall not be used for retail Customer settlement purposes.

- For IDR installation procedures reference Section 10.2.2, TDSP Metered Entities.

- TDSPs responsible for any Load transfer schemes between ERCOT and Non-ERCOT Regions shall install IDR metering capable of measuring the Load served during the period the Load transfer is implemented.

### 18.6.2	IDR Administration Issues

ERCOT shall produce a report informing the appropriate Market Participants of profiled Premises that have become subject to the provisions of Section 18.6, Installation and Use of Interval Data Recorders.  ERCOT shall put in place a system to track Market Participants’ timely adherence to this requirement.  This report shall be posted to MIS.

### 18.6.3	Adherence to Interval Data Recorder Requirements

Municipal Entities and Electric Cooperative Entities that opt-in to Customer Choice must install IDR meters at all Premises subject to the IDR Mandatory Installation Threshold for metering prior to the effective date of their participation in the testing and integration requirements of ERCOT systems for Customer Choice.

### 18.6.4	Technical Requirements

Regardless of data retrieval method, interval data shall be provided on a schedule that supports the requirements of final settlement (typical monthly billing cycle).

Interval data that is provided for settlement shall be consistent with the ERCOT defined Settlement Interval.

IDRs used for settlement shall meet technical metering requirements defined in the Load Profiling Guides.

### 18.6.5	Future Requirements for IDRs

ERCOT and the appropriate ERCOT TAC subcommittee shall evaluate the impact of the IDR Mandatory Installation Threshold as defined in this Section for possible revision prior to the introduction of competitive metering services to the market on January 1, 2004.

### 18.6.6	Peak Demand Determination for Non-IDR Premises

For the purpose of determining the peak demand level for the IDR Mandatory Installation Threshold in Section 18.6.1, Interval Data Recorder (IDR) Installation and Use in Settlement, the demand will be determined in accordance with PUCT rulemaking or through a consensus process with ERCOT and Market Participants.  In the absence of a clear definition of peak demand in the “price to beat” rulemaking, the following application shall be used in determining the peak demand level for IDR  Mandatory Installation Threshold in Section 18.6.1, Interval Data Recorder (IDR) Installation and Use in Settlement:

A Premise (ESI ID) has a peak demand greater than 1000 kW (or kVA) when a demand greater than 1000 kW (kVA) has been measured in any two (2) billing months of the most recent twelve (12) month period.  Competitive Retailers may dispute an IDR assignment through the ERCOT settlement dispute process, described in Section 9.5 Settlement and Billing Dispute Process.

ERCOT shall be responsible for receiving and storing demand information necessary for determining mandatory IDR installations.

- 18.6.7 IDR Optional Removal Threshold

A Customer, or the CR upon a Customer’s request, may request, in accordance with PUCT rules and regulations, removal of an IDR at a Premise. The IDR Optional Removal Threshold is established as follows:

a)  for an existing Customer, where the Load at the Premise has never exceeded  the IDR Optional Removal Threshold of one hundred and fifty (150) kW(kVA) during the most recent twelve (12) consecutive months; or

b) for a new Customer Move-In, where the request is submitted within one hundred and twenty (120) days of the Move-In provided the new Customer’s Demand at the Premise has remained below the IDR Mandatory Installation Threshold between the Move-In date and the request date and that meter readings covering at least forty five (45) consecutive days of usage at the Premise have been registered for the new Customer.

If a Customer or CR requests removal of an IDR meter at a Premise, the same Customer or CR may not request reinstallation of an IDR meter at that Premise for a period of twelve (12) months following such removal. Removal or re-installation of IDR meters may be subject to applicable tariff charges.

| PRR Evaluation |  |
| --- | --- |
| Non-ERCOT Market Comparison | No Comparison conducted. |
| Comments Author | Comments Summary |
| ERCOT (06/22/04) | Revised language |
| NEC (06/22/04) | Revised language |
| OPC (06/23/04) | Revised language |
| NEC (06/30/04) | Asked for TAC approval |
| ERCOT (07/01/04) | Revised language |

| Sponsor's Revision Description | Allow the removal of an IDR meter for settlement under certain conditions when load drops below a minimum threshold. And to allow a NOIE to remove IDRs that were used for NOIE load aggregation prior to implementing Customer Choice. |
| --- | --- |
| Sponsor's Reason for Revision | Allowing the removal of an IDR meter for settlement when a new Customer move in to a premise and that Customer’s load is much lower that the prior Customer who may have warranted the use of an IDR meter. The new Customer should not have to pay for an IDR meter needed by the prior Customer. In addition, some Customer’s load may have dropped for some time, which means the need for a mandatory IDR meter has also diminished. |